3GPP TSG-SA3 Meeting #122	S3-251943

# Fukuoka, Japan,  19 - 23 May 2025

Source:	ZTE

Title:	pCR against draft CR on Authentication and Authorization for CAPIF RNAA interconnection

Document for:	Approval

Agenda item:	4.22

Spec:	TS 33.122

Version:	19.0.0

Work Item:	Security aspects of CAPIF Phase3

Comments

This pCR proposes to update clause 6.X.3.4 on authentication and authorization for CAPIF RNAA interconnection.

Proposed Changes

* * * First Change * * * *

6.X.3.4		Authentication and Authorization for CAPIF RNAA interconnection

In CAPIF RNAA interconnection,  the API invoker is onboarded to CCF-B of the CAPIF provider B and the target AEF is registered to CCF-A of CAPIF provider A. The ROF connects with CCF-A of CAPIF provider A.

For authorization and authorization in CAPIF RNAA interconnection, clause 6.5.3 shall be followed with the following enhancement:

-	For authorization, the CCF-A has the role of the OAuth 2.0 authorization server, i.e., providing the access token used for RNAA. The CCF-A shall indicate the selected flows to the API invoker.	The CCF-A shall give service authorization which subscribers or users can use RNAA.

-	For revocation, the CCF-A can initiate the Authorization Revocation Request message as defined in clause 8.23.4 of TS 23.222 [3] with additional information to identify the RNAA-related revoked token. The CCF-B provides notification message to the API invoker after token revocation.

* * * End of Changes * * * *