# 3GPP TSG-SA3 Meeting #122																			S3-251951

# Fukuoka, Japan, 19 – 23 May 2025

Source:	ZTE

Title:	Update the clause 5.3

Document for:	Approval

Agenda item:	4.26

Spec:	3GPP TS 33.369

Version:	0.1.0

Work Item:	AmbientIoT-SEC

Comments

The procedure for the protection of information during AIoT service communication needs to be added.

Proposed Changes

* * * First Change * * * *

5.3	Protection of information during AIoT service communication

Editor’s Note: This clause contains the security procedures on the information protection in command message, including protection for disabling device operation.

0.	Credentials are securely stored in the ADM on the network side and AIoT device.

1-5.	These steps refer to the step 1-6 in the clause 6.2.3 in TS 23.369[2], the AIoTF gets the session key and the counter from the ADM if the AIoTF does not have the session key and counter related to the AIoT device.

6.	The AIoTF may perform inventory procedure.

7. The AIoT generate the Command request message and MAC1. If counter is used,  MAC1= HMAC (device ID, counter), the input key is session key Ks

8. The AIOTF sends the Command Request message include the MAC1 to the selected AIoT RAN.

9.	Upon reception of the Command Request message from the AIOTF, the AIoT RAN sends an Command Request to the AIoT device.

10. The AIoT Device generates the session key Ks based on the long term key and check the received MAC1. If successful, the AIoT device increase the counter by one and calculate a MAC2 use the new counter. Then the AIoT device increase the counter by one.

11. The AIoT device sends a Command Response message that includes MAC2.

12.	AIoT RAN responds an Command Response message  include the MAC2 to the AIOTF.

13.	The AIOTF increase the counter and calculate a XMAC and validates the MAC2 received. If successful, the AIoTF increase the counter by one.

14.	The AIOTF reports the result of the AIoT Command request to the NEF by sending the Nnef_AIoT_Command Notify message.

15.	The NEF informs the AF of the result of the AIoT_Command request by sending the Nnef_AIoT_Command Notify message.

* * * End of Changes * * * *