# 3GPP TSG-SA3 Meeting #122																			S3-251979

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on table content for OAuth 2.0

Document for:	Approval

Agenda item:	5.20

Spec:	3GPP TR 33.938

Version:	0.2.0

Work Item:	FS_CryptoInv

Comments

This contribution proposes table content for OAuth 2.0 for TR 33.938.

* * * First Change * * * *

## 4.2	3GPP Symmetric Cryptographic Algorithms

Editor’s Note: The current table is for example and placeholder purposes. It would be revised/refined once the detailed protocol list description has been agreed.

The following table summarizes the security related protocols used in 3GPP employing symmetric cryptographic algorithms including hash functions (5G System).

Table 4.2-1: Protocols Used in 3GPP Employing Symmetric Cryptographic Algorithms (5G System)

| Protocol/Function | Protocol Profile, Clauses | Cryptographic Algorithm(s) | Feature(s), Usage Type |
| --- | --- | --- | --- |
| e.g., PDCP (TS 38.323[]) | TS 33.501 [4] | 128-NxA1 | Confidentiality and Integrity Protection |
|  |  | 128-NxA2 | Confidentiality and Integrity Protection |
| OAuth 2.0 (IETF RFC 6749 [x1], 6750 [x2]) | TS 33.210 [2] clause 6.2 for TLS | 128-AES GCM SHA256 (IETF RFC 8442 [x3]). | Confidentiality and Integrity Protection |
|  |  | 256-AES GCM SHA384 (IETF RFC 8442 [x3]). | Confidentiality and Integrity Protection |
|  | TS 33.210 [2] clause 6.3 for JWE/JWS | 128-AES GCM | Confidentiality and Integrity Protection |
|  |  | 256-AES GCM | Confidentiality and Integrity Protection |

## 4.3	3GPP Asymmetric Cryptographic Algorithms

Editor’s Note: The current table is for example and placeholder purposes. It would be revised/refined once the detailed protocol list description has been agreed.

The following table summarizes the security related protocols used in 3GPP employing asymmetric cryptographic algorithms (5G System).

Table 4.3-1: Protocols Used in 3GPP Employing Asymmetric Cryptographic Algorithms (5G System)

| Protocol/Function | Protocol Profile, Clauses | Cryptographic Algorithm(s) | Feature(s), Usage Type |
| --- | --- | --- | --- |
| e.g., TLS (IETF RFC 8446) | TS 33.210 [2] | ECDHE (IETF RFC 8996) | Key Agreement |
|  |  | RSA (IETF RFC 8017) | Digital Signature and Authentication |
| OAuth 2.0 (IETF RFC 6749 [x1], 6750 [x2]) | TS 33.210 [2] clause 6.2 for TLS | ECDHE (IETF RFC 8996 [x3]) | Key Agreement |
|  |  | RSA (IETF RFC 8017 [x4]) | Digital Signature and Authentication |
|  | TS 33.210 [2] clause 6.3 for JWE/JWS | ECDHE (IETF RFC 8996 [x3]) | Key Agreement |
|  |  | ECDSA using P-256 and SHA-256 (IETF RFC 6979 [x4]) | Digital Signature and Authentication |

* * * End of Changes * * * *