# 3GPP TSG-SA3 Meeting #122																			S3-251980

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on table content for IKEv2

Document for:	Approval

Agenda item:	5.20

Spec:	3GPP TR 33.938

Version:	0.2.0

Work Item:	FS_CryptoInv

Comments

This contribution proposes table content for IKEv2 for TR 33.938.

* * * First Change * * * *

## 4.2	3GPP Symmetric Cryptographic Algorithms

Editor’s Note: The current table is for example and placeholder purposes. It would be revised/refined once the detailed protocol list description has been agreed.

The following table summarizes the security related protocols used in 3GPP employing symmetric cryptographic algorithms including hash functions (5G System).

Table 4.2-1: Protocols Used in 3GPP Employing Symmetric Cryptographic Algorithms (5G System)

| Protocol/Function | Protocol Profile, Clauses | Cryptographic Algorithm(s) | Feature(s), Usage Type |
| --- | --- | --- | --- |
| e.g., PDCP (TS 38.323[]) | TS 33.501 [4] | 128-NxA1 | Confidentiality and Integrity Protection |
|  |  | 128-NxA2 | Confidentiality and Integrity Protection |
| IKEv2 (IETF RFC 7296 [x1]) | TS 33.210 [2] clause 6 | 128-AES GCM SHA256 (IETF RFC 8442 [x2]). | Confidentiality and Integrity Protection |
|  |  | 256-AES GCM SHA384 (IETF RFC 8442 [x2]). | Confidentiality and Integrity Protection |
|  | TS 33.310 [3] clauses 5,6,7 |  |  |

## 4.3	3GPP Asymmetric Cryptographic Algorithms

Editor’s Note: The current table is for example and placeholder purposes. It would be revised/refined once the detailed protocol list description has been agreed.

The following table summarizes the security related protocols used in 3GPP employing asymmetric cryptographic algorithms (5G System).

Table 4.3-1: Protocols Used in 3GPP Employing Asymmetric Cryptographic Algorithms (5G System)

| Protocol/Function | Protocol Profile, Clauses | Cryptographic Algorithm(s) | Feature(s), Usage Type |
| --- | --- | --- | --- |
| e.g., TLS (IETF RFC 8446) | TS 33.210 [2] | ECDHE (IETF RFC 8996) | Key Agreement |
|  |  | RSA (IETF RFC 8017) | Digital Signature and Authentication |
| IKEv2 (IETF RFC 7296 [x1]) | TS 33.210 [2] clause 6 | DH group 19, 20, 31 | Key Agreement |
|  | TS 33.310 [3] clauses 5,6,7 | RSA Sha-256 (IETF RFC 8017 [x3]) | Digital Signature and Authentication |
|  |  | ECDSA SHA-256/384/512 (IETF RFC 4754 [x4]) | Digital Signature and Authentication |

* * * End of Changes * * * *