3GPP TSG-SA3 Meeting #122	S3-252015

Fukuoka, Japan, 19 - 23 May 2025

# Source:	KDDI

# Title:	New SID on supporting AEAD algorithms

# Document for:	Approval

# Agenda Item:	6.3 6G SID/WIDs

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on supporting AEAD algorithms

Acronym: tbd

Unique identifier:	tbd

Potential target Release:	Rel-20

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| N/A |  | {optional free text} |

Dependency on non-3GPP (draft) specification:

3	Justification

ETSI SAGE and 3GPP SA3 have recently completed specifications for 256-bit cryptographic algorithms. For the very first time in 3GPP, these specifications also include Authenticated Encryption with Associated Data (AEAD) algorithms [1][2][3]. With the industry’s increasing focus on higher throughput and data-intensive applications, SA3 should consider adoption of these AEAD algorithms to be used for NAS and AS security (including control and user plane security) in a 3GPP System.

Cryptographic algorithms operating in AEAD mode combine both encryption and integrity protection to a single operation. Generally speaking, this approach has two main benefits:

- Enhanced throughput and power consumption

  - Single-pass operation: Ideally, they provide data encryption and authentication in a single pass (in contrast to the separate ciphering and integrity protection algorithms used today).

  - Reduced power consumption: There is data that suggests that, for smaller payloads, AEAD algorithms can exhibit reduced power consumption compared to other symmetric key schemes that generate the MAC / authentication tag separately [4].

  - Enhanced throughput: When conducting both encryption and integrity, it is observed AEAD mode produces better efficiency compared to legacy mode encryption and integrity protection combined. [4]

- Simplified system design and key management

  - Fewer algorithms to implement: As encryption and integrity protection can be achieved by a single algorithm, the number of algorithms to be implemented can be reduced.

  - Fewer keys to manage: With AEAD, only one key is needed for both encryption and integrity protection, simplifying key management.

  - Less Error-Prone: Using separate algorithms can introduce complexity (e.g. security policy complexity) and potential errors in their combination. AEAD reduces the risk of such errors by providing a single, well-defined process.

Current NAS and AS security mechanisms use 32-bit Message Authentication Code (MAC) tags for the integrity protection. A 32-bit MAC is too short to provide enough security margin against forgery and longer length is recommended. Since new AEAD algorithms allow generating up to 128-bit tags, SA3 should also consider a longer MAC length.

Note that AEAD is merely a new operating mode for the 256-bit versions of the existing cryptographic algorithms, there is no reason to separate the introduction of AEAD mode from that of the 256-bit algorithms themselves. Therefore the normative phase for introducing 256-bit algorithms can be considered after this study.

Reference:

[1]	TS 35.240 Specification of the Snow 5G based 256-bits algorithm set

[2]	TS 35.243 Specification of the AES based 256-bits algorithm set

[3]	TS 35.246 Specification of the ZUC based 256-bits algorithm set

[4]	S3-250369, “Use of AEAD in Next-Generation 3GPP System”

4	Objective

This study aims to identify potential challenges and requirements for supporting AEAD algorithms [1, 2, 3] for NAS and AS security (including control and user plane security) in the 6G System, including the following:

- Studying whether the current order of encryption and integrity protection AS security, where the MAC tag is generated over the plaintext and both plaintext and MAC tag are encrypted, is suitable for AEAD (Encrypt-then-MAC).

- If the Mac-then-Encrypt order is to be maintained for AS security, studying how AEAD can be used to support this order without any change of AEAD algorithm and performance degradation.

- Key hierarchy and management to support AEAD algorithms

- Negotiation of encryption and/or integrity protection when using AEAD algorithms

- Creation and handling of AEAD algorithm inputs, such as Nonce and Associated Data

- Interworking between AEAD-compatible systems and legacy deployments

- Number of implementation of algorithms, only AEAD algorithms or both AEAD and dedicated algorithms

- Studying potential challenges and requirements concerning MAC length.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 33. XYZ | Study on supporting AEAD algorithms | SA#112 | SA#112 | tbd |

| TU count / planning |  |
| --- | --- |
| SA3#123 | 1/2 TU |
| SA3#124 | 1/2 TU |
| SA3#125 | 1/2 TU |
| SA3#126 | 1/2 TU |
| SA3#127 | 1/2 TU |
| SA3#128 | 1/2 TU |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| N/A |  |  |  |

6	Work item Rapporteur(s)

TBD

7	Work item leadership

SA3

8	Aspects that involve other WGs

For a Stage 2 WID requiring Stage 3 to be done by another group: on a best-effort basis, indicate which potential WG is expected to specify the Stage 3: {possible values: "Not applicable", " unknown", "CT WGs", etc}

9	Supporting Individual Members

Supporting IM name

KDDI