3GPP TSG-SA3 Meeting #122	S3-252018

Fukuoka, Japan, 19 - 23 May 2025												               (revision of S3-24abcd)

# Source:	vivo

# Title:	New Sol: KI#3, Combine UE-generated temp ID with network-assigned temp ID

# Document for:	Approval

Agenda Item:	5.9

1	Decision/action requested

Approve the pCR on new solution for key issue #3.

2	References

3		Rationale

This contribution proposes a new solution to combine UE-generated temp ID with network-assigned temp ID for KI#3.

The solution integrates both the UE-generated temp ID and the network-assigned temp ID methods. For individual paging (Inventory only, Inventory and command), the UE-generated temp ID method is suggested to reduce reliance on the command messages associated with the network-assigned temporary ID solution. Meanwhile, for group paging (Inventory only, Inventory and command), the network-assigned temp ID method is recommended to address the limitations of the UE-generated temp ID approach (i.e. significant computational demand on the network side).

4	Detailed proposals

*** BEGIN CHANGES ***

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.369: "Service Requirements for ambient power-enabled IoT".

[3]	RP-234058, RAN New SID for Study on Solution for Ambient IoT in NR.

[4]	3GPP TR 23-700-13: "Study on Architecture Support of Ambient power-enabled Internet of Things".

[5]					3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[6]					R2-2406202 RAN2#126 Meeting Report

[7]	RFC 4739: "Multiple Authentication Exchanges in the Internet Key Exchange (IKEv2) Protocol".

[8]	3GPP TR 38.848: "Technical Specification Group Radio Access Network; Study on Ambient IoT (Internet of Things) in RAN".

[9]	3GPP TR 38.789: “Study on solutions for Ambient IoT (Internet of Things) in NR”

[10]                      Jiao L, Wang N, Wang P, et al. Physical layer key generation in 5G wireless networks[J]. IEEE wireless communications, 2019, 26(5): 48-54.

[11]                     Zhao H, Zhang Y, Huang X, et al. A physical-layer key generation approach based on received signal strength in smart homes[J]. IEEE Internet of Things Journal, 2021, 9(7): 4917-4927.

[12]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[xx]	3GPP TS 23.369: “Architecture support for Ambient power-enabled Internet of Things”.

*** 2nd CHANGE  (all new text) ***

## 6.Y	Solution #Y: Combine UE-generated temp ID with network-assigned temp ID

### 6.Y.1	Introduction

This solution addresses KI#3.

The solution integrates both the UE-generated temp ID and the network-assigned temp ID methods. For individual paging (Inventory only, Inventory and command), the UE-generated temp ID method is suggested to reduce reliance on the command messages associated with the network-assigned temporary ID solution. Meanwhile, for group paging (Inventory only, Inventory and command), the network-assigned temp ID method is recommended to address the limitations of the UE-generated temp ID approach (i.e. significant computational demand on the network side).

### 6.Y.2	Solution details

6.Y.2.1 Individual Paging Procedure (Inventory only, Inventory and command)

Figure 6.Y.2.1-1: Individual paging protection

1.	The AF sends an Inventory/Command Request message towards the NEF as depicted in clause 6.2.2 or 6.2.3 in TS 23.369 [xx]. The information about the target AIoT Device(s) in the message includes permanent device ID (PID) meaning that the Inventory is for an individual AIoT device.

2.	The NEF sends Inventory/Command Request message with the information about the target AIoT Device to the AIOTF.

3.	The AIoTF generates NONCEnw, and sends the PID and NONCEnw to the ADM.

4.	The ADM calculates temp IDin-nw to page the individual AIoT device using permanent key, PID, NONCEnw.

NOTE:	The permanent key is also used for authentication, the device does not need to be pre-configured with two permanent keys (i.e. one is for ID protection, and another one is for authentication).

5.	The ADM sends temp IDin-nw to the AIOTF.

6.	The AIOTF sends individual inventory/command request towards the Reader, and the Reader sends individual paging with temp IDin-nw and NONCEnw.

7.	The AIoT device verifies temp IDin-nw with the permanent key, the NONCEnw, and the PID, if it matches, the AIoT devices generates temp IDin-d with permanent key, NONCEd, the NONCEnw, and the PID.

8.	The AIoT devices sends AIoT Inventory Response with the temp IDin-d and NONCEd to the AIOTF.

9.	The AIoTF sends received temp IDin-d, NONCEd and NONCEnw to ADM. This procedure is integrated with authentication procedure.

10.	The ADM verifies the temp IDin-d with permanent key, NONCEd, the NONCEnw, and the PID.

11.	If command is followed, the ADM determines to generate the network assigned temp IDgp-n. How to generate the temp IDgp-n is depicted in step 8 in 6.Y.2.2.

12	The ADM response with temp IDgp-n.

Case 1:	For inventory only

13.	Since command is not followed, the AIOTF will not update temp ID to the AIoT devices.

Case 2:	For inventory and command

14.	The The AIOTF sends AIoT Command Request to the AIoT device with temp IDgp-n.

15.	The AIoT device stores the received temp IDgp-n and replaces the latest temp IDgp-n-1.

16-18.	The AIoT device responds to the AIoTF.

6.Y.2.2 Group Paging Procedure (Inventory only, Inventory and command)

Figure 6.Y.2.2-1: Group Paging protection

1.	The AF sends an Inventory/Command Request message towards the NEF as depicted in clause 6.2.2 or 6.2.3 in TS 23.369 [xx]. The information about the target AIoT Device(s) in the message includes Filtering Information, as described in clause 5.8 in TS 23.369 [xx] meaning that the Inventory is for a group of AIoT devices.

2.	The NEF sends Inventory/Command Request message with the information about the target AIoT Device(s) to the AIOTF.

3.	The AIOTF sends group inventory/Command request towards the Reader, and the Reader sends group paging with (i.e. paging a group of AIoT devices) or without the AIoT Identification Information (i.e. paging all).

4.	If the AIoT Identification Information includes Filtering Information or paging does not include the AIoT Identification Information, and the AIoT device matches, the AIoT device uses the stored network assigned temp IDgp-n from the latest command. The AIoT device may be pre-configured a network assigned temp IDgp-0.

5.	The AIoT devices send AIoT Inventory Response with the temp IDgp-n(s) to the AIOTF.

6.	The AIoTF sends received temp IDgp-n(s) to ADM. This procedure is integrated with authentication procedure.

7.	The ADM gets PIDs from the temp IDgp-n(s) to identify the AIoT device. The ADM may generate temp IDgp in a restorable way, so that the ADM does not need to store the temp IDgp, and swiftly identify the AIoT device even if temp IDgp-n(s) is outdated, e.g. employ an asymmetric method to generate temp IDgp.

8.	If command is followed, the ADM determines to generates the network assigned temp IDgp-n+1. The ADM uses the same mechanism to generate temp IDgp-n+1 as depicted in step 7.

9.	The ADM response with PIDs and temp IDgp-n+1.

Case 1:	For inventory only

10.	Since command is not followed, the AIOTF will not update temp ID to the AIoT devices. The temp ID will be updated if command is followed in group or individual paging as depicted in step 11 in step 14 in 6.Y.2.1 or in 6.Y.2.2.

Case 2:	For inventory and command

11.	The AIOTF sends AIoT Command Request to the AIoT device with temp IDgp-n+1.

12.	The AIoT device stores the received temp IDgp-n+1 and replaces the latest temp IDgp-n.

13-15.	The AIoT device responds to the AIoTF.

### 6.Y.3	Evaluation

The solution address requirements of key issue#3.

For individual inventory only:

Temp ID will be fresh because two-side NONCE is used for ID calculation.

For individual inventory and command:

Temp ID will be fresh because two-side NONCE is used for ID calculation, and in this case, the network may also assign a new temp ID for potential group paging.

For group inventory and command:

Temp ID will be fresh because new temp ID is assigned.

For group inventory only:

The permanent ID will not be exposed. For a short period, the temp ID may remain unchanged if the command to update the network-assigned temp ID is not executed, but the temp ID will be fresh if the command is following for individual paging and group paging.

Especially for issues discussed in the last meeting:

ID Uniqueness: In inventory and command case, the update of temp ID of the AIoT device is managed by the operator, so the ID will not be clash.

Group Paging: This solution is effective for group paging scenarios (both MASK or paging ALL case) because the network-assigned solution is proposed to minimize the substantial computational demands associated with the UE-generated solution on the network side.

Re-Synchronization:	The ADM can generate temp ID in a recoverable manner, eliminating the need to store the temp ID while still allowing it to quickly identify the AIoT device, even if temp ID is outdated (for instance, the latest temp ID in the Command Request may not be successfully stored on the AIoT device side, the AIoT device uses the legacy one). This recoverable manner can be achieved by using an asymmetric method to generate temp ID.

Device Capabilities:	The ADM assigns a temp ID to AIoT devices only when the command is followed, ensuring that no extra signaling is required for ID protection.

Besides above, the more benefits of this solution are:

AIOTF is stateless: The AIOTF does not need to maintain a context of mapping between SUPI and temp ID, which simplifies ID management within the AIOTF.

Reuse the authentication key for ID protection: A extra permanent key is needed to simply AIoT device key management.

*** END OF CHANGES ***