3GPP TSG-SA3 Meeting #122	S3-252029

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Nokia

Title:	Resolution of EN in solution 35 concerning device constrains.

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 37.713

Version:	0.8.0

Work Item:	FS_Ambient_IoT_Sec

Comments

This pCR proposes a resolution to an editor’s notes in solution 35 concerning device constraints. The rationale to remove the EN is the following – The gNB is the initiator of the communication with the AIoT device and the provider of the backscatter signal powering the device. This implies, that the gNB can leave the backscatter connected until the device replies, indicating that the counter has been written. If the backscatter is disrupted, two possible outcomes exist, either the counter is written, but was not updated in the AIoTF or not written at all. The mitigation strategy to recover from the disruption, when backscatter is re-established, is to try with the old value presuming the value wasn’t written. If the device doesn’t reply, increment the counter and retry. This indicates the counter was written.

The above reflections are added to the evaluation and makes it complete.

Proposed Changes

* * * First Change * * * *

## 6.35	Solution #35: Configurable device/network authentication, data confidentiality, integrity and id privacy protection

### 6.35.1	Introduction

The solution proposes a solution to KI#3, KI#4 and KI#5. It gives a proposal to how a AIoT device can establish a secure connection with a reader, within the RAN procedure to resolve contention. This minimizes the signaling between the AIoT device and reader and thereby also the signaling overhead related to security establishment.

### 6.35.2	Solution details

The solution details how a shared session key can be derived while providing the means for mutual authentication (if required), confidentiality, integrity, reply and privacy protection. The explicit procedure is shown in figure 1. The solution is based on the 3-step contention based random access.

Figure 6.35-1: Security context establishment based on 3-step contention based random access

1. The AIoT device and AIoTF is provisioned with a share secret K and a configuration of which primitives to enable like replay protection, integrity protection, privacy protection etc. Furthermore, the AIoT device contains a non-volatile counter and the AIoTF keeps track of the counter.

The group key is shared among the group.

2. An AF requests the inventory from a AIoT device.

3. The AIoTF draws a random nonce1 and uses the nonce to derive a new key k_AIoT from the shared secret. K_AIoT is used to encrypt the device ID and optionally the counter value if reply protection is required. Please note that the same device ID produces different ciphertexts due to the nonce, hereby addressing the privacy concerns.

In case of a group request, the key used to derive the k_AIoT is the group key K_G. The derived group key will in the following be name k_AIoT_G. This implies, that the protected device id which in this case is a group ID, doesn’t reveal the group who is requested. The nonce will also ensure, that the same protected group ID will produce a different ciphertext for each request and thereby not disclose the group requested to eavesdroppers.

  - The Protected Id and nonce1 is send to the AIoT device.

6.	The AIoT device derives k_AIoT and decrypts the protected ID. In case of a group request the k_AIoT_G will be used to decrypt the protected ID. If the device ID doesn’t match the AIoT device ID or the device is not a member of the group rest of the steps can me omitted. If replay protection is enabled, the AIoT device compares the internal counter with count. If the counter values don’t match, the AIoT device can decide to omit rest of the steps or return the correct counter value to get the AIoTF back to sync (Please note that the counter is protected with the session key k_AIoT_ses). The decision is taken based on configuration and if the counter is s returned, the procedure restarts at step 3. Returning the protected counter enables the AIOTF to update the counter stored and hereby get back into sync. If returning the protected counter is disabled by configuration, the core can try counter+1, counter+2 to counter+N until the device replies or N is reached. N is defined by configuration. The synchronisation strategy can be left for deployment, but both proposed synchronisation strategies are viable options.

The AIoT device draws a random number Nonce2 and uses Nonce2 and k_AIoT to derive the session key k_AIoT_ses. Using the session key, the device creates a MAC of nonce1 called ARES. If authentication of the network isn’t required, the AIoT device ID can be encrypted/integrity protected using the session key. If the received request is a group request, the identifier of the key K is returned protected by the key k_AIoT_G. The key ID is uncorrelated with the device ID. The step finalises by incrementing the non-volatile counter.

7-8.	ARES and Nonce2 and optional protected counter and protected ID and key ID are sent to the AIoTF.

9.  The AIoTF derives the session k_AIoT_ses and calculates the MAC ARES*. If ARES equals ARES* the device is authenticated. This step can be omitted if device authentication isn’t required. If the reply is related to a group request, the AIoTF uses the key ID to lookup K enabling it to derive k_AIoT_ses. The AIoTF creates the MAC NRES using Nonce2 and the session key as input. If network authentication isn’t required, calculation of NRES can be omitted. If counter is returned form the AIoT device, the AIoTF re-adjust the internal counter and restarts at step 3.

10-11.NRES is send to the AIoT device.

12.The AIoT device calculates NRES* and compares it to NRES. If the values are equal, the AIoT device have authenticated the network. The device encrypts/integrity protects the AIoT device ID

13-14.	The protected ID is sent to the AIoTF.

15.	The reader might acknowledge the AIoT device – This is part of the contention based random access procedure.

16-17.	The AIoTF decrypts and verifies the integrity of the Protected ID and send it to the AF,

The key k_AIoT_ses can further be used to confidentiality and integrity protect command messages. If confidentiality and integrity protection isn’t required, NULL ciphering and integrity protection algorithms can be used.

.

NOTE: According to TR 38.769 [9] legacy is paging is not support and the baseline for the randomaccess procedure is slotted-ALOHA.

NOTE: The device will maintain state as long backscatter is applied, which implies if backscatter is removed, only the configuration, key K and group K_G and count maintain state. Procedure starts at step 3, when backscatter is reapplied.

### 6.35.3	Evaluation

The solution fulfils the requirements of KI#3 by protecting the device identifier such same device ID produces different ciphertexts in each paging message, hereby avoiding ciphertext attack. Furthermore, when the real device ID is transferred from the device to the AIoTF, the ID is protected by a session key, implying the same ID is producing different ciphertexts for each session, hereby also avoiding ciphertext and likability attacks.

The solution fulfills the requirements of KI#4 by providing the means for confidentiality, integrity and replay protecting. The enablement of the protection profile is based on configuration both in the AIoT device and AIoTF. The solution doesn’t mandate a given profile, but leaves it for deployment and implementation,

The solution fulfills the requirement of KI#5 by providing the means for authentication both one-way and mutual. The selection of authentication scheme is part of configuration of the AIoT and AIoTF.

The configurability of the protection means enables the AIoT framework to adapt to different use cases, and hereby also reduce the device complexity and encounter for the constraints. If applying all protection means, the execution time of a message will be prolonged, but quantitative measure cannot be given due to the influence by hardware optimizations. The prolonged time can be reduced by carefully evaluating the necessity of the protection mean and disable if not required. It’s not recommended to reduce execution time by disabling without careful evaluation.

The procedure proposes a method which stores a counter as part of step 6. If backscatter (power) is disrupted during this step, it’s not explicitly indicated by a reply, whether the counter was stored or not. If the device, due to constrains, cannot mitigate the ambiguity at next boot, the obligation of mitigating is delegate to the core. The issue is mitigated by initially retrying with the initial counter value, if device doesn’t reply, retry with an incremented counter.

* * * End of Change * * * *