3GPP TSG-SA3 Meeting #122	S3-252030

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Nokia

Title:	Refinement of EN in KI#5.

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 37.713

Version:	0.8.0

Work Item:	FS_Ambient_IoT_Sec

Comments

This pCR proposes a refinement to an editor’s note in KI#5 concerning the storage of credentials in the core network side. SA2 has taken a decision to store the device subscription profile in an ADM. This was informed in an LS on Device Subscription Data and therefore it’s proposed to follow the decision and store the credentials, together with the Device subscription data in the ADM. Additionally, SA2 has agreed to include the credential holder concept as an alternative to the core network base authentication. As the credentials need to be located as part of the authenticator, two options exist, either in the ADM or credential holders AAA server. It’s proposed to reflect this in the requirements. Furthermore, it’s proposed to provide this information to SA2 as requested in the LS.

Proposed Changes

* * * First Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ADM	Ambient IoT Data Management

<ABBREVIATION>	<Expansion>

* * * Next Change * * * *

## 5.5	Key Issue #5: Authentication in Ambient IoT service

### 5.5.1	Key issue details

The TR 23.700-13 [4] studies the architecture support of Ambient Internet of Things (AIoT) device, considering the service requirements for ambient power-enabled IoT device. In TR 23.700-13 [4], the validation of the AIoT device identity and authentication are explicitly mentioned.

### 5.5.2	Threats

In the air interface, an attacker may impersonate the victim device and report fake identification to the network side. If the billing is based on per AIoT device’s identity, the fake identity may lead to charging problem. This can be used by an adversary to steal an AIoT device by replacing the AIoT device with a fake device, which might cause a loss to the owner of the device. Whether fake identities pose a threat in case of inventory use case, is up to the individual deployments risk profile.

Over the air interface, without authentication, an adversary can impersonate a legitimate network and send a command — e.g., disablement command will permanently deactivate a device.

### 5.5.3	Potential security requirements

The 5G system shall provide a means to perform mutual authentication between the AIoT device and the network.

The 5G system shall store the credentials together with the device profile data in the ADM at network side or credential holder at the 3rd party side.

NOTE: If solution reuses the existing authentication framework (e.g., 5G-AKA, EAP-AKA’, other EAP methods for SNPN), the existing UE security requirements in TS 33.501[5] apply. Otherwise, the core network entities, if any, that are used for authentication are dedicated for Ambient IoT service.

Editor’s Note: Secure storage and processing of credentials (on the AIoT device) for solution not reusing existing authentication framework as per the note above is ffs.

Editor's Note: Whether performing authentication is to be mandated or optional, one way or mutual, or left to the application layer is to be discussed and decided in the conclusion.

* * * End of Change * * * *