3GPP TSG-SA3 Meeting #122																			S3-252041

Fukuoka, Japan, 19 – 23 May 2025

# Source:	Samsung

# Title:	pCR to update CAPIF nested API authorization procedure

# Document for:	Approval

Agenda Item:	4.22

# 1	Decision/action requested

It is proposed to add the following changes to the draftCR of nested API authorization.

# 2	References

[1]	S3-251868 - "Living CR on nested API"

# 3	Rationale

This pCR proposes to update the authorization for nested API procedure by resolving the following Editor’s Note:

As per RFC 8693, the access_token received in successful response (section 2.2.1) is termed as security token issued by the authorization server in response to the token exchange request. The access_token parameter from Section 5.1 of [RFC6749] is used to carry the requested token. TS 33.122 Annex C.2.2 already captures the standard claims of the access_token as per RFC 6749, hence for nested API case same Annex clause can be referred. Further, it is required to clearly mention if there is any IEs in addition to standard claims are required. For example.,

{

"aud":"urn:example:cooperation-context",

"iss":"https://as.example.com",

"exp":1441913610,

"scope":"status feed",

"sub":"user@example.net",

"act":

{

"sub":"admin@example.net"

}

}

As per the above snippet from RFC 8693, additionally the newly issued access token should include the information of the “sub”, which is the API invoker and “act sub” which is the AEF-1 in case of delegation. Hence the following EN can be addressed.

Editor’s Note:	The content of the newly issued token is FFS.

IETF RFC 8693 is considered for nested API invocation, the RFC includes both delegation semantics and impersonation semantics. Clause 1.1 in RFC gives clear differentiation between delegation and impersonation. In nested API case if delegation semantics are considered i.e., with delegation semantics, API invoker-1 still has its own identity separate from API invoker-2 (AEF-1), and it is explicitly understood that while API invoker-1 may have delegated some of its rights to AEF-1, any actions taken are being taken by AEF-1 representing API invoker-1 i.e., AEF-1 is an agent for API invoker-1.

But when impersonation is considered, AEF-1 impersonates API invoker, AEF-1 is given all the rights API invoker has and is indistinguishable from API invoker.

As delegation is not possible without the actor_token (also specified in RFC 8693), it is proposed to use an actor_token to validate the delegated authorization to access service API of API exposing function 2. In case of impersonation it is assumed that CCF is in possession of information to authorize the request of AEF-1 as part of local policy and how this local policy is obtained is upto implementation. With this understanding it is proposed to address the following Editor’s Note.

Editor’s Note:	How the CCF validates whether the requesting API exposing function 1 is allowed for delegated authorization to access service API of API exposing function 2 is FFS.

Further, the information flows in case of token exchange mechanism is not clear. It is proposed to add a clause in annex to specify the token exchange request/response information flow mapping it with IETF RFC 8693. With this update the following EN can be addressed.

Editor’s Note:	The parameters in the token exchange request and response is FFS.

# 4	Detailed proposal

* * * First Change * * * *

## 6.X	Authorization procedure in a nested API invocation

Editor’s Note:	This clause will include authorization procedure for nested API invocation cases.

The nested API invocation scenario is a scenario where an API invocation towards a first API exposing function (AEF-1) triggers that API exposing function to request an API invocation towards a second API exposing function (AEF-2), which is in the same API provider domain as the first API exposing function. The authorization of API invocation triggered towards second API exposing function shall use token exchange procedure as specified in IETF RFC 8693 [xx], where the access token of the API invoker to be used towards AEF-1 is used as the subject token as per the IETF RFC 8693 [xx]. AEF-1 before triggering API invocation towards AEF-2, invokes the token exchange request towards the CCF by sending the subject token to receive a new access token. AEF-1 uses the received new access token towards AEF-2 for nested API invocation. This is depicted in figure 6.X-1.

Figure 6.X-1: Authorization for nested API invocation

1.	API invoker -1 invokes the AEF-1 service by using the access token obtained from the CCF/Authorization Function.

2.	Based on the service API invocation request, API exposing function 1 verifies the access token and decides to invoke another service API exposed by API exposing function 2.

3.	The API exposing function 1 sends token exchange request message to CCF, to get a token to invoke the service API in API exposing function 2. The CCF validates the request from API exposing function 1 based on the actor token received in the request message (for delegation) or based on the local policy available at the CCF (for impersonation). The CCF validates whether the requesting API exposing function 1 is allowed for accessing service API of API exposing function 2. Also, the CCF validates the access token in the request message that is provided by the API invoker to AEF-1.  After successful validation, the CCF responds to API exposing function 1 with token exchange response message that includes a new access token to allow API exposing function 1 to invoke the service API on API exposing function 2. The details of the token exchange request and response message is specified in Annex C.X.

4.	API exposing function 1, shall send a service API invocation request to API exposing function 2 with the authorization information i.e., access token received in step 3.

5.	The API exposing function 1 receives the service API invocation response resulting from the service API invocation once API exposing function 2 has checked whether the API invoker is authorized to invoke that service API based on the received token.

6.	The API invoker receives the service API invocation response resulting from the service API invocation.

* * * Second Changes * * * *

# C.X	OAuth 2.0 token exchange

## C.X.1	General

For nested API invocation when AEF-1 (API invoker-2) decides to invoke another service API exposed by AEF-2, the AEF-1 may request a new access token from CAPIF Core Function to invoke another service API exposed by AEF-2 as defined in clause 6.X. Figure C.X.1-1 shows the token exchange request and response.

Figure C.X.1-1: Token exchange

## C.X.2	Token exchange request

To obtain an access token, the API Invoker makes a request to the CAPIF Core Function by sending a Token Exchange Request message with the following parameters using the "application/x-www-form-urlencoded" format, with a character encoding of UTF-8 in the HTTP request entity-body. The Token Exchange Request parameters are shown in table C.X.2-1.

Table C.X.2-1: Token Exchange Request message parameters

| Parameter | Values |
| --- | --- |
| grant_type | REQUIRED. The value shall be set to urn:ietf:params:oauth:grant-type:token-exchange indicates that a token exchange is being performed. |
| subject_token | REQUIRED. The access_token received from the API invoker in service API invocation request. |
| actor_token | OPTIONAL. A security token that represents the identity of the delegated AEF-1. |
| scope | OPTIONAL. A string containing a space-delimited list, of desired scope of the requested token. |

## C.X.3	Token exchange response

If the token exchange request is valid and authorized by the CAPIF Core Function, the CAPIF Core Function then returns a new access token to the AEF-1 in token exchange response message; otherwise it will return an error.

The token exchange response parameters are shown in table C.X.3-1.

Table C.X.3-1: Token exchange response message parameters

| Parameter | Values |
| --- | --- |
| access_token | REQUIRED. This is the newly issued access token to invoke service API exposed by AEF-2. (see NOTE 1) |
| token_type | REQUIRED. This field shall be “bearer”. |
| expires_in | REQUIRED. The lifetime in seconds of the access token. |
| scope | REQUIRED. The granted scope by the CAPIF core function, if the scope of the issued security token is not identical to the scope requested by the API invoker; otherwise OPTIONAL |
| NOTE 1: The newly issued access_token shall include the resource owner information, authorization information of the API invoker and delegated AEF-1 information in addition to the standard access token claims as specified in clause C.2.2. |  |

Upon receiving the token exchange response message, the AEF-1 may now use the newly issued access token to make authorized northbound API requests to AEF-2 as described in clause 6.X.

* * * End of Changes * * * *