3GPP TSG-SA3 Meeting #122	S3-252054

# Fukuoka, Japan, 19 - 23 May 2025

Source:	Samsung

Title:	Security procedure for inter-CU LTM

Document for:	Approval

Agenda item:	4.12

Spec:	3GPP TS 33.501

Version:	19.1.0

Work Item:	NR_Mob_Ph4_Sec

Comments

This pCR proposes normative text to the draftCR for security procedure for inter-CU LTM, based on the progress made in RAN WGs [R2-2502981 and S3-251824].

* * * First Change * * * *

#### 6.X.Y	Security mechanism and procedures for L1/L2 Triggered Mobility

##### 6.X.Y.1	When DC is not configured

For the case where CU is acting as MN and DC is not configured as specified in TS 38.300 [52], the source gNB upon generating the KNG-RAN* as described in Annex A.11, shall send the generated {KNG-RAN*, NCC} pair to the target gNB(s) of the candidate cells using XnAP message.

During LTM execution phase, the source gNB includes the NCC used for the derivation of KNG-RAN* in the Cell Switch Command MAC CE to the UE to avoid any de-synchronization issue due to NCC mismatch. If the NGAP PATH SWITCH REQUEST ACKNOWLEDGE message from the AMF contains a NSCI, then the keySetChangeIndicator field shall be set to true in the Cell Switch Command MAC CE. Upon receiving the cell switch command, the UE derives the KNG-RAN* as described in 6.9.2.3.4 of the present document.

For the gNBs with split DU-CU implementations, the CU provides the NCC via the UE Context Modification procedure to the DU. The DU includes the received NCC value in the Cell Switch Command to the UE.

The target gNB shall initiate an intra-gNB-CU handover, if there is mismatch in the UE security context received in the PATH SWITCH REQUEST ACKNOWLEDGE message from the AMF and the UE security context received from the source gNB. The target gNB shall indicate to the UE to change the current KgNB, to reconfigure the DRBs to activate or de-activate the UP integrity/confidentiality as per the received policy from SMF and selected algorithms in the intra-cell handover procedure. Further, the target gNB updates the UE’s 5G security capabilities and/or UE’s UP security policy for the active PDU session to all the candidate gNBs.

##### 6.X.Y.2	When DC is configured

In the case where CU is acting as SN and MN is unchanged as specified in TS 37.340 [51], the security mechanism and procedures as specified in clause 6.10.2.4 of this specification for SCPAC shall be applied.

* * * End of Changes * * * *