3GPP TSG-SA3 Meeting #122	S3-252055

Fukuoka, Japan, 19 – 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.256 | CR | 0053 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding support for multiple USSs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, InterDigital, Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3_Sec |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is no support for multiple USS in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | As the target USS is the only entity that knows whether it wants to authenticate a UAV when the UAV gets transitioned to it from a source USS, the target USS triggers an existing re-authorisation/re-authentication procedure with the UAV. This is aligned with TS 23.256, in clause 5.13.2 step 20, where the UUAA is initiated by target USS (i.e., USS 2). The UAS NF may store and maintain a list of authorized USSes as part of the UUAA context for each UAV |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for multiple USS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, 5.2.1.1, 5.2.2.1, 5.3.1, 5.3.2, 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3-251725 |  |  |  |  |  |  |  |  |

**** START OF CHANGES ****

# 4	Overview

TS 23.256 [3] describes the architecture enhancements for supporting Uncrewed Aerial Systems (UAS).TS 23.256 [3] contains the reference architecture and message flows to support this new functionality for UAVs. The present document describes the security of these new features including:

-	Authentication and authorization of a Uncrewed Aerial Vehicle (UAV) with the UAS Service Supplier (USS) during 5GS registration;

-	Authentication and authorization of a UAV with the USS during PDU session establishment and PDN connection establishment;

-	Support re-authentication, re-authorisation and revocation of the above;

-	Support for USS authorization of pairing of UAVs and UAV-Cs;

-	Support for authorisation of providing location information and providing network based location to mitigate against UAVs reporting false location data.

-	Support for security and privacy of Command and Control (C2) communications over PC5 interface in 3GPP system;

-	Support for security and privacy of transporting Broadcast Remote Identification;

-	Support for security and privacy for Detect And Avoid (DAA) traffic; and

-	Support for security of multiple USS serving different geographical areas corresponding to the UAV flight path, i.e. changeover of the UAV from one USS to another USS.

**** NEXT CHANGE ****

#### 5.2.1.1	General

The UAV USS authentication and authorization (UUAA) is the procedure to ensure that the UAV can be authenticated and authorized by a USS before the connectivity for UAS services is enabled. This clause specifies the relationship between primary authentication (as described in clause 6.1 in TS 33.501 [2]) and UUAA. An UAV is allowed to perform UUAA with the USS/UTM only after the UAV (UE) has completed successfully primary authentication.

It may be triggered by the AMF when UAV is registering with 5GS or triggered by the SMF during the PDU session establishment procedure. The UUAA procedure may also be triggered by a USS for re-authentication if the USS had authenticated the UAV. Network support for UUAA during registration is optional while it is mandatory during the PDU Session establishment. UE Support for UUAA during registration and during the PDU Session establishment is mandatory.

The AMF or SMF triggers the UUAA procedure if the UAV has an Aerial UE subscription and the UAV requests access to UAS services by providing the CAA-Level UAV ID of the UAV in the Registration Request or PDU Session Establishment Request.

The UUAA is performed between the UAV and the USS. The UAV is authenticated based on the CAA-Level UAV ID and credentials associated to the CAA-Level UAV ID. The authentication messages are included in a transparent container and conveyed between the UAV and the USS via a 3GPP UAS NF.

NOTE:	The provision of CAA-Level UAV ID, credentials, and the actual authentication methods and information that needs to be sent to perform the UUAA are out of scope of the 3GPP specifications.

On successful completion of a UUAA, the USS can send UAS security information in the UUAA Authorization Payload to the UAV. The contents of that security information are out of scope of the 3GPP specifications.

The UUAA procedure at registration in 5G is described in the clause 5.2.1.2 and the UUAA procedure during PDU session establishment procedure is described in the clause 5.2.1.3.

At any time after the initial registration, the USS or the AMF (when the networking supports UUAA during registration) may initiate the Re-authentication procedure for the UAV. The AMF initiated Re-authentication procedure is described in the clause 5.2.1.2, whereas the USS initiated Re-authentication procedure is described in the clause 5.1.2.4.

The support of multiple USS is given in clause 5.X of the present document.

Figure 5.2.1.1-1 provides an example of how UUAA fits into the 5GS procedures. The complete description of this flow is given in TS 23.256 [3].

Figure 5.2.1.1-1: UUAA in 5GS

1.	The UE sends a Registration Request message to the AMF. The UE may provide a CAA-Level UAV ID, and optionally a USS address/IP address, to indicate the request is registering for UAS services. In case the CAA-Level UAV ID and/or USS address/IP address is configured not to be sent in plain text, e.g., the USS address or an IP address not to be exposed in public, the CAA-Level UAV ID, and USS/IP address if available, shall be sent after the NAS security is established.

2.	AMF completes security set up including primary authentication as needed.

3.	After successful Primary authentication, AMF determines whether UUAA is required for the UE. UUAA shall only be triggered if the UE has provided a CAA-Level UAV ID and has a valid Aerial UE subscription. AMF may skip UUAA if the UE has completed UUAA successfully before and the UE UUAA is current, i.e., the UE's authentication and authorization has not been revoked after a previous successful UUAA.

4a. AMF shall return a Registration Accept message to the UE and indicate that UUAA is pending.

4b.	UE may send a Registration Complete message to acknowledge the AMF.

5. AMF triggers the UUAA procedure if determined needed in step 3 as described in clause 5.2.1.2.

The following procedure is for UUAA during PDU session establishment:

6. The UE sends a PDU Session Establishment Request message to the SMF including a CAA-Level UAV ID to indicate the request is for UAS services.

7. The SMF determines whether UUAA is required for the UE. UUAA shall only be triggered if the UE has provided a CAA-Level UAV ID and has a valid Aerial UE subscription. SMF may skip UUAA, the UE has completed UUAA successfully with the same USS/DN before, i.e., at registration as in step 5 or in previous PDU Session Establishment procedures and a successful UUAA result is available.

8. The SMF triggers the UUAA procedure if determined needed at step 7 as described in clause 5.2.1.3.

**** NEXT CHANGE ****

#### 5.2.2.1	General

The UAV USS authentication and authorization (UUAA) is the procedure to ensure that the UAV can be authenticated and authorized by a USS before the connectivity for UAS services is enabled. This clause specifies the relationship between authentication and UUAA. An UAV is allowed to perform UUAA with the USS/UTM only after the UAV (UE) has completed successfully authentication with EPC. The SMF+PGW-C triggers the UUAA procedure if the UAV has an Aerial UE subscription and the UAV requests access to UAS services by providing the CAA-Level UAV ID of the UAV when attaching to the network.

The UUAA is performed between the UAV and the USS. The UAV is authenticated based on the CAA-Level UAV ID and credentials associated to the CAA-Level UAV ID. The authentication messages are included in a transparent container and conveyed between the UAV and the USS via a 3GPP UAS NF.

NOTE:	The provision of CAA-Level UAV ID, credentials, and the actual authentication methods and information that needs to be sent to perform the UUAA are out of scope of the 3GPP specifications.

On successful completion of a UUAA, the USS sends UAS security information (if determined by the USS) in the UUAA Authorization Payload to the UAV. The contents of that security information are out of scope of the 3GPP specifications.

The UUAA procedure is described in the clause 5.2.2.2.

The support of multiple USS is given in clause 5.X of the present document.

**** NEXT CHANGE ****

## 5.3	Location Information Veracity and Location Tracking Authorization

### 5.3.1	General

There are three UAV tracking modes as follows (see TS 23.256 [3] for more details):

-	UAV location reporting mode;

-	UAV presence monitoring mode; and

-	Unknown UAV tracking mode.

The first two relate to obtaining location information about a particular UE while the latter one is about obtaining information about all the UEs in a particular geographic region.

For the first two mode before proceeding with the request for information about the particular UE, the UAS NF shall ensure that the requesting USS is the one that authorized the UE or is part of the list of authorized USSs in the UAV UE's context.

For the latter mode, a USS is authorized to receive the CAA level ID of all UAVs in a geographic area indicated by the USS. In addition, if the USS performed the UUAA of the UAV, or the UAS NF is configured to know the USS is authorized to receive such information, by checking that the USS is part of the list of authorized USSs in the UAV UE's context, then the 3GPP UAV ID of such UAVs is also included.

### 5.3.2	Location information veracity and location tracking authorization in 5GS

USS may receive the location information which is reported by UAV via the application layer. The USS may decide to check and verify the location information in order to prevent spoofed and forged location information. The location result from 5GS helps to verify the location information reported from UAV side. 5GS provides network-based location information by utilizing the Location Services (LCS) supported by AMF or GMLC as specified in TS 23.273 [4] and 23.502 [5], and the detailed procedures of location information veracity and location tracking authorization are described below.

Figure 5.3.2-1: Location information veracity and location tracking authorization in 5GS

Step 1-3 shows the procedure for the USS to obtain a network-based location for UAV(s).

1.	The USS sends the location request to UAS NF/NEF to request the UAV location or presence from network. The location request includes the GPSI of the UAV to request the location information or presence about an individual UE, or a geographic area when trying to find the information of all UAVs in an area. The LCS request also indicates the 5GS to obtain reliable UE location information, i.e. the location calculated and provided by the network.

If the USS/TPAE does not specify target 3GPP UAV ID and request UAS NF for a list of the UAVs in the geographic area and served by the PLMN, clauses 5.3.1.3 and 5.3.4 in TS 23.256 [3] apply.

2.	The UAS NF/NEF first verifies the request in step 1 is authorized. When the USS sends a GPSI, this is done by checking whether the identifier of the USS sending the request matches the previously associated mapping between the GPSI and the USS identifier or whether the identifier of the USS sending the request matches one of the authorized USSs in the UAV UE's context. When the USS request UAS NF for a list of the UAVs in the geographic area, this is done by checking the USS is authorized to receive the CAA level ID of all UAVs in a geographic area indicated by the USS. The UAS NF/NEF gets the relevant UAV(s) location information or presence from AMF or GMLC by the current location services supported by AMF or GMLC if passes the above authorization check. On the condition of the location services provided by AMF, the UE presence status is provided by reusing the Area of Interest mechanism. On the condition of the location services provided by GMLC, the GMLC indicates LMF via AMF to select Network Assisted Positioning method which relies on the location measurement from NG-RAN nodes, if receiving reliable location information request in step 1.

NOTE 1: Void

3.	The UAS NF/NEF provides the UAV(s) location information or presence to the USS. When the USS request UAS NF for a list of the UAVs in the geographic area, if the USS performed the UUAA of the UAV, or the USS is authorized to receive such information as part of the authorized USSs in the UAV UE's context, then the 3GPP UAV ID of such UAVs is also included. USS may make decisions to control the UAV based on the result output received from UAS NF/NEF.

NOTE 2: Use of LCS privacy feature (e.g. user consent) is applicable to UAVs as for normal UEs.

**** NEXT CHANGE ****

## 5.X	Security aspects supporting multiple USS

### 5.X.1	USS changeover procedure

The support of multiple USS is given in clause 5.13 of TS 23.256 [3]. The USS change procedure is about moving service of the UAV from the serving USS (called USS 1) to a target (called USS 2).

The procedure for USS change given in clause 5.13.2 of TS 23.256 [3] is followed with the following addition:

Step 5:	 The UAS NF/NEF shall check whether its UUAA context contains the UUAA results for candidate USSs.

Step 8:	 In the response message, the UAS NF/NEF shall include info about whether the candidate USSs require UUAA.

Step 9:	 The serving USS (i.e. USS 1) may choose the target USS (i.e. USS 2) considering the UUAA result of the candidate USSs stored in the UUAA context

Step 20: The USS2 may initiate the UUAA procedure. If USS 2 needs to authenticate the UAV, then one of the following procedures shall be used:

On 5GS, USS 2 triggers the procedure given in clause 5.X.2.1.

On EPS, USS 2 triggers the procedure given in clause 5.X.2.2.

Editor's Note:	The support for UAV triggered UUAA is FFS and needs to be checked in coordination with SA2.

NOTE 1: Triggering either of the above procedures results in an authentication between the UAV and target USS.

### 5.X.2	Multiple USS procedures

#### 5.X.2.1	5GS UUAA during USS changeover

This procedure runs between the UAV and USS 2 following the procedure in clause 5.2.1.4 of the present document using the information obtained by USS 2 at step 18 in clause 5.13.2 of TS 23.256 [3].

NOTE: Re-authentication procedure is re-used to perform authentication between the UAV and target USS.

#### 5.X.2.2	EPS UUAA during USS changeover

This procedure runs between the UAV and USS 2 following the procedure in clause 5.2.2.3 of the present document using the information obtained by USS 2 at step 18 in clause 5.13.2 of TS 23.256 [3].

NOTE: Re-authentication procedure is re-used to perform authentication between the UAV and target USS.

#### 5.X.2.5	UUAA procedures supporting multiple USSes

The following procedures are adapted to support multiple USSes

On 5GS in clause 5.2.1.2 (step 5) and clause 5.2.1.3 (step 5): the UAS NF/NEF may store and maintain a list of authorized USSes as part of the UUAA context for each UAV.

On EPS in clause 5.2.2.2 (step 5): the UAS NF/NEF may store and maintain a list of authorized USSes as part of the UUAA context for each UAV.

**** END OF CHANGES ****