3GPP TSG-SA3 Meeting #122	S3-252062

# Fukuoka, Japan, 19 – 23 May 2025

Source:	China Mobile

Title:	Pseudo-CR on updating AIoT system architecture and security assumptions of TS 33.369

Document for:	Approval

Agenda item:	4.26

Spec:	3GPP TS 33.369

Version:	0.1.0

Work Item:	AmbientIoT_Sec

Comments

This contribution proposes to update the system architecture and security assumption for TS 33.369.

Proposed Changes

* * * First Change * * * *

4	Overview of AIOT Security aspects

4.1	General

Two functional cases are supported: inventory, command.

The AIoT RAN reader is assumed to be trusted, i.e., authorized from network side to communicate with the AIoT device.

The AIoT domain and the 5G core network should be placed into separate security domains as depicted in Figure 4.1.

Editor’s note: If the existing authentication framework (e.g., 5G-AKA, EAP-AKA', other EAP methods for SNPN) is not reused, a dedicated network with separate NFs for ambient IoT service shall be needed, and security isolation mechanism between the AIoT domain and 5G core network shall be needed (e.g. a security gateway may be deployed) to isolate the 5G core network. The above are FFS.

NOTE: The possible interfaces needed to be protected include the interfaces between ADM and 5G core network, the interfaces between AIOTF and 5G core network.

NOTE: If multiple domains exist in the deployment of the architecture in figure 4.1, the above policy applies.

Figure 4.1 Domain separation

* * * End of Changes * * * *