3GPP TSG-SA3 Meeting #122	S3-252063

# Fukuoka, Japan, 19 – 23 May 2025

Source:	China Mobile

Title:	Pseudo-CR on updating clause 4.2.2 Requirements on the AIOTF of TS 33.369

Document for:	Approval

Agenda item:	4.26

Spec:	3GPP TS 33.369

Version:	0.1.0

Work Item:	Ambient_IoT_Sec

Comments

This contribution proposes the security requirements for AIoTF.

Proposed Changes

* * * First Change * * * *

4.2.2	Requirements on the AIOTF

For network layer authentication,

- AIOTF shall support the authentication functionality.

AIOTF shall support the signalling data confidentiality.

AIOTF shall support the signalling data integrity.

The traffic over AIOT2 interface between AIOTF and AIoT RAN Reader shall be integrity, confidentiality and replay-protected.

The traffic over Naiotf interfaces shall be integrity, confidentiality and replay-protected.

The AIOT3 interface between AIOTF and AMF shall support isolation if AIOTF and AMF are deployed in different security domains.

* * * End of Changes * * * *