# 3GPP TSG-SA3 Meeting #122																			S3-252078

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Xiaomi

Title:	pCR against interconnection baseline

Document for:	Approval

Agenda item:	4.22

Spec:	3GPP TS 33.122

Version:	19.0.0

Work Item:	CAPIF_Ph3_Sec

Comments

This contribution reuses the method 3 part in interconnection baseline to support RNAA interconnection.

The only difference is that the authorization request in the authorization code (optional PKCE) flow should also be transferred from CCF-B to CCF-A. Specifically, authorization request is used to get the authorization code while method 3 part in the baseline only considers the token request message.

.

* * * First Change * * * *

6.X.3.4		Authentication and Authorization for CAPIF RNAA interconnection

Clause 6.X.3.3 can be reused to support client credentials flow in RNAA scenarios.

For authorization code (optional PKCE) flow in RNAA scenarios, the clause 6.X.3.3 can be reused with the following additional enhancements related to the authorization request message.

- The API invoker shall send the authorization request message to the onboarded CCF-B, CCF-B determines that the service API requested is provided by the AEFs in CAPIF provider domain A.

- The CCF-B sends the authorization request message to the CCF-A in CAPIF provider domain A.

* * * End of Changes * * * *