# 3GPP TSG-SA3 Meeting #122																			S3-252080

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Xiaomi

Title:	Addressing EN in store and forward satellite operation

Document for:	Approval

Agenda item:	4.19

Spec:	3GPP TS 33.401

Version:	18.3.0

Work Item:	5GSAT_Ph3_SEC

Comments

Editor’s Note: Addressing (D)DOS attack is FFS.

The (D)DOS attack for the store and forward service operation can be divided into two parts: DOS attack on the UE and (D)DOS attack on the network entity onboard the satellite.

For the DOS attack on the UE, the threat arises due to providing the S&F Wait Timer and S&F Monitoring List in the unprotected Attach Reject message. The attacker can tamper with the value of S&F Wait Timer and S&F Monitoring List, leading the UE to receive incorrect values. However, based on the clarification in TS 23.401, the S&F wait Timer and S&F Monitoring List cannot prevent the UE from attempting to access another S&F satellite in the different PLMN or to another entity operating in non-S&F Mode. The impact of DOS attack on the UE is limited.

-	When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause, with a satellite of the same PLMN that is operating in S&F Mode.

The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE may (re)attempt NAS procedures or receive MT data from.

[Observation 1]: The restriction of S&F Wait Timer and S&F Monitoring List only applies to the S&F satellite in the same PLMN.

NOTE 6:	The S&F Wait Timer or S&F Monitoring List doesn't affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode.

[Observation 2]: The UE can attempt to access the entity operation in non-S&F mode after receiving the S&F wait timer or S&F Monitoring List.

NOTE 7:	How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one.

[Observation 3]: The UE can follow the information included in the S&F Monitoring List or not, which is up to the UE implementation.

In addition, based on TS 23.401, other timers (e.g., T3346, T3402, Lower bound timer value) may also be included in the ATTACH REJECT message, which is not protected. This information may be tampered by the attacker as well, but no protection method has been discussed and specified. Therefore, there is no need to define a security method dedicated for protecting the S&F Wait Timer and S&F Monitoring List in the ATTACH REJECT message.

For the (D)DOS attack on the network entity onboard the satellite, the threat arises due to the storage of received unprotected information. The attacker may send a large number of false Attach Requests to exhaust storage resources of the onboard entity. However, the (D)DOS attack has been discussed in the terrestrial network, which is a common risk for NTN network and TN network. The operator can determine how to address this risk based on its local policy. The specific behavior of the entity on-board should be left to implementation.

To summarize, it is proposed to change the Editor’s Note to Note: How to address (D)DOS attack on the entity on-board is determined by the operator’s policy and left to implementation.

* * * First Change * * * *

X.2	Security aspects of Split MME architecture

Mutual authentication between the UE and the network in a split MME architecture can involve more than one satellite (i.e., more than one MME-onboard), in which case the ground segment of the network is responsible for the selection and provisioning of MME-onboard the same, or another satellite, with the necessary information (e.g., Authentication Vector) to perform or finish an authentication procedure. The MME on-board obtains the EPS authentication vectors when the feeder link is available, and stores the authentication vectors when the service link is unavailable.

The NAS security shall be terminated on the MME-onboard. The ground segment of the network shall ensure that the latest NAS security context of the UE, or an Authentication Vector, is available at the MME-onboard.

NOTE 1: 	The distribution and synchronization of the latest NAS security context between the MME-ground and MME-onboard is out of the scope of 3GPP.

NOTE 2:	How to address (D)DOS attack on the entity on-board is determined by the operator’s policy and left to implementation.

* * * End of Changes * * * *