3GPP TSG-SA3 Meeting #122	S3-252084

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Xiaomi

Title:	Protection of information during AIoT service communication

Document for:	Approval

Agenda item:	4.26

Spec:	3GPP TS 33.369

Version:	0.1.0

Work Item:	AmbientIoT-SEC

Comments

Based on the conclusion in TR 33.713, it is proposed to define the information protection method during AIoT service communication.

* * * First Change * * * *

5.3	Protection of information during AIoT service communication

The information transmitted during AIoT service communication may be protected at the network layer. In this case, the AIoTF acts as the security termination. The information protection method for operator-managed AIoT devices is described below in Figure 5.3-1.

Figure 5.3-1: Protection of information during AIoT service communication for operator-managed AIoT device

- Steps 1-7 of clause 5.2 are performed. The session key is maintained by the AIoT device and AIoTF.

- The AIoTF may protect the NAS command (e.g., read, write, or disable) by using the session key associated with the AIoT device ID. If the integrity protection is activated, the AIoTF calculates the MAC for the NAS command. If the confidentiality protection is activated, the AIoTF encrypts the NAS command. The AIoTF sends the Command request to the Reader directly or indirectly. If the NAS command is for a group of AIoT devices (e.g., disable), the AIoTF aggregates the protected NAS command request in the Command request to the Reader.

Editor’s Note: The details of replay protection are FFS.

- The Reader sends the Command request to the selected AIoT device.

- If the MAC is included in the received Command request, the AIoT device verifies the integrity of NAS command by using the stored session key. If the encryption is activated, the AIoT device decrypts the NAS Command by using the stored session key. If the verification is passed, the AIoT device performs the command operation as requested by AIoTF.

The AIoT device may protect the NAS Command response by using the session key and return the Command response to the Reader.

- The Reader forwards the Command response to the AIoTF.

The AIoTF verifies the NAS Command response by using the corresponding session key and provides the information to the AF if any.

* * * End of Changes * * * *