3GPP TSG-SA3 Meeting #122 	S3-252098

# Fukuoka, Japan, 19 - 23 May 2025

# Source:	Apple

# Title:	Authentication procedure for Inventory

# Document for:	Approval

Agenda Item:	4.26

1	Decision/action requested

Approve the pCR to TS 33.369

- References

- TR 33.713 v0.8.0

- TS 23.369 v0.3.0

3	Rationale

This contribution proposes the authentication procedure for Inventory based on the conclusions in TR 33.713 and the procedure in TS 23.369.

4	Detailed proposal

*** Start of 1st Change ***

5	Security procedures for Ambient IoT service

5.1	General

Editor’s Note: This clause contains the general security aspects.

5.2	Authentication procedure

Editor’s Note: This clause contains the security procedures on the authentication.

5.2.1	General

5.2.2	Authentication procedure for Inventory

The following procedure illustrates the one-way authentication between AIoT device and AIOTF/ADM. The security credenticials are stored in AIoT device and ADM.

Figure 5.2.2-1 One-way Authentication procedure for Inventory

Step 1-4 are the same with TS 23.369.

Step 5. ADM generates the RAND, sends the AIoT device ID and RAND to AIOTF.

Step 6-7 are the same with TS 23.369.

Step 8. AIoTF sends the device ID and RAND to the NG-RAN.

Step 9. NG-RAN includes the device ID and RAND in the paging Request message.

Step 10. If the AIoT device determins it matches the AIoT Identification Information, AIoT device calculates the RES using K and device ID and RAND. K is pre-stored in AIoT device. AIoT device sends the RES to NG-RAN in Paging Response message.

Step 11. NG-RAN sends the AIoT NAS back to AIOTF in the Inventory Response message, which contains the device ID and RES.

Step 12. AIOTF sends data retrieve message to ADM including the device ID and RAND to calculate the XRES.

Step 13. ADM derives the XRES using K, device ID and RAND.

Step 14. ADM sends the XRES back to AIOTF using Data retrieve response message.

Step 15. AIOTF compares RES and XRES, if they are the same, the AIOTF determins the authentication successful.

Editor’s Note: the procedures may need to be updated based on SA2’s update on the Inventory procedure.

5.2.3	Authentication procedure for Command

Editor’s Note: This clause contains the security procedures on the authentication for Command procedure

5.3	Protection of information during AIoT service communication

*** End of 2nd Change ***