# 3GPP TSG-SA3 Meeting #122																			S3-252151

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Ericsson

Title:	Pseudo-CR on Privacy for group paging

Document for:	Approval

Agenda item:	4.26

Spec:	3GPP TS 33.369

Version:	0.1.0

Work Item:	AmbientIoT-SEC

Comments

AIoT procedures specified by SA2, RAN2 and RAN3 contain the option to page or perform inventory not only for a single device, but also for groups of devices. In clause 5.8 of v0.3.0 of TS 23.369, SA2 describes the identifiers that are used for group paging, called filtering information: "Filtering information is constructed by one or multiple components of the AIoT Device Permanent Identifier". An AIoT device implicitly belongs to the group of devices that is intended to be paged if its permanent identifier matches the filtering information. An Editor's Note in the same clause points out that the security is in the remit of SA3.

This contribution argues that there is a privacy issue with filtering information if too many bits of the permanent device identifier are revealed. The more cleartext bits are revealed, the smaller becomes the group of devices that is intended to reply to the paging (or inventory). If the size of the group is k, then the filtering information provides k-anonymity.

The size of the group is not completely determined by the number of cleartext bits that are revealed by the filtering information. Also, which value of k that is acceptable for k-anonymity depends on the assessment of the specific use case and deployment. It is, however, clear that filtering information corresponding to a whole permanent AIoT device id corresponds to paging a single device with cleartext permanent id, which is clearly not in line with the conclusions of TR 33.713 on identifier privacy.

Another aspect to consider is that a person could be carrying multiple AIoT devices belonging to different groups. It should be considered whether the combination of the corresponding k-anonymity sets could compromise a person's privacy by providing a kind of tracking fingerprint specific to that person.

This contribution proposes to capture the reasoning above in TS 33.369 as follows:

* * * First Change * * * *

### 4.2.2	Requirements on the AIOTF

Editor’s Note: This clause contains the security requirement on the AIOTF.

The AIoTF needs to use filtering information, as specified in clause 5.8 of TS 23.369 [2], such that the number of devices identified by each filtering information is sufficiently large to provide sufficient anonymity for the deployment and use case. The decision of the appropriate minimum group size is up to the deployment. Specifically, filtering information corresponding to a whole permanent AIoT device identifier corresponds to paging a single device with cleartext permanent AIoT device identifier. The AIoT device shall not use filtering information corresponding to a whole permanent AIoT device identifier.

* * * Next Change * * * *

## 5.4	Protection of AIoT device identifier privacy

Editor’s Note: This clause contains the security procedures for AIoT device identifier privacy.

### 5.4.x	AIoT device identifier privacy for groups of devices

The requirements on AIoT device identifier privacy are specified in clause 4.2.2.

* * * End of Changes * * * *