3GPP TSG-SA3 Meeting #122	S3-252163

Fukuoka, Japan, 19 - 23 May 2025

Source:	Qualcomm Incorporated

Title:	Addressing ENs in solution 34

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.8.0

Work Item:	FS_AIOT_Sec

Comments

This contribution proposes a few updates to address the Editor’s Notes.

First, this solution mainly addresses how AIoTF and AIoT device(s) enable network layer security when a third-party AF manages the AIoT device credential. This solution proposes that the AF derives the network layer security key(s) from the AIoT credential and provide them to the AIoTF. The AIoTF then provides the key derivation parameters to the AIoT device(s) so that the AIoT device(s) derives the same key as AIoTF. After this network layer security key provisioning, AIoTF and AIoT device(s) can perform authentication, device ID protection, and information protection during AIoT service using the provisioned key as in step 5. As the existing solution details already describe how the network layer security key for network layer authentication and information protection during AIoT service communication is derived and provisioned, this solution addresses the KI#4 and KI#5. Thus, it is proposed to remove the following three Editor’s Notes:

Editor’s Note: How the solution addresses the KI#4 is FFS.

Editor’s Note: how the solution addresses KI#5 is FFS.

Editor’s Note: Why this solution is considered network layer authentication solution without network layer authentication credentials is FFS.

Second, it is proposed to add a clarification that how the AIoT credentials are managed by a third party AF is out of 3GPP scope, following the principle of SNPN in the evaluation, and remove the following Editor’s Note:

Editor’s Note: How AIoT credentials are managed by the third party AF is FFS.

In addition, it is also proposed to add a NOTE clarifying that the AF can be replaced by a SNPN’s credential holder in case AIoT device credentials are stored in the credential holder in SNPN deployment.

Third, it is proposed to move the description of the following Editor’s Note to an evaluation clause.

Editor’s Note: To support Inventory to all potential devices (e.g., the Information about the target AIoT device is not included in the AF request message), each key associated with all potential devices has to be provided by the AF.

Lastly, it is proposed to add an evaluation text clarifying the above aspects.

Proposed Changes

* * * First Change * * * *

## 6.47	Solution #47: A key provisioning for network layer security

### 6.47.1	Introduction

This solution addresses key issues #4 and #5.

This solution provides a key provisioning mechanism to support network layer security for the scenario where long-term AIoT device credentials are managed by the AF. In such scenario, the AF generates a key for the AIoT device and provides it to the AIoTF. Then, Inventory and Command procedure is used to provision the key to the AIoT device. This key is used to protect the Inventory and Command procedures between the AIoTF and AIoT device. The AIoTF can perform the Inventory and Command procedure multiple times using the provided key until a new key is provided.

This solution only applies to SNPN deployment.

Note that it is assumed the AF includes the credential holder that stores AIoT device identifier and the associated credential.

### 6.47.2	Solution details

Figure 6.47.2 -1:  A key provisioning mechanism for network layer security

0.	Each AIoT device is provisioned with its AIoT device identifier and a AIoT device credential. An Application Function (AF) manages the AIoT device identifier and the associated AIoT device credential.

NOTE 1: In this solution, the AF includes the credential holder that stores AIoT device identifier and the associated credential.

The AIoTF is configured with the list of the AIoT device identifiers by the AF as part of AIoT service establishment. AIoTF can also be configured with the AIoT device key (KAIoTF) along with the AIoT device ID. When KAIoTF is provisioned during the service establishment, the key derivation parameter (e.g., freshness parameter) is also sent to the AIoTF, which is then included in the Inventory request message so that the AIoT device can derive the same KAIoTF.

The steps 1a to 1c can be performed whenever AIoTF decides to refresh KAIoTF.

1a.	The AIoTF requests a new KAIoTF to the AF when it does not have a valid key for the AIoT device. The request includes the AIoT device ID and a key derivation parameter.

1b. The AF generates a KAIoTF using the AIoT device credential associated with the AIoT device and the key derivation parameter (e.g., freshness parameter).

1c. The AF provides KAIoTF to the AIoTF.

The steps 2-7 (i.e., inventory and command procedure) is performed to provision the same KAIoTF to the AIoT device.

2.	The AIoTF sends the Inventory Request that contains the key derivation parameter and a freshness parameter1 to Reader(s). The Reader(s) broadcasts the received Inventory Request message.

3.	Upon receipt of the Inventory Request message, the AIoT device generates a KAIoTF based on the AIoT device credential, and the received key derivation parameter.

4.	The AIoT device sends an Inventory Response that contains the AIoT device ID, freshness parameter1 and the freshness parameter2.

NOTE 2: The protection of Inventory Response and/or privacy protection of AIoT device ID is not addressed in this solution.

5.	The AIoTF continues with the Command procedure for key confirmation. The Command request and response messages are protected based on the KAIoTF and freshness parameters.

NOTE 3: The AIoTF can perform the Inventory and Command procedure multiple times using the provided key until a new key is provided.

### 6.47.3	Evaluation

This solution addresses the KI#4 and KI#5 by providing a key provisioning mechanism to support network layer security when a third-party entity manages the AIoT device credential.

To support Inventory to all potential devices (e.g., the Information about the target AIoT device is not included in the AF request message), each key associated with potential AIoT devices has to be provided by the third-party entity.

The AIoT device credentials management by the third-party entity follows the principle of SNPN (i.e., out of 3GPP scope).

* * * End of Changes * * * *