3GPP TSG-SA3 Meeting #122																			S3-252165

Fukuoka, Japan, 19 – 23 May 2025

Source:	Qualcomm Incorporated

Title:	Security procedure on the information protection in command procedure

Document for:	Approval

Agenda item:	4.26

Spec:	3GPP TS 33.369

Version:	0.1.0

Work Item:	AmbientIoT-SEC

Comments

It is proposed to specify a security procedure on the information protection during command procedure in clause 5.3.

Proposed Changes

# * * * * First change * * * *

## 5.3	Protection of information during AIoT service communication

### 5.3.1 General

This clause describes the security protection of the command procedure.

### 5.3.2	Security procedure on information protection during command procedure

Figure 5.3.1-1:  Security procedure on the information protection during command procedure

1.	The AF initiates the command procedure as specified in clause 6.2.3 of TS 23.369 [2].

2.	The step 2 to step 5 of the inventory procedure described in clause 5.4.2 is performed.

3.	The AIoTF shall construct a NAS Command Request and protect the message based on the KAIoTF, the configured confidentiality and integrity algorithms for the AIoT device. The AIoTF shall send the Command Request containing the protected NAS Command Request to NG-RAN.

4.	The NG-RAN shall send a R2D message containing the protected NAS Command Request.

Editor’s Note: the details on the confidentiality and integrity protection of a NAS Command Request/Response is FFS.

5.	The AIoT device shall process the protected NAS Command Request. If the processing is successful, the AIoT device shall construct a NAS Command Response and protect the message based on the KAIoTF, the configured confidentiality and integrity algorithms.

6.	The AIoT device shall send a D2R message containing the protected NAS Command Response to the NG-RAN.

7.	The NG-RAN shall send the Command Response containing the protected NAS Command Response to the AIoTF.

8.	The AIoTF shall process the protected NAS Command Response. If the processing is successful, the AIoTF shall construct an AIoT_Command Notify message.

9.	The AIoTF shall send AIoT_Command Notify to the AF as specified in clause 6.2.3 of TS 23.369 [2].

# * * * * End of change * * * *