# 3GPP TSG-SA3 Meeting #122																			S3-252187

# Fukuoka, Japan, 19 – 23 May 2025

Source:	LG Electronics

Title:	Security mechanism and procedure for inter-CU-LTM

Document for:	Approval

Agenda item:	4.12

Spec:	3GPP TS 33.501

Version:	19.2.0

Work Item:	NR_Mob_Ph4_Sec

Comments

It is proposed to update normative text based to the draftCR for security procedure for inter-CU LTM.

* * * First Change * * * *

6.X.Y	Security mechanism and procedures for inter-CU LTM

Editor’s Notes: This clause contains the security procedure for inter-CU LTM in 5G.

6.X.Y.1	When DC is not configured

For the case where CU is acting as MN and DC is not configured as specified in TS 38.300 [52], some parameters in UE security context are synchronized via LTM candidate preparation phase, in which UE’s 5G security capabilities and UE’s UP security policy are sent by the initial/source gNB to all the candidate gNBs; the selected AS security algorithms and UE’s UP security activation status determined by the candidate gNBs are sent to the UE via the initial/source gNB.

During LTM execution phase, the source gNB includes the NCC used for the derivation of KNG-RAN* in the Cell Switch Command MAC CE to the UE to avoid any de-synchronization issue due to NCC mismatch. The new source gNB needs to provide the new KgNB*(s) to the corresponding candidate gNB via LTM Configuration Update procedure after path switching.

The target gNB shall initiate an intra-gNB-CU handover, if there is mismatch in the UE security context received in the PATH SWITCH REQUEST ACKNOWLEDGE message from the AMF and the UE security context received from the source gNB. The target gNB shall indicate to the UE to change the current KgNB, to reconfigure the DRBs to activate or de-activate the UP integrity/confidentiality as per the received policy from SMF and selected algorithms in the intra-cell handover procedure. Further, the target gNB updates the UE’s 5G security capabilities and/or UE’s UP security policy for the active PDU session to all candidate gNBs.

6.X.Y.2	When DC is configured

In the case where CU is acting as SN and MN is unchanged as specified in TS 37.340 [51], the security mechanism and procedures as specified in clause 6.10.2.4 of this specification for SCPAC shall be applied.

* * * End of Changes * * * *