# 3GPP TSG-SA3 Meeting #122																			S3-252205

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Thales, ORANGE

Title:	General conclusion: network layer

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.8.0

Work Item:	FS_AIOT_Sec

Comments

Following discussions during the previous SA3 meetings, this contribution proposes that the normative work focuses at the network layer that is required. There is no agreement for application layer, or combination of network layer and application layer.

* * * First Change * * * *

## 7.0	General conclusion

The study has reached conclusion on the following aspects, to be taken into account for the normative phase:

1.	For the protection of any new interfaces, CN or backhaul, existing mechanisms (i.e., NDS/IP from TS 33.310 [12] and SBA security from TS 33.501 [5]) are to be reused to the extent possible.

2.	 The protection of Ambient IoT Services between the AIoT device and the network are provided by mechanisms at the network layer.

Editor’s Note: Further conclusions on how to accommodate application layer mechanisms or whether that would be left to deployments, out of scope, etc. are ffs.

3.	The network layer security solution to be specified includes the following security capabilities: authentication (one-way and mutual), protection of the AIoT data (confidentiality, integrity and replay protection) and identifier privacy over the air interface.

NOTE: The exact security capabilities to be supported by a device depend on the functionalities supported by the device. Which device functionality mandates which security capabilities is to be clarified during the normative work. Whether every device supports every possible device functionality or not is out of SA3 scope.

4.	For network layer security, the AIoTF acts as the security termination point on the network side.

5.  A dedicated network for AIoT service shall be needed if the existing authentication framework (e.g., 5G-AKA, EAP-AKA’, other EAP methods for SNPN) is not reused. To secure the operator's core network, security isolation mechanism between the AIoT service domain and operator domain shall be needed (e.g., operators may choose to deploy a security gateway).

NOTE: If multiple domains exist in the deployment of the architecture, the above policy applies.

* * * End of Changes * * * *