3GPP TSG-SA3 Meeting #122																			S3-252211

Fukuoka, Japan, 19 – 23 May 2025

# Source:	Ericsson

# Title:	Issues about user consent for exposure and a proposal to address them

# Document for:	Endorsement

Agenda Item:	4.1.15

# 1	Decision/action requested

This document discusses the need of clarification, in the SA3 specifications, to reflect the understanding that TS 33.501 Annex V user consent mechanism is not for exposure and proposes to do necessary clarifications directly with CRs to the related SA3 specifications.

# 2	References

[1]	3GPP TS 33.501	: "Security architecture and procedures for 5G System"

[2] 	3GPP SP-241934	: "Reply LS on clarifications on consent management"

[3]	3GPP S3-233078	: "Discussion paper on the purpose and requirements of Annex V TS 33.501"

[4]	3GPP TS 33.558	: "Security aspects of enhancement of support for enabling edge applications"

[5]	3GPP TS 23.558	: "Architecture for enabling Edge Applications"

[6]	3GPP S3-252212	: "Clarification of the intended applicability and requirements for Annex V"

# 3	Rationale

SA3 have had several discussions on Annex V of TS 33.501 [1] and user consent for exposure and it is clarified in the LS out (ref. SA LSOut to GSMA OPG (SP-241934 [2]) that Annex V is for 5G Core internal usage. It also needs to be reflected in some of the specifications mentioned in the detailed proposal section.

Annex V (which uses UDM) specified in TS 33.501 [1] is a user consent mechanism for 5GC internal usage of the UE related data (ref. SA LSOut to GSMA OPG (SP-241934 [2]) and is not fit for exposure (i.e., consent for applications) due to the following reasons:

1.	User consent needs to be both per user resource and per AF. According to EU Guidance on consent sect. 3.3.1, it has to include the purpose, the data, to whom the permission is given (ref. S3-233078 [3] sect. 5.3).

2.	UDM's static subscription-based data and data provisioning model is incompatible with user-driven updates to consent for applications. User-driven updates are highly dynamic and frequent in nature (ref. GDPR art.7, where user can change his consent at any time, which does not fit with UDM provisioning and permanent data approach).

3.	Given the number of applications and the required information to store for each expression of consent, consent data and the number of read and write operations will grow considerably.

4.	Application awareness:

a.	UDM would need to be provisioned and maintained to keep up with all possible applications the users could use.

b.	Applications would need to be identified in a standard way to be recognizable across different UDMs.

5.	Separation of the network and exposure/application domains with respect to security:

a.	Application provisioning and related traffic (consent checks, consent updates) put a burden on UDM, which may impact mobile network traffic and UDM availability.

b.	Consent management for exposure solutions must be generation (e.g., 4G/5G/6G) agnostic. For example, Annex V of TS 33.501 is valid only for 5G.

It should be clear in SA3 specs TS 33.501 [1] and TS 33.558 [4] that the Annex V is not used for exposure.

1.	The mechanism specified for EDGEAPP (TS 33.558 [4]) may create a privacy breach because the consent was reduced to single ON/OFF configuration in UDM, ignoring the requirements to comply with the regulations mentioned in item#1 of the list above. In addition, the existing mechanism may not be enough to fulfil the requirements identified for the different user data in TS 23.558 [5] to be subjected to user consent per application.

2.	As stated above, Annex V of TS 33.501 [1] should be updated to clarify that it is for 5GC internal usage of UE related data as also stated in the LS reply to GSMA (SP-241934 [2]). A concrete proposal for TS 33.501 [1] Annex V is submitted in [6].

To make the specifications clear and to prevent any misinterpretation of the standard, the problems described above needs to be addressed.

# 4	Detailed proposal

It is proposed to agree on the CR [6] for clarification in Annex V of TS 33.501 [1] and revisit TS 33.558 [4] Clause 5.1.3 (User consent requirements) to make it clear that Annex V is not used for exposure.