3GPP TSG-SA3 Meeting #122																			S3-252213

Fukuoka, Japan, 19 – 23 May 2025

# Source:	Ericsson

# Title:	Updates to the living CR on finer level authorization

# Document for:	Approval

Agenda Item:	4.22

# 1	Decision/action requested

It is proposed to add the following changes to the living CR on finer level authorization (S3-251845).

# 2	References

[1]	3GPP TR 33.700-22	:	"Study on security aspects of Common API Framework (CAPIF) Phase3"

[2]	S3-251845			:	"Living CR on Finer level authorization"

# 3	Rationale

Based on the key issue #1.3 conclusion in TR 33.700-22 [1], this document proposes content to be included in the living document of finer level of authorization [2].

NOTE: The changes on top of the content of the living CR are done by using the user name "Ericsson" in track change.

Details on the rationale:

- 	purpose is considered in TS 23.222, so it is proposed to add purpose in the token request for client credential flow, and in the authorization request in the authorizatio code flow.

-	Resource is defined in TS 23.222 as "the object or component of the API on which the operations are acted upon", so it is a term used also in stage 2 specifications. "services operations, resources" terminology is also used in TS 33.501 in SBA security. Also, by specifying token parameters in TS 33.122, SA3 is doing stage 3 related job. Thus, it is proposed to add resource term in the token scope parameter definition.

-	To provide flexibility for further stage 3 details to be done by CT3, it is proposed to have a note for the feature level access.

-	The example provided in the scope parameter is not inline with the practice in the ecosystem. The example forces to have a 3GPP customized parsing operation to be done by the AEF. It is proposed to remove the example to give the flexibility to CT3 to define the format of the scope.

# 4	Detailed proposal

Approve the following updates.

*** 1st Change ***

## 6.Y	Authorization for finer level service API access

The authorization function shall support finer level authorization as specified in TS 23.222 [X]. In both RNAA and non-RNAA scenarios, finer level service API access authorization can be used to limit access to specific services and service operations.

To enable finer level service API access, the authorization request or access token request includes the requested service, service operations and resource at the respective granularity. For RNAA, the request may also include the resource owner ID.

If the API invoker is authorized, the CCF responds to the API invoker with an access token including finer level authorization information, or an authorization code.

The token request and token profiles are specified in Annex C for finer level service API access.

*** 2nd Change ***

#### 6.5.3.2	Authorization using oauth client credential flow

If client credential flow is used for authorization of the API invoker by the AEF, the procedures in RFC 6749 [4] shall be followed with the following profile:

-	The access token request message may include the resource owner ID.

-	The access token request message may include purpose of the data processing.

NOTE 1: If the API invoker is on a UE, the CCF obtains its GPSI during authentication.

NOTE 2: The mapping of API Invoker ID and GPSI is left for stage 3.

-	The CCF shall check whether the API invoker is entitled to consume the API and allowed to access the resources of the resource owner, by using authorization information available in the CCF.

-	If the API invoker is on a UE, the CCF shall check that the UE is accessing its own resources. If the API invoker is an AF not on a UE, the check is omitted.

NOTE 2: How to get the authorization from the resource owner and store it in the CCF is out of scope of the present document.

*** 3rd Change ***

#### 6.5.3.3	Authorization using authorization code (optional PKCE) flow

If authorization code flow, optionally with PKCE, is used by the AEF for authorization of the API invoker, the procedures in RFC 6749 [4] and optionally RFC 7636 [11] shall be followed, with the following profile:

-	The authorization token and/or authorization request may include the resource owner ID.

-	The authorization request may include purpose of the data processing.

NOTE 1: If the API invoker is on a UE, the CCF obtains its GPSI during authentication.

NOTE 2: The mapping of API Invoker ID and GPSI is left for stage 3.

-	The resource owner dynamically authorizes the API invoker to access the resource owner's resources as described in RFC 6749 [4] and optionally RFC 7636 [11].

-	If the API invoker is on a UE, the CCF shall check that the UE is accessing its own resources. The access token shall contain the resource owner ID (i.e. GPSI) and the API invoker ID. If the API invoker is an AF not on a UE, the check is omitted.

*** 4th Change ***

## C.2.2	Token claims

The CAPIF ‘Method–3 - TLS with OAuth token’ access token or an access token used in RNAA shall convey the following claims as defined in IETF RFC 7519 [6] and IETF RFC 6749 [4].

Table C.2.2-1: Access token standard claims

| Parameter | Description |
| --- | --- |
| exp | REQUIRED. The expiration time of the access token. Implementers MAY provide for some small leeway, usually no more than a few minutes, to account for clock skew (not to exceed 30 seconds). |
| client_id | REQUIRED. The identifier of the API Invoker making the API request as previously established with the CAPIF Core Function through onboarding. |
| scope | REQUIRED. A string containing a space-delimited list of AEF ID(s), service API name(s), service operation(s), API resource(s). |

NOTE:	Feature level access is left to stage 3 to decide.

The CAPIF OAuth  2.0 access token shall additionally convey the following claim for RNAA.

Table C.2.2-1: Access token customized claims

| Parameter | Description |
| --- | --- |
| resOwnerId | OPTIONAL. Resource owner ID. |

The ‘exp’and ‘scope’ parameters of the access token shall be determined by the CAPIF core function based upon the client_id of the API Invoker provided in the Access Token Request message.

The scope parameter ‘List of Services per AEF’ shall contain a full or partial list of services which the API Invoker is permitted to access at each AEF.

*** 5th Change ***

## C.3.2	Access token request

To obtain an access token, the API Invoker makes a request to the CAPIF Core Function by sending an Access Token Request message with the following parameters using the "application/x-www-form-urlencoded" format, with a character encoding of UTF-8 in the HTTP request entity-body. The access token request parameters are shown in table C.3.2-1.

Table C.3.2-1: Access token request message parameters

| Parameter | Values |
| --- | --- |
| grant_type | REQUIRED. The value shall be set to "client_credentials or “authorization_code”". |
| client_id | REQUIRED. The identifier of the API Invoker making the request. It shall match the value that was assigned to the API Invoker during the onboarding process. |
| client_cred | OPTIONAL. The client credential that was provided to the API Invoker during the onboarding process. |
| Redirect_uri | OPTIONAL. The value shall be identical with the value in authorization request once authorization code grant or PKCE is used. |
| code | OPTIONAL. The authorization code received from the CCF for RNAA once authorization code grant or PKCE is used. |
| code_verifier | OPTIONAL. If the authorization code grant with PKCE flow is selected, the code verifier is used by the CCF to check the code_challenge according to IETF RFC 7636 [11] once PKCE is used. |
| scope | OPTIONAL. A string containing a space-delimited list of AEF ID(s), service API name(s), service operation(s), API resource(s). |

NOTE:	Feature level access is left to stage 3 to decide.

If the token is used for RNAA (see clause 6.5.3), the parameter resOwnerID is used for the resource owner ID.

| resOwnerID | OPTIONAL. Resource owner ID |
| --- | --- |

*** End of Changes ***