3GPP TSG-SA3 Meeting #122																			S3-252218

Fukuoka, Japan, 19 – 23 May 2025

# Source:	Ericsson

# Title:	Updates to the living CR on Revocation

# Document for:	Approval

Agenda Item:	4.22

# 1	Decision/action requested

It is proposed to add the following changes to the living CR on revocation.

# 2	References

[1]	3GPP S3-251894			:	"Living CR on revocation"

# 3	Rationale

The proposed updates to the living CR [1] are shown in track change with "Ericsson" user name. Main motivation is to align with SA6 and leave the resource update part out of the scope of the specification.

# 4	Detailed proposal

Approve the following updates.

* * * First Change * * * *

#### 6.5.3.4	Revocation

The CCF can initiate the Authorization Revocation Request message as defined in clause 8.23.4 of TS 23.222 [3] with additional information to identify the RNAA-related revoked token.

NOTE:  The CCF can receive a revocation request message from the resource owner via the UE, resource owner function, web page etc. All these mechanisms are out of the scope of the present document.

AEF, storing the information about the RNAA-related revoked token, shall check whether the token presented by an API invoker is revoked or not, before responding to the API invoker’s invocation request.

The CCF provided notification message to the API invoker shall include the information to identify the RNAA-related revoked token.

The procedure illustrated in Figure 6.5.3.4-1 and explained below can be used for revoking RNAA-related token in RNAA scenarios.

Pre-conditions:

1.	The API invoker is authenticated and authorized to use the service API.

API invoker

Resource owner function

CAPIF Core function

AEF

0. TLS session establishment.

1. Resource owner authorization revocation request

2.  Identify the RNAA-related revoked token.

3. Revoke API invoker authorization request

4. Invalidate the authorization of API invoker for service API and resource

5-7: Same steps as the steps 4-6 in clause 8.23.4 of  TS 23.222

8. Resource owner authorization revocation response

Figure 6.5.3.4-1: Procedure for revoking resource owner authorization

- CCF and ROF shall establish TLS session over CAPIF-8 reference point as specified in clause 6.11.

- Triggered by the resource owner, the resource owner function sends resource owner authorization revocation request to the CCF. The resource owner authorization revocation information in request message includes the API invoker informationand information related to service API. The request can include Resource Owner ID (e.g., GPSI), the purpose for data processing, service operation information, API service resource information, and resource owner-related information.

Editor’s Note: Whether any additional the resource owner- related information is needed in resource owner authorization revocation information is ffs.

- The CCF uses the information in the resource owner authorization revocation request to identify the RNAA-related token to be revoked.

- The CCF sends the Revoke resource authorization notify message to the AEF as defined in clause 8.35 of TS 23.222 [3] with additional information to identify the RNAA-related revoked token.

Editor’s Note: clarification on additional information used for AEF to identify the RNAA-related token is to be done after clarification in the revocation procedure in SA6.

Editor’s Note: Whether the name of the message sent by CCF to AEF is Revoke API authorization Request or Revoke resource authorization notify is ffs.

- The AEF, storing the resource owner authorization revocation information about the RNAA-related revoked token, checks whether the token presented by an API invoker is revoked or not, before responding to the API invoker’s invocation request.

NOTE:	The need of updating the resource due to the token revocation and how to do it if resource update is needed are not in the scope of the present document.

5-7. Step 5-7 are the same as the steps 4-6 in clause 8.23.4 of TS 23.222 [3].

8.    The CCF sends a resource owner authorization revocation response as an acknowledgement to the ROF that the resource owner authorization has been revoked.

* * * End of Changes * * * *