3GPP TSG-SA3 Meeting #122	S3-252230

# Fukuoka, Japan,  19 - 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2149 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on naming of purposes for user consent |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI |  |  |  |  |  | Date: |  |  | 2025-5-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In clasue 8.5 of TR 33.867, conclusion for KI #5: Unambiguous naming of purposes is made as: “An NF acting as the user consent enforcement point is able to determine the actual purpose for data processing when other NFs request data subject to user consent. NF involved in user consent handling may include a list of purposes when requesting user consent related parameters from the enforcement point. However, this is left to the normative work and to be decided on a use case basis. It is recommended that the purpose of data processing is formatted as a blob. The content of such blob is out of scope. All NFs involved in processing data subject to user consent are assumed to be operated by the same entity and therefore have the same notion of what the purpose (formatted as blob) means. How to map such a blob to human readable format, should there be a need, is left out of scope.” However, since the purpose is determined based on use case, other groups do not standardize the blob content as suggested by SA3, but specify specific purpose names. Nonetheless, these purpose names specified in clause 6.1.6.3.20 of TS 29.503, such as "ANALYTICS," "MODEL_TRAINING," "NW_CAP_EXPOSURE," and "EDGEAPP_UE_LOCATION," are not easily understandable for users and may pose challenges for obtaining user consent. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add guidance on how to specify the purpose in a way that is understandable to users. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of guidance on how to name purposes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex V.1.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

Annex V (normative): 
User consent requirements and mechanisms

# V.1	General

## V.1.1	Scope

User consent can be required for 3GPP features depending on local regulations. Therefore, this annex describes the generic security requirements and procedures to support user consent enforcement in 3GPP services. While the use cases can differ, the annex focuses on the common and generic aspects related to the storage, checking and revocation of the user consent.

The user consent related requirements and mechanism in the present document are applicable only when it is required by regional regulations or operator’s local policy, not otherwise.

The term data processing in this annex is used to convey the same meaning as in [101].

## V.1.2	Relationship between end-users and subscriber

It is assumed that the user consent is obtained from the end-users. The end-user(s) is the subscriber itself or authorize the subscriber to provide consent on behalf of the end-users. Alternatively, the end-users are authorized by the subscriber to provide the consent. That means user consent is always tied to the subscription information. How authorization is provided between the subscriber and the end-users is out-of-scope of this specification.

NOTE: The term end-user is defined in TR 21.905 [1].

## V.1.X	Naming of purposes

The purpose of data processing in the user consent parameters shall be mapped to a human-understandable format when requesting user consent for the purpose from the user. The method for mapping these purposes to human-understandable format is operator’s implementation, and is out of scope.

Purpose naming should be use case specific and designed to be unambiguous, human-understandable after being transformed into user-understandable format.

End of Change