3GPP TSG-SA3 Meeting #122	S3-252254

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2151 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on verification of NFc for discovery in roaming scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MITRE-FFRDC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI-19 |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | **This is same changes as S3-251343-r1 from SA3#121. Additional rationale is provided. During the service discovery procedure in the roaming scenario, it’s not pretty clear who does the verification between query parameter from NFc and NFc Profile. The reason this check should be mandatory is because in the roaming case the target NRF (hNRF) receives the Nnrf_NFDiscovery_Request from the source NRF (as described in TS 23.502 clause 4.17.5). In that case the target NRF trust that the information received from the source NRF is verified. This verification of consumer cannot be done by the target NRF. The reason this check may be optional in non-roaming case, is because the NRF has authenticated the NF service consumer, it has the option to check. In roaming case, if it is optional check, this same level of assurance is not available (i.e., the target NRF cannot verify the consumer information in the discovery request). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that NRF of the NFc will do the verification above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear to the reader on the executor of NFc verification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.3.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************** START of CHANGE****************

#### 13.3.1.3	Authorization of discovery request and error handling

When NRF receives message from unauthenticated NF, NRF shall support error handling, and may send back an error message. The same procedure shall be applied vice versa.

After successful authentication between NRF and NF, the NRF shall decide whether the NF is authorized to perform discovery and registration.

NOTE 1a: 	The NRF verifies that the values of the parameters in the Nnrf_NFDiscovery_Request received from the NF Service Consumer match with the values of those parameters in the profile of the NF Service Consumer stored at the NRF. This verification is implementation specific.

The NRF shall check that the values of the authorization parameters in the NF (Service) Profile of an NF Service Producer allows an NF Service Consumer to discover the NF Service Producer. In the response message, the NRF shall only return information of those NF Service Producer instances that the NF Service Consumer is authorized to discover.

NOTE 2: The authorization parameters in NF Profile are those used by NRF to determine whether a given NF Instance/NF Service Instance can be discovered by an NF Service Consumer.

In the non-roaming scenario, the NRF authorizes the Nnrf_NFDiscovery_Request based on the profile of the expected NF/NF service and the type of the NF Service Consumer, as described in clause 4.17.4 of TS23.502 [8].

In the roaming scenario, the NRF of the NF Service Producer shall authorize the Nnrf_NFDiscovery_Request based on the profile of the expected NF/NF Service, the type of the NF Service Consumer and the serving network ID. If the requesting NF Service Consumer has registered its profile at its NRF, the NRF shall verify that the query parameters in the Nnrf_NFDiscovery_Request from the NF Service Consumer match with the values of those parameters in the profile of the NF Service Consumer stored at the NRF before sending the discovery request to the NRF of the NF Service Producer.

If the NRF finds NF Service Consumer is not allowed to discover the expected NF instances(s) as described in clause 4.17.4 of TS 23.502[8], NRF shall support error handling, and may send back an error message.

NOTE 1: 	Void.

When a NF consumes the Nnrf_NFManagement or the Nnrf_NFDiscovery services provided by the NRF, the usage of the OAuth 2.0 access token for authorization between the NF and the NRF is optional.

**************** END of CHANGE****************