3GPP TSG-SA3 Meeting #122																	              S3‑252269

Fukuoka, Japan, 19 – 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2148 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the intended applicability and requirements for Annex V |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA3 has answered several LS (e.g. S3-211338, S3-234267, S3-242374, S3-245363, S3-244306) on the intended usage of the user consent framework stating that: - “3GPP user consent framework, Annex V of TS 33.501: User Consent Subscription Data (UcSubscriptionData) is specified in 3GPP TS 29.503 as subscription data stored in the UDR. This data can be retrieved from UDR by UDM and from UDM by any other authorized NF (e.g. NWDAF, NEF). Permanent subscription data within this framework can be modified only by provisioning/administration means.” (S3-245363) - The user consent framework as defined in TS 33.501 relates to user consent attributes to be used within the 5G system. - It is suggested that the personal data and processes governed by user consent shall be configured by OAM (S3-211338, S3-234267). Data exposure from 5G core to external parties/entities/AF should be declared to be outside the scope of applicability of Annex V. The reasons are: - the regulatory implications of exposing personal data between data controllers were not studied at the time of specifying Annex V. The consent mechanism acquired a static nature, contrary to legal requirements for consent. There is no understanding of how a subscriber/user can (dynamically) authorize (or decline) personal data exposure to parties/entities/AFs outside the 5G core |  |  |  |  |  |  |  |  |
| Summary of change: |  | The suggested changes provide clarification of the intended applicability of the Annex V user consent framework in TS33.501. It is suggested to further clarify that: - Annex V is subscriber-based consent given to the operator so that the 5G core/RAN can be provisioned/configured based on the operator-subscriber agreed permissions. These permissions are stored in UDM and enable operators 5G System to comply with local laws and regulations. User Consent according to Annex V is considered for use within 5G Core/RAN system. It is proposed that user consent according to Annex V shall be configured by OAM (S3-211338, S3-234267). - User Consent according to Annex V is not inteded to cater to dynamic subscriber permissions to expose her personal data to systems outside 5G Core. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misunderstanding of the applicability of the user consent framework may lead to non-compliance to regulations, with possible privacy breach and inconsistency between different stages of normative work with respect to user consent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex V.1.1, Annex V.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S3-252212 |  |  |  |  |  |  |  |  |

*** Start of Change ***

Annex V (normative): 
User consent requirements and mechanisms

# V.1	General

## V.1.1	Scope

User consent can be required for 3GPP features depending on local regulations. Therefore, this annex

describes the generic security requirements and procedures to support user consent enforcement within 3GPP services.

While the use cases can differ, the annex focuses on the common and generic aspects related to the storage, checking and revocation of the user consent.

The user consent related requirements and mechanism in the present document are applicable only when it is required by regional regulations or operator’s local policy, not otherwise.

This annex relates to user consent data to be used within the 5G Core.

NOTE:	Usage of the annex for other needs is not specified.

The term data processing in this annex is used to convey the same meaning as in [101].

## V.1.2	Relationship between end-users and subscriber

It is assumed that the user consent is obtained from the end-users. The end-user(s) is the subscriber itself or authorize the subscriber to provide consent on behalf of the end-users. Alternatively, the end-users are authorized by the subscriber to provide the consent. That means user consent is always tied to the subscription information. How authorization is provided between the subscriber and the end-users is out-of-scope of this specification.

NOTE: The term end-user is defined in TR 21.905 [1].

# V.2	Requirements

The UDM shall support the following services related to the user consent.

-	Retrieval of user consent parameters.

-	Notification of user consent parameters change.

The user consent parameters shall be stored in the UDM/UDR as subscription data. Configuration of the parameters in the UDM/UDR can be done by OAM or other provisioningmeans.

The user consent parameters shall be bound to a SUPI/GPSI.

The user consent parameters shall be bound to the purpose of data processing.

The user consent parameters shall include whether the user consent is granted or not.

The user consent shall be effective only after the point in time that user consent was given.The user consent shall be effective until revoked. It means that there is no expiry/validity timer for the user consent parameters stored in the subscription data.

NOTE:	UDM does not provide user consent revocation service, it only provides notifications of user consent parameter changes.

*** End of Change ***