# 3GPP TSG-SA3 Meeting #122																			S3-252315

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Xiaomi, ZTE

Title:	Correction on the Authorization of DA services using CAPIF

Document for:	Approval

Agenda item:	4.21

Spec:	3GPP TS 33.434

Version:	18.2.0

Work Item:	Metaverse_Sec

Comments

In the current living document for Metavers_Sec, RNAA specified in TS 33.122 clause 6.5.3 is followed for digital asset services authorization when CAPIF is used. The user (UE subscriber) information contained in the RNAA access token is used by the DA server to authorize a user to access the requested DA service.

Based on the progress on CAPIF_Ph3-Sec, what can be authenticated in RNAA procedure are the API invoker and the resource owner. Hence what are included in the RNAA access token are API invoker ID and owner ID. The access token is used by the exposure function to authorize whether the resource of the owner can be exposed to the API invoker or not.

However, the user behind the API invoker is not considered in CAPIF_Ph3-Sec. It means that there is no defined method for CAPIF RNAA to authenticate the user behind the API invoker. Hence the identifier of the user behind the API invoker is not included in the RNAA access token. With only owner information but no other user information in the access token, the exposure function, i.e. DA server, is only able to authorize the DA service access by the DA owner, but not able to authorize a user different from the DA owner to access the requested DA service.

Therefore, it is proposed to correct the current text in the clause for “Authentication and authorization for digital asset services when CAPIF is used”.

* * * First Change * * * *

6.Y	Authentication and authorization for digital asset services

6.y.1 	Authentication and authorization for digital asset services when CAPIF is used

When CAPIF is used as specified in TS 23.434 [2], the security mechanism for CAPIF specified in TS 33.122 [29] is referred to protect interfaces between digital asset client and digital asset server specified in TS 23.438 [y], between VAL server and digital asset server. The digital asset (DA) requestor (e.g. digital asset client or VAL server) takes the role of API Invoker in CAPIF. Digital asset server takes role of AEF in CAPIF.

Authentication and authorization for RNAA specified in clause 6.5.3 of TS 33.122 [29] shall be followed for the DA server to authorize the DA requestor to access the DA service API allowed by the DA owner. In addition to verifying the access token, the DA server shall further check the access control list in the DA profile.

Figure 6.Y.1-1: Procedure for DA requestor authorization to access DA services with token

1. 	The DA requestor sends the DA service request (e.g. DA create/retrieve/update/delete request) to the DA server. The access token received from the CCF is sent along with the DA service request. The DA owner identity, e.g. GPSI if the DA is owned by a UE subscriber, is included in the access token, as per the procedure in TS 33.122 [29] clause 6.5.3.

2. 	Upon receiving the request with an access token, the DA server validates the integrity of the token using the CCF’s certificate. If validation of the token is successful, the DA server further verifies the DA service request against the authorization claims in the token.

In case the access token does not contain sufficient claims for authorizing the DA service API invocation, the DA server further checks its local DA profile for finer level authorization if needed. For example, if the DA profile contains the information of owner(s) or allowed operation(s) for resource level authorization, the DA server checks if the DA owner in the access token matches the owner(s) in the DA profile, and if the service operation in the access token matches the allowed operation(s) in the DA profile.

3.	If the DA service API invocation is successfully authorized by the DA server, it executes the service logic for the invoked DA service API and returns the DA service response as a result of the DA service API invocation.

NOTE x: The assumption is that the user behind the digital asset client is the owner of the digital asset.

NOTE y: How CAPIF RNAA supports service operation level and resource level authorization is specified in TS 33.122 [29].

* * * End of Change * * * *