# 3GPP TSG-SA3 Meeting #122																				S3-252375

# Fukuoka, Japan, 19 – 23 May 2025														revision of S3-252231

Source:	Thales

Title:	Resolution of ENs in solution #39

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.8.0

Work Item:	FS_AIOT_Sec

Comments

This contribution proposes to address the following editor’s notes in solution #39 of TR 33.713.

- Editor’s Note: It is FFS whether the solution applies to all types of AIoT devices.

The solution applies to all types of devices having the capacity to store credentials and perform cryptographic operations.

- Editor’s Note: Impact to the RAN paging message is FFS.

In case of 5G AKA, one possible solution could be the following: the paging message sent by the AIoT reader to the AIoT device contains AIoT Device Identifier and also RAND, AUTN values provided by the AIoT authentication server of an operator or a third party. Once the Ambient IoT device has verified the received SQN value and computed the RES response as specified for 5G AKA in TS 33.501, the D2R data transmission returns at least AIoT Device Identifier and RES value to the AIoT reader. Consequently, 5G AKA can fit AS procedure between AIoT device an AIoT reader described in RAN 2 conclusions for AIoT Service (confer S3-250005 – SA3#119Adhoc-e).

- It is proposed to add explanation in the clause on solution details and remove the editor’s note.

- Editor’s Note: It is FFS whether the number of device interactions are feasible for AIoT.

5G authentication frameworks were designed to address any type of UEs deployment scenarios. And, as described in 1), 5G AKA can fit AIoT procedure to perform mutual authentication between the AIoT device and the 5G network.

- It is proposed to add text in the evaluation part and delete the editor’s note.

- Editor’s Note: Since all AIoT devices require a subscription, the impact to the UDM is FSS.

The credentials can be owned by an operator or a third party. The authentication server can play the role of AUSF/UDM or AAA-S in case of reuse of existing authentication framework such as 5G AKA.

Part1 information of permanent Ambient IoT Device Identifier, assigned by an operator or by a third party, can be used to locate the authentication server, as described in conclusion on Key Issue #2 of TR 23.700-13.

Therefore, according to the number of AIoT devices, it is possible to allocate several authentication servers that could be located thanks to Part1 information of the permanent Ambient IoT Device Identifier that is used in this solution to perform the authentication procedure.

- It is proposed to add text in the evaluation part and delete the editor’s note.

- Editor’s Note: It is FFS how the existing mechanisms referred to in this solution fit within RAN2 procedures.

The explanations provided in 1) and 4) follow RAN2 and SA2 conclusions.

- It is proposed to delete the editor’s note.

* * * First Change * * * *

## 6.39	Solution #39: reuse of existing authentication frameworks

### 6.39.1	Introduction

This solution addresses Key Issue #5.

This solution applies to topology 1 and topology 2. The solution takes into account the presence of RAN reader and AIoTF.

### 6.39.2	Solution details

The following authentication frameworks specified in TS 33.501 [12] can be used to perform authentication between an AIoT device and AUSF/UDM (or AAA-S): 5G AKA, EAP-AKA’, EAP-TLS or any other key-generating EAP-method.

The choice of the authentication framework depends on the deployment scenario and is compliant with TS 33.501 [12].

In case of 5G AKA, one possible solution could be the following: the paging message sent by the AIoT reader to the AIoT device contains AIoT Device Identifier and also RAND, AUTN values provided by the AIoT authentication server (AUSF/UDM) of an operator or a third party. Once the Ambient IoT device has verified the received SQN value and computed the RES response as specified in TS 33.501 [12], the D2R data transmission returns at least AIoT Device Identifier and RES value to the AIoT reader.

RAN reader and AIoTF do not perform any cryptographic computation of the authentication procedure.

The key hierarchy resulting from the authentication procedure is adapted to AIoT service, e.g. there is no key derived for handover and mobility.

Editor’s Note: It is FFS whether the key hierarchy resulting from the authentication could be further reduced depending on the conclusions of other key issues, e.g. key issue #4 "Protection of information during AIoT service communication".

### 6.39.3	Evaluation

This solution addresses Key Issue #5 by reusing existing authentication frameworks.

The solution applies to all types of devices having the capacity to store credentials and perform cryptographic operations.

5G authentication frameworks were designed to address any type of UEs deployment scenarios. 5G AKA can fit AIoT AS procedure to perform mutual authentication between the AIoT device and the 5G network.

The authentication server can play the role of AUSF/UDM or AAA-S in case of reuse of existing authentication framework such as 5G AKA. Part1 information of permanent Ambient IoT Device Identifier can be used to locate the authentication server for permanent Ambient IoT Device Identifier assigned by an operator or by a third party. Therefore, according to the number of AIoT devices, it is possible to allocate several authentication servers that could be located thanks to Part1 information of the permanent Ambient IoT Device Identifier that is used in this solution to perform the mutual authentication.

NOTE: The evaluation is not complete.