3GPP TSG-SA3 Meeting #122	S3-252383

Fukuoka, Japan 19 – 23 May 2025

Source:	OPPO

Title:	Update AIoT conclusion #4

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.8.0

Work Item:	FS_Ambient_IoT_Sec

Comments

This pCR proposes to update AIoT conclusion #4 in TR 33.713. Since the conclusions on KI#4 are general and sufficient as a guide for carrying out the corresponding TS work, it is proposed to remove the Editor’s Note and add one NOTE to address people’s concerns.

Proposed Changes

* * * Start of Change * * * *

7.4	Conclusion on KI #4: Protection of information during AIoT service communication

The following aspects are agreed for the protection AIoT information carried in the messages during inventory and command procedure:

NOTE: The information considered in KI#3 is not considered in this KI.

-  The AIoTF acts as the network layer security termination point for information protection.

-  The command request and response messages shall be confidentiality, integrity and replay protected based on the pre-shared key and freshness parameter or session keys derived from them.

-  The specified device security capabilities that are needed to support the security features are known/preconfigured in the network side in advance. Therefore, there is no need for security capability negotiation.

* * * End of Change * * * *