3GPP TSG-SA3 Meeting #122																			S3-252414

Fukuoka, Japan, 19 – 23 May 2025

# Source:	KPN

# Title:	Resolving ENs in Solution #42 of TR 33.713

# Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This contribution proposes changes to resolve ENs in solution #42 of TR 33.713.

# 2	References

[1]		3GPP TR 33.713 Study on security aspects of Ambient Internet of Things (AIoT) services in 5G

# 3	Rationale

This contribution provides an update to the solution to address the following Editor’s Notes.

Editor’s Note: Applicability and addressing Nonce exhaustion attack in this solution is FFS

This solution provides a means to update the nonces in the device and in the authentication server independently based on shared secret information between them and thereby addressing nonce exhaustion attack.

Editor’s Note: Applicability and addressing (D)DoS attack in this solution is FFS

The Authentication Server checks if any previously eavesdropped message is replayed by checking the freshness of the nonce in the incoming message even before performing key lookup and decryption, thereby addressing (D)DoS attack.

# 4	Detailed proposal

***** START OF 1st CHANGE *****

## 6.42	Solution #42: Combined authentication and data protection for Ambient IoT services

### 6.42.1	Introduction

This solution addresses key issue #5 on authentication and key issue #4 on protection of information during AIoT service communication. It combines authentication and data transmission into a single message so that low complexity/ low energy devices are not required to perform multiple exchange of messages in order to first authenticate and then to send a message securely. A successful decryption of the message is an indication of authentication of the device.

### 6.42.2	Solution details

The solution makes use of nonces and associated nonce-IDs. The nonce is used to create unique encryption keys which are used to encrypt the device ID of an AIoT device, and the encrypted device ID is used to authentication a message. This prevents that an attacker can forge a valid message. By marking used nonces as used, replay attacks are prevented.

The use of nonce-IDs in addition to nonces facilitates the lookup of nonces in the network and thus eases the computational burden on the network, especially when the number of stored nonces has grown during time.

Figure 6.42.2-1 Procedure for combined authentication and data protection

0.	The device is provisioned with a device ID, shared symmetric key K_AIoT, an initial set of nonces and a set of associated nonce-IDs. For each device, the Authentication Server stores its device ID and provisions it with the associated key K_AIoT, the initial set of nonces and the associated set of nonce-IDs. The initial set of nonces and associated nonce-IDs are the same for the device and for the Authentication Server.

1.	AF to NEF: Request data (device information, [type of data])

Device information contains the information about the devices that needs to provide data to the AF, this could be for instance a device ID or a set of device IDs or a group ID. Optionally, type of data expected from the devices can be part of this request.

The NEF selects an appropriate AIoT function.

2.	NEF to AIoT function: Request data (device information, [type of data]).

The AIoT function selects a Reader capable of interacting with the required device(s).

3.	AIoT function to Reader: Request data (device information, [type of data]).

4.	Reader to AIoT device: Paging ([device information], [type of data]).

The paging message optionally contains device information indicatng the devices that need to be paged.

5.	The AIoT device prepares a message to be sent to the network by performing the following steps:

5a.	The AIoT device selects the next nonce from the set of stored nonces (Nonce1, with associated Nonce1_ID) and derives a symmetric key K_d using K_AIoT and Nonce1 as inputs. If the set of stored nonces is empty, the AIoT device prepares and stores the next set of nonces with associated nonce_IDs as described in step 13.

5b.	The AIoT device marks Nonce1 as used and removes it from the set of stored nonces.

5.c The AIoT device encrypts data that needs to be sent to the AF along with device ID using K_d as the encryption key or compute a keyed hash of the device ID using K_d, resulting in Enc_K_d (data) and Enc_K_d (device ID), respectively.

5d. The AIoT device generates a hash of data, resulting in Hash(data).

6.	AIoT device to Reader: Send_data (device ID, [Enc_K_d(data)], Enc_K_d(device ID), Nonce1, Nonce1_ID, [Hash(data)])

7.	Reader to AIoT function: Send_data (device ID, [Enc_K_d(data)], Enc_K_d(device ID), Nonce1, Nonce1_ID, [Hash(data)])

AIoT function selects AIoT specific Authentication Server holding the K_AIoT associated with the device ID.

8.	AIoT function to Authentication Server: Decrypt_data_request (device ID, [Enc_K_d(data)], Enc_K_d(device ID), Nonce1, Nonce1_ID, [Hash(data)]).

9.	The Authentication Server verifies the received message and (if valid) decrypts the data by performing the following steps:

9a.	The Authentication Server looks up the nonce associated with the received Nonce1_ID in its set of stored nonces.

-	If found and matching with the received Nonce1, the Authentication Server verifies whether the nonce is marked as used.

-	If the nonce is marked as used, the received message is discarded (as being a replay), and the rest of the steps in 9 and subsequent steps 11 to 12 are skipped.

-	If the nonce is not marked as used, continue with the rest of the steps in 9.

-	If found and not matching, the received message is discarded (as being invalid), and the rest of the steps in 9 and subsequent steps 11 to 12 are skipped.

-	If not found, continue with the rest of the steps in 9.

9b.	The Authentication Server obtains K_AIoT based on received device ID.

9c.	The Authentication Server derives K_d using K_AIoT and Nonce1 as inputs.

NOTE 1:	The algorithm used by the Authentication Server to derive K_d is the same as the one used by the AIoT device to derive K_d.

9d.	The Authentication Server decrypts the Enc_K_d(device ID), using K_d, and checks if the decrypted device ID matches the unencrypted device ID received.

-	If the decrypted device ID matches the received unencrypted device ID, the AIoT device is considered to be authenticated, and the received message is considered to be valid. Continue with the rest of the steps in 9 and further.

-	If the decrypted device ID doesn’t match the received unencrypted device ID, the AIoT device is considered to be not authenticated, and the received message is discarded (as being invalid). The rest of the steps in 9 and steps 11 to 12 are skipped.

9e.	If the received nonce has not been found in the set of stored nonces, the Authentication Server prepares the next set of nonces with associated nonce_IDs as described in step 14 and stores them along with the device ID.

9f.	The Authentication Server marks Nonce1 as used.

9g.	The Authentication Server decrypts Enc_K_d(data) if present, using K_d, resulting in (unencrypted) data.

9h.	The Authentication Server computes the hash of the data and checks if it matches the hash sent in the message. If the hash does not match, the received data is discarded.

10.	Authentication Server to AIoT function: Decrypt_data_response (Authentication_result, [data])

Authentication result is Successful if the match in step 9d is successful, else it is Failed.

data contains the decrypted data obtained in step 9g if present and if not discarded in step 9h.

11.	AIoT function to NEF: Send data ([data]).

Message containing data is sent from AIoT function to NEF, if authentication result is Successful and if data is present in the message received from the Authentication Server in step 10. If authentication result is Failed, an appropriate response is sent to the NEF.

12.	NEF to AF: Send data ([data]).

Message containing datais sent from NEF to AF, if authentication result is Successful and if data is present in the message received from the AIoT Function in step 11. If authentication result is Failed, an appropriate response is sent to the AF.

13.	[Optional] The AIoT device prepares the next set of nonces and their associated nonce-IDs and stores it in the device. The new set of nonces is created based on any shared data available in the device, e.g. the K_AIoT and the previous set of nonces, or on the latest nonce_ID of the previous set.

NOTE 2:	The AIoT can perform step 13 at any time, e.g. if it considers its set of stored nonces too low and it has enough energy to create a new set of nonces. This step can also be performed on demand as mentioned in step 5a.

14.	[Optional] The Authentication Server prepares the next set of nonces and their associated nonce-IDs and stores it with the device-ID. The new set of nonces is created using the same algorithm used in step 13 and using the same set of shared data. This implies that the set created by the Authentication Server and by the AIoT device is identical if the same shared data is used.

NOTE 3:	The Authentication Server can perform step 14 at any time, e.g. if it considers its set of stored nonces too low. This step can also be performed on demand as mentioned in step 9e.

### 6.42.3	Evaluation

This solution addresses key issue #5 on authentication and key issue #4 on information protection. The solution is applicable when the amount of data that needs to be sent from device to network/application function is small enough to be embedded in a single message. Use of cleartext device IDs in messages can create a privacy vulnerability, but this depends on the use case. If needed, this vulnerability can be addressed by using some solutions that protect device ID privacy.

The solution does not involve a handshake to perform the authentication, instead it is based on implicit authentication. In this case, only the authenticated device can successfully encrypt the device ID. Similarly, only the authenticated network can decrypt the encrypted device ID thus verify it. Replay protection is achieved by marking used nonces as used. The use of nonce-IDs is not fully evaluated.

The solution remains valid even if the paging message does not contain any device specific information and provides one way authentication.

The solution using only the finite number of initially provisioned nonces is uniquely positioned to work for a single-use or limited-use AIoT devices (e.g., disposable AIoT device designed to answer the inventory page only once or limited number of times). Using the option to create new set of nonces for remediating nonce exhaustion attack adds extra complexity that might or might not be satisfactory for general AIoT use cases. This added complexity creates weakness in this solution.

***** END OF 1st CHANGE *****