3GPP TSG-SA3 Meeting #122	S3-252428

# Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | 0107 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security procedures for CAPIF interconnection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, China Telecom, Nokia, Nokia Shanghai Bell, Ericsson, Xiaomi, Lenovo, ZTE, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3_sec |  |  |  |  |  | Date: |  |  | 2025-02-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This draftCR proposes normative texts for security procedures for CAPIF interconnection based on the conclusions made in TR 33.700-22. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This draftCR will capture the security procedures for CAPIF interconnection |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Security procedure for CAPIF interconnection is not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (new), 6.X.1 (new), 6.X.2(new), 6.X.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is an implementation of draftCR S3-252405 approved in SA3#122. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 6.X	Security procedures for CAPIF interconnection

### 6.X.1	General

The CAPIF provider A and CAPIF provider B host the CAPIF in their trust domains as specified in clause 6.2.2 in TS 23.222 [3]. The designated CAPIF core function of the CAPIF provider A interconnects with the designated CAPIF core function of the CAPIF provider B over CAPIF-6/6e interface.

The following clauses 6.X.2 and 6.X.3 details security aspects of the scenario where, the API invoker is onboarded to CCF-B of the CAPIF provider B and the target AEF is registered to CCF-A of CAPIF provider A.

### 6.X.2	Security method negotiation

For security method negotiation procedure in CAPIF interconnection, clause 6.3.1.2 shall be followed with the following enhancement:

- The API invoker shall send the security method request to the CCF-B.

- In case where CCF-B is in possession of the security method(s) as specified in clause 8.25.3.1 of TS 23.222 [3], CCF-B shall select a security method to be used over CAPIF-2/2e reference point for each AEF based on the access scenarios and AEF capabilities.

- In case where CCF-B is not in possession of the security method(s), based on the AEF details received from the API invoker, CCF-B identifies the CCF-A where the AEF is registered and sends the request to CCF-A to either get the supported list of security method(s) of AEF or to get a selected security method. The request to CCF-A shall include AEF details and may include the API invoker ID and security method supported by API invoker (e.g., to enable CCF-A to select the security method). The CCF-A shall provide to CCF-B either the list of supported security methods of AEF or the selected security method. If the list of supported security methods of AEF is received, the CCF-B shall select a security method to be used over CAPIF-2/2e reference point for each AEF based on the access scenarios and AEF capabilities.

- The CCF-B shall send Security Method Response message to the API invoker indicating the selected security method for each AEF.

### 6.X.3	Authentication and authorization procedure

For the authentication and authorization between the API invoker onboarded to CCF-B in CAPIF provider B and AEF registered to the CCF-A in CAPIF provider A, the procedures as defined in clause 6.5.2 shall be followed with the enhancements as specified in this clause.

NOTE:	It is assumed authorization information is available at the CCF-A.

#### 6.X.3.1	Method 1: Using TLS-PSK

- The API invoker in CAPIF provider domain B shall include the API invoker ID in the authentication initiation request message sent to the target AEF in CAPIF provider domain A for CAPIF interconnection.

- Based on the AEF details available at the API invoker, which indicates the AEF belongs to CCF-A, the authentication initiation request message shall include the information for identification of the CCF-B.

- The AEF in CAPIF provider domain shall request for security information (AEFPSK) from CCF-A to perform authentication and secure connection establishment with the API invoker, if the AEF does not have a security information. The request shall include the API invoker ID and the CCF-B ID (if received from the API invoker).

- When CCF-A receives the request message for security information from the AEF, CCF-A fetches the security information related to the chosen security method (TLS-PSK: AEFPSK), based on API invoker ID and CCF-B ID.

- If the CCF-A does not have security information of the API invoker locally available, CCF-A shall request the security information from CCF-B over CAPIF-6/6e reference point based on the received API invoker ID, and the available AEF details (including the service API interface information). The CCF-B shall provide the TLS-PSK related security information (AEFPSK) to CCF-A.

NOTE: The CCF-A can check the received API Invoker ID and CCF-B ID (if any) based on the information locally available, to contact the right CCF-B to fetch the security information.

- The CCF-A shall provide the received security information to the AEF.

- After sending the authentication initiation response, API invoker in CAPIF provider domain B and AEF in CAPIF provider domain A establish a TLS connection using the security information obtained.

- After successful authentication of API invoker and AEF, the AEF shall authorize the API invoker’s service API invocation request based on authorization information obtained from CCF-A. If CCF-A does not have sufficient information for authorization, CCF-A shall fetch additional information related to the API invoker from CCF-B based on the business relationship.

#### 6.X.3.2	Method 2: Using PKI

The API invoker onboarded to CCF-B and the AEF registered to CCF-A shall follow the procedure in subclause 6.X.3.1 with the following adaptation to establish dedicated secure session over CAPIF-2e using TLS based on certificate based mutual authentication.

- For fetching the security information related to the chosen security method (TLS-PKI) the CCF-B includes only the API invoker ID.

- The CCF-B shall provide the security information (API invoker's root CA certificate) to the AEF via CCF-A, for allowing the AEF to validate the API invoker's certificate.

#### 6.X.3.3		Method 3: TLS with OAuth Token

- The API invoker shall send the access token request message to the onboarded CCF-B, CCF-B determines that the service API requested is provided by the AEFs in CAPIF provider domain A. In interconnection scenario, the parameter ‘scope’ in Access token request message is required.

- The CCF-B sends the access token request message to the CCF in CAPIF provider domain A with the information as specified in Annex C.3.2. In interconnection scenario, the Onboard_secret is not included in the access token request message to the CCF-A. The CCF-A generates and provides an access token and returns it in an Access Token Response message to the API invoker via CCF-B as specified in clause 6.5.2.3.

- On CAPIF-2e, the API invoker authenticates to the AEF by establishing a TLS session with the AEF as specified in clause 6.X.3.1 or 6.X.3.2.

- With successful authentication to the AEF on CAPIF-2e, the API invoker shall initiate invocation of a 3GPP northbound API with the AEF including the access token in northbound API invocation request as per OAuth 2.0. The AEF verifies the integrity of the access token by verifying the CCF’s signature to validate the access permission for the requested service API as specified in clause 6.5.2.3.

NOTE: Since the API invoker is authenticated by CCF-B, and CCF-A and CCF-B authenticated to each other, the API invoker is implicitly authenticated by CCF-A.

* * * End of Change * * * *