3GPP TSG-SA3 Meeting #123	S3-252709

Goteborg, Sweden, 25 – 29 August 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.519 | CR | 0006 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add additional TLS tests to TC_CP_AUTH_AF_NEF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BSI (DE) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5GA |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test case TC_CP_AUTH_AF_NEF is not complete as it does not take into account all possible methods in which certificate authentication can be avoided by an application function, such as omitted or empty TLS Certificate messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add subtests for verifying that certificate-based authentication is correctly implemented for the NEF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When certificate-based authentication can be avoided by an application function, the NEF may be accessed by an unauthorized application function. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.2.2.1.1	Authentication on application function

Requirement Name: Authentication on application function

Requirement Reference: TS 33.501 [2], clause 5.9.2.3, and clause 12.2

Requirement Description: Mutual authentication between the NEF and Application Function is supported as specified in TS 33.501 [2], clause 5.9.2.3. For authentication between NEF and an Application Function that resides outside the 3GPP operator domain, mutual authentication based on client and server certificates is performed between the NEF and AF using TLS and Certificate based authentication  follows the profiles given in TS 33.210 [7], clause 6.2 as specified in TS 33.501 [2], clause 12.2.

Threat References: TR 33.926 [5], clause I.2.2.1, No authentication on application function

Test Case:

Test Name: TC_CP_AUTH_AF_NEF

Purpose: To verify that the NEF can authenticate application function and establish TLS connection towards the application server with certificate based authentication, and may authenticate application function and establish TLS connection towards the application server with pre-shared key based authentication.

Pre-Condition:

-	The NEF network product shall be connected in emulated/real network environments.

-	In order to establish TLS connections to the NEF network product, the application function shall offer a feature that is supported by the NEF network product, including protocol version and combination of cryptographic algorithms.

-	The application function and the NEF network product shall support certificate based authentication, and may support pre-shared key based authentication.

-	If the NEF network product does not support CAPIF as specified in clause 6.2.5.1 in TS 23.501 [3], the certificates or the pre-shared key shall be provisioned in the NEF network product.

-	If the NEF network product supports CAPIF, the certificates or the pre-shared key shall be provisioned in the CAPIF core function, the CAPIF core function shall be able to select appropriate authentication method as defined in the sub-clause 6.5.2 in TS 33.122 [4].

-	Test cases 3 and 4 is only applicable if the NEF network product supports pre-shared-key-based authentication.

Execution Steps:

NOTE:	All test cases below must be executed for TLS 1.2 and TLS 1.3 by configuring the application function to only use the respective TLS version.

Test case 1: Certificate-based authentication with a correct client certificate:

1.	The tester provisions a valid client certificate on the application function.

2.	The tester triggers the application function to initiate a TLS connection towards the NEF.

3.	During the handshake, the NEF sends a CertificateRequest message as specified in RFC 5246, section 7.3 for TLS 1.2 or RFC 8446, section 2 for TLS 1.3.

4.	The tester triggers the application function to respond with a Certificate message to the NEF that includes the client certificate from step 1.

5.	Complete the TLS handshake.

Test case 2: Certificate-based authentication with an incorrect client certificate:

1.	The tester provisions an invalid client certificate on the application function.

2.	The tester triggers the application function to initiate a TLS connection towards the NEF.

3.	During the handshake, the NEF sends a CertificateRequest message as specified in RFC 5246, section 7.3 for TLS 1.2 or RFC 8446, section 2 for TLS 1.3.

4.	The tester triggers the application function to respond with a Certificate message to the NEF that includes the client certificate from step 1.

5.	Complete the TLS handshake.

Test case 3: Pre-shared-key-based authentication with a valid pre-shared key:

The following steps should be taken when TLS 1.2 is in use:

1.	The tester provisions a valid pre-shared key on the application function.

2.	The tester triggers the application function to initiate a TLS connection towards the NEF by sending a ClientHello message as described in section 2 of RFC 4279.

3.	The NEF sends a ServerKeyExchange message as specified in section 2 of RFC 4279.

4.	The tester triggers the application function to send a ClientKeyExchange message with the pre-shared key from step 1.

5.	Complete the TLS handshake.

The following steps should be taken when TLS 1.3 is in use:

1.	The tester provisions a valid pre-shared key on the application function.

2.	The tester triggers the application function to initiate a TLS connection towards the NEF by sending a ClientHello message with the pre_shared_key extension as described in section 2.2 of RFC 8446. The extension includes the pre-shared key set in step 1.

3.	The NEF sends a ServerHello message with the pre_shared_key extension as specified in section 2 of RFC 8446.

4.	Complete the TLS handshake.

Test case 4: Pre-shared-key-based authentication with an invalid pre-shared key:

The following steps should be taken when TLS 1.2 is in use:

1.	The tester provisions an invalid pre-shared key on the application function.

2.	The tester triggers the application function to initiate a TLS connection towards the NEF by sending a ClientHello message as described in section 2 of RFC 4279.

3.	The NEF sends a ServerKeyExchange message as specified in section 2 of RFC 4279.

4.	The tester triggers the application function to send a ClientKeyExchange message with the pre-shared key from step 1.

5.	Complete the TLS handshake.

The following steps should be taken when TLS 1.3 is in use:

1.	The tester provisions an invalid pre-shared key on the application function.

2.	The tester triggers the application function to initiate a TLS connection towards the NEF by sending a ClientHello message with the pre_shared_key extension as described in section 2.2 of RFC 8446. The extension includes the pre-shared key set in step 1.

3.	The NEF sends a ServerHello message with the pre_shared_key extension as specified in section 2 of RFC 8446.

4.	Complete the TLS handshake.

Test case 5: Certificate-based authentication while the application function does not send the ClientCertificate message:

1. Do not configure a client certificate on the application function.

2. The tester triggers the application function to initiate a TLS connection towards the NEF.

3. During the handshake, the NEF sends a CertificateRequest message as specified in RFC 5246, section 7.3 for TLS 1.2 or RFC 8446, section 2 for TLS 1.3.

4. The tester triggers the application function to continue the handshake without sending a ClientCertificate message. The application function should behave as if no CertificateRequest message was received.

5. Complete the TLS handshake.

Test case 6: Certificate-based authentication while the application function sends an empty ClientCertificate

1. Do not configure a client certificate on the application function.

2. The tester triggers the application function to initiate a TLS connection towards the NEF.

3. During the handshake, the NEF sends a CertificateRequest message as specified in RFC 5246, section 7.3 for TLS 1.2 or RFC 8446, section 2 for TLS 1.3.

4. The tester triggers the application function to continue the handshake by sending an empty ClientCertificate message.

5. Complete the TLS handshake.

Expected Results:

For test case 1 and 3, the application function and NEF successfully complete the TLS handshake and establish a TLS connection.

For test case 2, 4, 5, and 6, the NEF rejects the TLS connection attempt with a handshake_failure fatal alert as defined in the respective TLS specification.

Expected format of evidence:

Evidence suitable for the interface, e.g. evidence can be presented in the form of log messages or a packet trace. A packet trace contains at least the full communication of the TLS handshake sent between the NEF network function and application function on the N33 interface.