3GPP TSG-SA3 Meeting #123	S3-252776

# Goteborg, Sweden, 25 – 29 August 2025

Source:	Huawei, HiSilicon

Title:	New KI on Security protection for Multi-Satellites Store and Forward Operation in Split MME Architecture.

Document for:	Approval

Agenda item:	6.1.10

Spec:	None

Version:	None

Work Item:	None

Comments

This contribution proposes a new key issue on the security protection for Multi-Satellites Store and Forward Operation in Split MME Architecture.

* * * First Change * * * *

2	References

[x1]	3GPP TR 23.700-29: “Study on integration of satellite components in the 5G architecture; Phase 3”.

[x2]	3GPP TS 23.401: “General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access”.

* * * Second Change * * * *

Y.X	Key Issue #X: Security protection for Multi-Satellites Store and Forward (S&F) Operation in Split MME Architecture

Y.X.1	Key issue details

In Store and Forward (S&F) satellite operation as described in 3GPP TR 23.700-29 [x1], the UE-satellite-ground connectivity is intermittent, and no Inter-Satellite Links (ISL) are assumed. Under the split-MME architecture introduced in 3GPP TS 23.401 (Annex O.2) [x2], the MME is divided between an MME-onboard (hosted on each satellite) and an MME-ground (in the core network). In multi-satellite scenarios, a UE may interact with different MME-onboard instances during separate satellite passes.

Y.X.2	Security threats

In a multi-satellite S&F architecture, if NAS counters are maintained independently on each onboard MME, an attacker could exploit the lack of synchronization between satellites to replay NAS messages across different serving satellites. This could allow bypassing replay protection mechanisms.

Y.X.3	Potential security requirements

The 3GPP system shall support means to provide confidentiality, integrity, and anti-replay protection for NAS messages in store and forward satellites operation.

* * * End of Changes * * * *