3GPP TSG-SA3 Meeting #123	S3-252853

# Gothenburg, Sweden, 25 - 29 August 2025

# Source:	Apple

# Title:	Addressing EN on authentication anchor in Authentication procedure

# Document for:	Approval

Agenda Item:	4.1.1

# 1	Decision/action requested

Approve the pCR to TS 33.369

# References

- TR 33.713 v1.0.0

- TS 23.369 v0.2.0

# 3	Rationale

This contribution addresses the EN on authentication anchor. It is proposed ADM should be the security anchor to generate the RAND_n and compares the XRESAIOT and RESAIOT

# 4	Detailed proposal

*** Start of 1st Change ***

## 5.2	Authentication procedure

Editor’s Note: This clause contains the security procedures on the authentication.

### 5.2.1	General

This clause describes the authentication procedure for Ambient IoT devices for both Inventory procedure and Command procedure when authentication is triggered by the network.

Editor’s Note: The alignment with the ID privacy procedure is FFS.

NOTE: KAIOT is the long-term key.

### 5.2.2	Authentication procedure

The authentication procedure is aligned with inventory procedure and command procedure in 6.2.2 and 6.2.3 of TS 23.369[2].

Figure 5.2.1-1: Authentication procedure

0. Step 1-6 of clause 6.2.2 Procedure for Inventory or clause 6.2.3 Procedure for command in TS 23.369 [2] is performed.

1. ADM shall generate RANDAIOT_n.

2. AIOTF shall send inventory request message including RANDAIOT_n to NG-RAN.

Editor’s Note: The inclusion of RANDAIOT_n in Paging Request and the size of RANDAIOT_n needs RAN confirmation.

3. NG-RAN shall send the paging request message including RANDAIOT_n to the AIoT device.

Editor’s Note: Whether replay attack is possible is FFS.

4. Upon receiving the paging request message, AIoT device shall derive RESAIOT using KAIoT and RANDAIOT_n for network authenticating AIoT Device.

Editor’s Note: How RESAIOT is derived and whether it is derived from KAIoT or intermediate key is FFS.

Editor’s Note: Where the authentication credentials are processed in AIOT device is FFS.

5. AIoT device sends D2R message to the NG-RAN, including RESAIOT and RANDAIOT_d from device.

Editor’s Note: The security requirements of generating RANDAIOT_d are FFS.

Editor’s Note: Whether RANDAIOT_d is required for inventory procedure is FFS.

6.  NG-RAN sends Inventory report message to AIOTF, including the RESAIOT and RANDAIOT_d.

7. AIOTF sends device identifier, RESAIOTand RANDAIOT_d to ADM.

Editor’s note: The impact of interaction between AIOTF and ADM is FFS. If the authentication is expected to be run more often than normal UE, (e.g., during each inventory procedure), the analysis of load of ADM is FFS.

8. ADM derives XRESAIOT using the same method as in AIoT device, and compares with XRESAIOT

Editor’s Note: Where the authentication credential is processed in AIOT device is FFS.

9. ADM sends authentication result to AIOTF.

10. If the verification is successful, the steps 12-14 in clause 6.2.2 for inventory procedure or the step 8-11of clause 6.2.3 for command procedure in TS 23.369 [2] continues.

Editor’s note: How and where to derive keys is FFS.

Editor’s note: How to perform the mutual authentication for command procedure will be specified.

*** End of 2nd Change ***