3GPP TSG-SA3 Meeting #123	S3-252913

Goteborg, Sweden, 25 – 29 August 2025

# Source:	IIT Bombay

# Title:	pCR to TS 33.502 - Security related Events for Reconnaissance

# Document for:	Approval

Agenda Item:	5.1.1

# 1	Decision/action requested

It is proposed to approve this pCR for incorporation into the draft TS 33.502

# 2	References

[1]	3GPP TS 33.502 v0.0.1

# 3	Rationale

Based on approved WID (SP-250876), the working task needs to specify the general requirements for security events handling and collection.

In WT2, it will specify the events that need to be reported including the following.

- 	The NF collects information on the SBA layer about malformed messages it receives that deviate from the 3GPP specified messages or are considered invalid according to the protocol specification and network state.

- 	The NF collects information about events involving receiving a massive number of incoming messages on the SBA layer.

- 	The NF collects information about failed authentication and authorization attempts from inbound connections on the SBA layer.

- 	The NF collects information about potential replay attacks on the SBA layer.

- 	The NF collects information about potential abnormal SBI call flows as defined for the communication models in Annex E of TS 23.501.

It is proposed to add security requirements for configuration of security related events.

# 6	Security related Events

Editor’s Note: This clause addresses the list and description of the events as well as naming convention for the events.

* * * First Change * * * *

## 6.x	Security events related to Reconnaissance

To ensure the integrity of the 5G Service-Based Architecture (SBA), it is important to monitor both successfully established and failed TLS connections between Network Functions (NFs).

- Successful TLS connections without subsequent API use may indicate probing behavior by a compromised NF, attempting to map out network topology or gather certificates from other NFs.

- Failed or partially established TLS connections may signal malicious attempts to initiate unauthorized sessions, certificate misuse, or reconnaissance activities

A malicious NF can establish TLS connections to multiple peers, collect their certificates, and later exploit this information for targeted attacks. If such events are not monitored or logged, operators lose the ability to identify early indicators of NF compromise. Moreover, the absence of trustworthy TLS session data reduces the accuracy of security event analysis and hinders timely detection of malicious attacks.

Monitoring both successful and failed TLS session attempts provides operators with threat intelligence to strengthen SBA security.

For each TLS connection attempt, whether successful or unsuccessful, the following information should be captured when available:

- Source IP address of the initiating NF.

- TLS certificate details of the NF consumer (issuer, subject, and validity information).

- Timestamp of the handshake attempt or session establishment and duration for which the connection was active.

- API usage details:

- APIs invoked over the TLS session (if any), and point in the TLS establishment procedure it was terminated or

- If TLS connection failed or if it was successful but no APIs were invoked, collect the duration of the TLS session.

- Connection outcome classification:

- Fully established and used (API calls made)

- Fully established but unused (no API calls)

- Partially established or aborted (e.g., handshake terminated or empty certificate message).

The following identifiers can be used to standardize logs associated with reconnaissance-related TLS events:

- NF_TLS_RECON_SUCCESS – TLS session established successfully, no APIs invoked.

- NF-_TLS_RECON_FAIL – TLS handshake aborted or incomplete.

- NF_TLS_RECON_USED – TLS session established and APIs invoked

Reconnaissance related security events can use the following log template

<NF_TLS_RECON_(SUCCESS|FAIL|USED)>::<CertificateDetails>::<APIsInvoked>::<Duration>::<Timestamp>::<Outccome>

* * * End of Changes * * * *