3GPP TSG-SA3 Meeting #123	S3-252664

# Goteborg, Sweden, 25 – 29 August 2025

Source:	Huawei, HiSilicon

Title:	New key issue of the PSK usage in QUIC or TLS

Document for:	Approval

Agenda item:	6.1.6

Spec:	None

Version:	None

Work Item:	None

Comments

It is proposed to add a new key issue about studying how to use the pre-shared key (PSK) for QUIC or TLS in ATSSS scenario.

* * * First Change * * * *

A.X	Key issue #X: New key issue of the PSK usage in QUIC or TLS

A.X.1	Key issue details

After both parties possess a PSK, how to securely use the PSK for MPQUIC in ATSSS scenario needs to be studied. For example, how to identify the PSK for both parties, whether different MPQUIC connections use the same PSK, etc.

A.X.2	Security threats

If the PSK of both parties cannot be securely identified, a QUIC connection cannot be established; if different MPQUIC connections use the same PSK, different UPFs may obtain the same key, leading to security risks.

A.X.3	Potential security requirements

The 5G system shall be able to securely use the PSK between UE and UPF to establish the MPQUIC connection.

* * * End of Change * * * *