3GPP TSG-SA3 Meeting #123	S3-252671

# Goteborg, Sweden, 25 - 29 August 2025

Source:	Huawei, HiSilicon

Title:	New Key Issue on exposure security for UE data towards OTT servers

Document for:	Approval

Agenda item:	6.1.7

Spec:	3GPP TS/TR <TS/TR number>

Version:	<TS Version>

Work Item:	Study on Security for Core Network Enhanced Support for Artificial Intelligence (AI) / Machine Learning (ML) Phase 2

Comments

As per KI#1 in TR 23.700-04, which aims to investigate and develop architectural enhancements to support UE data collection to meet the requirements for AI/ML for NR air interface with UE-side model training. The work involves training ML models at the UE side, where the OTT server trains the ML model for the UEs. This process requires the transfer of training data from the UE to the 5G core and then to the OTT server

The motivation is to investigate means to perform authentication and secure the communication between 5GC and OTT server before exposure of UE related data as highlighted in KI#1 of TR 23.700-04.

* * * First Change * * * *

## Y.X.  Key Issue #X: Exposure security for UE data towards OTT servers

### Y.X.1	Key issue details

The key issue aims to address the security issues related to exposure of UE related data towards the OTT server as studied in KI#1 of TR 23.700-04 to meet requirements for AI/ML for NR air interface operation with UE-side model training. Specifically focused on the authenticating the OTT server before establishing a communication channel and to secure the underlying transport layer.

### Y.X.2	Security threats

If UE related data is leaked in transit information may be disclosed.

Having no authentication between 5GC component i.e. (NEF) and the OTT servers can lead to unnecessary UE data disclosure.

### Y.X.3	Potential security requirements

The 5GS shall support integrity and confidentiality protection for UE related data exposed towards OTT server.

The 5GS shall support mutual authentication between 5GC component i.e. (NEF) and OTT servers that handles UE related data.

* * * End of Change * * * *