3GPP TSG-SA3 Meeting #123	S3-252691

# Gothenburg, Sweden, 25 - 29 August 2025

Source:	Huawei, HiSilicon

Title:	pCR on Threats on Bidding down attacks during coexistence of PQC and classical algorithms

Document for:	Approval

Agenda item:	5.2.1

Spec:	3GPP TR 33.703

Version:	0.0.0

Work Item:	FS_CryptoPQC

Comments

This pCR presents a potential threat to SUPI privacy in PQC bidding-down attacks.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[x1]	3GPP TR 33.938: "3GPP Cryptographic Inventory".

* * * Second Change * * * *

7.1	Threats

7.1.X	Bidding down attacks during coexistence of PQC and classical algorithms

Mobile networks have to maintain backward compatibility with older generations. In the process of PQC migration, it’s assumed that both UE and networks need to support both classical and quantum-resistant algorithms. During the coexistence, in 3GPP networks, many signalling procedures containing cryptographic inventories listed in [x1] will need to agree on what cipher suites to be used for security and compatibility reason. This shows a vulnerability to bidding-down attacks because a Man-in-the-Middle (MitM) attacker can fool both sides to agree on using a classical rather than PQC algorithm for following interaction.

As an example, for SUCI calculation, an attacker may fool both the UE and the network side to agree on using a classical cryptographic algorithm rather than PQC. If so, the UE will calculate SUCI from its SUPI with a classical encryption algorithm (e.g., ECIES) and send the SUCI information to the network side for authentication, which will be captured by the attacker. After that, the attacker can reveal the SUPI information. 3GPP system has to consider how to prevent such a serious SUPI privacy breach in PQC bidding-down attacks.

* * * End of Changes * * * *