3GPP TSG-SA3 Meeting #123	S3-252741

Goteborg, Sweden, 25 – 29 August 2025

Source:	OPPO

Title:	Update clause 5.3 in TS 33.369

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	AmbientIoT-SEC

Comments

This contribution tries to update clause 5.3 in TS 33.369. The remaining ENs are addressed one by one.

According to RAN2, there is only one command request after every inventory procedure. Hence, the KAIOTF key is fresh and the replay attack and key stream reuse are not considered in R19.

According to RAN2, the paging message is more than 1000 bits and segmentation is needed, Hence, the length of the key stream is not fixed.

The ADM derives the KAIOTF key before it returns the authentication data. The AIoT device derives the KAIOTF key after it receives the command request message. The KAIOTF key is derived based on the KAIOT and the second network generated random number RANDAIOT_n2.

One indication or the selected ciphering algorithm identifier is needed to be carried in command request message, e.g., ‘0’ for NEA0 and ‘1’ for 128-NEA2.

Proposed Changes

* * * Start of Change * * * *

5.3	Protection of information during AIoT service communication

5.3.1	General

This clause describes the security procedures for the information protection in command message. The protection of information is provided as part of the AIoT NAS protocol between AIoT device and AIOTF. The AIOTF acts as the security termination point for AIoT information protection.

5.3.2	Security procedure on information protection during command procedure

Figure 5.3.2-1:  Security procedure on the information protection during command procedure

1.	The command procedure is initiated as specified in step 1-6 of clause 6.2.3 of TS 23.369 [2].

2.	The procedure as described in clause 5.2.2 shall be performed. The AIOTF acquire the KAIOTF key from ADM to be used for command protection. The KAIOTF key is derived using the KAIOT and second network generated RANDAIOT_n2 at ADM during the authentication procedure for Inventory and Command.

3.	The AIoTF shall construct a NAS Command Request and protect the message based on the KAIoTF, the confidentiality and integrity algorithms for the AIoT device. The AIoTF shall send the Command Request containing the protected NAS Command Request and the selected ciphering algorithm identifier, i.e., NEA0 or 128-NEA2, to NG-RAN.

4.	The NG-RAN shall send a R2D message containing the protected NAS Command Request as specified in as specified in TS 38.300 [3] and TS 38.391 [6] .

5.	The device shall derive KAIOTF key using KAIOT and second network generated RANDAIOT_n2, and then verify the integrity of the command message. If the verification of integrity is successful, the AIoT device shall decipher it in case it is confidentiality protected. The AIoT device shall construct a NAS Command Response and protect the message based on the KAIoTF key using the same algorithms.

6.	The AIoT device shall send a D2R message containing the protected NAS Command Response to the NG-RAN as specified in as specified in TS 38.300 [3] and TS 38.391 [6].

7.	The NG-RAN shall forward the Command Response containing the protected NAS Command Response to the AIoTF.

8-9.	The AIoTF shall verify the integrity of the command message. If the verification of integrity is successful, the AIoT device shall decipher it in case it is confidentiality protected. Then, the AIOTF shall continue the procedure as specified in clause 6.2.3 of TS 23.369 [2].

5.3.3	Input parameters to integrity algorithm

The input parameters to the integrity algorithm as described in Annex D.3 in TS 33.501[5] shall be set as follows.

The KEY input is equal to the KAIOTF key.

The DIRECTION bit is set to 0 for uplink and 1 for downlink.

The BEARER is set to all zeros.

The COUNT is set to all zeros.

5.3.4	Input parameters to ciphering algorithm

The input parameters for the ciphering algorithms shall be the same as the ones used for NAS integrity protection as described in clause 5.3.3, with the exception that there is an additional input parameter, namely the length of the key stream to be generated by the ciphering algorithms.

* * * End of Change * * * *