3GPP TSG-SA3 Meeting #123	S3-252823

Goteborg, Sweden, 25 – 29 August 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.210 | CR | 0092 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | JWS profile requirements alignment by use case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some of the existing JWS profile requirements are applicable only for the N32 interface. Therefore, it is unnecessary to make those requirements applicable for all 3GPP entities and functions that support JWS. Additionally, TS 33.501, in Clause 13.3.8.2, specifies that CCA shall include either x5u to refer to a resource for the X.509 public key certificate or certificate chain, or x5c to include the X.509 public key certificate or certificate chain used for signing the CCA. However, JWS profile does not specify currently the use of JOSE header parameters for the CCA. This change aims to enhance the clarity of JWS requirements by consolidating them in a single location and organizing them by use case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Separate generic and N32 interface JWS profile requirements. Introduce JWS profile requirements for CCA by adding x5u and x5c. Minor corrections to keep terminology consistent with RFC 7515. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3, 6.3.3.X (new), 6.3.3.Y (new), 6.3.3.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGE ***

### 6.3.3	JWS profile

#### 6.3.3.X	General

All entities and functions that support JWS according to RFC 7515 [46] shall follow the following restrictions and extensions:

The "jwk" header parameter shall not be supported.

The "alg" header parameter value ES256 (ECDSA using P-256 and SHA-256) shall be supported.

The "none" "alg" header parameter value shall not be supported.

The "kid" header parameter shall be supported.

If the "kid" header parameter is used, the end point shall check the indicated "alg" matches the one specified by the parameters.

#### 6.3.3.Y	N32 profile requirements

End points may establish the expected signing algorithm and associated keys out-of-band (e.g. N32-c) and use the "kid" header parameter to pass a key identifier.

If an end point has established a public key and algorithm out of band (e.g. N32-c) and the "kid" header parameter is not used, then the end point shall check the indicated "alg" header parameter against the established algorithm.

#### 6.3.3.Z	CCA profile requirements

The "x5u" header parameter shall be supported.

The "x5c" header parameter shall be supported.

*** END OF CHANGE ***