3GPP TSG-SA3 Meeting #123	S3-252848

# Goteborg, Sweden, 25 – 29 August 2025

Source:	OPPO

Title:	Authentication Procedure for Inventory Only

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	AmbientIoT-SEC

Comments

- It is proposed to resolve ENs to 5.2 Authentication procedure in TS 33.369.

- Considering the limited power and energy for Device 1, the RES should be generated based on the long-term key KAIoT.

- Since paging message is more than 1000bit in RAN2 AIoT specs, the size of 128-bit RANDAIOT_n is suitable for paging message.

- In SA LS SP-250851, for Rel-19, the AIoT system is defined as private network (isolated network deployment that does not interact with a public network), so the ADM load is not as challenge as in the public network.

- RANDAIOT_d may be redundant for the following reasons. Alternatively, RN 16 could be used as the RANDAIOT_d.

- Since RES is used to authenticate the device, RANDAIOT_n would provide enough challenge, and RANDAIOT_d is not needed for RES derivation.

- There is no indication of whether subsequent command procedure to device in the inventory procedure, generating RANDAIOT_d would be redundant for inventory only case since there is no following command message and no command protection.

- Furthermore, using RANDAIOT_n for RES derivation is align with 5G AKA.

Proposed Changes

* * * First Change * * * *

5.2	Authentication procedure

Editor’s Note: This clause contains the security procedures on the authentication.

## 5.2.Y	Authentication procedure for inventory only

The authentication procedure is aligned with inventory procedure and command procedure in 6.2.2 and 6.2.3 of TS 23.369[2].

Figure 5.2.x: Authentication procedure

0. Step 1-6 of clause 6.2.2 Procedure for Inventory or clause 6.2.3 Procedure for command in TS 23.369 [2] is performed.

1. ADM shall generate RANDAIOT_n.

2. AIOTF shall send inventory request message including RANDAIOT_n of 128-bit to NG-RAN.

3. NG-RAN shall send the paging request message including RANDAIOT_n to the AIoT device.

4. Upon receiving the paging request message, AIoT device shall derive RESAIOT using KAIoT and RANDAIOT_n for network authenticating AIoT Device.

RES = fx KAIoT (RANDAIOT_n, RN 16)；

Editor’s Note: Whether fx is as same as f2 in 33501 is FFS.

5. AIoT device sends D2R message to the NG-RAN, including RESAIOT from device.

6.  NG-RAN sends Inventory report message to AIOTF, including the RESAIOT and RN 16.

7. AIOTF sends device identifier and RN 16 to ADM.

8. ADM derives XRESAIOT using the same method as in AIoT device.

9. ADM sends XRESAIOT to AIOTF.

10. AIOTF verifies RESAIOT. If the verification is successful, the steps 12-14 in clause 6.2.2 for inventory procedure or the step 8-11of clause 6.2.3 for command procedure in TS 23.369 continues.

Editor’s note: How and where to derive keys is FFS.

Editor’s note: How to perform the mutual authentication for command procedure will be specified.

* * * End of Change * * * *