3GPP TSG-SA3 Meeting #123	S3-252940

Goteborg, Sweden, 25 – 29 August 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | 0115 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating tables for token claims and access token request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3_sec |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Finer level granular authorization has been specified but it has not been reflected in token and token request profiles in Annex C. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating tables for token claims and access token request |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity in the specification about finer level granularity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex C.2.2, Annex C.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | revision of S3-252876 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## C.2.2	Token claims

The CAPIF ‘Method–3 - TLS with OAuth token’ access token or an access token used in RNAA shall convey the following claims as defined in IETF RFC 7519 [6] and IETF RFC 6749 [4].

Table C.2.2-1: Access token standard claims

| Parameter | Description |
| --- | --- |
| exp | REQUIRED. The expiration time of the access token. Implementers MAY provide for some small leeway, usually no more than a few minutes, to account for clock skew (not to exceed 30 seconds). |
| client_id | REQUIRED. The identifier of the API Invoker making the API request as previously established with the CAPIF Core Function through onboarding. |
| scope | REQUIRED. A string comprising a List of Services per AEF with optionally additional information as described in clause 6.12. |

The CAPIF OAuth  2.0 access token shall additionally convey the following claim for RNAA.

Table C.2.2-1: Access token customized claims

| Parameter | Description |
| --- | --- |
| resOwnerId | OPTIONAL. Resource owner ID. |

The ‘exp’and ‘scope’ parameters of the access token shall be determined by the CAPIF core function based upon the client_id of the API Invoker provided in the Access Token Request message.

The scope parameter ‘List of Services per AEF’ shall contain a full or partial list of services which the API Invoker is permitted to access at each AEF.

* * * Next Change * * * *

## C.3.2	Access token request

To obtain an access token, the API Invoker makes a request to the CAPIF Core Function by sending an Access Token Request message with the following parameters using the "application/x-www-form-urlencoded" format, with a character encoding of UTF-8 in the HTTP request entity-body. The access token request parameters are shown in table C.3.2-1.

Table C.3.2-1: Access token request message parameters

| Parameter | Values |
| --- | --- |
| grant_type | REQUIRED. The value shall be set to "client_credentials or “authorization_code”". |
| client_id | REQUIRED. The identifier of the API Invoker making the request. It shall match the value that was assigned to the API Invoker during the onboarding process. |
| client_cred | OPTIONAL. The client credential that was provided to the API Invoker during the onboarding process. |
| Redirect_uri | OPTIONAL. The value shall be identical with the value in authorization request once authorization code grant or PKCE is used. |
| code | OPTIONAL. The authorization code received from the CCF for RNAA once authorization code grant or PKCE is used. |
| code_verifier | OPTIONAL. If the authorization code grant with PKCE flow is selected, the code verifier is used by the CCF to check the code_challenge according to IETF RFC 7636 [11] once PKCE is used. |
| scope | OPTIONAL. As defined in IETF RFC 6749 [4], a string comprising a List of Services per AEF with optionally additional information as described in clause 6.12. |

If the token is used for RNAA (see clause 6.5.3), the parameter resOwnerID is used for the resource owner ID.

| resOwnerID | OPTIONAL. Resource owner ID |
| --- | --- |

* * * End of Changes * * * *