3GPP TSG SA WG3 Security — SA3#65	S3-111118

7-11 November 2011, San Diego, US

Source:	Radisys

Title:	iFire Candidate Solution

Document for:	Discussion and decision

Agenda Item:	8.7 Study on IMS Firewall Traversal

Work Item / Release:	iFire / Rel-11

# 1	Introduction

It is accepted that some firewalls may be highly restrictive to prevent UDP based traffic from traversing a firewall and that many candidate solutions will in turn use TCP and/or other transports to traverse firewalls.  Equally well there are many firewalls that will continue to support IKE/IPsec and pass traffic without difficulty. IKE/IPsec have enjoyed more than a decade of operation to support client-based corporate VPN access and within 3GPP, have enjoyed support across GAN/UMA, 3GPP system interworking to Wireless Local Area Network (WLAN) interworking, as well non-3GPP IP access (as in 23.402) and additionally as Network Domain Security.

This proposal in summary illustrates a TCP encapsulation technique for IKE/IPsec and procedures that allow for the re-use of IKE/IPsec.  It is known both existing commercial and opensource implementations (such as OpenVPN) employ TCP encapsulation as an alternative method for IKE/IPsec firewall traversal.  Such a reuse of IKE/IPsec would offer many advantages, principally re-use of existing technology and well-understood procedures, and provide a means for general IP tunnelling to be supported.  Its key disadvantage is the additional overhead for the encapsulation.

# 2	Proposal

The following text is proposed for inclusion in the TR.

# 3	PCR

## X     Candidate Solution—Reuse of IKE/IPsec

## X.1  Background

Re-use of IKE/IPsec is given consideration due to IKE/IPsec have enjoyed more than a decade of operation to support client-based corporate VPN access.  Within 3GPP, IKE/IPsec have enjoyed support across GAN/UMA, 3GPP system to Wireless Local Area Network (WLAN) interworking, and as well non-3GPP IP access (as in 23.402).  Many firewalls handle IKE/IPsec without difficulty; however, it is recognized that very restrictive firewalls (such as those that permit TCP traffic only) may block IKE/IPsec.  The reuse of IKE/IPsec procedures as well as proposals which address IKE/IPsec’s inability to traverse firewalls that permit TCP traffic only merit consideration.

## X.2  eSEG-enhanced Security Gateway-Candidate Solution

This candidate solution is based upon enhancing Security Gateway (SEG) operations which are modified to address IKE/IPsec deficiencies with respect to UDP transport and as well permit reuse of IKE/IPsec where the firewall allows such to operate.  This enhancement of existing SEG functions is termed eSEG.

A new element termed an enhanced SEG (eSEG) is introduced to support IP tunnelling of existing IMS services within a TCP encapsulation designed to carry IKE and IPsec through restrictive firewalls.

The following diagram illustrates the eSEG in relation to UE, access, and IMS core.  A Tunnelling Client (TC) handles the establishment of IKE/IPsec over TCP using a TPKT-like (TPKT’) framing.  IKE/IPsec ESP tunnel mode packets that would have been framed over UDP per RFC3948 are now framed over TPKT’ over TCP.  This framing of IKE/IPsec packets using TPKT’ over TCP is termed TrIKESec (Tcp transport for IKE & IPsec).

Figure X: Deployment model for eSEG

The figure below illustrates transport for SIP, RTP, and other applications following the above method.

Figure Y: SIP, RTP, & other applications transport

Should it not be desirable for SIP (control plane) and bearer (e.g., RTP) to share the same authentication, integrity, and/or confidentiality measures, multiple IPsec SA may be negotiated.

For completeness, IKE carriage follows.

Figure Z: IKE Carriage

### X.2.1 eSEG Packet Format

The following packet formats are used and illustrate the framing for IPsec and IKE packets.

Figure a: IPsec ESP format in TPKT’ encapsulation

Figure b: IKE packet format in TPKT’ encapsulation

IKEv2 features such as key exchange and configuration are preserved.

The TPKT’ header is as a TPKT header per RFC983 with version set to 1.  The TPKT’ header is 4 bytes.

### X.2.2 eSEG Firewall Traversal Procedures

The following procedures are used to support firewall traversal, both for permissive (firewalls that allow for the passage of IKE/IPsec) and restrictive firewalls (firewalls that do not allow for unmodified passage of IKE/IPsec).  If the UE does not support access to IMS services via non-3GPP IP access as in 23.402, 3GPP system to Wireless Local Area Network (WLAN) interworking, or packet switched access via GAN/UMA, the following procedure is followed.

1. The UE invokes the TC.

2. The TC starts a regular (RFC 5996) IKE negotiation assuming UDP encapsulation of ESP.  If the IKE negotiation indicates NAT and firewall traversal is successful, IPsec SA are also established.

If step 2 fails due to failure to establish NAT and firewall traversal, the TC may proceed to step 3.

3. The TC attempts to establish a TCP session towards port 80 on an eSEG.  IKE negotiation proceeds with IKE messages encapsulated by TPKT’ over TCP illustrated in sections X.2 and X.2.1.  Assuming a successful IKE negotiation, IPsec SA would be created and IPsec ESP tunnel mode packets are framed over TPKT’ over TCP.

If step 3 fails due to TCP failing to establish or should IKE or IPsec traffic not be observed, the TC may attempt other methods.  The failure of step 3 may indicate the use of HTTP proxies or other methods which may be interfering with session establishment.

### X.2.3 Packet Overheads and Impact

Assuming the use of IPv4, the average overhead of running IPsec with TPKT’ over TCP per packet will be 89 bytes (20 byte IP header + 20 byte TCP header +4 byte TPKT’ Header + 16 Byte ESP Header + 2Byte ESP Trailer + 12 byte ESP Authentication data + 15 bytes for maximum padding for AES_128).  The TCP and TPKT’ framing adds 16 bytes to UDP encapsulation of ESP.

IKE/IPsec with TPKT’ over TCP has several key properties:

- Application neutrality.  It tunnels any IP based protocol.

- NIMSFW Traversal.  It traverses NIMSFWs as framed by appearing as TCP port 80 traffic.

- Reuse of IKE/IPsec procedures.  IKE/IPsec procedures are reused, there is no need for a new protocol for IP address configuration or other special control measures.

- Allowance for separate security measures to be applied to SIP signalling and RTP via use of multiple IPsec SA, tunnelled traffic need not share a single authentication, integrity, or confidentiality measure.

Reuse of IKE/IPsec of course have the properties of reusing IKE/IPsec.  While it is recognized there are restrictive firewalls that do not pass IKE/IPsec there are many that will without modification.

### X.2.4 Detection of IKE/IPsec with TPKT’ over TCP

TPKT’ framing is readily detectable and contains a 2 byte header followed by a 2 byte packet length header.  A fixed header has the advantage of not requiring any additional state or network data (such as IP addresses of eSEG) to block TrIKESec packets.