3GPP TS 33.107 V12.10.0 (2015-03)
Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

3G security;

Lawful interception architecture and functions

(Release 12)

[image: image1.jpg]




[image: image2.emf] 


The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Keywords

LTE, UMTS, Security, Architecture

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2015, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.


UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

11Foreword

Introduction
11
1
Scope
12
2
References
12
3
Definitions, symbols and abbreviations
14
3.1
Definitions
14
3.2
Abbreviations
14
4
Functional architecture
16
5
Activation, deactivation and interrogation
21
5.0
General
21
5.1
Activation
22
5.1.0
General
22
5.1.1
X1_1-interface
22
5.1.2
X1_2-interface (IRI)
23
5.1.3
X1_3-interface (CC)
24
5.2
Deactivation
25
5.2.0
General
25
5.2.1
X1_1-interface
25
5.2.2
X1_2-interface (IRI)
25
5.2.3
X1_3-interface (CC)
26
5.3
Interrogation
26
5.3.0
General
26
5.3.1
Interrogation of the 3G ICEs
26
5.3.2
Interrogation of Delivery Functions
27
6
Invocation of Lawful Interception for Circuit Switched Services
28
6.0
General
28
6.1
Provision of Intercept CC - Circuit Switched
29
6.2
Provision of CC - Short Message Service
29
6.3
Provision of Intercept Related Information
30
6.3.0
General
30
6.3.1
X2-interface
30
6.3.2
Structure of the events
31
6.3.3
Call Related events
33
6.3.3.1
Call establishment
33
6.3.3.2
Answer
33
6.3.3.3
Supplementary Services
34
6.3.3.4
Handover
34
6.3.3.5
Release
34
6.3.4
Non Call Related events
35
6.3.4.1
SMS
35
6.3.4.2
Location update
35
6.3.4.3
Subscriber Controlled Input (SCI)
35
6.4
Intercept cases for circuit switched supplementary services
36
6.4.1
Interception of Multiparty call
36
6.4.2
Interception for Call Forwarding / Call Deflection / ECT
36
7
Invocation of Lawful Interception for GSN Packet Data services
37
7.0
General
37
7.1
Provision of Intercept Product - Short Message Service
38
7.2
Provision of Intercepted Content of Communications – Packet data GSN services
39
7.2.0
General
39
7.2.1
X3-interface
39
7.3
Provision of Intercept Related Information
40
7.3.0
General
40
7.3.1
X2-interface
40
7.3.2
Structure of the events
41
7.4
Packet Data related events
44
7.4.1
Mobile Station Attach
44
7.4.2
Mobile Station Detach
44
7.4.3
Packet Data PDP context activation
45
7.4.4
Start of interception with PDP context active
45
7.4.5
Packet Data PDP context deactivation
46
7.4.6
RA update
46
7.4.7
SMS
47
7.4.8
Packet Data PDP context modification
47
7.4.9
Serving System
47
7.4.10
Start of interception with mobile station attached
48
7.4.11
Packet Header Information Reporting
48
7.4.11.0
Introduction
48
7.4.11.1
Packet Data Header Report
48
7.4.11.2
Packet Data Summary Report
48
7.5
Void
49
7.6
Interception of the Multimedia Messaging Service (MMS)
50
7A
Invocation of Lawful Interception for Packet Data Multi-media Service
50
7A.1
Provision of content of communications
50
7A.1.A
Decryption for IMS Media Plane Security
50
7A.2
Provision of IRI
51
7A.2.1
Provision of IRI with SIP messaging
51
7A.2.2
Provision of IRI with XCAP messages
51
7A.3
Multi-media events
52
7A.3.0
General
52
7A.3.1
Mid IMS Session Interception
53
7A.3.1.0
General
53
7A.3.1.1
SDES Media Security
53
7A.4
Multi-media Call State Control Service Scenarios
54
7A.5
Push to talk over Cellular (PoC)
54
7A.6
SMS over IMS
54
7A.7
LI for KMS based IMS Media Security
54
7A.7.1
LI Architecture and functions
54
7A.7.2 
Signalling over the Xk interfaces and LI events
55
7A.7.3
Cooperating KMSs
56
7A.7.4
Security
56
7A.7.5
Start of interception for an already established IMS media secured session
56
7A.8
IMS IMEI Interception
57
8
Security
58
8.0
General
58
8.1
Administration security
58
8.2
IRI security
58
8.2.1
Normal operation
58
8.2.2
Communication failure
58
8.3
CC security
59
8.4
Security aspects of Lawful Interception billing
59
8.5
Other security issues
59
8.5.1
Log files
59
8.5.2
Data consistency
59
9
Invocation of Lawful Interception for 3GPP WLAN Interworking Services
59
9.0
General
59
9.1
Provision of Intercept Product - Short Message Service
60
9.2
Provision of Intercepted Content of Communications - 3GPP WLAN Interworking services
60
9.2.0
General
60
9.2.1
X3-interface
61
9.3
Provision of Intercept Related Information
61
9.3.0
General
61
9.3.1
X2-interface
62
9.3.2
3GPP WLAN Interworking LI Events and Event Information
62
9.4
Structure of I-WLAN Events
67
9.4.1
I-WLAN Access Initiation
67
9.4.2
WLAN Access Termination
68
9.4.3
I-WLAN Tunnel Establishment
68
9.4.4
I-WLAN Tunnel Disconnect
69
9.4.5
Start of Intercept with I-WLAN Communication Active
70
9.4.6
Packet Header Information Reporting
71
9.4.6.0
Introduction
71
9.4.6.1
Packet Data Header Report
71
9.4.6.2
Packet Data Summary Report
72
10
Interception of Multimedia Broadcast/MultiCast Service (MBMS)
74
10.0
General
74
10.1
Provision of Content of Communications
74
10.2
Provision of Intercept Related Information
74
10.2.0
General
74
10.2.1
X2-interface
75
10.2.2
MBMS LI Events and Event Information
75
10.3
Structure of MBMS Events
77
10.3.1
Service Joining
77
10.3.2
Service Leaving
77
10.3.3
Start of Interception with Service Active
78
10.3.4
Subscription Activation
78
10.3.5
Subscription Modification
78
10.3.6
Subscription Termination
79
11
IMS Conference Services
80
11.1
Background for IMS Conference Services
80
11.1A
Start of Interception for IMS Conference Services
80
11.2
Provision of Intercepted Content of Communication – IMS Conference Services
80
11.2.0
General
80
11.2.1
X3-interface
81
11.3
Provision of Intercept Related Information for IMS Conference Service
82
11.3.0
General
82
11.3.1
X2-interface
82
11.3.2
IMS Conference Events and Event Information
82
11.3.3
Structure of Conference Events
85
11.3.3.1
Start of Conference
85
11.3.3.2
Party Join
85
11.3.3.3
Party Leave
86
11.3.3.3A
Conference Bearer Modification
86
11.3.3.4
Start of Intercept on an Active Conference
87
11.3.3.5
Conference End
87
11.3.3.6
Creation of Conference
88
11.3.3.7
Update of Conference
88
12
Lawful Interception for Evolved Packet System
90
12.1
LI functional architecture for EPS
90
12.2
Functional requirements for LI in case of E-UTRAN access and GTP based S5/S8.
92
12.2.0
General
92
12.2.1
Provision of Intercept Related Information
92
12.2.1.0
 General
92
12.2.1.1
 X2-interface
93
12.2.1.2
 Structure of the events
93
12.2.2
X3-interface
97
12.2.3
EPS related events
98
12.2.3.1
Attach
98
12.2.3.2
Detach
98
12.2.3.3
Bearer activation
99
12.2.3.4
Bearer deactivation
99
12.2.3.5
Bearer modification
100
12.2.3.6
 Start of interception with active bearer
101
12.2.3.7
Tracking Area Update
101
12.2.3.8
Serving Evolved Packet System
101
12.2.3.9
UE requested PDN connectivity
101
12.2.3.10
UE requested PDN disconnection
102
12.2.3.11
UE requested Bearer Resource Modification
102
12.2.3.12
Void
103
12.2.3.13
Start of interception with E-UTRAN attached UE
103
12.2.3.14
Packet Header Information Reporting
103
12.2.3.14.0
Introduction
103
12.2.3.14.1
Packet Data Header Report
103
12.2.3.14.2
Packet Data Summary Report
104
12.3
Functional requirements for LI in case of E-UTRAN access and PMIP based S5/S8 interfaces
105
12.3.0
General
105
12.3.1
Provision of Intercept Related Information
106
12.3.1.0
General
106
12.3.1.1
X2 interface
106
12.3.1.2
Structure of the events
106
12.3.2
X3-interface
108
12.3.3
LI events for E-UTRAN access with PMIP-based S5 or S8
108
12.3.3.1
Initial E-UTRAN Attach and UE PDN requested connectivity with PMIP-based S5 or S8
108
12.3.3.2
Detach and PDN disconnection for PMIP-based S5/S8
109
12.3.3.3
Start of interception with active tunnel for PMIP based S5/S8
109
12.3.3.4
Dedicated Bearer Procedures for E-UTRAN Access with PMIP-based S5/S8
109
12.3.3.5
PDN-GW initiated PDN-disconnection Procedure
109
12.3.3.6
PMIP Session modification
110
12.3.3.7
Packet Header Information Reporting
110
12.3.3.7.0
Introduction
110
12.3.3.7.1
Packet Data Header Report
110
12.3.3.7.2
Packet Data Summary Report
111
12.4
Functional requirements for LI in case of trusted non-3GPP IP access
112
12.4.0
General
112
12.4.1
Provision of Intercept Related Information
113
12.4.1.0
General
113
12.4.1.1
X2-interface
113
12.4.1.2
 Structure of the events
113
12.4.2
X3-interface
116
12.4.3
LI events for trusted Non-3GPP IP access
117
12.4.3.1
Initial Attach and PDN connection activation with PMIPv6 on S2a
117
12.4.3.2
Initial Attach and PDN connection activation procedures with MIPv4 FACoA on S2a
117
12.4.3.3
Initial Attach and PDN connection activation procedures with DSMIPv6 over S2c
117
12.4.3.4
Detach and PDN disconnection with PMIPv6 on S2a
118
12.4.3.5
Detach and PDN disconnection with MIPv4 FACoA
118
12.4.3.6
Detach and PDN disconnection with DSMIPv6 on S2c
118
12.4.3.7
PDN-GW reallocation upon initial attach on s2c
118
12.4.3.8
PDN GW initiated Resource Allocation Deactivation with S2a PMIP
119
12.4.3.9
PDN GW initiated Resource Allocation Deactivation with S2a MIP v4
119
12.4.3.10
Serving Evolved Packet System
119
12.4.3.11 
Start of interception with active tunnel or bearer
119
12.4.3.12
PMIP session modification
120
12.4.3.13
DSMIP session modification
120
12.4.3.14
Bearer activation
120
12.4.3.15
Bearer deactivation
120
12.4.3.16
 Bearer modification
120
12.4.3.17
Packet Header Information Reporting
121
12.4.3.17.0
Introduction
121
12.4.3.17.1
Packet Data Header Report
121
12.4.3.17.2
Packet Data Summary Report
121
12.5
Functional requirements for LI in case of untrusted non-3GPP IP access
122
12.5.0
Introduction
122
12.5.1
Provision of Intercept Related Information
123
12.5.1.0
General
123
12.5.1.1
X2-interface
123
12.5.1.2
Structure of the events
123
12.5.2
X3-interface
126
12.5.3
LI events for untrusted Non-3GPP IP access
127
12.5.3.1
Initial Attach and PDN connection activation with PMIPv6 on S2b
127
12.5.3.2
Initial attach and PDN connection activation for S2c in untrusted non-3GPP IP access
127
12.5.3.3
UE/ePDG-initiated Detach Procedure and UE Requested PDN disconnection with PMIP
127
12.5.3.4
Detach and PDN Disconnection for S2c in Un-trusted Non-3GPP IP access
128
12.5.3.5
Serving Evolved Packet System
128
12.5.3.6 
Start of interception with active tunnel/bearer
128
12.5.3.7
PDN-GW reallocation upon initial attach on s2c
128
12.5.3.8
PDN GW initiated Resource Allocation Deactivation with S2b PMIP
129
12.5.3.9
PMIP session modification
129
12.5.3.10
DSMIP session modification
129
12.5.3.11
Packet Header Information Reporting
130
12.5.3.11.0
General
130
12.5.3.11.1
Packet Data Header Report
130
12.5.3.11.2
Packet Data Summary Report
130
12.5.3.12
Bearer activation
132
12.5.3.13
Bearer deactivation
132
12.5.3.14
 Bearer modification
132
12.6
Functional requirements for LI in case of Handovers between E-UTRAN and CDMA2000 Accesses.
132
12.7
Functional requirements for LI in case of interworking between SGSN and EPS nodes over S4/S12 interfaces
132
12.8
Functional requirements for LI in case of interworking between SGSN and PDN-GW over Gn/Gp interfaces
132
13
Lawful Interception for 3GPP H(e)NBs
133
13.0
General
133
13.1
Provision of Intercepted Content of Communications for 3GPP H(e)NBs
133
13.2
Provision of Intercept Related Information for 3GPP H(e)NBs
133
13.2.1
X2-interface
133
13.3
 3GPP H(e)NB LI Events and Event Information
133
13.4
UMTS Home Node B (HNB)
134
13.4.0
General
134
13.4.1
Intercepted Content of Communications for 3GPP UMTS HNBs
135
13.4.2
Intercept Related Information
135
13.4.2.0
General
135
13.4.2.1
X2-interface
135
13.4.3
3GPP UMTS HNB LI Events and Event Information
136
13.4.4
Structure of HNB Events
136
13.4.4.1
Target UE Registration to HNB
136
13.4.4.2
Target UE De-Registration from HNB
137
13.4.4.3
Start of Intercept with HNB attached UE
138
13.4.4.4
Target UE HNB Handover
138
13.5
Home enhanced Node B (HeNB)
139
14
Interception of Generic Bootstrapping Architecture (GBA) Secured Communications
139
14.1
Introduction
139
14.2
Provision of Content of Communications
140
14.3
Provision of Intercept Related Information
140
14.3.1
Provision of Intercept Related Information Data Flow
140
14.3.2
X2-interface
141
14.3.3
GBA LI Events and Event Information
141
14.4
Structure of GBA Events
143
14.4.1
Bootstrapping
143
14.4.2
Query from NAF
143
14.4.3
Start of Interception with GBA key
144
15
Invocation of Lawful Interception for IMS-based VoIP
144
15.1
Overview of VoIP Interception
144
15.2
Provision of Content of Communications
144
15.2.1
General Principles of CC Interception
145
15.2.1.1
Intercept Trigger
145
15.2.1.2
X3-Interface
145
15.2.2
VoIP CC Interception
146
15.3
Provision of Intercept Related Information for VoIP
147
16
LI for Group Communications using GCSE
147
16.1
Background
147
16.2
GCSE AS in Operator Network
147
16.2.0
General
147
16.2.1
Provision of Content of Communications
148
16.2.1.0
General
148
16.2.1.1
X3-interface
148
16.2.2
Provision of Intercept Related Information
149
16.2.2.0
General
149
16.2.2.1
X2-interface
149
16.2.2.2
GCSE AS LI Events and Event Information
149
16.2.2.2.0
General
149
16.2.2.2.1
Activation of GCSE Communications Group
151
16.2.2.2.2
Deactivation of GCSE Communications Group
151
16.2.2.2.3
User Added
152
16.2.2.2.4
User Dropped
152
16.2.2.2.5
Start of Intercept with an Active GCSE Communications Group
152
16.2.2.2.6
End of Intercept with an Active GCSE Communications Group
153
16.2.2.2.7
Modification of Target Connection to GCS AS
154
16.3
GCS AS outside Intercepting CSP Network
154
17
Interception for Proximity Services
155
17.1
ProSe Direct Discovery
155
17.1.1
General
155
17.1.2
Provision of Inteception of Call Content
155
17.1.3
Provision of Intercept Related Information
155
17.1.3.1
General
155
17.1.3.2
X2-interface
156
17.1.3.3
ProSe LI Events and Event Information
156
17.1.3.3.1
ProSe LI Events
156
17.1.3.3.2
ProSe LI Event Information
156
17.1.3.3.3
Structure of ProSe Events
157
17.1.3.3.3.1
Discovery Request
157
17.1.3.3.3.2
Match Report
158
17.2
ProSe One To Many Communications – In Network
158
17.2.1
General
158
17.2.2
Provision of Intercept Product – One-To-Many Communications
159
17.2.2.1
General
159
17.2.2.2
X2-interface
159
17.2.2.3
ProSe LI One-To-Many Events and Event Information
160
17.2.2.3.1
Overview of ProSe LI One-To-Many Events
160
17.2.3.3.2
Structure of ProSe LI One-To-Many Event Information
160
17.2.2.3.3
ProSe LI One-To-Many Events
160
Annex A (informative):
Information flows for Lawful Interception invocation of circuit switched services
162
A.1
Mobile originated circuit switched calls
162
A.2
Mobile terminated circuit switched calls
163
A.3
Call hold / call waiting
164
A.4
Multiparty calls
166
A.5
Call forwarding / call deflection
169
A.5.0
General
169
A.5.1
Unconditional call forwarding
169
A.5.2
Call forwarding on not reachable (IMSI detached)
170
A.5.3
Call forwarding on busy (network determined)
171
A.5.4
Call forwarding on not reachable (no response to paging/radio channel failure)
172
A.5.5
Call forwarding on no reply
172
A.5.6
Call forwarding on busy (user determined)/call deflection
173
A.5.7
Call waiting / call forwarding on no reply
174
A.6
Explicit call transfer
177
Annex B (informative):
Information flows for Lawful Interception invocation of GSN Packet Data services
179
B.0
General
179
B.1
Mobile Station Attach
179
B.2
Mobile Initiated Mobile Station Detach
180
B.3
Network initiated Mobile Station Detach
180
B.4
Intra 3G GSN Routing Area Update
181
B.5
Inter 3G GSN Routing Area Update
181
B.6
PDP Context Activation
182
B.7
Start of interception with PDP context active
182
B.8
MS initiated PDP Context Deactivation
183
B.9
Network initiated PDP Context Deactivation
183
B.10
SMS
184
Annex C (informative):
Information flows for the invocation of Lawful Interception for Packet Data with multimedia
186
C.0
General
186
C.1
Multimedia registration
186
C.2
Multimedia Session Establishment and Answer
188
C.3
Multimedia Release
189
C.4
Multimedia with Supplementary Service – Call Forwarding
189
C.5
Multimedia with Supplementary Service – Explicit Call Transfer
189
C.6
Multimedia with Supplementary Service – Subscriber Controlled input
189
Annex D (informative):
Information flows for Lawful Interception invocation at the MGW using H.248
190
D.0
General
190
D.1
Mobile to Mobile call, originating side is target
190
Annex E (Informative) IMS-based VoIP Lawful Interception Call Scenarios
192
E.1
Overview
192
E.2
Background
192
E.3
Originating Call from the Target with CC Interception at the PDN-GW/GGSN
194
E.3.0
General
194
E.3.1
Originating Call from the Target with CC Interception at the MRF
195
E.4
Originating Call from the Target with CC Interception at the IMS-AGW
196
E.5
Terminating Call to the Target with CC Interception at the PDN-GW/GGSN
197
E.5.0
General
197
E.5.1
Terminating Call to the Target with CC Interception at the MRF
198
E.6
Terminating Call to the Target with CC Interception at the IMS-AGW
199
E.7
Intra-CSP Forwarded Call with CC Interception at the PDN-GW/GGSN
200
E.7.0
General
200
E.7.1
Intra-CSP Forwarded Call with CC Interception at the MRF
201
E.8
Intra-CSP Forwarded Call with CC Interception at the IMS-AGW
202
E.9
Inter-CSP Forwarded Call to a CS Domain
203
E.10
Inter-CSP Forwarded Call to an IMS Domain
204
E.11
Originating Call from the Target  with IMS Roaming
205
E.12
Terminating Call to the Target  with IMS Roaming
206
E.13
Intra-CSP Forwarded Call  with IMS Roaming
207
Annex F (Informative) Examples of IMS-based VoIP Lawful Interception Call Flows
209
F.1
General Remarks
209
F.2
Call Originations from Target in Home CSP
209
F.2.0
Introduction
209
F.2.1
Target Originated Call - Target (Party_A) Calls Party_B
210
F.2.2
Target Originated Call – Target (Party_A) dials a Special Number
211
F.3
Call Terminations to Target – Home CSP
211
F.3.0
Introduction
211
F.4
Call Forwarding – Non Roaming
212
F.4.0
Introduction
212
F.4.1
Intra-CSP Call Forwarding Unconditional
213
F.4.2
Intra-CSP Call Forwarding No Answer
214
F.4.3
Inter-CSP Call Forwarding Unconditional
216
F.5
IMS Roaming
217
F.5.0
General
217
F.5.1
Roaming Target Originates a Call
217
F.5.2
Call Termination to a Roaming Target
218
F.6
Interception in Visited CSP
218
F.6.0
General
218
F.6.1
Interception in Visited CSP – Target Originated Call
219
F.6.2
Interception in Visited CSP – Target Terminating Calls.
220
F.6.3
Incoming Call to Roaming Target is forwarded due to Call Forwarding No Answer
221
Annex G (informative):
Change history
222


Foreword

This Technical Specification (TS) has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x
the first digit:

1
presented to TSG for information;

2
presented to TSG for approval;

3
or greater indicates TSG approved document under change control.

y
the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z
the third digit is incremented when editorial only changes have been incorporated in the document.

Introduction

This Technical Specification has been produced by the 3GPP TSG SA to allow for the standardisation in the area of lawful interception of telecommunications. This document describes in general the architecture and functions for lawful interception. Laws of individual nations and regional institutions (e.g. European Union), and sometimes licensing and operating conditions define a need to intercept telecommunications traffic and related information in modern telecommunications systems. It has to be noted that lawful interception shall always be done in accordance with the applicable national or regional laws and technical regulations.

1
Scope

The present document describes the architecture and functional requirements within a Third Generation Mobile Communication System (3GMS) and the Evolved Packet System (EPS).

The specification shows the service requirements from a Law Enforcement point of view only. The aim of this document is to define a 3GMS and EPS interception system that supports a number of regional interception regulations, but these regulations are not repeated here as they vary. Regional interception requirements shall be met in using specific (regional) mediation functions allowing only required information to be transported.

The handover interfaces for Lawful Interception (LI) of Packet-Data Services, Circuit Switched Services, and Multimedia Services within the UMTS network and Evolved Packet System for Stage 3 are described in TS 33.108 [11].

2
References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

· References are either specific (identified by date of publication, edition number, version number, etc.) or non‑specific.

· For a specific reference, subsequent revisions do not apply.

· For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]
Void

[2]
ETSI ES 201 158 (V1.2.1 April 2002): "Lawful Interception; Requirements for network functions".

[3]
ETSI ES 201 671 (V3.1.1 May 2007): "Handover Interface for the lawful interception of telecommunications traffic".

[4]
Void

[5]
Void

[6]
Void

[7]
3GPP TS 33.106: "3G Security; Lawful Interception Requirements".

[8]
ANSI J-STD-025‑A (April 2003): "Lawfully Authorised Electronic Surveillance".

[9]
 VOID

[10]
3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description".

[11]
3GPP TS 33.108: "3G Security; Handover interface for Lawful Interception".

[12]
 Void

[13]
3GPP TS 21.905: "Vocabulary for 3GPP Specifications".

[14]
3GPP TS 23.234: "3GPP system to Wireless Local Area Network (WLAN) Interworking; System Description".

[15]
3GPP TS 23.008: "Organization of subscriber data".

[16]
3GPP TS 29.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; Stage 3".

[17]
3GPP TS 24.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; User Equipment (UE) to network protocols; Stage 3".

[18]
IETF RFC 1122 (October 1989): "Requirements for Internet Hosts -- Communication Layers".

[19]
IETF RFC 1123 (October 1989): "Requirements for Internet Hosts -- Application and Support".

[20]
3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and Functional Description".

[21]
3GPP TS 24.147: "Conferencing Using the IP Multimedia (IM) Core Network (CN) subsystem 3GPP Stage 3".
[22]
3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[23]
3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[24]
3GPP TS 29.273: "Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[25]
3GPP TS 33.328: "IP Multimedia Subsystem (IMS) media plane security".

[26]
3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[27]
Void

[28]
3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[29]
3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[30]
3GPP TS 23.272: " Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2"
[31]
3GPP TS 22.220: " Service Requirements for Home NodeBs and Home eNodeBs".

[32]
3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network  (E-UTRAN); Overall description; Stage 2".

[33]
3GPP TS 25.467: "UTRAN architecture for 3G Home Node B (HNB); Stage 2"

[34]
3GPP TS 33.320: "Security of Home Node B (HNB) / Home evolved Node B (HeNB) ".

[35]
3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)". 

[36]
IETF RFC 3966 (December 2004): "The Tel URILs for Telephone Numbers ". 
[37]
3GPP TS 29.060: "General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface". 

[38]
3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[39]
IETF RFC 791: "Internet Protocol".

[40]
IETF RFC 2460: "Internet Protocol, Version 6 (IPv6) Specification".

[41]
IEFT RFC 3697: "IPv6 Flow Label Specification"
[42]
3GPP TS 29.334: "IMS Application Level Gateway (IMS-ALG) – IMS Access Gateway (IMS-AGW); Iq Interface (Stage 3)"

[43]
3GPP TS 23.228: "IP Multimedia Subsystem; Stage 2"

[44]
3GPP TS 23.203: "Policy Charging and Control Architecture"

 [45]
3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 2"

 [46]
3GPP TS 29.162: "Interworking between IM CN subsystem and IP Networks"

[47]
3GPP TS 29.163:"Interworking between IP Multimedia Core Network (CN) subsystem and Circuit Switched (CS) Networks"

[48]
3GPP TS 23.334: "IP Multimedia Subsystem (IMS) Application Level Gateway (IMS-ALG) – IMS Access Gateway (IMS-AGW) interface: Procedures descriptions"
[49]
3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3"
[50]
3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)"

[51]
3GPP TS 22.468: "Group Communication System Enablers for LTE (GCSE_LTE)"

[52]
3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2"
[53]
3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2"
[54]
3GPP TS 23.303: "Proximity based Services; Stage 2".
[55]
3GPP TS 24.623: "Technical Specification Group Core Network and Terminals; Extensible Markup Language (XML) Configuration Access Protocol (XCAP) over the Ut interface for Manipulating Supplementary Services".

[56]
IETF RFC 4825: "The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".
[57]
3GPP TS 33.303: "Proximity-based Services (ProSe); Security aspects".
[58]
3GPP TS 24.333: "Proximity-services (ProSe) Management Objects (MO)".
[59]
3GPP TS 32.277: "Telecommunication management; Charging management; Proximity-based Services (ProSe) charging".
3
Definitions, symbols and abbreviations

3.1
Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [13] and the following apply.

Application layer: As defined by Internet Engineering Task Force (IETF) in RFC 1123 [19]. 

Closed access mode: H(e)NB provides services only to its associated CSG members.  A H(e)NB configured for closed access broadcasts a CSG Indicator and a specific CSG Identity.
Hybrid access mode: H(e)NB provides services to its associated CSG members and to non-CSG members.  A H(e)NB configured for hybrid access does not broadcast a CSG Indicator but does broadcast a CSG Identity.

IP layer: As defined by Internet Engineering Task Force (IETF) in RFC 1122 [18]

Interception Area: is a subset of the network service area comprised of a set of cells which defines a geographical zone.

Location Dependent Interception: is interception of a target mobile within a network service area that is restricted to one or several Interception Areas (IA). 

Open access mode: H(e)NB operates as a normal NodeB or eNodeB.  A H(e)NB configured for open access does not broadcast either a CSG Indicator or CSG Identity.

Other LI specific definitions are given in TS 33.108 [11].

3.2
Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [13] and the following apply:

3GMS
3rd Generation Mobile Communications System

3G GGSN
3rd Generation Gateway GPRS Support Node

3G GSN

3rd Generation GPRS Support Node (GGSN/SGSN)

3G MSC

3rd Generation Mobile Switching Center

3G SGSN
3rd Generation Serving GPRS Support Node

3G UMSC
3rd Generation Unified Mobile Switching Centre

AAA
Authentication, Authorization, and Accounting

ADMF
Administration Function

AGW
Access Gateway

AN
Access Network

AP
Access Provider

AS
Application Server

BM-SC
Broadcast-Multicast Service Centre

BSF
Bootstrapping Serving Function 

B-TID
Bootstrapping Transaction Identifier 

CC
Content of Communication

CS
Circuit Switched

CSCF
Call Session Control Function 

CSG
Closed Subscriber Group

DF
Delivery Function

DSMIP

Dual Stack Mobile IP

ECT
Explicit Call Transfer

EPC
Evolved Packet Core

ePDG
Evolved PDG

EPS
Evolved Packet System

E-UTRAN
Evolved UTRAN

FTP
File Transfer Protocol

GBA
Generic Bootstrapping Architecture

GGSN
Gateway GPRS Support Node

GPRS
General Packet Radio Service

GSM
Global System for Mobile communications

GSN
GPRS Support Node (SGSN or GGSN)

HA
Home Agent 

HeMS
HeNB Management System

HeNB
Home enhanced NodeB

HeNB GW
HeNB Gateway

H(e)NB
Home and Home enhanced NodeB

HI
Handover Interface 

HMS
HNB Management System

HNB
Home NodeB

HNB GW
HNB Gateway

HRPD
High Rate Packet Data

HSS
Home Subscriber Server

IA
Interception Area

IBCF
Interconnecting Border Control Function

ICEs
Intercepting Control Elements (3G MSC Server, 3G GMSC Server, P-CSCF, S-CSCF, SGSN, GGSN, HLR, AAA Server, PDG, MME, S-GW, PDN-GW, HSS)

IETF
Internet Engineering Task Force

IM-MGW
IMS Media Gateway

IMEI
International Mobile station Equipment Identity

IMS
IP Multimedia Core Network Subsystem

IMS-AGW
IMS Access Gateway

IMSI
International Mobile Subscriber Identity

INEs
Intercepting Network Elements (3G MSC Server, 3G GMSC Server, P-CSCF, S-CSCF, SGSN, GGSN, MGW, HLR, AAA Server, PDG)

IP
Internet Protocol

IRI
Intercept Related Information

I-WLAN
Interworking WLAN (3GPP WLAN interworking subnetwork)

LAN
Local Area Network

LDI
Location Dependent Interception

LEA
Law Enforcement Agency

LEMF
Law Enforcement Monitoring Facility 

LIPA
Local IP Access

LTE
Long Term Evolution

MBMS
Multimedia Broadcast/Multicast Service

MGCF
Media Gateway Control Function

MGW
Media Gateway

ME
Mobile Entity

MIP
Mobile IP

MME
Mobility Management Entity

MN
Mobile Node

MRF
Media Resource Function
MSISDN
Mobile Subscriber ISDN Number

NAF
Network Application Function

NAI
Network Access Identifier

NO
Network Operator

PCRF
Policy and Charging Rules Function

P-CSCF
Proxy CSCF

PDG
Packet Data Gateway

PDN 
Packet Data Network

PDN-GW
PDN Gateway

PMIP
Proxy Mobile IP

PoC
Push to talk over Cellular

PS
Packet Switched

RA
Routing Area

RAI
Routing Area Identity

SAI
Service Area Identity 

S-CSCF
Serving CSCF

SeGW
Security Gateway

SGSN
Serving GPRS Support Node

SIP
Session Initiation Protocol

SMS
Short Message Service

S-GW
Serving Gateway 
SR-VCC
Single Radio Voice Call Continuity
TEL URI
"tel" URI, as defined in RFC 3966  [ 36]

TLS
Transport Layer Security

TrGW
Transit Gateway

UE
User Equipment

UMTS
Universal Mobile Telecommunication System

URI
Universal Resource Identifier

URL
Universal Resource Locator

VoIP
Voice over IP

VoLTE
Voice over LTE

WLAN
Wireless LAN
WAF
WebRTC Authorisation Function

WebRTC
Web Real Time Communications

WIC
WebRTC IMS Client

WWSF
WebRTC Web Server Function

4
Functional architecture

The following figures contain the reference configuration for the lawful interception. The circuit-switched configuration is shown in figure 1a. The packet-switched configuration is shown in figure 1b. Intercept configurations for HLR and IMS are shown in figures 1c and 1d. The WLAN interworking configuration is shown in figure 1e. The intercept configurations for IMS conferencing is shown in figure 1f. The CC intercept configuration for IMS-based VoIP is shown in figure 1g.The various entities and interfaces are described in more detail in the succeeding clauses. The additional intercept configurations for Evolved 3GPP Packet Switching Domain are described in clause 12. 

PS domain of the UMTS system (GSN and Multimedia Packet Data services), 3GPP-WLAN interworking network and Evolved Packet Switching Domain provide UMTS/GSM/EPS customer's mobile equipment (UE) with connectivity service to another end of the communication. Another end of the communication may be a network element (server) or another UE. Therefore, UMTS/EPS system provides IP layer TS 23.008 [15] services. Hence, UMTS/EPS NO/AP is responsible only for IP layer interception of CC data. In addition to CC data, the LI solution for UMTS/EPS offers generation of IRI records from respective control plane (signalling) messages. The IP layer connectivity service is needed to support application layer TS 29.234 [16] service provision to UMTS/GSM/EPS customers. For instance, the following are examples of application layer services: email service; web browsing service; FTP service; audio services  (e.g. VoIP, PoC); other multimedia services (MBMS, video telephony); The majority of the application layer services require addition of respective server functionality to the network. Note that it is not necessary that such application layer SP should be the same commercial entity as the UMTS/EPS AP/NO in question. 
When location information of the target is delivered by an ICE, the MF may need to add the civic address associated with the access network point as known by the CSP. The method used to obtain the civic address will depend on the CSP implementation. (eg by accessing a remote database). National regulations define whether the civic address needs to be provided.
NOTE 1:
For instance in MBMS a BM-SC and especially content providing server may be operated by different commercial entity than UMTS network.


[image: image3.wmf] 

LEMF

 

Mediation

 

Function

 

Delivery

 

Function 2

 

Mediation

 

Function

 

ADMF

 

MGW

 

MSC Server/

 

GMSC Server

 

Mc

 

X2

 

X3

 

X1_2

 

X1_1

 

X1_3

 

HI1

 

HI2

 

Mediation

 

Function

 

Del

ivery

 

Function 3

 

Signaling

 

HI3

 

Mediation

 

Function

 

Delivery

 

Function 3

 

Bearer

 


Figure 1a: Circuit switched intercept configuration


[image: image4.wmf] 

LEMF

 

Mediation

 

Function

 

Delivery

 

Function 2

 

Mediation

 

Function

 

ADMF

 

GSN

 

X2

 

X3

 

X1_2

 

X1_1

 

X1_3

 

HI1

 

HI2

 

Mediation

 

Function

 

Delivery

 

Function 3

 

HI3

 


Figure 1b: Packet Switched Intercept configuration


[image: image5.wmf] 

LEMF

 

Mediation

 

Function

 

Delivery

 

Function 2

 

Mediation

 

Function

 

ADMF

 

HLR

 

X2

 

X1_2

 

X1_1

 

HI1

 

HI2

 


Figure 1c: HLR Intercept configuration


[image: image6.wmf] 

LEMF

 

Mediation

 

Function

 

Delivery

 

Function 2

 

Mediation

 

Function

 

ADMF

 

X2

 

X1_2

 

X1_1

 

HI1

 

HI2

 

P

-

CSCF/

 

S

-

CSCF

 


Figure 1d: IMS-CSCF Intercept configuration


[image: image7.emf]   

HI1  

HI3  

HI2  

X3  

X1_3  

X1_2  

X2  

            LEMF  

      Delivery       Function 3    

          ADMF  

      Delivery       Function 2  

Mediation   Function  

Mediation   Function  

Mediation   Function  

    PDG /WAG  

    AAA  

X1_1  


Figure 1e: WLAN Interworking Intercept configuration


[image: image8.emf]   

HI1  

HI3  

HI2  

X3  

X1_3  

X1_2  

X2  

            LEMF  

      Delivery       Function 3    

          ADMF  

      Delivery       Function 2  

Mediation   Function  

Mediation   Function  

Mediation   Function  

    MRFP  

    AS/ MRFC  

X1_1  


Figure 1f: IMS Conferencing Intercept configuration

[image: image9.emf]LEMF

LEMF

LEMF

ADMF

Mediation

Function

HI3

X1_3

X3

CC Intercept 

Function

CC Interception 

Triggering 

Function

X1_1

HI1

Mediation

Function

Delivery 

Function 3


Figure 1g: VoIP CC Intercept Configuration  

The reference configuration is only a logical representation of the entities involved in lawful interception and does not mandate separate physical entities. 

Regional Mediation Functions, which may be transparent or part of the administration and delivery functions, are used to convert information on the HI1, HI2 and HI3 interfaces in the format described in various national or regional specifications. For example, if ETSI ES 201 671 [3] or ANSI J-STD-025 [8] is used, then the adaptation to HI1, HI2 and HI3 will be as defined in those specifications.

There is one Administration Function (ADMF) in the network. Together with the delivery functions it is used to hide from the 3G ICEs that there might be multiple activations by different Law Enforcement Agencies (LEAs) on the same target. The administration function may be partitioned to ensure separation of the provisioning data from different agencies.

See the remaining clauses of this document for definitions of the X1_1, X1_2, X1_3, X2 and X3 interfaces. These interfaces are specifically defined for LI and are not related to other interfaces/reference points having the same name specified in other 3GPP specifications (such as e.g. X2 interface specified in the e-UTRAN architecture).
Interception at the Gateways is a national option. However, if 3G direct tunnel functionality with the GGSN, as defined in TS 23.060 [10], is used in the network, then the GGSN shall perform the interception of IRI and the content of communications.

In figure 1a DF3 is responsible for two primary functions:

-
Call Control (Signalling) for the Content of Communication (CC); and

-
Bearer Transport for the CC.

HI3 is the interface towards the LEMF. It must be able to handle the signalling and the bearer transport for CC.

In figures 1a, 1b, 1e, 1f and 1g, the HI2 and HI3-interfaces represent the interfaces between the LEA and two delivery functions. The delivery functions are used:

-
to distribute the Intercept Related Information (IRI) to the relevant LEA(s) via HI2 (based on IAs, if defined);

-
to distribute the Content of Communication (CC) to the relevant LEA(s) via HI3 (based on IAs, if defined).

In figures 1c and 1d the HI2 interface represents the interface between the LEA and the delivery function. The delivery function is used to distribute the Intercept Related Information (IRI) to the relevant LEA(s) via HI2.

Figure 1g shows the CC interception configuration for VoIP. The trigger for the CC interception is provided by a SIP signalling node and identified within the figures as CC Interception Triggering Function.  

NOTE 2:
With reference to figure 1c, CC interception does not apply to HLR.

NOTE 3:
For IMS, figure 1d relates to the provision of IRI for SIP messages handled by the CSCF. Interception of CC for this case can be done at the GSN under a separate activation and invocation, according to the architecture in Figure 1b (see also clause 7.A.1). For CC interception of VoIP, see figure 1g.
NOTE 4:
If an operator is required to support "HI1 notification over HI2" [11], the X1_2 interface may carry the information coming from the ADMF to the DF2/MF that will be conveyed to the LEMF.
5
Activation, deactivation and interrogation

5.0
General

Figure 2 is an extraction from the reference intercept configuration shown in figures 1a through to 1e which is relevant for activation, deactivation and interrogation of the lawful interception.


[image: image10.wmf] 

Delivery

 

Function 2

 

 

 

3G ICE

 

ADMF

 

Delivery

 

Function 3

 

X1_2

 

X1_3

 

X1_1

 

HI1

 


Figure 2: Functional model for Lawful Interception activation, deactivation and interrogation

In addition to the typical 3G ICEs functional entities, a new functional entity is introduced - the ADMF - the Lawful Interception administration function. The ADMF:

-
interfaces with all the LEAs that may require interception in the intercepting network;

-
keeps the intercept activities of individual LEAs separate;

-
interfaces to the intercepting network.

Every physical 3G ICE is linked by its own X1_1-interface to the ADMF. Consequently, every single 3G ICE performs interception (activation, deactivation, interrogation as well as invocation) independently from other 3G ICEs. The HI1-interface represents the interface between the requester of the lawful interception and the Lawful administration function; it is included for completeness, but is beyond the scope of standardisation in this document.

For VoIP CC Interception, the CC Interception Triggering Function and the CC Intercept Function are treated as one 3G ICE from a Lawful Interception administration perspective.

The target identities for 3GMS CS and PS interception at the SGSN, GGSN, 3G MSC Server and 3G GMSC Server can be at least one of the following: IMSI, MSISDN or IMEI. 
NOTE 1:
Some communication content during a mobility procedure may not be intercepted when interception is based on MSISDN (only PS interception) or IMEI. The use of the IMSI does not have this limitation. For the availability of the target identities IMSI, MSISDN and IMEI (PS interception), refer to TS 23.060 [10].

The target identities for multi-media at the CSCF can be one or more of the following: SIP URI,  TEL URI, or IMEI. Other identities are not defined in this release. 
The target identities for 3GPP WLAN Interworking interception can be MSISDN, IMSI or NAI. For the availability of the target identities in the I-WLAN nodes (AAA server, PDG, WAG), refer to TS 23.234 [14], TS 23.008 [15], TS 29.234 [16] and TS 24.234 [17].

NOTE 2:
The NAI may be a temporary ID, therefore the use of MSISDN or IMSI is recommended.

NOTE 3:
 Void

The target identities for 3GPP HNB interception can be IMSI, MSISDN, IMEI, or ME Id.  

Use of the HNB ID or the CSG Identity as a target identity is FFS.

In the case of location dependent interception the following network/national options exist:

-
target location versus Interception Areas (IAs) check in the 3G ICEs and Delivery Functions (DFs);

-
target location versus IAs check in the DFs (physical collocation of the DFs to the 3G ICEs may be required by national law);

-
location dependent interception is not applicable to CSCF.

NOTE 4:
The IA is previously defined by a set of cells. From the location of the target this set of cells permits to find the relevant IA.

NOTE 5:
It is not required that the 3G GMSC or the 3G GGSN are used for interception when Location Dependent Interception is invoked and the location of the target is not available.

Editors' note:
Location dependent intercept for the 3G MSC Server is not defined for this release.

The ADMF shall be able to provision P-CSCFs independently from S-CSCFs. If both P-CSCFs and S-CSCFs are administered within the network for intercept, redundant multi-media IRI may be presented to the agency as a result.

5.1
Activation

5.1.0
General

Figures 3, 4 and 5 show the information flow for the activation of Lawful Interception.

5.1.1
X1_1-interface

The messages sent from the ADMF to the 3G ICEs (X1_1-interface) contain the:

-
target identities (MSISDN, IMSI, IMEI, SIP URI or TEL URI, NAI) (see notes 4, 5, 6);

-
information whether the Content of Communication (CC) shall be provided (see note 1);

-
address of Delivery Function 2 (DF2) for the intercept related information (see note 2);

-
address of Delivery Function 3 (DF3) for the intercepted content of communications (see note 3);

-
IA in the case of location dependent interception.

NOTE 1:
As an option, the filtering whether intercept content of communications and/or intercept related information has to be provided can be part of the delivery functions. (Note that intercept content of communications options do not apply at the CSCF, HLR and AAA server). If the option is used, the corresponding information can be omitted on the X1_1-interface, while "information not present" means "intercept content of communications and related information has to be provided" for the ICE. Furthermore the delivery function which is not requested has to be "pseudo-activated", in order to prevent error cases at invocation.

NOTE 2:
As an option, only a single DF2 is used by and known to every 3G ICE. In this case the address of DF2 can be omitted.

NOTE 3:
As an option, only a single DF3 is used by and known to every 3G ICE (except at the CSCFs, HLR and AAA server). In this case the address of DF3 can be omitted.

NOTE 4:
Since the IMEI is not available, interception based on IMEI is not applicable at the 3G Gateway. Moreover, in case the IMEI is not available, interception based on IMEI is not applicable at 3G ICEs.

NOTE 5:
Interception at the CSCFs is based upon either SIP URI,  TEL URI or IMEI. SIP URI and TEL URI as target identities are not supported by the other ICEs. The related CC interception also uses the SIP URI,  TEL URI or IMEI.
NOTE 6:
Interception based on NAI is only applicable at AAA server, PDG, and WAG. As the NAI could be encrypted or based on temporary identity at the PDG and WAG, interception based on the NAI is not applicable in those cases in these nodes.

NOTE 7:
Void

If after activation subsequent Content of Communications (CC) or Intercept Related Information (IRI) has to be activated (or deactivated) an "activation change request" with the same identity of the target is to be sent.


[image: image11.wmf] 

... activation of delivery 

functions ...

 

request for lawful 

interception activation

 

response for lawful 

interception activation

 

ADMF

 

           

 

           

3G ICE

 

lawful interception 

activation

 

lawful interception 

activation ack

 


Figure 3: Information flow on X1_1-interface for Lawful Interception activation

Interception of a target can be activated on request from different LEAs and each LEA may request interception via a different identity. In this case, each target identity on which to intercept will need to be sent via separate activation messages from ADMF to the 3G ICEs on the X1_1-interface. Each activation can be for IRI only, or both CC and IRI.

When several LEAs request activation on the same identity and the ADMF determines that there is an existing activation on the identity, the ADMF may (as an implementation option) send additional activation message(s) to the 3G ICEs. When the activation needs to change from IRI only to CC and IRI an activation change message will be sent to the 3G ICEs.

In the case of a secondary interception activation only the relevant LEAs will get the relevant IRIs.

5.1.2
X1_2-interface (IRI)

For the activation of IRI the message sent from the ADMF to the DF contains:

-
the target identity;

-
the address for delivery of IRI (= LEMF address);

-
which subset of information shall be delivered;

-
a DF2 activation identity, which uniquely identifies the activation for DF2 and is used for further interrogation or deactivation, respectively;

-
the IA in case of location dependent interception;

-
the warrant reference number if required by national option.

If a target is intercepted for several LEAs and/or several identities simultaneously, a single activation of delivery is necessary for each combination of LEA and identity.


[image: image12.wmf]... activation on X1_3 ...

... activation on X1_1

request for lawful

interception activation

response for lawful

interception activation

ADMF

DF 2

lawful interception

activation

lawful interception

activation ack


Figure 4: Information flow on X1_2-interface for Lawful Interception activation

5.1.3
X1_3-interface (CC)

For the activation of intercepted Content of Communications the message sent from the ADMF to the Delivery Function contains:

-
the target identity;

-
the address of delivery for CC (= LEMF address);

-
a DF3 activation identity, which uniquely identifies the activation for DF3 and is used for further interrogation or deactivation, respectively;

-
the IA in case of location dependent interception;

-
the warrant reference number if required by national option.

If a target is intercepted by several LEAs and/or several identities simultaneously, a single activation of delivery is necessary for each combination of LEA and identity.


[image: image13.wmf]... activation on X1_2 ...

... activation on X1_1...

request for lawful

interception activation

response for lawful

interception activation

ADMF

DF 3

lawful interception

activation

lawful interception

activation ack


Figure 5: Information flow on X1_3-interface for Lawful Interception activation

5.2
Deactivation

5.2.0
General
Figures 6, 7 and 8 show the information flow for the deactivation of the Lawful interception.

5.2.1
X1_1-interface

The messages sent from the ADMF to the 3G ICEs for deactivation contain:

-
the target identity;

-
the possible relevant IAs in case of location dependent interception.


[image: image14.wmf] 

... deactivation of 

delivery functions ...

 

request for lawful 

interception deactivation

 

response for lawful 

interception deactivation

 

ADMF

 

 

           3G ICE

 

lawful interception 

deactivation

 

lawful interception 

deactivation ack

 


Figure 6: Information flow on X1_1-interface for Lawful Interception deactivation

If interception of a target has been activated via different identities then a separate deactivation message will need to be sent from the ADMF to the 3G ICEs for each identity.

When several LEAs requested activation on the same identity and subsequently request deactivation then the ADMF determines that there are remaining activations on the identity. In this case, the ADMF will not send a deactivation message to the 3G ICEs except when the activation needs to change from CC and IRI to IRI only. In that case an activation change message will be sent to the 3G ICEs.

5.2.2
X1_2-interface (IRI)

The message(s) sent from the ADMF to Delivery Function 2 for the deactivation of the Intercept Related Information contains:

-
a DF2 activation ID, which uniquely identifies the activation to be deactivated for DF2.

If a target is intercepted by several LEAs and/or several identities simultaneously, a single deactivation is necessary for each combination of LEA and identity.


[image: image15.wmf]... deactivation on X1_3

... deactivation on X1_1

request for lawful

interception deactivation

response for lawful

interception deactivation

ADMF

 DF 2

lawful interception

deactivation ack

lawful interception

deactivation


Figure 7: Information flow on X1_2-interface for Lawful Interception deactivation

5.2.3
X1_3-interface (CC)

For deactivating the delivery of the CC the message(s) sent from the ADMF to DF3 contains:

-
a DF3 activation ID, which uniquely identifies the activation to be deactivated for DF3.


[image: image16.wmf]... deactivation on X1_1

and X1_2 ...

request for lawful

interception deactivation

response for lawful

interception deactivation

ADMF

DF 3

lawful interception

deactivation ack

lawful interception

deactivation


Figure 8: Information flow on X1_3-interface for Lawful Interception deactivation

5.3
Interrogation

5.3.0
General
Interrogation provides the current status of the interception activation in the system. Interrogation of all activations for a given LEA is an ADMF function.

5.3.1
Interrogation of the 3G ICEs

Figure 9 shows the information flow for the interrogation of the Lawful Interception. It shall be possible to interrogate:

-
a specific activation at each relevant 3G ICEs;

-
all activations at each relevant 3G ICEs.

As a result of the interrogation the activation status and data are returned.


[image: image17.wmf] 

... interrogation for 

activation status on 

X1_1

 

request for lawful 

interception interrogation

 

 

response for lawful 

interception interrogation

 

ADMF

 

        3G ICE

 

lawful interception 

interrogation ack

 

lawful interception 

interrogation

 


Figure 9: Interrogation of the Lawful Interception (3G ICEs)

5.3.2
Interrogation of Delivery Functions

Figure 10 shows the information flow for the interrogation of the Lawful Interception. It shall be possible to interrogate:

-
a specific activation at a DF;

-
all activations at a DF for a given target identity;

-
all activations at a DF.

As a result of the interrogation the activation status and data are returned.


[image: image18.wmf]... interrogation for

activation status on

X1_2 and X1_3 ...

request for lawful

interception interrogation

response for lawful

interception interrogation

ADMF

DF2/DF3

lawful interception

interrogation ack

lawful interception

interrogation


Figure 10: Interrogation of the Lawful Interception (Delivery Functions)

6
Invocation of Lawful Interception for Circuit Switched Services

6.0
General

Figure 11 shows an extraction from the reference configuration in figure 1a which is relevant for the invocation of the lawful interception.

[image: image19.wmf] 

Delivery

Function 2

X2

HI2

HI3

LEMF

3G MSC

Server

X3

Delivery

Function 3

Signaling

3G MSC

Server

Delivery

Function 3

Bearer

3G MGW

Mc


Figure 11: Functional model for Lawful Interception invocation

The HI2 and HI3 interfaces represent the interfaces between the LEMF and two delivery functions. Both interfaces are subject to national requirements. They are included for completeness, but are beyond the scope of standardization in this document. The delivery functions are used:

-
to convert the information on the X2-interface to the corresponding information on the HI2-interface;

-
to convert the information on the X3-interface to the corresponding information on the HI3-interface;

-
to distribute the intercept related information to the relevant LEA(s) (based on IAs, if defined);

-
to distribute the intercept content of communicationsto the relevant LEA(s) (based on IAs, if defined).

For the delivery of the CC and IRI, the 3G MSC Server provides a correlation number and target identity to the DF2 and DF3 which is used to select the different LEAs to which the product shall be delivered.

NOTE:
If interception has been activated for both parties of the call both CC and IRI will be delivered for each party as separate intercept activity.

The Mc interface between the 3G MSC Server and MGW is used to establish intercept and deliver the bearer to DF3.

For Location Dependent Interception, the location dependency check occurs at the establishment of each call. Subsequent dependency checks for simultaneous calls are not required, but can be a national option.

If a target is marked using an IA in the 3G MSC Server, the 3G MSC Server shall perform a location dependency check at call set-up. Only if the target's location matches the IA then the call is intercepted.

If a target is marked using an IA in the DF2, the DF2 shall perform a location dependency check at reception of the first IRI for the call. Only if the target's location matches the IA for certain LEAs is IRI the relayed to these LEAs. All subsequent IRIs for the call are sent to the same LEAs.

If a target is marked using an IA in the DF3, the DF3 signalling function shall perform a location dependency check at reception of the CC. Only if the target's location matches the IA for certain LEAs is the CC relayed to these LEAs.

6.1
Provision of Intercept CC - Circuit Switched

Figure 12 shows the access method for the delivering of CC. The access method shall be a bridged/ T-connection.




[image: image21.emf] 

LEMF  

Delivery  Function 3  

o ther  party  

MGW  

Target  

Two stub lines  


Figure 12: Delivery configuration to the LEMF for the interception of a circuit switched call

The signals of both parties of the configuration to be intercepted are delivered separately to the LEMF. The delivery function has no impact on the connection between the subscribers.

The two stublines towards the LEMF are established in parallel to the call set up. For both stublines the address is used which has been provided during activation.

Bearer, and only bearer, is sent from the MGW to the bearer function of DF3.

NOTE 1:
For data calls it is necessary to provide means for fast call establishment towards the LEMF to help ensure that the beginning of the data transmission is delivered.

The following information needs to be transferred from the 3G MSC Server to the DF3 in order to allow the DF3 to perform its functionality:

-
target identity (MSISDN, IMSI or IMEI); note 2

-
the target location (if available) or the IAs in case of location dependent interception. note 2

-
correlation number (IRI <-> CC);

-
direction indication - (Signal from target or signal to target).

NOTE 2:
For DF3 internal use only.

Additional information may be provided if required by national laws.

6.2
Provision of CC - Short Message Service

Figure 14 shows an SMS transfer from the 3G MSC Server to the LEMF. Quasi-parallel to the delivery from / to the mobile subscriber a message, which contains the contents of the SMS with the header, is generated and sent via the Delivery Function 2 to the LEMF in the same way as the Intercept Related Information.

The IRI will be delivered to the LEMF:

-
for a SMS-MO. Dependent on national requirements, delivery shall occur either when the 3G MSC receives the SMS from the target MS, or when the 3G MSC receives notification that the SMS-Centre successfully received the SMS;

-
for a SMS-MT. Dependent on national requirements, delivery shall occur either when the 3G MSC receives the SMS from the SMSC, or when the 3G MSC receives notification that the target MS successfully received the SMS.




[image: image23.emf] 

3G  MSC  

SMS   service   center   Target  

Delivery   Function  2  

LEMF  


Figure 14: Provision of Content of Communication - Short Message Service

6.3
Provision of Intercept Related Information

6.3.0
General

Intercept Related Information (Events) are necessary at the Begin and End of the call, for all supplementary services during a call and for information which is not call associated. There are call related events and non call related events.

Figure 15 shows the transfer of intercept related information to the DF2. If an event for / from a mobile subscriber occurs, the 3G MSC Server sends the relevant data to the DF2.




[image: image25.emf] 

3G  MSC  

Other   party   Target  

Delivery   Function  2  

LEMF  


Figure 15: Provision of Intercept Related Information

6.3.1
X2-interface

The following information needs to be transferred from the 3G MSC Server to the DF2 in order to allow a DF2 to perform its functionality:

-
target identity (MSISDN, IMSI or IMEI);

-
in case of location dependent interception, the IAs and/or target cell ID shall be provided;

-
events and associated parameters as defined in clauses 6.3.3 and 6.3.4 may be provided.

The IRI should be sent to DF2 with a reliable transport mechanism.

6.3.2
Structure of the events

The information sent to DF2 is triggered by up to eight different call related and non-call related events. Details are described in following clause. The events for interception are configurable (if they are sent to DF2) in the 3G MSC Server and can be suppressed in the DF2. The events are listed as follows:

Call Related Events:

-
Call Establishment

-
Answer

-
Supplementary Service

-
Handover

-
Release

Non Call Related Events:

-
SMS

-
Location Update

-
Subscriber Controlled Input

Table 1 below shows the set of information that can be associated with the events. The events trigger the transmission of the information from the 3G MSC Server to DF2. Available IEs from this set of information can be extended in the 3G MSC Server, if this is necessary in a specific country. DF2 can extend available information if this is necessary in a specific country e.g. a unique number for each surveillance warrant.

Table 1: Information Elements for Circuit Event records

	Observed MSISDN


Target Identifier with the MSISDN of the target.

	Observed IMSI


Target Identifier with the IMSI of the target.

	Observed IMEI


Target Identifier with the IMEI of the target,


It shall be checked for each call over the radio interface

	event type


Description which type of event is delivered: Establishment, Answer, Supplementary service,

Handover, Release, SMS, Location update, Subscriber controlled input

	event date


Date of the event generation in the 3G MSC Server

	event time


Time of the event generation in the 3G MSC Server

	dialled number


Dialled phone number before digit modification, IN-modification etc.

	Connected number


Number of the answering party

	other party address


Directory number of the other party for MOC


Calling party for MTC

	call direction


Information if the target is calling or called e.g. MOC/MTC or originating/ terminating


In or/out

	Correlation number


Unique number for each call sent to the DF, to help the LEA, to have a correlation between each

Call and the IRI

	Network Element Identifier


Unique identifier for the element reporting the ICE.

	Location Information


Location information is the service area identity and/or location area identity that is present at the 3G MSC Server

at the time of event record production. 


In some traffic cases the available location information can be the one received from the MME, i.e. the Tracking 
Area Identity (TAI) and/or the E-UTRAN Cell Global Identification (ECGI) as specified in the 3GPP TS 23.272 [30].

	basic service


Information about Tele service or bearer service.

	Supplementary service


Supplementary services used by the target e.g. CF, CW, ECT

	Forwarded to number


Forwarded to number at CF

	call release reason


Call release reason of the target call

	SMS initiator


SMS indicator whether the SMS is MO, MT, or undefined

	SMS Message


The SMS content with header which is sent with the SMS-service

	Redirecting number


The number which invokes the call forwarding towards the target. This is provided if available. 

	SCI


Non call related Subscriber Controlled Input (SCI) which the 3G MSC Server receives from the ME


6.3.3
Call Related events

6.3.3.1
Call establishment

For call establishment a call establishment-event is generated. This event is generated at the beginning of a call when the 3G MSC Server attempts to reach the subscriber. This information will be delivered to the DF2 if available:

	Observed MSISDN 

	Observed IMSI

	Observed IMEI

	event type

	event date

	event time

	dialled number

	other party address

	call direction

	Correlation number 

	Redirecting number

	Network Element Identifier

	Location Information

	basic service

	Supplementary service


6.3.3.2
Answer

If the called party answers, an answer- event is generated. This information will be delivered to the DF2 if available:

	Observed MSISDN 

	Observed IMSI 

	Observed IMEI

	event type

	event date

	event time

	dialled number

	other party address

	Connected party

	call direction

	Correlation number

	Redirecting number

	Network Element Identifier

	Location Information

	basic service

	Supplementary service


6.3.3.3
Supplementary Services

For supplementary services events are generated with the information which supplementary service is used e.g. Call Forwarding (CF), Call Waiting (CW), Explicit Call Transfer (ECT), Multi Party (MPTY), Call Hold and information correlated to the service like the forwarded to number. This information will be delivered to the DF2 if available:

	Observed MSISDN 

	Observed IMSI

	Observed IMEI

	event type

	event date

	event time

	dialled number

	other party address

	call direction

	Correlation number

	Network Element Identifier

	Location Information

	basic service

	Supplementary service

	Forwarded to number


6.3.3.4
Handover

For each handover that is realised at the 3G MSC Server due to a change in target location information, a handover-event with the new location information is generated. This information will be delivered to the DF2 if available:

	Observed MSISDN 

	Observed IMSI 

	Observed IMEI

	event type

	event date

	event time

	Correlation number

	Network Element Identifier

	Location Information


6.3.3.5
Release

For the release or failed attempt of a target call, a release event with the following information is generated. This information will be delivered to the DF2 if available:

	Observed MSISDN

	Observed IMSI

	Observed IMEI

	event type

	event date 

	event time

	dialled number

	other party address

	call direction

	Correlation number

	Network Element Identifier

	Location Information

	basic service

	call release reason


6.3.4
Non Call Related events

6.3.4.1
SMS

For MO-SMS the event is generated in the 3G MSC Server. Dependent on national requirements, event generation shall occur either when the 3G MSC Server receives the SMS from the target MS or when the 3G MSC Server receives notification that the SMSC successfully receives the SMS; for MT-SMS the event is generated in the 3G MSC Server. Dependent on national requirements, event generation shall occur either when the 3G MSC Server receives the SMS from the SMSC or when the 3G MSC Server receives notification that the target MS successfully received the message. This information will be delivered to the DF2 if available:

	Observed MSISDN 

	Observed IMSI 

	event type

	event date

	event time

	Network Element Identifier

	Location Information

	SMS initiator

	SMS Message


6.3.4.2
Location update

For location updates a Location update-event is generated, with the new location information. This information will be delivered to the DF2 if available:

	Observed MSISDN 

	observed IMSI 

	event type

	event date

	event time

	Network Element Identifier

	Location Information


6.3.4.3
Subscriber Controlled Input (SCI)

SCI includes subscriber initiated changes in service activation and deactivation. SCI does not include any information available in the CC. For subscriber controlled inputs - a SCI-event is generated with information about the SCI. This information will be delivered to the DF2 if available:

	observed MSISDN 

	observed IMSI 

	event type

	event date

	event time

	Network Element Identifier

	Location Information

	SCI


6.4
Intercept cases for circuit switched supplementary services

6.4.1
Interception of Multiparty call

[image: image26.wmf]A

B

C

D

3G MGW 

DF3

bearer

 


Figure 16: Interception of Multiparty for CC

Figure 16 shows the delivery of CC from intercepted multiparty call where party A is the target of interception.

One pair of call content channels are delivered to the delivery function. Party A is delivered to the DF3 on one channel and the sum of the balance of the parties, B,C and D is delivered on the second channel.

It should be noted that if parties B,C or D is a target of interception, that intercept is treated as a simple call intercept.

The events contain information about B, C and D if subscriber A is monitored. If one of B, C or D is monitored, events contain the information about A but not the other parties of the conference.

6.4.2
Interception for Call Forwarding / Call Deflection / ECT

[image: image27.wmf]                                 

Signalling

       

A                                         B

                   

Bearer Traffic

                                                C


Figure 17: Interception for Call Forwarding / Deflection / ECT

The interception of party B once the supplementary service is invoked is a national option.

For Intercept Related Information it depends who is monitored:

-
If subscriber A is monitored the number of A and B are mandatory in the event information and the number of C if available.

-
If subscriber B is monitored the number of B and C are mandatory in the event information and the number of A if available.

-
If subscriber C is monitored the number of C is mandatory in the event information and the number of A and B if available.

Intercept requirements for CS multi-media is not defined in this release.

7
Invocation of Lawful Interception for GSN Packet Data services

7.0
General

Figure 18 shows the extract from the reference configuration which is relevant for the invocation of the Lawful Interception of the packet data GSN network.


[image: image28]
Figure 18: Functional model for Packet Data GSN Network Lawful Interception invocation

The HI2 and HI3 interfaces represent the interfaces between the LEA and two delivery functions. Both interfaces are subject to national requirements. They are included for completeness, but are beyond the scope of this specification. The delivery functions are used:

-
to convert the information on the X2-interface to the corresponding information on the HI2 interface;

-
to distribute the intercept related information to the relevant LEA(s);

-
to distribute the intercept product to the relevant LEA(s).

For the delivery of the CC and IRI the 3G SGSN and/or, per national option 3G GGSN provides correlation number and target identity to the DF2 and DF3 which is used there in order to select the different LEAs where the product shall be delivered. When the SGSN connects an UE to a S-GW through the S4 interface ([10], see also note 3), the SGSN is not required to provide CC for that communication (see note 4).

The correlation number is unique in the whole PLMN and is used to correlate CC with IRI and the different IRI's of one PDP context.

The correlation number shall be generated by using existing parameters related to the PDP context. 

When the SGSN connects an UE to a S-GW through the S4 interface ([10], see also note 3), the SGSN is not required to provide IRIs for PDP contexts associated with CC and correlation for that communication (see note 4).

NOTE 1:
If interception has been activated for both parties of the Packet Data communication both CC and IRI will be delivered for each party as separate intercept activity.

In case of location dependent interception:

-
for each target, the location dependency check occurs at each Packet Data session establishment or release and at each Routing Area (RA) update to determine permanently the relevant IAs (and deduce, the possible LEAs within these IAs);

-
concerning the IRI:

-
when an IA is left, either a Mobile Station Detach event is sent when changing servicing 3G GSNs, or an RA update event is sent;

-
RA update event is sent to DF2 when changing IAs inside the same servicing 3G SGSN;

-
when a new IA is entered a RA update event is sent to DF2 and, optionally, a "Start of interception with PDP context active" event for each PDP context;

-
concerning the CC, when crossing IAs, the CC is not sent anymore to the DF3 of the old IA but sent to the DF3 of the new IA.

Both in case of location dependent and location independent interception:

"Start of interception with PDP context active" event is sent by the new SGSN if an Inter-SGSN RA update procedure, which involves different PLMNs, takes place for a target, which has at least one active PDP context.

NOTE 2:
An SGSN can differentiate "Inter PLMN" type of Inter-SGSN RA update procedure from "Intra PLMN" type of Inter-SGSN RA update procedure by inspecting the old RAI parameter, which is being received by the SGSN as part of the procedure (see TS 23.060 [10], clause 6.9.1.2.2 and TS 23.003, clause 4.2).

Optionally, it is possible to send "Start of interception with PDP context active" for all cases of inter- SGSN RA update when at least one PDP context is active. 

NOTE 3: S4 is an intra-PLMN reference point between the SGSN and the S-GW.

NOTE 4: When the SGSN connects an UE to a S-GW through the S4 interface, the S-GW provides IRI, CC and correlation for the EPS bearer associated to the PDP context, as specified in clause 12.
7.1
Provision of Intercept Product - Short Message Service

Figure 19 shows an SMS transfer from the 3G SGSN node to the LEA. Quasi-parallel to the delivery from / to the mobile subscriber a SMS event, which contains the content and header of the SMS, is generated and sent via the Delivery Function 2 to the LEA in the same way as the Intercept Related Information. National regulations and warrant type determine if a SMS event shall contain only SMS header, or SMS header and SMS content.

The IRI will be delivered to the LEA:

-
for a SMS-MO. Dependent on national requirements, delivery shall occur either when the 3G SGSN receives the SMS from the target MS or when the 3G SGSN receives notification that the SMS-Centre successfully received the SMS;

-
for a SMS-MT. Dependent on national requirements, delivery shall occur either when the 3G SGSN receives the SMS from the SMS-Centre or when the 3G SGSN receives notification that the target MS successfully received the SMS.


[image: image29]
Figure 19: Provision of Intercept Product - Short Message Service
7.2
Provision of Intercepted Content of Communications – Packet data GSN services

7.2.0
General

The access method for the delivering of Packet Data GSN Intercept Product is based on duplication of packets without modification at 3G GSN. The duplicated packets with additional information in a header, as described in 7.2.1, are sent to DF3 for further delivery to the LEA.


[image: image30]
Figure 20: Configuration for interception of Packet Data GSN product data

7.2.1
X3-interface

In addition to the intercepted content of communications, the following information needs to be transferred from the 3G GSN to the DF3 in order to allow the DF3 to perform its functionality:

-
target identity;

-
correlation number;

-
time stamp - optional;

-
direction (indicates whether T-PDU is MO or MT) - optional;

-
the target location (if available) or the IAs in case of location dependent interception.
As a national option, in the case where the 3G GGSN is performing interception of the content of communications, the target is handed off to another SGSN and the same 3G GGSN continues to handle the content of communications subject to roaming agreements, the 3G GGSN shall continue to perform the interception of the content of communication.

If 3G direct tunnel functionality with the GGSN, as defined in TS 23.060 [10], is used in the network, then the GGSN shall perform the interception of the content of communications.
7.3
Provision of Intercept Related Information

7.3.0
General
Intercept Related Information (Events) are necessary at the Mobile Station Attach, Mobile Station Detach, PDP Context Activation, Start of intercept with PDP context active, PDP Context Deactivation, RA update, Serving System, Packet Header Information Reporting,  and SMS events.

Serving System event reporting is a national option
Packet Header Information Reporting is a national option.  

Figure 21 shows the transfer of intercept related information to the DF2. If an event for / from a mobile subscriber occurs, the 3G GSN or the Home Location Register (HLR) sends the relevant data to the DF2. For Packet Header Information Reporting, a 3G GSN either isolates the relevant data and sends it to the DF2 or sends the packet stream to another entity in the network (e.g., DF3) for isolation which then provides the relevant data to the DF2.

See clause 7A for multi-media Intercept Related Information produced at the CSCF.




[image: image32.emf] 

HLR  

  3G GSN  

other   party   Target  

Delivery   Function 2  

LEMF  


Figure 21: Provision of Intercept Related Information

7.3.1
X2-interface

The following information needs to be transferred from the 3G GSN or the HLR to the DF2 in order to allow a DF2 to perform its functionality:

-
target identity (MSISDN, IMSI, IMEI);

-
events and associated parameters as defined in clauses 7.3.2 and 7.4 may be provided;

-
the target location (if available) or the IAs in case of location dependent interception;
-
Correlation number;

-
Quality of Service (QoS) identifier;
-
Encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.
The IRI should be sent to DF2 using a reliable transport mechanism.

The 3G GSN detects packets containing packet header information in the communications path but the information needed for Packet Header Information Reporting may need to be transferred from the 3G GSN either directly to the DF2 or via another entity in order to allow the DF2 to perform its functionality.

7.3.2
Structure of the events

There are several different events in which the information is sent to the DF2 if this is required. Details are described in the following clause. The events for interception are configurable (if they are sent to DF2) in the 3G GSN or the HLR and can be suppressed in the DF2.

The following events are applicable to 3G SGSN:

-
Mobile Station Attach;

-
Mobile Station Detach;

-
PDP context activation; 

-
Start of interception with mobile station attached (national option);

-
Start of intercept with PDP context active;

-
PDP context modification;

-
PDP context deactivation;

-
RA update;

-
SMS;

-
Packet Header Information Reporting.

NOTE:
3G GGSN interception is a national option. Location information may not be available in this case.  If interception is performed at the 3G GGSN, then Packet Header Information Reporting shall also be performed at the 3G GGSN and not at the 3G SGSN.
If 3G direct tunnel functionality with the GGSN, as defined in TS 23.060 [10], is used in the network, then both the SGSN and the GGSN shall perform the interception of intercept related information.

When the SGSN connects an UE to a S-GW through the S4 interface ([10]), the SGSN is not required to report events PDP context activation (successful), Start of intercept with PDP context active, PDP context modification, PDP context deactivation; the SGSN shall report unsuccessful PDP context activation event.

The following events are applicable to the 3G GGSN:

-
PDP context activation;

-
PDP context modification;

-
PDP context deactivation;

-
Start of interception with PDP context active ,

-
Packet Header Information Reporting.
The following events are applicable to the HLR:

-
Serving System.

A set of elements as shown below can be associated with the events. The events trigger the transmission of the information from 3G GSN or HLR to DF2, perhaps via a MF in the case of Packet Header Information Reporting. Available IEs from this set of elements as shown below can be extended in the 3G GSN or HLR, if this is necessary as a national option. DF2 can extend available information if this is necessary as a national option e.g. a unique number for each surveillance warrant.

Table 2: Information Events for Packet Data Event Records

	Observed MSISDN

MSISDN of the target.

	Observed IMSI

IMSI of the target.

	Observed IMEI

IMEI of the target,it shall be checked for each activation over the radio interface.

	Event type

Description which type of event is delivered: MS attach, MS detach, PDP context activation, Start of intercept with PDP context active, PDP context deactivation, SMS, Serving System, Packet Data Header Information, Cell and/or RA update.

	Event date

Date of the event generation in the 3G GSN or the HLR.

	Event time

Time of the event generation in the 3G GSN or the HLR. Timestamp shall be generated relative to GSN or HLR internal clock.

	PDP address

The PDP address of the target . Note that this address might be dynamic. In case the PDP type is IPv4v6, the parameter may carry two IP addresses.

	Access Point Name

The APN of the access point. (Typically the GGSN of the other party).

	Location Information

Location Information is the Service Area Identity (SAI), RAI and/or location area identity that is present at the GSN at the time of event record production.

	Old Location Information

Location Information of the subscriber before Routing Area Update

	PDP Type

The used PDP type.

	Correlation Number

The correlation number is used to correlate CC and IRI.

	SMS

The SMS content with header which is sent with the SMS-service. The header also includes the SMS-Centre address.

	Network Element Identifier

Unique identifier for the element reporting the ICE.

	Failed attach reason

Reason for failed attach of the target.

	Failed context activation reason

Reason for failed context activation of the target .

	IAs

The observed Interception Areas.

	Initiator

The initiator of the PDP context activation, deactivation or modification request either the network or the 3G MS.

	SMS Initiator

SMS indicator whether the SMS is MO or MT.

	Deactivation / termination cause

The termination cause of the PDP context.

	QoS

This field indicates the Quality of Service associated with the PDP Context procedure.

	Serving System Address

Information about the serving system (e.g. serving SGSN number or serving SGSN address).

	NSAPI

Network layer Service Access Point Identifier

The NSAPI information element contains an NSAPI identifying a PDP Context in a mobility management context specified by the Tunnel Endpoint Identifier Control Plane.

This is an optional parameter to help DF/MF and LEA's to distinguish between the sending mobile access networks when the GGSN is used as element of the PDG according TS 23.234 [14], Annex F.

	ULI Timestamp

Indicates the time when the User Location Information was acquired.

The parameter is specified in 3GPP TS 29.060 [37].

	Destination IP Address

The IP address, including type IPv4 or IPv6, of the destination of the IP packet.

	Destination Port Number

The port number of the destination of the IP packet.

	Flow Label (IPv6 only)

The field in the IPv6 header that is used by a source to label packets of a flow (see RFC 3697 [41]).

	Packet Count 

The number of packets detected and reported (for a particular summary period).

	Packet Data Summary Reason

The reason for a Packet Data Summary message being sent to the LEMF (e.g., timed out, counter expiration, end of session)

	Packet Size

The size of the packet. (i.e., Total Length Field in IPv4 or Payload Length field in IPv6)

	Source IP Address

The IP address, including type IPv4 or IPv6, of the source of the IP packet.

	Source Port Number

The port number of the source of the IP packet.

	Sum of Packet Sizes (for a particular summary period)

The sum of values contained in the Total Length fields of the IPv4 packets or the sum of the values contained in the Payload Length fields of the IPv6 packets.

	Summary Period

Includes the dates and times of the first and last packets in a particular packet data interval.  

	Transport Protocol (e.g., TCP)

The identification of the transport protocol of the packet or packet flow being reported.


7.4
Packet Data related events

7.4.1
Mobile Station Attach

For attach an attach-event is generated. When an attach activation is generated from the mobile to serving 3G G SN this event is generated. These elements will be delivered to the DF2 if available:

	Observed MSISDN 

	Observed IMSI

	Observed IMEI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Location Information

	Failed attach reason

	IAs (if applicable)


7.4.2
Mobile Station Detach

For detach a detach-event is generated, this is for the common (end) detach. These elements will be delivered to the DF2 if available:

	Observed MSISDN

	Observed IMSI

	Observed IMEI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Location Information

	IAs (if applicable)


7.4.3
Packet Data PDP context activation

When a PDP context activation is generated a PDP context activation-event is generated. These elements will be delivered to the DF2 if available:

	Observed MSISDN

	Observed IMSI

	Observed IMEI

	PDP address of observed party

	Event Type 

	Event Time

	Event Date

	Correlation number

	Access Point Name

	PDP Type

	Network Element Identifier

	Location Information

	Failed context activation reason

	IAs (if applicable)

	Initiator (optional)

	QoS (optional)

	NSAPI (optional)


7.4.4
Start of interception with PDP context active

This event will be generated if interception for a target is started and if the target has at least one PDP context active. If more than one PDP context is open, for each of them an event record is generated. These elements will be delivered to the DF2 if available:

	Observed MSISDN 

	Observed IMSI

	Observed IMEI

	PDP address of observed party

	Event Type 

	Event Time

	Event Date

	Correlation number

	Access Point Name

	PDP Type

	Network Element Identifier

	Location Information

	Old Location Information (optional)

	IAs (if applicable)

	QoS (optional)

	Initiator (optional)

	NSAPI (optional)


Presence of the optional Old Location Information field indicates that PDP context was already active, and being intercepted. However, the absence of this information does not imply that interception has not started in the old location SGSN for an active PDP context. 

Start of interception with PDP context active shall be sent regardless of whether a Start of interception with mobile station attached has already been sent.

7.4.5
Packet Data PDP context deactivation

At PDP context deactivation a PDP context deactivation-event is generated. These elements will be delivered to the DF2 if available:

	Observed MSISDN 

	Observed IMSI

	Observed IMEI

	PDP address of observed party

	Event Type

	Event Time

	Event Date

	Correlation number

	Access point name

	Network Element Identifier

	Location Information

	IAs (if applicable)

	Deactivation cause

	Initiator (optional)

	NSAPI (optional)

	ULI Timestamp


7.4.6
RA update

For each RA update an update-event with the elements about the new location is generated. New SGSN shall send the event, and the old SGSN may optionally send the event as well. These elements will be delivered to the DF2 if available:

	Observed MSISDN 

	Observed IMSI 

	Observed IMEI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Location Information (only for the new SGSN)

	Old Location Information (only for the old SGSN)

	IAs (if applicable)


NOTE:
Once target moves out of the interception area, old SGSN may report the RAU event. Normally, however, the old SGSN does not receive the new SGSN's RAI, while the new SGSN does receive the old SGSN's RAI from UE with the RAU Request message.

7.4.7
SMS

For MO-SMS the event is generated in the 3G SGSN. Dependent on national requirements, event generation shall occur either when the 3G SGSN receives the SMS from the target MS or when the 3G SGSN receives notification that the SMS-Centre successfully receives the SMS; for MT-SMS the event is generated in the 3G SGSN. Dependent on national requirements, event generation shall occur either when the 3G SGSN receives the SMS from the SMS-Centre or when the 3G SGSN receives notification that the target MS successfully received the message. These elements will be delivered to the DF2 if available:

	Observed MSISDN 

	Observed IMSI 

	Observed IMEI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Location Information

	SMS

	SMS Initiator

	IAs (if applicable)


7.4.8
Packet Data PDP context modification

This event will be generated if an active PDP context for the target is modified. These elements will be delivered to the DF2 if available:

	Observed MSISDN 

	Observed IMSI

	Observed IMEI

	PDP address of observed party

	Event Type 

	Event Time

	Event Date

	Correlation number

	Access Point Name

	PDP Type

	Network Element Identifier

	Location Information

	IAs (if applicable)

	Initiator

	QoS


7.4.9
Serving System

The Serving System report event is generated at the HLR, when the HLR has detected that the target has roamed. The elements will be delivered to the DF2 if available:

	Observed MSISDN

	Observed IMSI

	Observed IMEI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Serving System Address


7.4.10
Start of interception with mobile station attached

This event will be generated if interception has started for the already attached target. These elements will be delivered to the DF2 if available:

	Observed MSISDN 

	Observed IMSI 

	Observed IMEI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Location Information

	IAs (if applicable)


7.4.11
Packet Header Information Reporting

7.4.11.0
Introduction
Packet Header Information Reporting can be done either on a per-packet (i.e., non-summarized) basis or in a summary report.  

7.4.11.1
Packet Data Header Report

This event is used to provide packet header reports on a per packet basis (non-summarized reporting) and is triggered by each packet sent or received by the target.  These elements will be delivered either directly to DF2 or via another network entity if available:

	Observed MSISDN 

	Observed IMSI 

	Observed IMEI

	PDP address of observed party

	Event Type

	Event Time

	Event Date

	Correlation Number

	Access Point Name

	PDP Type

	Network Element Identifier

	Source IP Address

	Source Port Number

	Destination IP Address

	Destination Port Number

	Transport Protocol (e.g., TCP)

	Packet Size

	Flow Label (IPv6 only)


7.4.11.2
Packet Data Summary Report

This event is used to report:

1)
the source and destination information derived from the packet headers, including:

a)
source and destination IP Addresses,

b)
IP next-layer protocol,

c)
Layer-4 ports, and

d)
Flow label, if the packet is IPv6

2)
summary information for the number of packets and bytes transmitted or received by the target for each unique packet flow within a PDP context,  and 

3)
the date and the time of the first and last packets associated with that packet flow.  A packet flow is defined as the 6-tuple of source/destination IP address/port number and the layer 4 protocol, and PDP Context.

IP addresses and the IP next-layer protocol are always reported, the flow label is reported if the packet is IPv6, and the layer-4 ports are reported.    

The event provides packet summary reports for each unique packet data session (PDP context) and packet flow, and is triggered by one of the following:

-
start of a packet flow associated with a PDP Context

-
an interim report for a packet flow associated with a PDP Context is to be reported

-
end of a packet flow associated with a PDP Context (including end of the PDP Context itself).

An interim report can be triggered by

-
The expiration of a configurable timer per intercept (called a Summary Timer). The Summary Timer is configurable in units of seconds;

-
A per-intercept configurable count threshold is reached;

These elements will be delivered either directly to DF2 or via an MF for each packet flow if available:
	Observed MSISDN 

	Observed IMSI 

	Observed IMEI

	PDP address of observed party

	Event Type

	Event Time

	Event Date

	Correlation Number

	Access Point Name

	PDP Type

	Network Element Identifier

	Source IP Address

	Source Port Number

	Destination IP Address

	Destination Port Number

	Transport Protocol (e.g., TCP)

	Flow Label (IPv6 only)

	Summary Period

	Packet Count (for this summary period)

	Sum of Packet Sizes (for this summary period)


If the packets are IPv4, the sum of all observed packet sizes is the sum of the values contained in the Total Length field of each packet as specified in IETF RFC 791[39].

If the packet is IPv6, the sum of all observed packet sizes is the sum of the values contained in the Payload Length field for each packet as specified in IETF RFC 2460 [40].

If no packets were detected for the duration of the Summary Timer, then the Packet Data Summary Report shall not be sent.
7.5
Void

7.6
Interception of the Multimedia Messaging Service (MMS)

The Multimedia Messaging Service (MMS) is a service running over the 3GPP PS-domain. Both mobile originating and mobile terminating MMS messages must pass through PS domain GSN nodes en route to or from Multimedia Message Service Centres (MMSCs). Therefore, interception of MMS messages shall be performed at the GSN in exactly the same way as for other PS-domain bearer services.

The GSN is not responsible for recovering individual MMS messages from the user PDP context IP stream.

No MMS specific HI2 records are defined to be delivered to the LEMF over the DF2 other than those listed in clause 7.4 of this specification. CC records shall be sent to the LEMF over the DF3 as specified in clause 7.3. 

Interception of a user PDP context IP stream will occur as described in clause 7.2. Such a stream may or may not contain MMS messages.

7A
Invocation of Lawful Interception for Packet Data Multi-media Service

7A.1
Provision of content of communications

Interception of the content of communications for GSN packet data services is explained in clause 7.2.. Activation and invocation of lawful interception for multi-media service only at the CSCF(s) does not produce interception of content of communications.  Consequently,  a separate activation and invocation of lawful interception must occur at a node that has access to the CC (e.g., in case of GPRS / UMTS (PS domain) interception of CC occurs at the GSN).

Interception at the GSN is only possible for a basic call. For the interception of content of communications of IMS-based voice services including CC for forwarded and transferred calls, refer to clause 15. 
7A.1.A
Decryption for IMS Media Plane Security

This clause describes how the TSP can meet the national requirements in Clause 5.1.2 of TS 33.106 [7] to deliver intercepted communications decrypted when the TSP uses [25] IMS Media Plane Security options. If an ICE, in TSP IMS network using Security options [25], allows interception of Content of Communication in clear then this clause does not apply. 

If Session Description Protocol (SDP) Security Descriptions for Media Streams (SDES) is used, the DF2 shall identify the SDES keys from the SDP offer and SDP answer messages and provide the DF3 with the necessary SDES related parameters.  In this case, the DF3 shall perform the decryption prior to delivery to the LEMF.  The interface between the DF2 and DF3 to support the transfer of session keys is outside the scope of this specification. 
When SDES is used in end-to-access edge mode, the P-CSCF shall intercept SDES keys from SDP messages and shall deliver them to the DF2.
If a Key Management Service (KMS) and Multimedia Internet KEYing ticket (MIKEY-TICKET) is used, the TSP may use the mechanism as defined in Clause 7A.7.1, which results in the DF2 receiving the sessions keys needed to decrypt the intercepted communications.  Clause 7A.7.1 defines that the DF2 delivers the keys to the LEMF as IRI in order for the LEMF to decrypt the intercepted traffic.  

If the network is to decrypt the content of communications prior to delivery to the LEMF via HI3, the DF2 shall provide the DF3 with the sessions keys as defined in Clause 7A.7.1 instead of to the LEMF.  In this case, the DF3 shall perform the decryption prior to delivery to the LEMF.  The interface between the DF2 and DF3 to support the transfer of session keys is outside the scope of this specification.

7A.2
Provision of IRI
7A.2.1
Provision of IRI with SIP messaging
SIP messaging is reported as Intercept Related Information for the interception of multi-media service. As shown in figure 22 below, all SIP messages executed on behalf of a target are subject to intercept at the S-CSCF and Optionally P-CSCF. Based upon network configuration, the ADMF shall provision P-CSCFs, or S-CSCFs, or both P-CSCFs and S-CSCFs with SIP URI, TEL URI, or IMEI target identifiers. These resulting intercepted SIP messages shall be sent to DF2 for mediation prior to transmittal across the HI2 interface.  The enhanced P-CSCF (eP-CSCF) shall adhere to all LI requirements pertaining to the P-CSCF. Any additional LI requirements pertaining to the support of WebRTC Interworking, as specified in TS 23.228 [43], that only apply to the eP-CSCF are described distinctly.  

WebRTC Web Server Function (WWSF), if provided by the CSP, is an ICE that is used to copy and transmit via the DF to the LEMF the IP address and port used by the target as viewed by the WWSF.  This IP address may be a public or private address depending on how the target accesses the WWSF. 
WebRTC Authorisation Function (WAF), if provided by the CSP, is an ICE that creates a time-stamped authentication event associated with the target including relevant information such as the user’s identity provided to the WAF.   

Further details of the WWSF and WAF are FFS.

Editor’s Note:  The text in this clause related to the WWSF and WAF need to be moved to a more appropriate part of this specification.
For roaming scenarios, interception at the P-CSCF shall be Mandatory, in order to provide IRI Interception in the visited network, where the P-CSCF is located in the Visited Network. Where the P-CSCF is located in the Home Network, interception at the P-CSCF shall be Optional, subject to national regulation.


[image: image33]
Figure 22: Provision of Intercept Related Information for multi-media

7A.2.2
Provision of IRI with XCAP messages
The AS that store the XCAP data of the target shall intercept and transmit to the DF2 any XCAP based messages related to actions by the target, related to the supplementary service and other target’s service settings, defined in 3GPP TS 24.623 [55]:

-
on the Ut interface, 

-
on other interface to any AS with XCAP server capability that uses XCAP protocol. 

The DF2 will encapsulate the information as an IRI to the LEMF. 
NOTE:
The XCAP services separation through XCAP filtering or the application of Operator Policy function for national regulation is outside of the scope of this specification as an implementation issue.

Every successful or unsuccessful IMS supplementary services setting modification management request and response between UEs and IMS service nodes, or from other access to the target’s XCAP servers shall be reported. In case of IRI only, any filtering of XCAP messages based on operator policy or national regulation is for further studies.

NOTE:
Report of events related to target’s XCAP data and resources access by non XCAP protocol are for further studies. 

7A.3
Multi-media events

7A.3.0
General

-
All SIP messages to or from a target, and all SIP messages executed on behalf of a target for multi-media session control are intercepted by the S-CSCF and Optionally P-CSCF and sent to DF2. The target identifier used to trigger the intercept will also be sent with the SIP message. This standard does not require nor prohibit redundant information from being reported to DF2.

-
Where a CSCF which provides lawful interception makes changes to a SIP message, sent to or from or executed on behalf of a target then the CSCF shall report both the original message and the modified message to the DF2.

-
Where a CSCF which provides lawful interception changes identities within a SIP message (eg IMPI/IMPU changes or due to call forwarding etc) and the new identity is the target, then both the original and modified SIP messages shall be reported to DF2.

-
Where a CSCF which provides lawful interception changes identities within a SIP message (eg IMPI/IMPU changes or due to call forwarding etc) and the new identity is not the target, then both the original and modified SIP messages shall be reported to DF2.

-
P-CSCF event reports may be redundant with S-CSCF event reports when the P-CSCF and S-CSCF reside in the same network, however, this standard does not require nor prohibit redundant information from being reported to DF2.

-
The IRI should be sent to DF2 with a reliable transport mechanism.

-
Correlation for SIP to bearer shall be supported within the domain of one provider. 

-
An intercepted SIP event sent to DF2 is shown below:

-
Observed SIP URI

-
Observed TEL URI
-
Observed IMEI
-
Event Time and Date

-
Network element identifier

-
SIP Message Header

-
SIP Message Payload

NOTE: The Observed IMEI is obtained from the +sip.instance.id of the intercepted SIP message (as defined in 3GPP TS 24.229 [49]).-
All IMS XCAP messages to or from a target for multi-media or supplementary services are intercepted by the AS, or the group of AS in charge to transmit, manipulate and store any IMS XCAP of that target. The data have to be transmitted either “en clair” or encrypted with all elements to let the LEMF decrypt the data. The generated IRI should be sent in any case to DF2. 

Editor’s note:The data related to XCAP management and the XCAP documents modification of the target, as supplementary services, or as the 3GPP or OMA presence services (3GPP TS 24 141, OMA Presence SIMPLE specification and IETF RFC 4827), have to be reported through the DF2. However, these are points for further studies: 1) other data (XCAP management and the XCAP documents modification by the target) to be transmitted but related to other multimedia services; 2) the case of XCAP messages that are based on different interfaces than Ut interface; 3) the specific architecture related to encrypted data; 4) Detailed XCAP events, related to authentication. 

-
An intercepted XCAP report sent to DF2 is shown below:

-
Observed SIP URI or Tel URI, based on XUI (described in IETF RFC 4825 [56]) or information in the XCAP payload (if available)

-
Observed XUI or any other identities (if available)
-
Event Time and Date

-
Network element identifier

-
XCAP Message (the entire elements of the HTTP Header and the XCAP payload),

NOTE: 
The interpretation of XCAP messages, such as HTTP request through the Ut interface between the targets’s UE and related XCAP server may sometime be insufficient to let the LEA to understand what was modified as directed by the UE a later HTTP response is needed to understand the success or failure of the request.

7A.3.1
Mid IMS Session Interception

7A.3.1.0
General
Mid IMS Session interception functionality applies in addition to other IMS LI functional requirements as defined in section 7A. 

Where LI is activated on a target within a CSCF after an IMS session has already been established the CSCF shall do one of the following;

-
Where the CSCF has stored the media session information which occurred prior to the interception activiation, the CSCF shall provide a “start of interception with IMS session” event message, to the DF2/MF over the X2 interface, including the parameter and information listed in table 7A.3.1, if available.

-
Where the CSCF has not stored media session information which occurred prior to the interception activation, the CSCF shall report all future SIP messages which the CSCF is able to identify as associated with an ongoing target session. In this case, the event “start of interception with IMS session” is not applicable.

It is a national option whether the CSCF shall be mandated to store the necessary information to support reporting of session establishment parameters, in order to support mid IMS session interception, or whether the CSCF shall only report SIP messages which occur after the interception is applied and the CSCF is able to identify as related to an ongoing target session. If information is stored then it shall be possible to set a maximum storage time according to national and/or operator requirements.
Table 7A.3.1 Start of interception with established IMS session event

	Observed SIP URI

	Observed TEL URL

	Observed IMEI

	Event type

	Event Time

	Event Date

	Network Element Identifier

	SIP message header offer (NOTE)

	SIP message header answer (NOTE)

	SDP offer

	SDP answer

	Correlation information


NOTE: The SIP messages that carry the SDP offer and answer shall be reported. In case there are multiple SDP offers/answers during the session establishment, the SIP messages that carry the latest SDP offer/answer shall be provided.

7A.3.1.1
SDES Media Security

If an SDES crypto attribute is included in the SDP, the DF2/MF forwards the “start of interception with IMS session”  event message to the LEMF over HI2 without additional key processing. 

NOTE:
The SDES Crypto attribute contains the cryptographic key required for decrypting the encrypted IMS media.

If SDES mid session support is required then storing of media information as per 7A.3.1 is mandatory. 
7A.4
Multi-media Call State Control Service Scenarios

Annex C shows examples of the delivery of intercepted events and product under various call scenarios.

7A.5
Push to talk over Cellular (PoC)

PoC is a service of the IMS Domain and interception is done according the definitions in clause 7A.3. Interception of CC is available with the current implementations in the GSNs.

7A.6
SMS over IMS

SMS over IMS shall be intercepted in accordance with normal IMS interception as described in 7A.3.  SMS IRI (including originating and destination addresses, SMS direction, and SMS Centre Address) are reported, if available, for IRI-only intercepts.

7A.7
LI for KMS based IMS Media Security

7A.7.1
LI Architecture and functions

KMS based IMS media security is specified in [25]. The present clause specifies LI architecture and functions needed to provide session encryption keys generated by the KMS to protect IMS media for a subscriber who is a target for interception in the IMS nodes. This section is applicable to the cases in which the KMS is under responsibility of the Operator providing the IMS network infrastructure. Other scenarios such as the one in which the KMS is run by an independent legal entity are outside the scope of this specification.

NOTE 1: It is FFS whether the Xk interface defined in this section can be used also by the LEMF to directly query the KMS as an additional option.

NOTE 2: This section covers the scenario in which encrypted content of communication is provided to the LEMF together with encryption keys, to allow decryption at LEMF. 

Figure 7A.7.1 shows the LI architecture for the case in which decryption is performed by the LEMF and a KMS is used to support IMS media security, with a Xk interface defined between the DF2/MF and the KMS, in addition to the interfaces and functional entities needed to support LI in the P-CSCF/S-CSCF.


[image: image34.wmf] 

KMS

 

Xk

 

LEMF

 

Mediation

 

Function

 

Delivery

 

Function 2

 

Mediation

 

Function

 

ADMF

 

X2

 

X1_2

 

X1_1

 

HI1

 

HI2

 

P

-

CSCF/

 

S

-

CSCF

 


Figure 7A.7.1: KMS Intercept configuration

When LI has been activated in the P/S-CSCF for a target, the node will report SIP messages events on the X2 interface, as specified in section 7.A and subsections. The DF2/MF shall extract from the intercepted SIP signaling the information related to the encryption and send a request over the Xk interface to the KMS to derive the encryption keys; the request will carry also the reference to the ticket transferred by the SIP signalling between the parties involved in the communication. The KMS shall then, based on the information received from the DF2, resolve the ticket and provide the session keys to the DF2/MF over the Xk interface.

7A.7.2 
Signalling over the Xk interfaces and LI events

The following messages are defined over the Xk interface:

· get_keys

· get_keys_response

The message get_keys shall be sent by the DF2/MF to the KMS in order to ask the KMS to provide session keys for an ongoing communication.

The message get_keys_response shall be sent by the KMS to the DF2/MF in order to provide the session keys. 

The message get_key_response defines a LI event provided by the KMS to the DF2/MF which shall then be sent by the DF2/MF to the LEMF in a proper IRI record over the HI2 interface. 

Table 7A.7.2.1 provides the list of parameters, which shall be carried by the message get_keys, in order to transfer to the KMS the information, as specified in [25], needed to provide the session encryption keys:

Table 7A.7.2.1 Parameters and information in message get_keys

	Public KMS Identity of the target user

	TRANSFER_INIT

	TRANSFER_RESP


Upon reception of get_keys message, the KMS shall verify that the key management information is related to the targeted user. 
A timer may be defined in the DF2/MF in order to specify the amount of time that the DF2/MF shall wait for the response from the KMS. If this timer expires, a failure indication shall be sent to the LEMF.
Table 7A.7.2.2 provides the list of parameters, which shall be carried by the message get_keys_response, in order to provide the DF2/MF with the session keys:

Table 7A.7.2.2 Parameters and information in message get_keys_response

	Crypto Session ID

	Session key

	Salt

	Failure indication (optional)


With reference to table 7A.7.2.2, in case of failure in providing any of the decryption information, the KMS may provide a decryption failure indication.

Upon reception of get_keys_response message or in case of timer expiry, the following information shall be provided to the LEMF by the DF2/MF:

-
Lawful interception identifier

-
Observed target identity(ies)

-
Correlation number (in order to correlate the keys to IMS session under interception at the CSCF(s))

-
Event type (session encryption keys available) 
-
Crypto Session ID (if provided by the KMS)

-
Session key (if provided by the KMS)

-
Salt (if provided by the KMS)

-
MediaSec key retrieval failure indication (in case of e.g. timer expiry, or failure indication received from the KMS).
7A.7.3
Cooperating KMSs

As specified in [25], in some scenarios the parties involved in an encrypted IMS based communication may use two different KMSs. In these cases, no additional LI specific signalling between the KMSs shall take place. The KMS may need to cache the session keys retrieved as result of the ticket resolution for possible LI needs at later stage.

7A.7.4
Security

Xk interface and its configuration shall only be accessible to authorized personnel.

The Xk interface shall have strong integrity and confidentiality protection. The Xk interface shall be protected by TLS  unless protected by IPsec for LI purposes. TLS and certificate profiling shall be according to TS 33.310 [28]; IPsec profiling shall be according to TS 33.310 [28] and TS 33.210 [29]. 

7A.7.5
Start of interception for an already established IMS media secured session

This function is invoked when LI is activated in the network for a target who has already established an IMS session with secure media.

In order to provide information needed to decrypt the content of communication, the LI function in the CSCFs needs to have access to SDP information and SIP headers exchanged in the SIP signalling between the parties during the IMS session setup for possible later retrieval in case LI is activated during the ongoing session.

With reference to fig. 7A.7.1, if LI is activated by the ADMF over the X1_1 interface for a target, the CSCF shall check if the given target has an ongoing IMS media secured session. In this case, the CSCF shall provide a “Start of interception with established IMS  session” event message to the DF2/MF over the X2 interface, as specified in section 7A.3.1. 

Upon reception of Start of interception with established IMS secure session event, the DF2/MF shall check if a MIKEY-TICKET is included in the SDP. In this case the DF2/MF, in addition to forwarding the event to the LEMF over HI2, shall contact the KMS to resolve the ticket and retrieve the session keys and additional encryption related information as specified in in section 7A.7.2.

7A.8
IMS IMEI Interception

The use of Instance ID in TS 24.229[49] is mandatory in IMS in order to support IMEI based LI in the CSCF. The CSCF is required to have access to the Instance IDs for all active IMS registrations regardless of whether LI has been requested prior to UE registration for a given IMEI. The CSCF shall be responsible for extracting the IMEI from the Instance ID where required for a specific target interception and providing the IMEI to the DF2. The IMEI (when available) shall be provided by the CSCF to the DF2 for all intercepted communication regardless of whether the IMEI or another identifier has been used as the target for interception. 
Based on the national regulations, IMEI-based LI shall be possible for IMS sessions originated from, or terminated to, the UE with that IMEI.
8
Security

8.0
General
The security requirements are valid for the whole Lawful Interception system, i.e. rules and procedures shall be used for all involved entities, 3G GSN and the DF.

8.1
Administration security

The administration of the LI function, i.e. Activation, Deactivation and Interrogation of Lawful Interception, in the 3G ICEs and the DFs shall be done securely as described below:

-
It shall be possible to configure the authorised user access within the serving network to Activate, Deactivate and Interrogate Lawful Interception separately for every physical or logical port at the 3G ICEs and DF. It shall be possible to password protect user access.

-
Only the ADMF is allowed to have access to the LI functionality in the 3G ICEs and DF.

-
The communication links between ADMF, 3G GSN,3G MSC Server, CSCF, DF2, and DF3 may be required by national option to support security mechanisms. Options for security mechanisms include:

-
CUG / VPN;

-
COLP;

-
CLIP;

-
authentication;

-
encryption.

Through the use of user access restrictions, no unauthorised network entities or remote equipment shall be able to view or manipulate LI data in the 3G GSN, 3G MSC Server, CSCF or the DFs.

8.2
IRI security

8.2.1
Normal operation

The transmission of the IRI shall be done in a secure manner.

When DFs are physically separate from the 3G ICEs, the X2-interface may be required by national option to support security mechanisms. Options for security mechanisms include:

-
CUG/VPN;

-
COLP;

-
CLIP;

-
authentication;

-
encryption.

8.2.2
Communication failure

Depending on the national law in case of communication failure IRI may be buffered in the 3G INEs. After successful transmission of IRI the whole buffer shall be deleted. It shall be possible to delete the content buffer via command or a timer, in an un-restorable fashion.

8.3
CC security

The transmission of the CC shall be done in a secure manner.

When DFs are physically separate from the 3G INEs, the X3-interface may be required by national option to support security mechanisms. Options for security mechanisms include:

-
CUG/VPN;

-
COLP;

-
CLIP;

-
authentication;

-
encryption.

In case of transmission failure no buffering is required within the intercepting network.

8.4
Security aspects of Lawful Interception billing

Billing information may be suppressed or made available at the DFs and the ADMF. Billing information for Lawful Interception shall be separated from "regular" billing data.

Billing data transmission to the Lawful Interception billing system may be done in a secure manner per national option.

In case of transmission failure billing-data shall be buffered/stored in a secure way. After successful transmission billing data shall be deleted in an un-restorable fashion.

8.5
Other security issues

8.5.1
Log files

Log files shall be generated by the ADMF, DF2, DF3, 3G MSC Server, CSCF and the 3G GSN. All log files are retrievable by the ADMF, and are maintained by the ADMF in a secure manner.

8.5.2
Data consistency

The administration function in the 3GMS shall be capable of performing a periodic consistency check to ensure that the target list of target identities in all involved 3G MSC Servers, CSCFs, 3G GSNs in the 3GMS and the DFs contain the appropriate target Ids consistent with the intercept orders in the ADMF. The reference data base is the ADMF data base.

9
Invocation of Lawful Interception for 3GPP WLAN Interworking Services

9.0
General

Figure 23 shows the extract from the reference configuration which is relevant for the invocation of the Lawful Interception of the packet data 3GPP WLAN Interworking network.


[image: image35]
Figure 23: Functional model for invocation of Lawful Interception for 3GPP WLAN Interworking Services

The HI2 and HI3 interfaces represent the interfaces between the LEA and two delivery functions. Both interfaces are subject to national requirements. They are included for completeness, but are beyond the scope of this specification.

The delivery functions are used:

-
to convert the information on the X2-interface to the corresponding information on the HI2 interface;

-
to distribute the intercept related information to the relevant LEA(s);

-
to distribute the intercept product to the relevant LEA(s).

Interception at a WAG applies for the roaming users where the PDG is not in the visited network.

For most WLAN Interworking cases, the Packet Data Gateway (PDG) handles the bearer level interception, specifically interception of CC and IRI related to tunnel establishment and release in which case there is no need to perform interception at a WAG.  This includes the case where the PDG is in the intercepting carrier’s network (whether it be home or visited).  For the case where a visited network is to intercept WLAN related tunnel and the PDG for the tunnel is not in the visited network, the Wireless Access Gateway (WAG) is used to intercept the CC and IRI related to tunnel establishment and release. It should be noted that the CC available at the WAG may be encrypted.

9.1
Provision of Intercept Product - Short Message Service

LI for SMS in the 3GPP-WLAN Interworking case is described in Clause 7A.4. 

9.2
Provision of Intercepted Content of Communications - 3GPP WLAN Interworking services

9.2.0
General

The access method for the delivering of 3GPP WLAN Interworking Intercept Product is based on duplication of packets without modification at the PDG or WAG. The duplicated packets with additional information in the header, as described in the following sections, are sent to DF3 for further delivery. Note that CC available at the WAG is likely to be encrypted.

[image: image36]
Figure 24: Configuration for interception of 3GPP WLAN Interworking product data

9.2.1
X3-interface

In addition to the intercepted content of communications, the following information needs to be transferred from the PDG or WAG to the DF3 in order to allow the DF3 to perform its functionality:

-
target identity;

-
correlation number;

-
time stamp - optional;

-
direction (indicates whether T-PDU is MO or MT) - optional;

-
the target location (if available in the intercepting node).

9.3
Provision of Intercept Related Information

9.3.0
General
Figure 25 shows the transfer of intercept related information to the DF2. If an event for / from a mobile subscriber occurs, the PDG, WAG, or the AAA Server sends the relevant data to the DF2. Packet Header Information Reporting is a national option. For Packet Header Information Reporting, a PDG/WAG either isolates the relevant data and sends it to the DF2 or sends the packet stream to another entity in the network (e.g., DF3) for isolation which then provides the relevant data to the DF2.  




[image: image38.emf] 

AAA Server  

  PDG/WAG  

Other   Party  

Target  

LEMF  

DF2  


Figure 25: Provision of Intercept Related Information

9.3.1
X2-interface

The following information needs to be transferred from the PDG, WAG or the AAA server to the DF2 in order to allow a DF2 to perform its functionality:

-
target identity (IMSI, NAI, or MSISDN);

-
events and associated parameters as defined in section 9.3.2 may be provided;

-
the target location (if available);

-
Correlation number;

-
Quality of Service (QoS) identifier (if available).

The IRI should be sent to DF2 using a reliable transport mechanism. 
The PDG/WAG detects packets containing packet header information in the communications path but the information needed for Packet Header Information Reporting may need to be transferred from the PDG/WAG either directly to the DF2 or via another network entity in order to allow the DF2 to perform its functionality. 
9.3.2
3GPP WLAN Interworking LI Events and Event Information

The following events are applicable to AAA Server:

-
I-WLAN Access Initiation;

-
I-WLAN re-authentication, 

-
I-WLAN Access Termination;

-
I-WLAN Tunnel Establishment;

-
I-WLAN Tunnel Disconnect;

-
Start of Intercept with I-WLAN Communication Active;

The following events are applicable to the PDG and WAG:

-
I-WLAN Tunnel Establishment;

-
I-WLAN Tunnel Disconnect;

-
Start of Intercept with I-WLAN Communication Active. 
-
Packet Header Information Reporting.

A set of possible elements as shown below is used to generate the events. Information associated with the events are transmitted from the PDG, WAG or AAA server to DF2.

NOTE:
Some of these parameters apply to the PDG or WAG and some apply to the AAA server.  Parameters sent from the PDG, WAG or AAA server is dependent on what is available at the network element. If interception is performed at the PDG, then Packet Header Information Reporting shall also be performed at the PDG and not at the WAG.
Table 3: Information Events for WLAN Interworking Event Records

	Element
	PDG
	AAA Server

	Observed MSISDN

MSISDN of the target.
	Available, see TS 29.234
	Available, see TS 29.234

	Observed NAI

NAI of the target. 
	Not available
	Available, see TS 29.234

	Observed IMSI

IMSI of the target.
	Available, see TS 24.234
	Available, see TS 29.234

	Event type

Description which type of event is delivered: I-WLAN Access Initiation, I-WLAN Access Termination, I-WLAN Tunnel Establishment, I-WLAN Tunnel Disconnect, Start of Intercept with I-WLAN Communication Active, Packet Data Header Information.
	Available from ICE
	Available from ICE

	Event date

Date of the event generation in the PDG or the AAA server.
	Available from ICE
	Available from ICE

	Event time

Time of the event generation in the PDG or the AAA server. Timestamp shall be generated relative to the PDG or AAA server internal clock.
	Available from ICE
	Available from ICE

	WLAN UE Local IP address

The WLAN UE Local IP address of observed party.  The WLAN UE Local IP address field specified in TS 24.234 and IETF RFC 2409, represents the IPv4/IPv6 address of the WLAN UE in the WLAN AN.  It is an address used to deliver the packet to a WLAN UE in a WLAN AN.  Note that this address might be dynamic.
	Available, see TS 24.234 and IETF RFC 2409
	Not available

	WLAN UE MAC address

The WLAN MAC address of the target. Note that this address might be dynamic and the validity of the MAC Address is outside of the scope of 3GPP.
	Not available
	Available, see TS 29.234

	WLAN UE Remote IP address

The WLAN UE Remote IP address of observed party.  The WLAN UE Remote IP address field specified in TS 24.234, represents the IPv4/IPv6 address of the WLAN UE in the network being accessed by the WLAN AN.  It is an address used in the data packet encapsulated by the WLAN UE-initiated tunnel and is the source address used by applications in the WLAN UE.  Note that this address might be dynamic.
	Available, see TS 24.234
	Not available

	WLAN Access Point Name

The W-APN of the access point. 
	Available, see TS 24.234
	Available, see TS 29.234

	WLAN Operator Name

The name of the WLAN operator name serving the target. 
	Not available
	Available, see TS 29.234

	WLAN Location Data
The location of the WLAN serving the target (e.g., string like "coffee shop" or "airport", etc.).
	Not available
	Available, see TS 29.234

	WLAN Location Information

Location Information regarding the WLAN as provided in RADIUS or DIAMETER signalling exchanged with the AAA server. 
	Not available
	Available, see TS 29.234

	Correlation Number

The correlation number is used to correlate CC and IRI. The correlation number is also used to allow the correlation of IRI records.  In case of the AAA server, the Correlation Number is only used to correlate IRI records.
	Generated for LI by PDG
	Generated for LI by AAA server

	Network Element Identifier

Unique identifier for the element reporting the ICE.
	Generated for LI by PDG
	Generated for LI by AAA server

	Initiator

The initiator of the request either the network or the WLAN UE.
	Generated for LI by PDG
	Generated for LI by AAA server

	NAS IP/IPv6 address

The IP or IPv6 address of the NAS in the WLAN.
	Not available
	Available, see TS 29.234

	Visited PLMN ID

Identity of the visited PLMN to which the user is terminating their WLAN tunnels or through which the user is establishing their WLAN tunnels.
	Not available
	Available, see TS 29.234

	Session Alive Time

The amount of time in seconds during which the target can be registered for WLAN access.
	Not available
	Available, see TS 29.234

	Failed access reason

Provides the reason for why a WLAN access attempt failed ("Authentication Failed").
	Not available
	Available from ICE

	Session termination reason

Provides a reason for why a WLAN access session is terminated.
	Not available
	Available, see TS 29.234

	Failed tunnel establishment reason

Provides a reason for why a WLAN tunnel establishment failed ("Authentication failed" or "Authorization failed").
	Available from ICE
	Available from ICE

	NSAPI

Network layer Service Access Point Identifier

The NSAPI information element contains an NSAPI identifying a PDP Context in a mobility management context specified by the Tunnel Endpoint Identifier Control Plane.

This is an optional parameter to help DF/MF and LEA's to distinguish between the sending mobile access networks
	Optional available according 23.234 Annex F; defined 29.060 7.7.17
	Not available

	Destination IP Address

The IP address, including type IPv4 or IPv6, of the destination of the IP packet.
	Available from ICE
	Available from ICE

	Destination Port Number

The port number of the destination of the IP packet.
	Available from ICE
	Available from ICE

	Flow Label (IPv6 only)

The field in the IPv6 header that is used by a source to label packets of a flow (see RFC 3697 [41]).
	Available from ICE
	Available from ICE

	Packet Count 

The number of packets detected and reported (for a particular summary period).
	Available from ICE
	Available from ICE

	Packet Data Summary Reason

The reason for a Packet Data Summary message being sent to the LEMF (e.g., timed out, counter expiration, end of session)
	Available from ICE
	Available from ICE

	Packet Size

The size of the packet. (i.e., Total Length Field in IPv4 or Payload Length field in IPv6)
	Available from ICE
	Available from ICE

	Source IP Address

The IP address, including type IPv4 or IPv6, of the source of the IP packet.
	Available from ICE
	Available from ICE

	Source Port Number

The port number of the source of the IP packet.
	Available from ICE
	Available from ICE

	Sum of Packet Sizes (for a particular summary period)

The sum of values contained in the Total Length fields of the IPv4 packets or the sum of the values contained in the Payload Length fields of the IPv6 packets.
	Available from ICE
	Available from ICE

	Summary Period

Includes the dates and times of the first and last packets in a particular packet data interval.  
	Available from ICE
	Available from ICE

	Transport Protocol (e.g., TCP)

The identification of the transport protocol of the packet or packet flow being reported.
	Available from ICE
	Available from ICE


Table 3a: Information Events for WLAN Interworking Event Records - WAG

	Element
	WAG

	Observed MSISDN

MSISDN of the target.
	Available, see 3GPP TS 29.234

	Observed IMSI

IMSI of the target.
	Available, see 3GPP TS 29.234 

	Event type

Description which type of event is delivered: I-WLAN Tunnel Establishment, I-WLAN Tunnel Disconnect, Start of Intercept with I-WLAN Communication Active, Packet Data Header Information.
	Available from ICE

	Event date

Date of the event generation in the PDG/WAG or the AAA server.
	Available from ICE

	Event time

Time of the event generation in the PDG/WAG or the AAA server. Timestamp shall be generated relative to the PDG/WAG or AAA server internal clock.
	Available from ICE

	WLAN UE IP address

The WLAN UE IP addess of observed party.  The WLAN UE IP address field contains the IPv4/IPv6 address (specified by 3GPP TS 29.234) of the WLAN UE tunnel endpoint as seen by the WAG.  Note that this address might be dynamic.
	Available, see 3GPP TS 29.234 

	WLAN PDG Tunnel Endpoint IP address

The WLAN PDG Tunnel Endpoint IP address field contains the IPv4/IPv6 address of the PDG (as specified in 3GPP TS 29.234) as seen by the WAG. Note that this address might be dynamic.
	Available, see 3GPP TS 29.234

	WLAN Access Point Name

The W-APN of the access point. 
	Available, see 3GPP TS 29.234



	Correlation Number

The correlation number is used to correlate CC and IRI. The correlation number is also used to allow the correlation of IRI records.  
	Generated for LI by WAG

	Network Element Identifier

Unique identifier for the element reporting the ICE.
	Generated for LI by WAG

	NAS IP/IPv6 address

The IP or IPv6 address of the NAS in the WLAN.
	Available, see 3GPP TS 29.234



	Tunnel Protocol

The Tunnel Protocol as defined in the Routing-Policy AVP in 3GPP TS 29.234.
	Available, see 3GPP TS 29.234

	Source Ports

The list or range of source ports as specified in the Routing-Policy AVP provided by the AAA server in 3GPP TS 29.234.
	Available, see 3GPP TS 29.234

	Destination Ports

The list or range of destination ports as specified in the Routing-Policy AVP provided by the AAA server in 3GPP TS 29.234.
	Available, see 3GPP TS 29.234

	Session Alive Time

The amount of time in seconds during which the target can be registered for WLAN access.
	Available, see 3GPP TS 29.234



	Destination IP Address

The IP address, including type IPv4 or IPv6, of the destination of the IP packet.
	Available from ICE

	Destination Port Number

The port number of the destination of the IP packet.
	Available from ICE

	Flow Label (IPv6 only)

The field in the IPv6 header that is used by a source to label packets of a flow (see RFC 3697 [41]).
	Available from ICE

	Packet Count 

The number of packets detected and reported (for a particular summary period).
	Available from ICE

	Packet Data Summary Reason

The reason for a Packet Data Summary message being sent to the LEMF (e.g., timed out, counter expiration, end of session)
	Available from ICE

	Packet Size

The size of the packet. (i.e., Total Length Field in IPv4 or Payload Length field in IPv6)
	Available from ICE

	Source IP Address

The IP address, including type IPv4 or IPv6, of the source of the IP packet.
	Available from ICE

	Source Port Number

The port number of the source of the IP packet.
	Available from ICE

	Sum of Packet Sizes (for a particular summary period)

The sum of values contained in the Total Length fields of the IPv4 packets or the sum of the values contained in the Payload Length fields of the IPv6 packets.
	Available from ICE

	Summary Period

Includes the dates and times of the first and last packets in a particular packet data interval.  
	Available from ICE

	Transport Protocol (e.g., TCP)

The identification of the transport protocol of the packet or packet flow being reported.
	Available from ICE


9.4
Structure of I-WLAN Events

9.4.1
I-WLAN Access Initiation

For I-WLAN Access Initiation including I-WLAN re-authentication, for both  I-WLAN Access Initiation-event is generated. The elements, shown in Table 4, will be delivered to the DF2, if available, by the AAA server.

Table 4: I-WLAN Access Initiation – AAA Server

	Observed MSISDN 

	Observed IMSI

	Observed NAI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	WLAN Operator Name

	WLAN LocationData

	WLAN Location Information

	NAS IP/IPv6 Address

	WLAN UE MAC Address

	Visited PLMN ID

	Session Alive Time

	Failed Access reason


9.4.2
WLAN Access Termination

For WLAN Access Termination or the immediate purging of a user from a WLAN access, a WLAN access termination-event is generated. The elements, shown in Table 5, will be delivered to the DF2, if available, by the AAA server.

Table 5: I-WLAN Access Termination – AAA Server

	Observed MSISDN 

	Observed IMSI

	Observed NAI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	WLAN Operator Name

	WLAN Location Data

	WLAN Location Information

	NAS IP/IPv6 Address

	WLAN UE MAC Address

	Session Termination reason


9.4.3
I-WLAN Tunnel Establishment

For I-WLAN Tunnel Establishment, a I-WLAN tunnel establishment-event is generated. The elements, shown in Table 6, 6a, and Table 7, will be delivered to the DF2 if available, by the PDG, WAG or AAA server, respectively.

Table 6: I-WLAN Tunnel Establishment - PDG

	Observed MSISDN

	Observed IMSI

	Observed NAI

	Event Type 

	Event Time

	Event Date

	Correlation number

	WLAN UE Local IP address

	WLAN UE Remote IP address

	WLAN Access Point Name

	Network Element Identifier

	Failed tunnel establishment reason

	NSAPI (optional)


Table 6a: I-WLAN Tunnel Establishment - WAG
	Observed MSISDN

	Observed IMSI

	Event Type 

	Event Time

	Event Date

	Correlation number

	WLAN UE IP address

	WLAN PDG Tunnel Endpoint IP address

	WLAN Access Point Name

	NAS IP/IPv6 address

	Tunnel Protocol

	Source Ports

	Destination Ports

	Session Alive Time

	Network Element Identifier


Table 7: I-WLAN Tunnel Establishment - AAA Server

	Observed MSISDN

	Observed IMSI

	Observed NAI

	Event Type 

	Event Time

	Event Date

	Correlation number

	WLAN Access Point Name

	Network Element Identifier

	Visited PLMN ID

	Failed tunnel establishment reason


9.4.4
I-WLAN Tunnel Disconnect

At I-WLAN Tunnel Disconnect, a I-WLAN tunnel disconnect event is generated. The elements, shown in Table 8, 8a, and Table 9, will be delivered to the DF2, if available, by the PDG, WAG or AAA server, respectively.

Table 8: I-WLAN Tunnel Disconnect - PDG
	Observed MSISDN 

	Observed IMSI

	Observed NAI

	Event Type

	Event Time

	Event Date

	Correlation number

	WLAN UE Local IP Address

	WLAN UE Remote IP address

	WLAN Access Point Name

	Network Element Identifier

	Initiator (optional)


Table 8a: I-WLAN Tunnel Disconnect – WAG
	Observed MSISDN 

	Observed IMSI

	Event Type

	Event Time

	Event Date

	Correlation number

	WLAN UE IP address

	WLAN PDG Tunnel Endpoint IP address

	WLAN Access Point Name

	NAS IP/IPv6 address

	Tunnel Protocol

	Source Ports

	Destination Ports

	Network Element Identifier


Table 9: I-WLAN Tunnel Disconnect - AAA Server

	Observed MSISDN 

	Observed IMSI

	Observed NAI

	Event Type

	Event Time

	Event Date

	Correlation number

	Tunnel address of observed party

	WLAN Access Point Name

	Network Element Identifier

	Initiator (optional)


9.4.5
Start of Intercept with I-WLAN Communication Active

This event will be generated if interception for a target is started and if the target has one or more active I-WLAN Access sessions or one or more I-WLAN Tunnels established. The elements, shown in Table 10,10a, and Table 11, will be delivered to the DF2, if available, by the PDG, WAG or AAA server, respectively.

Table 10: Start of Intercept with I-WLAN Communication Active - PDG
	Observed MSISDN 

	Observed IMSI

	Observed NAI

	Event Type

	Event Time

	Event Date

	Correlation Number

	WLAN UE Local IP Address

	WLAN UE Remote IP address

	WLAN Access Point Name

	Network Element Identifier


Table 10a: Start of Intercept with I-WLAN Communication Active – WAG
	Observed MSISDN

	Observed IMSI

	Event Type 

	Event Time

	Event Date

	Correlation number

	WLAN UE IP address

	WLAN PDG Tunnel Endpoint IP address

	WLAN Access Point Name

	NAS IP/IPv6 address

	Tunnel Protocol

	Source Ports

	Destination Ports

	Session Alive Time

	Network Element Identifier


Table 11: Start of Intercept with I-WLAN Communication Active - AAA Server

	Observed MSISDN 

	Observed IMSI

	Observed NAI

	Event Type

	Event Time

	Event Date

	Correlation Number

	WLAN Access Point Name

	Network Element Identifier

	WLAN Operator Name

	WLAN Location Data

	WLAN Location Information

	NAS IP/IPv6 address

	Visited PLMN ID


9.4.6
Packet Header Information Reporting

9.4.6.0
Introduction
Packet Header Information Reporting can be done either on a per-packet (i.e., non-summarized) basis or in a summary report.  

9.4.6.1
Packet Data Header Report

This event is used to provide packet header reports on a per packet basis (non-summarized reporting) and is triggered by each packet sent or received by the target.  These elements will be delivered either directly to DF2 or via another network entity if available:

Table A: I-WLAN Packet Header Information Reporting - PDG
	Observed MSISDN 

	Observed IMSI

	Observed NAI

	Event Type

	Event Time

	Event Date

	Correlation number

	WLAN UE Local IP Address

	WLAN UE Remote IP address

	WLAN Access Point Name

	Network Element Identifier

	Initiator (optional)

	Source IP Address

	Source Port Number

	Destination IP Address

	Destination Port Number

	Transport Protocol (e.g., TCP)

	Packet Size

	Flow Label (IPv6 only)


Table B: I-WLAN Packet Header Information Reporting – WAG
	Observed MSISDN 

	Observed IMSI

	Event Type

	Event Time

	Event Date

	Correlation number

	WLAN UE IP address

	WLAN PDG Tunnel Endpoint IP address

	WLAN Access Point Name

	NAS IP/IPv6 address

	Tunnel Protocol

	Source Ports

	Destination Ports

	Network Element Identifier

	Source IP Address

	Source Port Number

	Destination IP Address

	Destination Port Number

	Transport Protocol (e.g., TCP)

	Packet Size

	Flow Label (IPv6 only)


9.4.6.2
Packet Data Summary Report

This event is used to report:

1)
the source and destination information derived from the packet headers, including:

a)
source and destination IP Addresses,

b)
IP next-layer protocol,

c)
Layer-4 ports, and

d)
Flow label, if the packet is IPv6

2)
summary information for the number of packets and bytes transmitted or received by the target for each unique packet flow within a WLAN tunnel, and 

3)
the date and the time of the first and last packets associated with that packet flow. A packet flow is defined as the 6-tuple of source/destination IP address/port number and the layer 4 protocol and WLAN tunnel.   
IP addresses and the IP next-layer protocol are always reported, the flow label is reported if the packet is IPv6, and the layer-4 ports are reported.    

The event provides packet summary reports for each unique packet data session (PDP context) and packet flow, and is triggered by one of the following:

-
start of a packet flow associated with a WLAN Tunnel

-
an interim report for a packet flow associated with a WLAN Tunnel is to be reported

-
end of a packet flow associated with a WLAN Tunnel (including end of the WLAN Tunnel itself).

An interim report can be triggered by

-
The expiration of a configurable timer per intercept (called a Summary Timer). The Summary Timer is configurable in units of seconds;

-
A per-intercept configurable count threshold is reached.

These elements will be delivered either directly to DF2 or via an MF for each packet flow if available:
Table C: I-WLAN Packet Header Summary Reporting - PDG
	Observed MSISDN 

	Observed IMSI

	Observed NAI

	Event Type

	Event Time

	Event Date

	Correlation number

	WLAN UE Local IP Address

	WLAN UE Remote IP address

	WLAN Access Point Name

	Network Element Identifier

	Initiator (optional)

	Source IP Address

	Source Port Number

	Destination IP Address

	Destination Port Number

	Transport Protocol (e.g., TCP)

	Flow Label (IPv6 only)

	Summary Period

	Packet Count (for this summary period)

	Sum of Packet Sizes (for this summary period)


Table D: I-WLAN Packet Header Summary Reporting – WAG
	Observed MSISDN 

	Observed IMSI

	Event Type

	Event Time

	Event Date

	Correlation number

	WLAN UE IP address

	WLAN PDG Tunnel Endpoint IP address

	WLAN Access Point Name

	NAS IP/IPv6 address

	Tunnel Protocol

	Source Ports

	Destination Ports

	Network Element Identifier

	Source IP Address

	Source Port Number

	Destination IP Address

	Destination Port Number

	Transport Protocol (e.g., TCP)

	Flow Label (IPv6 only)

	Summary Period

	Packet Count (for this summary period)

	Sum of Packet Sizes (for this summary period)


If the packets are IPv4, the sum of all observed packet sizes is the sum of the values contained in the Total Length field of each packet as specified in IETF RFC 791[39].
If the packet is IPv6, the sum of all observed packet sizes is the sum of the values contained in the Payload Length field for each packet as specified in IETF RFC 2460 [40].
If no packets were detected for the duration of the Summary Timer, then the Packet Data Summary Report shall not be sent.
10
Interception of Multimedia Broadcast/MultiCast Service (MBMS)

10.0
General

MBMS provides video or similar streamed services via either point to point multicast or cell broadcast mechanisms between an operator content server (BM-SC) and UEs as defined in TS 23.246 [20]. This section details the stage 2 Lawful Interception requirements for MBMS.

NOTE:
Generic Broadcast services where the UE receives the broadcast in IDLE mode and there is no subscription relationship between the UE and the BM-SC are out of scope. In addition 3rd party BM-SC services where the operator is not responsible for content encryption and subscription management are out of scope. 
Figure 10.1 shows the extract from the reference configuration which is relevant for the invocation of the Lawful Interception of the MBMS Services.

[image: image39]
Figure 10.1: Functional model for invocation of Lawful Interception for MBMS Services

10.1
Provision of Content of Communications

Interception of the content of communications for MBMS services if available, may be provided by the underlying transport bearer interception functionality (e.g. GSN, PDG or NGN network) and is therefore subject to the current transport bearer interception functionality detailed in other parts of this specification. 

10.2
Provision of Intercept Related Information

10.2.0
General

Figure 10.2 shows the transfer of intercept related information to the DF2. If an event for / from a mobile subscriber occurs, the BM-SC shall send the relevant data to the DF2.


[image: image40]
Figure 10.2: Provision of Intercept Related Information

10.2.1
X2-interface

The following information needs to be transferred from the BM-SC to the DF2 in order to allow a DF2 to perform its functionality:

-
target identity;

-
events and associated parameters as defined in clauses 10.3.2 may be provided;

-
For Further Study:- Encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.
The IRI should be sent to DF2 using a reliable transport mechanism.

10.2.2
MBMS LI Events and Event Information

Intercept Related Information (Events) are necessary are necessary for the following;

· Service Joining. 

· Service Leaving

· Start of Interception with Service Active 

· Subscription Activation 

· Subscription Modification 

· Subscription Termination

Events shall include changes resulting from direct communication between the UE and BM-SC and off-line subscription changes (e.g. changes made by operator customer services on behalf of the subscriber).

A set of possible elements as shown in Table 10.2.2 are used to generate the events.

Table 10.2.2: Information Events for MBMS Event Records

	Element

	Observed IMSI

IMSI of the target.

	Observed Other Identity

Other Identity of the target.

	Event type

Description which type of event is delivered:- Service Joining; Service Leaving; Subscription Activation; Subscription Modification; Subscription Termination.

	Event date

Date of the event generation in the BM-SC.

	Event time

Time of the event generation in the BM-SC. Timestamp shall be generated relative to the BM-SC server internal clock.

	MBMS Subscribed Service

Details of the MBMS Service to which the target has subscribed.

	MBMS Service Joining Time

Requested MBMS Service Joining Time

	MBMS Service Subscription List

List of all users subscribed to MBMS Service to which target has requested Joining. 

	Correlation Number

The correlation number is used to correlate CC and IRI. The correlation number is also used to allow the correlation of IRI records.  

	Network Element Identifier

Unique identifier for the element reporting the ICE.

	Initiator

The initiator of the request either the UE or Off-line BM-SC access (eg customer services agent or internet).

	Visited PLMN ID

Identity of the visited PLMN to which the user is registered

	APN

Access Point Name on which this IP multicast address is defined.

	Multicast/Broadcast Mode

MBMS bearer service in broadcast or multicast mode

	IP IP/IPv6 multicast address(multicast mode only)

IP or IPv6  multicast address identifying the MBMS bearer described by this MBMS Bearer Context.

	List of Downstream Nodes

List of downstream nodes that have requested the MBMS bearer service and to which notifications and MBMS data have to be forwarded.

	MBMS Leaving Reason

Indicates whether UE initiated/requested leaving, or whether BM-SC/network terminated the Service to the UE (e.g. GSN session dropped or BM-SC subscription expired etc).


NOTE: Generation of Correlation Number is FFS. 

10.3
Structure of MBMS Events

10.3.1
Service Joining

For MBMS Service Joining, a Service Joining event is generated. The elements, shown in Table 10,3.1 will be delivered to the DF2, if available, by the BM-SC. A new Service Joining Event shall be generated for each individual service joined.

Table 10.3.1: Service Joining
	Observed IMSI

	Event Type

	Event Time

	Event Date

	MBMS Subscribed Service

	MBMS Service Joining Time

	Network Element Identifier

	Initiator

	IP/IPv6 Multicast Address 




(If Applicable)

	Visited PLMN ID 







(If Applicable)

	Multicast/Broadcast Mode

	APN (If Available)

	List of Downstream Nodes 




  (If Available)

	MBMS Service Subscription List 



(Optional)


10.3.2
Service Leaving

For MBMS Service Leaving, a Service Leaving event is generated. The elements, shown in Table 10.3.2 will be delivered to the DF2, if available, by the BM-SC. A new Service Leaving Event shall be generated for each individual service leaving.

Table 10.3.2: Service Leaving
	Observed IMSI

	Event Type

	Event Time

	Event Date

	MBMS Subscribed Service

	Network Element Identifier

	Initiator

	IP/IPv6 Multicast Address 



     (If Applicable)

	Visited PLMN ID 






     (If Applicable)

	MBMS Service Subscription List 



(Optional)

	MBMS Service Leaving Reason


10.3.3
Start of Interception with Service Active

For Start of Interception where MBMS Service Joining has already occurred prior to start of interception, a Start of Interception with Service Active event is generated. The elements, shown in Table 10.3.3 will be delivered to the DF2, if available, by the BM-SC. A new Start of Interception with Service Active Event shall be generated for each individual service the target is subscribed to.

Table 10.3.3: Start of Interception with Service Active

	Observed IMSI

	Event Type

	Event Time

	Event Date

	MBMS Subscribed Service

	MBMS Service Joining Time

	Network Element Identifier

	Initiator

	IP/IPv6 Multicast Address 



     (If Applicable)

	Visited PLMN ID 






     (If Applicable)

	Multicast/Broadcast Mode

	APN (If Available)

	List of Downstream Nodes 




 (If Available)

	MBMS Service Subscription List 



(Optional)


10.3.4
Subscription Activation

For MBMS Subscription Activation, a Subscription Activation event is generated. The elements, shown in Table 10.3.4 will be delivered to the DF2, if available, by the BM-SC. If Subscription Activation is performed simultaneously for more than one service, a separate event shall be generated for each service activated.

Table 10.3.4: Subscription Activation

	Observed IMSI

	Event Type

	Event Time

	Event Date

	MBMS Subscribed Service

	Network Element Identifier

	Initiator

	IP/IPv6 Address 






     (If Applicable)

	Visited PLMN ID 






     (If Applicable)

	MBMS Service Subscription List 



(Optional)


10.3.5
Subscription Modification

For MBMS Subscription Modification, a Subscription Modification event is generated. The elements, shown in Table 10.3.5, will be delivered to the DF2, if available, by the BM-SC. If Subscription Modification is performed simultaneously for more than one service, a separate event shall be generated for each service modified.

Table 10.3.5: Subscription Modification

	Observed IMSI

	Event Type

	Event Time

	Event Date

	MBMS Subscribed Service

	Network Element Identifier

	Initiator

	IP/IPv6 Address 






     (If Applicable)

	Visited PLMN ID 






     (If Applicable)

	MBMS Service Subscription List 



(Optional)


10.3.6
Subscription Termination

For MBMS Subscription Termination, a Subscription Termination event is generated. The elements, shown in Table 10.3.6 will be delivered to the DF2, if available, by the BM-SC. If Subscription Termination is performed simultaneously for more than one service, a separate event shall be generated for each service performed.

Table 10.3.6: Subscription Modification

	Observed IMSI

	Event Type

	Event Time

	Event Date

	MBMS Subscribed Service

	Network Element Identifier

	Initiator

	IP/IPv6 Address 







(If Applicable)

	Visited PLMN ID 







(If Applicable)

	MBMS Service Subscription List 



(Optional)


11
IMS Conference Services

11.1
Background for IMS Conference Services

The entire clause 11 is a national option and is subject to national regulations. The covered cases are where the conference services are in the domain of the intercepting operator. The following cases are covered.

1. A target’s conference call is the target. This may be where the target is the head of the conference. IRI and CC for this conference is reported. The following are examples of information that is reported.

a. For example, the starting and ending of a conference as well as any parties joined or removed from the conference call are reported.

b. Reporting of CC for held conferences initiated by the target.

2. A conference that itself is directly the target of interception.  This case is applicable only provided that the conference is identified by a proper identity for LI in IMS domain (Conference URI or Conference Factory URI).  The IRI and CC for this conference is reported.

a. For example, the starting and ending or a conference as well as any parties joined or removed from the conference call are reported.

The case when an target joins an associate’s conference is for further study.

The key elements for interception of conference services are the AS/MRFC and MRFP. IRI associated with the conference services that are to be intercepted is reported by the AS/MRFC while the CC associated with the conference service is reported by the MRFP.  

11.1A
Start of Interception for IMS Conference Services

Interception (as defined in 11.1) for IMS Conference Services is started when the first of any one of the following occurs:

-
When a target requests that a conference be created

-
When a target successfully provisions a conference 

-
When a target provisioned or requested conference is started (i.e., the first party is joined to the conference)

-
When a conference that is a target of interception is started (i.e., the first party is joined to the conference)

-
When interception is activated (on a conference or a conference owner) during an ongoing conference 

-
When parties have joined a conference and communication is started or enabled by the conference server in cases where the conference is a target of interception or when it is a target’s conference.  

If the target of interception has provisioned or requested a conference to be created, interception on IMS Conference Services shall begin regardless whether the target of interception has joined the conference. Interception of IMS Conference Services shall continue if the target of interception is on hold and the conference continues.

11.2
Provision of Intercepted Content of Communication – IMS Conference Services

11.2.0
General
The access method for the delivery of IMS conference services intercept content of communication (CC) is based on duplication of packets without modification at the MRFP for conferences that are to be intercepted. The duplicated packets with additional information in the header, as described in the following sections, are sent to DF3 for further delivery. For a target’s conference call held by the target, the MRFP duplicates the CC for conference call held by the target, in accordance with national regulations.  For a conference call that is the target of interception, the MRFP duplicates the CC for the conference.
NOTE:
There is an issue of combined versus separated delivery. With combined delivery, one method for intercepting the CC would be to create a virtual conference port (not visible to others) through which a copy of the combined CC could be passed over the X3 interface (Y conferees means 1 content stream).  With the separated delivery approach, each conferee’s connection to the conference would need to be intercepted and passed over the X3 interface (Y conferees, means Y pairs of bi-directional content streams).




[image: image42.emf] 

MRFP  

Other   party  

Target  

LEMF  

Delivery   Function  3  

Other   party  

Duplicator of   packets  


Figure 11.1: Configuration for interception of IMS Conference Services CC

11.2.1
X3-interface

In addition to the intercepted content of communications, the following information may need to be transferred from the MRFP to the DF3 in order to allow the DF3 to perform its functionality:

-
identity used for interception;

-
correlation number;

NOTE 1:
Information passed between the MRFC and MRFP for correlation has to uniquely identify the mixing of associated media streams for a conference distinct from any other mixing or media handling.  An example is how H.248 uses a context identifier to do this. 

-
the identity of source (i.e., conference party identity) of a media stream;

-
time stamp - optional;

-
direction (incoming or outgoing stream) – optional.

NOTE 2:
When the media is delivered in a mixed format, the identity of the media stream source may be unknown.

11.3
Provision of Intercept Related Information for IMS Conference Service

11.3.0
General

Figure 11.2 shows the transfer of intercept related information to the DF2. If an event for / from or associated with a conference server occurs, the AS/MRFC sends the relevant data to the DF2.

NOTE:
Reporting of non-transmission related actions of a target’s subscriber controlled input (e.g., signalling "mute" commands) is for further study.


[image: image43]
Figure 11.2: Provision of Intercept Related Information for IMS Conferencing

11.3.1
X2-interface

The following information may need to be transferred from the AS/MRFC to the DF2 in order to allow a DF2 to perform its functionality:

-
target identity (IMPU, IMPI, Conference URI);

-
events and associated parameters as defined in section 11.3.3 "Structure of Conference Events" may be provided;

-
Correlation number;

-
Bandwidth and media descriptions (e.g., as associated with SDP negotiation) associated with the parties’ bearer connection to the conference.

The IRI should be sent to DF2 using a reliable transport mechanism.

11.3.2
IMS Conference Events and Event Information

The following events are applicable to AS/MRFC:

-
Start of Conference

-
Party Join;

-
Party Leave;

-
Conference Bearer Modification;

-
Start of Intercept on an Active Conference;

-
End of Conference;

-
Creation of Conference;

-
Update of Conference.

NOTE 1:
Reporting of Floor Control events from the MRFP is FFS.

A set of possible elements as shown below that may be reported with the events. Information associated with the events is transmitted from the AS/MRFC server to DF2. 

Table 11.3.1 Information Elements for Conference Events

	Element

	Observed IMPU

IMS Public User identity (IMPU) of the target.  In some cases, this identity may not be observed by the MRFC. Also see Note 1.

	Observed IMPI

IMS Private User identity (IMPI) of the target. In some cases, this identity may not be observed by the MRFC.  Also see Note 1.

	Observed Other Identity

Target Identifier with the NAI of the target.

	Event Type

Description which type of event is delivered: Start of Conference, Party Join, Party Leave, Start of Intercept on an Active Conference, Conference End.

	Event Date

Date of the event generation in the AS/MRFC.

	Event Time

Time of the event generation in the AS/MRFC server. Timestamp shall be generated relative to the AS/MRFC internal clock.

	Correlation Number

The correlation number is used to correlate CC and IRI. The correlation number is also used to allow the correlation of IRI records.  

	Network Element Identifier

Unique identifier for the element reporting the ICE.

	Initiator

The initiator of a request, for example, the target, the network, a conferee.

	Join Party ID

Identity of the party successfully joining or attempting to join the conference.

	Leave Party ID

Identity of the party leaving or being requested to leave the conference.

	List of Potential Conferees

Identifies each of the parties to be invited to a conference or permitted to join the conference (if available).

	Observed Conference URI

A URI associated with the conference being monitored.

	Temporary Conference URI

A temporarily allocated URI associated with a conference being monitored.

	List of Conferees

Identifies each of the conferees currently on a conference (e.g., via SIP URI or TEL URI).

	Failed Conference Start Reason

Provides a reason for why a conference start attempt failed.

	Failed Party Join Reason

Provides a reason for why a party join attempt failed.

	Party Leave Reason 

Provides a reason for the party leaving.

	Failed Party Leave reason

Provides a reason for why a party leave attempt failed.

	Conference End Reason

Provides a reason for why the conference ended.

	Potential Conference Start Date and Time

The expected start date and time of the conference, if start time information is configured in the system.

	Potential Conference End Date and Time

The expected end date and time of the conference, if such end information is configured in the system.

	Recurrence Information

Information indicating the recurrence pattern for the event as configured for the created conference.

	Identity(ies) of Conference Controller

Identifies the parties that have control privileges on the conference, if such information is configured in the system.

	Bearer Modify ID

Identifies the party modifying a conference bearer.

	Failed Bearer Modify Reason

Provides a reason for a bearer modification attempt failed.

	Failed Conference End Reason

Provides a reason why a conference end attempt failed.

	Join Party Supported Bearers

Identifies the bearer types supported by the party joining the conference.

	List of Waiting Conferees

Identifies each of the parties that have called into a conference but have not yet joined.

	Media Modification

Identifies how the media was modified (i.e., added, removed, changed) 

	Parties Affected by Bearer Modification

Identifies all conference party identities affected by the bearer modification.

	Supported Bearers

Identifies all bearer types supported by a conferee in a conference.

	Update Type

Indicates what update was done to a conference (e.g., update List of Potential Conferees, update of Start Time, update of End Time, Update of Recurrence Information, Cancellation of Conference, etc.).


Editor’s Note:  We should consider whether H.248 Context Identifier should be added to help correlate CII and CC

NOTE 2:
In most cases, either the IMPU or IMPI may be available, but not necessarily both.

11.3.3
Structure of Conference Events

11.3.3.1
Start of Conference

For the start of a conference, a Start of Conference-event is generated in the following cases:

-
When a target provisioned or requested conference or a conference that is the target of interception is started. The conference is started when the first party is joined to the conference.;

-
When a conference that is a target of interception or when a target provisioned or requested conference fails to start.

The fields, shown in Table 11.3.2, will be delivered to the DF2, if available, by the AS/MRFC.

Table 11.3.2. Start of Conference

	Observed IMPU 

	Observed IMPI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Correlation Number

	List of Potential Conferees

	List of Conferees

	List of Waiting Conferees

	Supported Bearers

	Observed Conference URI

	Temporary Conference URI

	Failed Conference Start Reason


11.3.3.2
Party Join

A Party Join-event is generated in the following cases:

-
When a party successfully joins the target’s conference or a conference that is the target of interception.

-
When a party unsuccessfully attempts to join the target’s conference or a conference that is the target of interception.

The fields, shown in Table 11.3.3, will be delivered to the DF2, if available, by the AS/MRFC.

Table 11.3.3 Party Join

	Observed IMPU 

	Observed IMPI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Correlation Number

	Join Party ID

	Join Party Supported Bearers

	Initiator (of the Party Join request) 

	Observed Conference URI

	Temporary Conference URI

	Failed Party Join Reason (e.g., not available)


11.3.3.3
Party Leave

A Party Leave-event is generated in the following cases:

-
When a party leaves a target’s conference or a conference that is the target of interception. This includes situations where the party simply disconnects themselves from the conference (hang up), the party’s connection to the conference is broken (e.g., party leaves wireless coverage area), and where the party’s connection to the conference is forcefully terminated due to another party’s drop request or operator policy.

-
When a party unsuccessfully attempts to drop another party from the conference. This applies to all the conferencing scenarios described earlier.

The fields, shown in Table 11.3.4, will be delivered to the DF2, if available, by the AS/MRFC.

Table 11.3.4 Party Leave

	Observed IMPU 

	Observed IMPI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Correlation Number

	Leave Party ID

	Supported Bearers (of Leaving Party)

	Initiator (of the Party Leave request) 

	Observed Conference URI

	Temporary Conference URI

	Party Leave Reason – see Note.

	Failed Party Leave Reason


NOTE:
A party could drop off the conference for normal reasons (e.g., just hang up) or could be removed by a conference controller.

11.3.3.3A
Conference Bearer Modification

A Conference Bearer Modification-event is generated for the following cases:

-
When a party to a conference successfully modifies (i.e., add, remove, change) a bearer stream in the conference;

-
When a party to a conference unsuccessfully attempts to modify (i.e., add, remove, change) a bearer stream in the conference.

The fields, shown in Table 11.3.4A, will be delivered to the DF2, if available, by the AS/MRFC.

Table 11.3.4A Conference Bearer Modification

	Observed IMPU 

	Observed IMPI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Correlation Number

	Observed Conference URI

	Temporary Conference URI

	Bearer Modify ID

	Media Modification

	Parties Affected by Bearer Modification

	Failed Bearer Modify Reason


11.3.3.4
Start of Intercept on an Active Conference 

A Start of Intercept on an Active Conference-event (a conference with at least one party) is generated for the following cases:

-
When interception is activated during an ongoing conference call.

The fields, shown in Table 11.3.5, will be delivered to the DF2, if available, by the AS/MRFC.  

Table 11.3.5 Start of Intercept with an Active Conference

	Observed IMPU 

	Observed IMPI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Correlation Number

	List of Conferees

	Supported Bearers

	Observed Conference URI

	Temporary Conference URI


11.3.3.5
Conference End

When a conference is terminated, a Conference End-event is generated in the following cases:

-
When a target provisioned or requested conference is terminated.  This occurs when the last party on the conference leaves or the conference is terminated by the conference server;

-
When there is an unsuccessful attempt to terminate a target provisioned or requested conference or a conference that is the target of interception.

The fields, shown in Table 11.3.6, will be delivered to the DF2, if available, by the AS/MRFC.

Table 11.3.6 End of Conference

	Observed IMPU 

	Observed IMPI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Correlation Number

	Initiator (e.g., target, network, conferee) – see Note

	Observed onference URI

	Temporary Conference URI

	Conference End Reason

	Failed Conference End Reason


NOTE:
The initiator can indicate that the decision to end the conference was the target or conferee, if the target or conferee sends an explicit command to end the conference.  It could be the network, if it determines the time length for the conference is ended. 

11.3.3.6
Creation of Conference 

When a conference is created, a Creation of Conference-event is generated in the following cases:

-
When a target successfully provisions or requests a conference to be created.  

This event is applicable provided that at least one of the two identities (IMPU, IMPI) are available at the AS/MRFC. Other scenarios, such as in case the creation is done via a web interface and the IMPU/IMPI cannot be seen are outside the scope of this specification.

The fields, shown in Table 11.3.7, will be delivered to the DF2, if available, by the AS/MRFC.
Table 11.3.7 Creation of Conference
	Observed IMPU 

	Observed IMPI

	Observed Other Identity

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	List of Potential Conferees (if available)

	Observed Conference URI

	Temporary Conference URI

	Potential Conference Start Date and Time (if available) – See Note 1

	Potential Conference End Date and Time (if available) – See Note 1

	Recurrence Information – See Note 2.

	Identity(ies) of Conference Controller


NOTE 1:
This information is statically provisioned information and is not correlated to the timestamp requirements for LI.

NOTE 2:
Recurrence information indicates the frequency or pattern of recurrence of the created conference.

11.3.3.7
Update of Conference 

When a conference is updated, an Update of Conference-event is generated in the following cases:

-
When a target successfully provisions or requests a conference to be updated (e.g., changes to List of Potential Conferees, Start Time, End Time, Recurrence Information, or Cancellation of Conference).  

This event is applicable provided that at least one of the two identities (IMPU, IMPI) are available at the AS/MRFC. Other scenarios, such as in case the update is done via a web interface and the IMPU/IMPI cannot be seen are outside the scope of this specification.

The fields, shown in Table 11.3.8, will be delivered to the DF2, if available, by the AS/MRFC.

Table 11.3.8 Update of Conference

	Observed IMPU 

	Observed IMPI

	Observed Other Identity

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Update Type

	List of Potential Conferees (if available)

	Observed Conference URI

	Temporary Conference URI

	Potential Conference Start Date and Time (if available) – See Note 1

	Potential Conference End Date and Time (if available) – See Note 1

	Recurrence Information – See Note 2.

	Identity(ies) of Conference Controller


NOTE 1:
This information is statically provisioned information and is not correlated to the timestamp requirements for LI.

NOTE 2:
Recurrence information indicates the frequency or pattern of recurrence of the created conference.

12
Lawful Interception for Evolved Packet System

12.1
LI functional architecture for EPS 

In addition to the reference configurations applicable to PS interception, the following figures contain the reference configuration applicable for the lawful interception in the EPS nodes ([22], [23]):



[image: image44]


Figure 12.1.1:
MME Intercept configuration


[image: image45.emf] 

X1_1  

Xv  

X1_2  

HI1  

   

   

    LEM F   ADMF  

Mediation   Function  

Delivery    Function 2  

Mediation   Function  

      MME  

  HeNB location  verifying node  

HI 2  

X2  


Figure 12.1.1a:
  Configuration for Intercept of HeNB

[image: image46.emf]   

LEMF  

Mediation    

Function    

Delivery    

Function 2    

Mediation    

Function    

ADMF    

X2  

X1_2  

X1_1  

HI1  

HI2  

 HSS  


Figure 12.1.2:
HSS Intercept configuration

[image: image47.emf]Mediation

Function

Delivery

Function 2

Mediation

Function

ADMF

LEMF

Mediation

Function

Delivery

Function 3

X1_2

S-GW,

PDN-GW,

ePDG

X3

X2

X1_3

X1_1 HI1

HI2

HI3


Figure 12.1.3:
S-GW, PDN-GW, ePDG Intercept configuration
The definition of the LI functional entities (ADMF, DF, MF, LEMF) and interfaces (X, HI) is the same as for 3G as given in chapter 4. Packet Header Information Reporting is a national option. For Packet Header Information Reporting, a S-GW/PDN-GW either isolates the relevant data and sends it to the DF2 or sends the packet stream to another entity in the network (e.g., DF3) for isolation which then provides the relevant data to the DF2.  
Procedures for LI activation, deactivation and interrogation are the same as for 3G as given in chapter 5, provided that:

-
the 3G ICE is replaced by the EPS node;

-
the proper target identity applicable to EPS node is used.

When the SGSN is used as node in the Evolved Packet System, to support 2G/3G access and mobility between E-UTRAN and pre-E-UTRAN 3GPP radio access technologies, it is subjected to all the related PS requirements specified throughout this document.

Figure 12.1.1a depicts how the HeNB location information is transferred from the HeNB location verifying node per TS 33.320 [34] to the DF2 via an Xv interface, in order to allow the DF2 to perform its functionality.  The public IP Address of the HeNB is provided to the HeNB location verifying node.  The manner that the HeNB location verifying node provides the DF2 with the HeNB location and HeNB IP Address is outside the scope of this document.  Additional information on HeNB interception is found in Clause X.

The target identities for 3GPP HeNB interception can be IMSI, MSISDN, IMEI, or ME Id.  Use of the HeNB ID or the CSG ID as a target identity is FFS.
12.2
Functional requirements for LI in case of E-UTRAN access and GTP based S5/S8.

12.2.0
General 

The target identities for interception at the MME, HSS, S-GW and PDN-GW are IMSI, MSISDN and ME (Mobile Equipment) Identity. 

NOTE 1:
Details about information included in the ME Identity and the relationship with IMEI needs to be considered. The term Mobile Equipment Identity is used in this text according to [22] so as to indicate that the EPC should support multiple equipment identity formats (e.g. those from 3GPP2, WiMAX, etc) as well as the IMEISV.

NOTE 2: 
In case of local breakout the PDN Gateway is in the VPLMN. In this case LI relevant information in the H-PLMN might be available at the H-PCRF.  Interception at the H-PCRF is FFS.

NOTE 3:
In case the ME Identity and/or MSISDN is not available in a node, interception based on the missing identity is not applicable at that node. 
NOTE 4:
MSISDN is a possible identity available in the EPC nodes, which may be provided by the HSS to the MME and then forwarded to the S-GW/PDN-GW.

As the MME only handles control plane, interception of Content of Communication is applicable only at the S-GW and PDN-GW. As the HSS only handles signaling, interception of Content of Communication is not applicable at this node.

LI in the PDN-GW is a national option.

For the delivery of the CC and IRI the S-GW and/or, per national option PDN-GW provides correlation number and target identity to the DF2 and DF3 which is used there in order to select the different LEAs where the product shall be delivered.

The correlation number is unique in the whole PLMN and is used to correlate CC with IRI and the different IRI's of one EPS bearer.

The correlation number shall be generated by using existing parameters related to the EPS bearer.

NOTE 5:
If interception has been activated for both parties of the Packet Data communication both CC and IRI will be delivered for each party as separate intercept activity.

Location Dependent Interception for EPC is FFS.

12.2.1
Provision of Intercept Related Information 

12.2.1.0

General
Intercept Related Information (Events) shall be sent at the Mobile Entity Attach, Mobile Entity Detach, Tracking Area Update, Bearer activation (valid for both Default and Dedicated bearer), Start of Intercept with bearer active, Start of Interception with E-UTRAN attached UE, Bearer Modification, Bearer Deactivation, Serving Evolved Packet System (applicable to the HSS), UE requested PDN connectivity, UE requested PDN disconnection, and UE requested bearer resource modification.

Serving Evolved Packet System event reporting is a national option.

12.2.1.1

X2-interface 

The following information needs to be transferred from the EPS nodes or the HSS to the DF2 in order to allow a DF2 to perform its functionality:

-
target identity (IMSI, MSISDN, ME identity);

-
events and associated parameters as defined in clause 12.2.1.2 and 12.2.3 may be provided;

-
the target location (if available) or the IAs in case of location dependent interception;
-
correlation number;

-
Quality of Service (QoS) information (if available);
-
encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.

For HeNB interception, the MME shall provide in addition the following: 

-
HeNB Identity;

-
HeNB location. 

HeNB location information needs to be transferred from the HeNB location verifying node to the DF2 in order to allow the DF2 to perform its functionality.

The EPS nodes detect packets containing packet header information in the communications path but the information needed for Packet Header Information Reporting may need to be transferred from the EPS nodes either directly to the DF2 or via another network entity in order to allow the DF2 to perform its functionality. 
The IRI should be sent to DF2 using a reliable transport mechanism.

12.2.1.2

Structure of the events 

There are several different events in which the information is sent to the DF2 if this is required. Details are described in the following clause. The events for interception are configurable (if they are sent to DF2) in the EPC nodes or the HSS and can be suppressed in the DF2. The network procedures for which the events are generated are defined in [22].

The following events are applicable to the MME:

-
Attach;

-
Detach;

- 
Tracking Area Update;

- 
UE requested PDN connectivity; 

- 
UE Requested PDN disconnection;

- 
Start of interception with E-UTRAN attached UE.

The following events are applicable to the Serving GW and PDN GW:
-
Bearer activation (valid for both Default and Dedicated bearer); 

-
Start of intercept with bearer active;

-
Bearer modification;

-
Bearer deactivation;

-
UE Requested Bearer Resource Modification;

-
Packet Data Header Information..

The following events are applicable to the HSS:

· Serving Evolved Packet System.

A set of elements as shown below can be associated with the events. The events trigger the transmission of the information from the nodes to DF2. Available IEs from this set of elements as shown below can be extended in the nodes, if this is necessary as a national option. DF2 can extend available information if this is necessary as a national option. If interception is performed at the PDN GW, then Packet Header Information Reporting shall also be performed at the PDN GW and not at the Serving GW.
	Observed MSISDN

MSISDN of the target.

	Observed IMSI

IMSI of the target.

	Observed ME Id

ME Id of the target; when it coincides with the IMEI, it shall be checked for each activation over the radio interface.

	Event type

Indicates which type of event is delivered: Attach, Detach, Tracking Area Update, UE requested PDN connectivity, UE Requested PDN disconnection, UE Requested Bearer Resource Modification, Bearer activation, Start of intercept with bearer active, Start of interception with E-UTRAN attached UE, Bearer deactivation, Bearer modification, Serving Evolved Packet System, Packet Data Header Information.

	Event date

Date of the event generation in the ICE.

	Event time

Time of the event generation in the ICE. Timestamp shall be generated relative to ICE internal clock.

	PDN Type

The parameter is applicable to the MME only and provides the IP version (IPv4, IPv4/IPv6, IPv6) requested by the UE.

	PDN Address Allocation

The parameter is applicable to the S-GW and PDN-GW; it provides the IP version (IPv4, IPv4/IPv6, IPv6) and IP address(es) allocated for the UE.

	Protocol Configuration Options

Are used to transfer parameters between the UE and the PDN-GW (e.g. Address Allocation Preference by DHCP).

	Attach type

Indicates the type of attach (may carry indication of handover in case of mobility with non-3GPP access).

	Location Information

Location Information is the Tracking Area Identity (TAI), TA List assigned to the UE, E-CGI and/or location area identity that is present at the node at the time of event record production. In case of Tracking Area Update event, the last visited TAI of the UE may be applicable.

	PDN address(es)

The UE IP address(es) for the PDN connection.

	APN

When provided by the MME, the parameter carries the Access Point Name provided by the UE. When provided by the S-GW/PDN-GW, it is the Access Point Name used for the connection.

	RAT type

The Radio Access Type

	APN-AMBR

The Aggregate Maximum Bit Rate for the APN.

	Handover indication

Provides information from  the GTPv2 protocol that the procedure is triggered as part of a handover.

	Procedure Transaction Identifier

Identifies a set of messages belonging to the same procedure; the parameter is dynamically allocated by the UE.

	EPS bearer identity

An EPS bearer identity uniquely identifies an EPS bearer for one UE accessing via E-UTRAN. The EPS Bearer Identity is allocated by the MME.

	Bearer activation/deactivation type

Indicates the type of bearer being activated/deactivated, i.e. default or dedicated.

	Linked EPS bearer identity

Indicates, in case of dedicated bearer, the EPS bearer identity of the default bearer.

	Initiator

The initiator of the procedure, either the network, HeNB, or the UE.

	Switch off indicator

Indicates whether a detach procedure is due to a switch off situation or not.

	Detach type

Parameter sent by the network to the UE to indicate the type of detach.

	Traffic Flow Template (TFT)

The EPS bearer traffic flow template (TFT) is the collection of all packet filters associated with that EPS bearer.

	Traffic Aggregate Description (TAD)

The TAD consists of the description of the packet filter(s) for the traffic flow aggregate.

	Serving MME address

The address of the serving MME.

	Old Location Information

Location Information of the subscriber before Tracking Area Update.

	Correlation Number

The correlation number is used to correlate CC and IRI.

	Network Element Identifier

Unique identifier for the ICE reporting the event.

	Logical Function Information

Used to distinguish between multiple logical functions operating in a single physical network element.

	Failed attach reason

Reason for failed attach of the target.

	Failed bearer activation reason

Reason for failed bearer activation for the target.

	Failed Bearer Modification reason

The reason for failure of Bearer Modification.

	IAs

The observed Interception Areas.

	Bearer Deactivation cause

The cause of deactivation of the PDP context.

	EPS Bearer QoS

This field indicates the Quality of Service associated with the Bearer procedure.

	Request type

Indicates the type of request in an UE requested PDN connectivity, i.e. initial request or handover.

	CSG Identity 

Uniquely identifies a CSG within one PLMN.  Note:  Open HeNBs do not have associated CSGs.

	CSG List

Identifies the membership of a given CSG (i.e., CSG Identities and associated expiration data for the UEs).

	HeNB Identity

Uniquely identifies a HeNB (i.e., HeNB equipment ID and HeNB name).

	HeNB IP Address

The public IP address of the HeNB provided to the HeNB location verifying node

	HeNB Location

Reports the location of the HeNB used during location verification.

	ISP Operator Identity

Identifies the ISP through which the HeNB is connected to the SeGW (e.g., IP address).

	Security Gateway IP Address

The IP Address of the Security Gateway that terminates the tunnel from the HeNB.

	Tunnel Protocol

The tunnel protocol used between the HeNB and the SeGW.

	ULI Timestamp

Indicates the time when the User Location Information was acquired.

The parameter is specified in 3GPP TS 29.274 [38].

	Destination IP Address

The IP address, including type IPv4 or IPv6, of the destination of the IP packet.

	Destination Port Number

The port number of the destination of the IP packet.

	Flow Label (IPv6 only)

The field in the IPv6 header that is used by a source to label packets of a flow (see RFC 3697 [41]).

	Packet Count 

The number of packets detected and reported (for a particular summary period).

	Packet Data Summary Reason

The reason for a Packet Data Summary message being sent to the LEMF (e.g., timed out, counter expiration, end of session)

	Packet Size

The size of the packet. (i.e., Total Length Field in IPv4 or Payload Length field in IPv6)

	Source IP Address

The IP address, including type IPv4 or IPv6, of the source of the IP packet.

	Source Port Number

The port number of the source of the IP packet.

	Sum of Packet Sizes (for a particular summary period)

The sum of values contained in the Total Length fields of the IPv4 packets or the sum of the values contained in the Payload Length fields of the IPv6 packets.

	Summary Period

Includes the dates and times of the first and last packets in a particular packet data interval.  

	Transport Protocol (e.g., TCP)

The identification of the transport protocol of the packet or packet flow being reported.


Editor’s note:  Identification of which HeNB IP Address is reported is needed.

12.2.2
X3-interface

The access method for the delivering of S-GW and/or PDN-GW Intercept Product is based on duplication of packets without modification at the S-GW and/or PDN-GW. The duplicated packets with additional information in a header are sent to DF3 for further delivery to the LEA.


[image: image48]
Figure 12.2.2.1: Configuration for interception of S-GW/PDN-GW product data

In addition to the intercepted content of communication, the following information needs to be transferred from the S-GW and/or the PDN-GW to the DF3 to perform its functionality:

· target identity;

· correlation number;

· time stamp (optional);

· direction (indicates whether T-PDU is MO or MT) – optional;

· the target location (if available) or the IAs in case of location dependent interception.

NOTE: location dependent interception for EPC is FFS.

12.2.3
EPS related events

12.2.3.1
Attach

When an attach activation is generated from the mobile an attach event is generated by the MME. These elements will be delivered to the DF2 if available:

	Observed MSISDN 

	Observed IMSI

	Observed ME Id

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Logical Function Information

	Location Information

	Failed attach reason

	IAs (if applicable)

	PDN Type

	APN

	Protocol Configuration Options

	Attach type 

	EPS bearer identity

	CSG Identity (if closed/hybrid H(e)NB)*

	CSG List (if closed/hybrid H(e)NB)* 

	HeNB Identity*

	HeNB IP Address* 

	HeNB Location*

	Security Gateway IP address*

	Tunnel Protocol*

	ISP Operator Identity*


* These elements are applicable for HeNB interception only.

12.2.3.2
Detach

For detach a detach-event is generated. The following elements will be delivered by the MME to the DF2 if available:

	Observed MSISDN

	Observed IMSI

	Observed ME Id

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Logical Function Information

	Location Information

	IAs (if applicable)

	Detach initiator

	Switch off indicator

	Detach type

	CSG Identity (if closed or hybrid HeNB)*

	HeNB Identity*

	HeNB IP Address*

	HeNB Location*


* These elements are applicable for HeNB interception only.

12.2.3.3
Bearer activation

When a bearer activation is generated a bearer activation-event is generated by the S-GW/PDN-GW. These elements will be delivered to the DF2 if available:

	Observed MSISDN

	Observed IMSI

	Observed ME Id

	RAT type (note 1)

	PDN address allocation (note 1)

	Event Type 

	Event Time

	Event Date

	Correlation number

	APN (Access Point Name) (note 1)

	Bearer activation Type (default, dedicated)

	Network Element Identifier

	Logical Function Information

	Location Information

	Failed bearer activation reason

	IAs (if applicable)

	EPS bearer QoS (note 2)

	APN-AMBR (note 3)

	EPS bearer id  (NSAPI)

	Protocol Configuration Options 

	Initiator

	Procedure Transaction Identifier

	Linked EPS bearer identity (note 2)

	Traffic Flow Template(s) (TFT) ( note 4)

	Handover indication


NOTE 1: Only in case of default bearer activation; the parameter includes both PDN type and PDN address(es).

NOTE 2: In case of unsuccessful default bearer activation, the parameter carries the requested EPS bearer QoS, otherwise it carries the EPS bearer QoS associated to the established bearer. 

NOTE 3: In case of unsuccessful default bearer activation, the parameter carries the subscribed APN-AMBR, otherwise it carries the APN-AMBR used for the established bearer. 

NOTE 4: TFT is applicable only in the case of dedicated bearer.

12.2.3.4
Bearer deactivation

When a bearer deactivation is generated a bearer deactivation-event is generated by the S-GW/PDN-GW. These elements will be delivered to the DF2 if available:

	Observed MSISDN

	Observed IMSI

	Observed ME Id

	Event Type 

	Event Time

	Event Date

	Correlation number

	Bearer deactivation Type (default, dedicated)

	Network Element Identifier

	Logical Function Information

	Location Information

	IAs (if applicable)

	EPS bearer id

	Initiator

	Procedure Transaction Identifier

	Bearer deactivation Cause (note )

	ULI Timestamp


In case all the bearers belonging to the same PDN connection are released at the same time, one event shall be sent for each bearer.

NOTE:
Cause can be present e.g. in case of inter S-GW TAU, when the new S-GW sends a bearer deactivation request to the old S-GW.

12.2.3.5
Bearer modification

When a bearer modification is detected, a bearer modification event shall be generated. These elements will be delivered by the S-GW/PDN-GW to the DF2 if available:

	Observed MSISDN 

	Observed IMSI

	Observed ME Id

	Event Type 

	Event Time

	Event Date

	Correlation number

	Network Element Identifier

	Logical Function Information

	Location Information

	IAs (if applicable)

	Initiator

	EPS Bearer QoS (Note 1)

	EPS bearer id

	Procedure Transaction Identifier

	RAT type

	APN-AMBR (Note 2)

	Traffic Flow Template(s) (TFT)

	Handover indication

	Failed Bearer Modification reason


NOTE 1:
In case of unsuccessful default bearer modification, the parameter carries the requested EPS bearer QoS, otherwise it carries the EPS bearer QoS associated to the modified bearer.

NOTE 2:
In case of unsuccessful default bearer modification, the parameter carries the subscribed APN-AMBR, otherwise it carries the APN-AMBR used for the modified bearer. 
The event may also be used by the PDN-GW to indicate a handover between different accesses. In this case, the RAT type indicates the new access after the handover.
12.2.3.6

Start of interception with active bearer

This event will be generated if interception for a target is started and if the target has at least the default bearer active. If more then one bearer is active, for each of them an event record is generated. The parameters which are defined for bearer activation (see related section) will be sent, if available, by the S-GW/PDN-GW to the DF2.

As an option, in case the event is sent due to a change of the involved S-GW, the new S-GW may provide as additional parameter, the "old location information". However, the absence of this information does not imply that interception has not started in the old location S-GW for an active bearer.

12.2.3.7
Tracking Area Update
For each TA update an update-event with the elements about the new location is generated. New MME shall send the event, and the old MME may optionally send the event as well. These elements will be delivered to the DF2 if available:

	Observed MSISDN 

	Observed IMSI 

	Observed ME

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Logical Function Information

	Location Information (only for the new MME)

	Old Location Information (only for the old MME)

	IAs (if applicable)

	Failure reason

	HeNB Identity*

	HeNB IP Address*

	HeNB Location*


* These elements are applicable for HeNB interception only.
12.2.3.8
Serving Evolved Packet System
The Serving Evolved Packet System report event is generated at the HSS, when the HSS has detected that the target has roamed. The elements will be delivered to the DF2 if available:

	Observed MSISDN

	Observed IMSI

	Observed ME 

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Logical Function Information

	Serving MME Address


12.2.3.9
UE requested PDN connectivity

When a PDN connectivity is requested from the mobile to allow multiple PDN connections ([22]), an UE requested PDN connectivity event is generated by the MME. These elements will be delivered to the DF2 if available: 

	Observed MSISDN 

	Observed IMSI

	Observed ME 

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Logical Function Information

	Location Information

	APN

	Request type

	PDN type

	Failed reason

	IAs (if applicable)

	Protocol Configuration Options

	EPS bearer identity

	HeNB Identity*

	HeNB IP Address*

	HeNB Location*


* These elements are applicable for HeNB interception only.

12.2.3.10
UE requested PDN disconnection

When a PDN disconnection is requested from the mobile to request for disconnection from one PDN ([22]), an UE requested PDN disconnection event is generated by the MME. These elements will be delivered to the DF2 if available:

	Observed MSISDN 

	Observed IMSI

	Observed ME 

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Logical Function Information

	Location Information

	IAs (if applicable)

	Linked EPS bearer identity

	HeNB Identity*

	HeNB IP Address*

	HeNB Location*


* These elements are applicable for HeNB interception only.
12.2.3.11
UE requested Bearer Resource Modification

When UE requested Bearer Resource Modification [22] is detected at the S-GW/PDN-GW, an UE requested Bearer Resource Modification event is generated. These elements will be delivered to the DF2 if available:

	Observed MSISDN 

	Observed IMSI

	Observed ME

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Logical Function Information

	Location Information

	IAs (if applicable)

	Linked EPS bearer identity

	EPS bearer identity

	Procedure Transaction Identifier

	EPS bearer QoS

	Traffic Aggregate Description

	Failed Bearer Modification reason

	Protocol Configuration Options


12.2.3.12
Void

12.2.3.13
Start of interception with E-UTRAN attached UE

This event will be generated if interception for a target is started and if the target is already E-UTRAN attached. If there are multiple PDN connections active for the target then for each them an event report is generated.

These elements will be delivered to the DF2 if available:

	Observed MSISDN 

	Observed IMSI

	Observed ME id 

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Logical Function Information

	Location Information

	APN

	PDN type

	IAs (if applicable)

	EPS bearer identity  of the default bearer

	CGS Identity (if closed or hybrid HeNB)*

	CSG List (if closed or hybrid HeNB)*

	HeNB Identity*

	HeNB IP Address*

	HeNB Location *

	Security Gateway IP address*

	Tunnel Protocol*

	ISP Operator Identity*


* These elements are applicable for HeNB interception only.

12.2.3.14
Packet Header Information Reporting

12.2.3.14.0
Introduction

Packet Header Information Reporting can be done either on a per-packet (i.e., non-summarized) basis or in a summary report.  

12.2.3.14.1
Packet Data Header Report

This event is used to provide packet header reports on a per packet basis (non-summarized reporting) and is triggered by each packet sent or received by the target.  These elements will be delivered by the S-GW/PDN-GW either directly to DF2 or via another network entity, if available:

	Observed MSISDN 

	Observed IMSI

	Observed ME Id

	Event Type 

	Event Time

	Event Date

	Correlation number

	Network Element Identifier

	Logical Function Information

	Location Information

	IAs (if applicable)

	Initiator

	EPS bearer id

	Handover indication

	PDN Address Allocation

	PDN address(es)

	APN

	Source IP Address

	Destination IP Address

	Destination Port Number

	Transport Protocol (e.g., TCP)

	Packet Size

	Flow Label (IPv6 only)


12.2.3.14.2
Packet Data Summary Report

This event is used to report:

1)
the source and destination information derived from the packet headers, including:

a)
source and destination IP Addresses,

b)
IP next-layer protocol,

c)
Layer-4 ports, and

d)
Flow label, if the packet is IPv6

2)
summary information for the number of packets and bytes transmitted or received by the target for each unique packet flow within an EPS bearer,  and 

3)
the date and the time of the first and last packets associated with that packet flow.  A packet flow is defined as the 6-tuple of source/destination IP address/port number and the layer 4 protocol and EPS bearer.   

IP addresses and the IP next-layer protocol are always reported, the flow label is reported if the packet is IPv6, and the layer-4 ports are reported.    

The event provides packet summary reports for each unique packet data session (EPS bearer) and packet flow, and is triggered by one of the following:

-
start of a packet flow associated with an EPS bearer

-
an interim report for a packet flow associated with an EPS bearer is to be reported

-
end of a packet flow associated with an EPS bearer (including end of the EPS bearer itself).

An interim report can be triggered by

-
The expiration of a configurable timer per intercept (called a Summary Timer). The Summary Timer is configurable in units of seconds;

-
A per-intercept configurable count threshold is reached.

These elements will be delivered either directly to DF2 or via a MF for each packet flow if available:

	Observed MSISDN 

	Observed IMSI

	Observed ME Id

	Event Type 

	Event Time

	Event Date

	Correlation number

	Network Element Identifier

	Logical Function Information

	Location Information

	IAs (if applicable)

	Initiator

	EPS bearer id

	Handover indication

	PDN Address Allocation

	PDN address(es)

	APN

	Care of address

	Source IP Address

	Source Port Number

	Destination IP Address

	Destination Port Number

	Transport Protocol (e.g., TCP)

	Flow Label (IPv6 only)

	Summary Period

	Packet Count (for this summary period)

	Sum of Packet Sizes (for this summary period)


If the packets are IPv4, the sum of all observed packet sizes is the sum of the values contained in the Total Length field of each packet as specified in IETF RFC 791[39].
If the packet is IPv6, the sum of all observed packet sizes is the sum of the values contained in the Payload Length field for each packet as specified in IETF RFC 2460 [40].
If no packets were detected for the duration of the Summary Timer, then the Packet Data Summary Report shall not be sent.
12.3
Functional requirements for LI in case of E-UTRAN access and PMIP based S5/S8 interfaces

12.3.0
General 

Functional requirements for LI in the MME, S-GW and HSS do not differ from the ones applicable to the case of GTP based S5-S8 interfaces, as specified in clause 12.2 and subclauses.

LI in the PDN-GW is a national option.

Interception in the PDN-GW shall be based on one or more of NAI, MSISDN, IMEI.

For the delivery of the CC and IRI, the PDN-GW provides correlation number and target identity to the DF2 and DF3 which is used there in order to select the different LEAs where the product shall be delivered.

The correlation number is unique in the whole PLMN and is used to correlate CC with IRI and the different IRI's of one IP-CAN session. However, when different protocols (i.e. GTP and PMIP) are used in the network, different values can be generated by different nodes.

The correlation number shall be generated by using existing parameters related to the IP-CAN session.

NOTE:
If interception has been activated for both parties of the Packet Data communication both CC and IRI will be delivered for each party as separate intercept activity.

12.3.1
Provision of Intercept Related Information 

12.3.1.0
General
Intercept Related Information (Events) shall be sent at attach/tunnel activation, detach/tunnel deactivation, start of interception with active PMIP tunnel, PMIP session modification, PDN-GW initiated PDN-disconnection, UE requested PDN connectivity, Serving Evolved Packet System.

Serving Evolved Packet System reporting is a national option. Requirements on the HSS specified in section 12.2 and subsections apply also to the case in which S5/S8 interfaces are PMIP based. 

12.3.1.1
X2 interface

The following information needs to be transferred from the PDN-GW to the DF2 in order to allow a DF2 to perform its functionality:

-
target identity;

-
events and associated parameters as defined in clause 12.3.1.2 and 12.3.3 may be provided;

-
the target location (if available) or the IAs in case of location dependent interception; (FFS)
-
correlation number;

-
Quality of Service (QoS) information, if available;
-
encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.
The PDN-GW detect packets containing packet header information in the communications path but the information needed for Packet Header Information Reporting may need to be transferred from the PDN-GW either directly to the DF2 or via another network entity in order to allow the DF2 to perform its functionality. 
The IRI should be sent to DF2 using a reliable transport mechanism.

12.3.1.2
Structure of the events

There are several different events in which the information is sent to the DF2 if this is required. The events for interception are configurable (if they are sent to DF2) in the PDN-GW and can be suppressed in the DF2. The network procedures for which the events are generated are defined in [23].

The following events are applicable to the PDN-GW:

-
PMIP Attach/tunnel activation;

-
PMIP Detach/tunnel deactivation;

-
PMIP session modification

- 
Start of interception with active PMIP tunnel;

-
PMIP PDN-GW initiated PDN-disconnection;

-
Packet Data Header Information..

A set of elements as shown below can be associated with the events. The events trigger the transmission of the information from the nodes to DF2. Available IEs from this set of elements as shown below can be extended in the nodes, if this is necessary as a national option. DF2 can extend available information if this is necessary as a national option.

	Observed MN NAI

The Network Access Identifier of the Mobile Node (target identity).

	Observed MSISDN

MSISDN of the target.

	Observed IMEI

IMEI  of the target

	Event type

Indicates which type of event is delivered: PMIP attach/tunnel activation, PMIP detach/tunnel deactivation, PMIP Session modification, Start of interception with active PMIP tunnel, PMIP PDN-GW initiated PDN disconnection.

	Event time

Time of the event generation in the ICE. Time stamp shall be generated relative to ICE internal clock.

	Event date

Date of the event generation in the ICE.

	Correlation number

The correlation number is used to correlate CC and IRI.

	Network Element Identifier

Unique identifier for the ICE reporting the event.

	Logical Function Information

Used to distinguish between multiple logical functions operating in a single physical network element.

	Lifetime

Indicates the lifetime of the tunnel; it is set to a nonzero value in the case of registration; is set to zero in case of deregistration.

	Failed attach reason

Reason for the failed attach/tunnel deactivation of the target.

	Access technology type

Indicates the Radio Access Type.

	Handover indicator

Provides information on whether the procedure is triggered as part of a handover.

	APN

The Access Point Name used for the connection.

	UE address info

Includes one or more IP addresses allocated to the UE.

	Additional Parameters

Additional information provided by the UE, such as protocol configuration options.

	PDN address(es)

The UE IP address(es) for the PDN connection.

	Revocation trigger

Indicates the reason which triggered the PDN-GW initiated PDN-disconnection procedure

	Serving Network

Identifies the serving network the UE is attached to

	DHCP v4 Address Allocation Indication

Indicates that DHCPv4 is to be used to allocate the IPv4 address to the UE

	Location Information

Provides, if received from the PCRF, location information of the target.

	Destination IP Address

The IP address, including type IPv4 or IPv6, of the destination of the IP packet.

	Destination Port Number

The port number of the destination of the IP packet.

	Flow Label (IPv6 only)

The field in the IPv6 header that is used by a source to label packets of a flow (see RFC 3697 [41]).

	Packet Count 

The number of packets detected and reported (for a particular summary period).

	Packet Data Summary Reason

The reason for a Packet Data Summary message being sent to the LEMF (e.g., timed out, counter expiration, end of session)

	Packet Size

The size of the packet. (i.e., Total Length Field in IPv4 or Payload Length field in IPv6)

	Source IP Address

The IP address, including type IPv4 or IPv6, of the source of the IP packet.

	Source Port Number

The port number of the source of the IP packet.

	Sum of Packet Sizes (for a particular summary period)

The sum of values contained in the Total Length fields of the IPv4 packets or the sum of the values contained in the Payload Length fields of the IPv6 packets.

	Summary Period

Includes the dates and times of the first and last packets in a particular packet data interval.  

	Transport Protocol (e.g., TCP)

The identification of the transport protocol of the packet or packet flow being reported.


12.3.2
X3-interface

The access method for the delivering of PDN-GW Intercept Product is based on duplication of packets without modification at the PDN-GW. The duplicated packets with additional information in a header are sent to DF3 for further delivery to the LEA.


[image: image49]
Figure 12.3.2.1: Configuration for interception of PDN-GW product data
In addition to the intercepted content of communication, the following information needs to be transferred from the PDN-GW to the DF3 to perform its functionality:

· target identity;

· correlation number;

· time stamp (optional);

· direction (indicates whether T-PDU is MO or MT) – optional;

· the target location (if available) or the IAs in case of location dependent interception.

NOTE: location dependent interception for EPC is FFS.

12.3.3
LI events for E-UTRAN access with PMIP-based S5 or S8

12.3.3.1
Initial E-UTRAN Attach and UE PDN requested connectivity with PMIP-based S5 or S8

When the E-UTRAN Attach or UE requested PDN connectivity is detected at the PMIP based PDN-GW, a PMIP attach/tunnel activation event shall be generated by the PDN-GW. The following elements will be delivered to the DF2 if available:

	Observed MN NAI

	Observed MSISDN

	Observed IMEI

	Event Type

	Event Time

	Event Date

	Correlation number

	Network Element Identifier

	Logical Function Information

	Lifetime

	Failed attach reason

	Access Technology Type

	Handover Indicator

	APN

	UE Address Info

	Additional Parameters

	Serving Network

	DHCPv4 Address Allocation Indication 

	Location information


12.3.3.2
Detach and PDN disconnection for PMIP-based S5/S8

When the Detach or PDN disconnection is detected at the PMIP based PDN-GW, a PMIP detach/tunnel deactivation event shall be generated by the PDN-GW. The following elements will be delivered to the DF2 if available:

	Observed MN NAI

	Observed MSISDN

	Observed IMEI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Logical Function Information

	Correlation number

	APN

	Additional Parameters

	Failed reason

	Location information


12.3.3.3
Start of interception with active tunnel for PMIP based S5/S8

This event shall be generated by the PDN-GW if interception for a target is started and if the target has an active PMIP tunnel. If more then one connection is active, for each of them an event record is generated. The parameters which are defined for PMIP attach/tunnel activation (see related section) will be sent, if available, by the PDN-GW to the DF2.
12.3.3.4
Dedicated Bearer Procedures for E-UTRAN Access with PMIP-based S5/S8

All the procedures can be intercepted at the S-GW according to the requirements specified for LI in case of GTP based S5/S8. 

PDN-GW is not involved in these procedures, except for the case of PDN-GW initiated PDN-disconnection Procedure.

12.3.3.5
PDN-GW initiated PDN-disconnection Procedure

When a PDN-GW initiated PDN-disconnection procedure is detected, a PMIP PDN-GW initiated PDN-disconnection event shall be generated by the PDN-GW. The following elements will be delivered to the DF2:

	Observed MN NAI

	Observed MSISDN

	Observed IMEI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Logical Function Information

	Correlation number

	PDN Address(es)

	Revocation trigger

	Location information


12.3.3.6
PMIP Session modification

When a session modification is detected at the PDN-GW, a PMIP Session modification event shall be generated by the PDN-GW. The following elements will be delivered to the DF2:

	Observed MN NAI

	Observed MSISDN

	Observed IMEI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Logical Function Information

	Correlation number

	Lifetime

	UE Address Info

	Access Technology Type

	Additional Parameters

	Failed reason

	Serving Network

	Handover indicator

	DHCPv4 Address Allocation Indication 

	Location information


12.3.3.7
Packet Header Information Reporting
12.3.3.7.0
Introduction

Packet Header Information Reporting can be done either on a per-packet (i.e., non-summarized) basis or in a summary report.  

12.3.3.7.1
Packet Data Header Report

This event is used to provide packet header reports on a per packet basis (non-summarized reporting) and is triggered by each packet sent or received by the target.  These elements will be delivered by the PDN-GW either directly to DF2 or via another network entity if available:

	Observed MN NAI

	Observed MSISDN

	Observed IMEI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Logical Function Information

	Correlation number

	Lifetime

	UE Address Info

	Access Technology Type

	Additional Parameters

	Serving Network

	Handover indicator

	DHCPv4 Address Allocation Indication 

	Location information

	Source IP Address

	Destination IP Address

	Destination Port Number

	Transport Protocol (e.g., TCP)

	Packet Size

	Flow Label (IPv6 only)


12.3.3.7.2
Packet Data Summary Report

This event is used to report:

1)
the source and destination information derived from the packet headers, including:

a)
source and destination IP Addresses,

b)
IP next-layer protocol,

c)
Layer-4 ports, and

d)
Flow label, if the packet is IPv6

2)
summary information for the number of packets and bytes transmitted or received by the target for each unique packet flow within an EPS bearer,  and 

3)
the date and the time of the first and last packets associated with that packet flow.  A packet flow is defined as the 6-tuple of source/destination IP address/port number and the layer 4 protocol and EPS bearer.   

IP addresses and the IP next-layer protocol are always reported, the flow label is reported if the packet is IPv6, and the layer-4 ports are reported.    

The event provides packet summary reports for each unique packet data session (EPS bearer) and packet flow, and is triggered by one of the following:

-
start of a packet flow associated with an EPS bearer

-
an interim report for a packet flow associated with an EPS bearer is to be reported

-
end of a packet flow associated with an EPS bearer (including end of the EPS bearer itself).

An interim report can be triggered by

-
The expiration of a configurable timer per intercept (called a Summary Timer). The Summary Timer is configurable in units of seconds;

-
A per-intercept configurable count threshold is reached.

These elements will be delivered either directly to DF2 or via DF3 for each packet flow if available:

	Observed MN NAI

	Observed MSISDN

	Observed IMEI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Logical Function Information

	Correlation number

	Lifetime

	UE Address Info

	Access Technology Type

	Additional Parameters

	Serving Network

	Handover indicator

	DHCPv4 Address Allocation Indication 

	Location information

	Source IP Address

	Source Port Number

	Destination IP Address

	Destination Port Number

	Transport Protocol (e.g., TCP)

	Flow Label (IPv6 only)

	Summary Period

	Packet Count (for this summary period)

	Sum of Packet Sizes (for this summary period)

	Packet Summary Reason


If the packets are IPv4, the sum of all observed packet sizes is the sum of the values contained in the Total Length field of each packet as specified in IETF RFC 791[39].
If the packet is IPv6, the sum of all observed packet sizes is the sum of the values contained in the Payload Length field for each packet as specified in IETF RFC 2460 [40].
If no packets were detected for the duration of the Summary Timer, then the Packet Data Summary Report shall not be sent.
12.4
Functional requirements for LI in case of trusted non-3GPP IP access

12.4.0
General

Differently to what happens in E-UTRAN case, in which the user traffic passes through the S-GW and then through the PDN-GW, there are two cases of access to the network through S2a (trusted Non-3GPP access) that require additional consideration.  Specifically, the PDN-GW is the only possible ICE in the 3GPP network in the case of non-roaming (PDN-GW in the HPLMN) and in the case of roaming with local breakout (PDN-GW is located in the VPLMN) Therefore, in these cases, interception at the PDN-GW is required.  

In case of access to the network through S2a (trusted Non-3GPP access) for roaming without local breakout (PDN-GW in the HPLMN and S-GW in the VPLMN), interception at the PDN-GW is a national option. 

Interception in the S-GW and PDN-GW shall be based on IMSI or NAI.

NOTE:
The NAI may be a temporary ID, therefore the use of  IMSI is recommended.

For the delivery of the CC and IRI, the S-GW and/or PDN-GW provides correlation number and target identity to the DF2 and DF3 which is used there in order to select the different LEAs where the product shall be delivered.

The correlation number is unique in the whole PLMN and is used to correlate CC with IRI and the different IRI's of one IP-CAN session. However, when different protocols (i.e. GTP and PMIP) are used in the network, different values can be generated by different nodes

The correlation number shall be generated by using existing parameters related to the IP-CAN session.

NOTE:
If interception has been activated for both parties of the Packet Data communication both CC and IRI will be delivered for each party as separate intercept activity.

12.4.1
Provision of Intercept Related Information

12.4.1.0
General

Intercept Related Information (Events) shall be sent at attach/tunnel activation on interfaces s2a and s2c, session modification, detach/tunnel deactivation, start of interception with active tunnel, PDN-GW reallocation upon initial attach on s2c, PDN GW initiated resource allocation Deactivation on s2a, Serving Evolved Packet System.

Serving Evolved Packet System reporting is a national option.

12.4.1.1
X2-interface

The following information needs to be transferred from the S-GW, PDN-GW or the HSS to the DF2 in order to allow a DF2 to perform its functionality:

-
target identity;

-
events and associated parameters as defined in clause 12.4.1.2 and 12.4.3 may be provided;

-
the target location (if available) or the IAs in case of location dependent interception; (FFS)
-
correlation number;

-
Quality of Service (QoS) information, if available;
-
encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.
The PDN-GW/S-GW detect packets containing packet header information in the communications path but the information needed for Packet Header Information Reporting may need to be transferred from the PDN-GW/S-GW either directly to the DF2 or via another network entity in order to allow the DF2 to perform its functionality. 
The IRI should be sent to DF2 using a reliable transport mechanism.

12.4.1.2

Structure of the events

There are several different events in which the information is sent to the DF2 if this is required. The events for interception are configurable (if they are sent to DF2) in the S-GW, PDN-GW or the HSS and can be suppressed in the DF2.

The following events are applicable to the S-GW:

-
PMIP attach/tunnel activation;

-
PMIP detach/tunnel deactivation;

-
PMIP session modification;

-
Start of interception with active PMIP tunnel;-
Packet Data Header Information.
The following events are applicable to the PDN-GW:

-
PMIP attach/tunnel activation;

-
PMIP detach/tunnel deactivation;

-
PMIP session modification;
-
Start of interception with active PMIP tunnel;

-
MIP registration/tunnel activation;

-
DSMIP registration/tunnel activation;

-
DSMIP session modification;

-
MIP deregistration/tunnel deactivation;

-
DSMIP deregistration/tunnel deactivation;

-
Start of interception with active MIP tunnel;

-
Start of interception with active DSMIP tunnel;

-
DSMIP HA Switch;

-
PMIP Resource Allocation Deactivation;

-
MIP Resource Allocation Deactivation;

-
Bearer activation;

- 
Bearer deactivation;

-
Bearer modification;

-
Start of interception with active bearer;
-
Packet Data Header Information..

NOTE: Bearer activation, bearer deactivation, bearer modification and start of interception with active bearer are applicable to trusted non-3GPP access when the GTP protocol is used over s2a interface as specified in 3GPP TS 23.402 [23].

The following event is applicable to the HSS:


-
Serving Evolved Packet System.

A set of elements as shown below can be associated with the events. The events trigger the transmission of the information from the nodes to DF2. Available IEs from this set of elements as shown below can be extended in the nodes, if this is necessary as a national option. DF2 can extend available information if this is necessary as a national option. In case GTP protocol is used over s2a interface, elements from table 12.2.1.2 are included in the applicable events. If interception is performed at the PDN GW, then Packet Header Information Reporting shall also be performed at the PDN GW and not at the Serving GW.
	Observed MN NAI

The Network Access Identifier of the Mobile Node (target identity).

	Observed IMSI

The IMSI of the target

	Event type

Indicates which type of event is delivered: PMIP attach/tunnel activation, PMIP detach/tunnel deactivation, PMIP session modification, Start of interception with active PMIP tunnel, MIP registration/tunnel activation, DSMIP registration/tunnel activation, DSMIP session modification, MIP deregistration/tunnel deactivation, DSMIP deregistration/tunnel deactivation, Start of interception with active MIP tunnel, Start of interception with active DSMIP tunnel, DSMIP HA Switch, PMIP resource Allocation Deactivation, MIP Resource Allocation Deactivation, Serving Evolved Packet System, Packet Data Header Information.

	Event time

Time of the event generation in the ICE. Time stamp shall be generated relative to ICE internal clock.

	Event date

Date of the event generation in the ICE.

	Correlation number

The correlation number is used to correlate CC and IRI.

	Network Element Identifier

Unique identifier for the ICE reporting the event.

	Logical Function Information

Used to distinguish between multiple logical functions operating in a single physical network element.

	Lifetime

Indicates the lifetime of the tunnel; must be set to a nonzero value in the case of registration or lifetime extension; is set to zero in case of deregistration.

	Failed attach reason

Reason for the failed attach/tunnel deactivation of the target.

	Session modification failure reason

Reason for a failure of a session modification attempt for the target 

	Access technology type

Indicates the Radio Access Type.

	Handover indicator

Provides information on whether the triggered as part of a handover.

	APN

The Access Point Name used for the connection.

	UE address info

Includes one or more IP addresses allocated to the UE.

	Additional Parameters

Additional information provided by the UE, such as protocol configuration options.

	PDN address(es)

The UE IP address(es) for the PDN connection.

	Home address

Contains the UE Home IP address.

	Home Agent address

Contains the IP address of the Home Agent.

	Requested IPv6 Home Prefix

The IPv6 Home Prefix requested by the UE.

	IPv6 home prefix

The IPv6 home prefix assigned by the PDN GW to the UE.

	Care of Address

The Local IP address assigned to the UE by the Access Network, used as Care of Address for DSMIPv6 over S2c reference point.

	HSS/AAA address

The address of the HSS/AAA triggering the PDN-GW reallocation.

	Target PDN-GW address

The address of the PDN-GW which the UE will be reallocated to.

	Revocation trigger

Contains the cause for the revocation procedure.

	Foreign domain address

The relevant IP address in the foreign domain.

	Visited network identifier

An identifier that allows the home network to identify the visited network [24]

	Location Information

Location information of the target, e.g. 3GPP2-BSID [26]. Provided if available from the PCRF.

	Initiator

The initiator of the procedure, either the network or the UE.

	Destination IP Address

The IP address, including type IPv4 or IPv6, of the destination of the IP packet.

	Destination Port Number

The port number of the destination of the IP packet.

	Flow Label (IPv6 only)

The field in the IPv6 header that is used by a source to label packets of a flow (see RFC 3697 [41]).

	Packet Count 

The number of packets detected and reported (for a particular summary period).

	Packet Data Summary Reason

The reason for a Packet Data Summary message being sent to the LEMF (e.g., timed out, counter expiration, end of session)

	Packet Size

The size of the packet. (i.e., Total Length Field in IPv4 or Payload Length field in IPv6)

	Source IP Address

The IP address, including type IPv4 or IPv6, of the source of the IP packet.

	Source Port Number

The port number of the source of the IP packet.

	Sum of Packet Sizes (for a particular summary period)

The sum of values contained in the Total Length fields of the IPv4 packets or the sum of the values contained in the Payload Length fields of the IPv6 packets.

	Summary Period

Includes the dates and times of the first and last packets in a particular packet data interval.  

	Transport Protocol (e.g., TCP)

The identification of the transport protocol of the packet or packet flow being reported.


12.4.2
X3-interface

The access method for the delivering of S-GW and/or PDN-GW Intercept Product is based on duplication of packets without modification at the S-GW and/or PDN-GW. The duplicated packets with additional information in a header are sent to DF3 for further delivery to the LEA.


[image: image50]
Figure 12.4.2.1: Configuration for interception of S-GW/PDN-GW product data
n addition to the intercepted content of communication, the following information needs to be transferred from the S-GW and/or the PDN-GW to the DF3 to perform its functionality:

· target identity;

· correlation number;

· time stamp (optional);

· direction (indicates whether T-PDU is MO or MT) – optional;

· the target location (if available) or the IAs in case of location dependent interception.

NOTE:
location dependent interception for EPC is FFS.

12.4.3
LI events for trusted Non-3GPP IP access

12.4.3.1
Initial Attach and PDN connection activation with PMIPv6 on S2a

When the Attach or PDN connectivity activation is detected over PMIP at the S-GW, PDN-GW, a PMIP attach/tunnel activation event shall be generated. The following elements will be delivered to the DF2 if available:

	Observed MN NAI

	Observed IMSI

	Event Type

	Event Time

	Event Date

	Correlation number

	Network Element Identifier

	Logical Function Information

	Lifetime

	Failed attach reason

	Access Technology Type

	Handover Indicator

	APN

	UE Address Info

	Additional Parameters

	Location Information


12.4.3.2
Initial Attach and PDN connection activation procedures with MIPv4 FACoA on S2a

When the Attach or PDN connectivity activation is detected over MIP at the PDN-GW, a MIP registration/tunnel activation event shall be generated. The following elements will be delivered to the DF2 if available:

	Observed MN NAI

	Observed IMSI

	Event Type

	Event Time

	Event Date

	Correlation number

	Network Element Identifier

	Logical Function Information

	Lifetime

	Failed attach reason

	Home Address

	Care of Address

	Home Agent Address

	APN


NOTE: As the S-GW has no Home Agent function, the event is not applicable to the S-GW. The use of MIPv4 in roaming case requires Local Breakout (PDN-GW in VPLMN), so LI in the PDN-GW is mandatory in order to intercept in this scenario.

12.4.3.3
Initial Attach and PDN connection activation procedures with DSMIPv6 over S2c

When the Attach or PDN connectivity activation is detected over DSMIP at the PDN-GW, a DSMIP registration/tunnel activation event shall be generated. The following elements will be delivered to the DF2 if available:

	Observed MN NAI

	Observed IMSI

	Event Type

	Event Date

	Event Time

	Correlation number

	Network Element Identifier

	Logical Function Information

	Lifetime

	Requested IPv6 home prefix

	IHome address

	APN

	Care of Address

	Failed attach reason


12.4.3.4
Detach and PDN disconnection with PMIPv6 on S2a

When a Detach or PDN disconnection is detected over PMIP at the S-GW, PDN-GW, a PMIP detach/tunnel deactivation event shall be generated. The following elements will be delivered to the DF2 if available:

	Observed MN NAI

	Observed IMSI

	Event Type

	Event Date

	Event Time

	Correlation number

	Network Element Identifier

	Logical Function Information

	APN

	Initiator

	Location Information


12.4.3.5
Detach and PDN disconnection with MIPv4 FACoA

When a Detach or PDN disconnection is detected over MIP at the PDN-GW, a MIP deregistration/tunnel deactivation event shall be generated. The following elements will be delivered to the DF2 if available:

	Observed MN NAI

	Observed IMSI

	Event Type

	Event Date

	Event Time

	Correlation number

	Network Element Identifier

	Logical Function Information

	Home Address

	Home Agent Address

	Care of address

	Initiator


12.4.3.6
Detach and PDN disconnection with DSMIPv6 on S2c

When a Detach or PDN disconnection is detected over DSMIP at the PDN-GW, a DSMIP deregistration/tunnel deactivation event shall be generated. The following elements will be delivered to the DF2 if available:

	Observed MN NAI

	Observed IMSI

	Event Type

	Event Date

	Event Time

	Correlation number

	Network Element Identifier

	Logical Function Information

	Home Address

	Initiator


12.4.3.7
PDN-GW reallocation upon initial attach on s2c

When a PDN GW reallocation procedure is detected by the PDN-GW, a DSMIP HA Switch event shall be generated. The following elements will be delivered to the DF2 if available:

	Observed MN NAI

	Observed IMSI

	Event Type

	Event Date

	Event Time

	Network Element Identifier

	Logical Function Information

	HSS/AAA address

	Target PDN-GW address


12.4.3.8
PDN GW initiated Resource Allocation Deactivation with S2a PMIP

When a PDN GW initiated resource allocation deactivation is detected by the S-GW/PDN-GW, a PMIP Resource Allocation Deactivation event shall be sent. The following elements will be delivered to DF2 if available
	Observed MN NAI

	Observed IMSI

	Event Type

	Event Date

	Event Time

	Network Element Identifier

	Logical Function Information

	Revocation trigger

	UE address info

	Correlation number

	Location Information


12.4.3.9
PDN GW initiated Resource Allocation Deactivation with S2a MIP v4

When a PDN GW initiated resource allocation deactivation is detected, a MIP Resource Allocation Deactivation event shall be sent. The following elements will be delivered to DF2 if available
	Observed MN NAI

	Observed IMSI

	Event Type

	Event Date

	Event Time

	Network Element Identifier

	Logical Function Information

	Home Address

	Foreign domain address

	Correlation number


12.4.3.10
Serving Evolved Packet System

The Serving Evolved Packet System report event is generated at the HSS, when the HSS has detected that the target has roamed. The elements will be delivered to the DF2 if available:

	Observed MSISDN

	Observed IMSI

	Observed ME Identity

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Logical Function Information

	Visited Network Identifier


12.4.3.11 
Start of interception with active tunnel or bearer

When interception is started at the S-GW, PDN-GW and the target has an already active tunnel or bearer, a start of interception with active tunnel/bearer shall be generated. Separate events are defined for the different protocols. The event shall be detected by the same node for which tunnel/bearer activation reporting is applicable and reported with the same parameters required for the specific protocol (PMIP, MIP, DSMIP, GTP) tunnel/bearer activation event, as defined in the related sections. One event shall be sent for each active tunnel/bearer.

12.4.3.12
PMIP session modification

When a session modification is detected at the S-GW/PDN-GW, a PMIP session modification event shall be generated by the S-GW/PDN-GW. The following elements will be delivered to the DF2 if available:

	Observed MN NAI

	Observed MSISDN

	Observed IMEI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Logical Function Information

	Correlation number

	Lifetime

	UE Address Info

	Access Technology Type

	Additional Parameters

	Session modification failure reason

	Serving Network

	Handover indicator

	DHCPv4 Address Allocation Indication 

	Location information


12.4.3.13
DSMIP session modification

When the session modification is detected over DS-MIPv6 at the PDN-GW, a DSMIP session modification event shall be generated. The following elements will be delivered to the DF2 if available:

	Observed MN NAI

	Observed IMSI

	Event Type

	Event Time

	Event Date

	Correlation number

	Network Element Identifier

	Logical Function Information

	Lifetime

	Session modification failure reason

	Home address

	Care of Address

	APN

	Requested IPv6 Home Prefix


12.4.3.14
Bearer activation
When the Initial attach in WLAN on GTP S2a (TS 23.402 [23]) or the Dedicated bearer activation in WLAN on GTP S2a (TS 23.402 [23]) is detected in the PDN-GW, a Bearer activation event shall be generated. The elements listed in the section 12.2.3.3 will be delivered to the DF2 if available.

12.4.3.15
Bearer deactivation
When the Detach and PDN disconnection in WLAN on GTP S2a (TS 23.402 [23]) or the PDN GW initiated Resource Allocation Deactivation in WLAN on GTP S2 (TS 23.402 [23] is detected in the PDN-GW, a Bearer deactivation event shall be generated. The elements listed in the section 12.2.3.4 will be delivered to the DF2 if available.

12.4.3.16

Bearer modification

When the Network initiated bearer modification in WLAN on GTP S2a (TS 23.402 [23]) is detected in the PDN-GW, a Bearer modification event shall be generated. The elements listed in the section 12.2.3.5 will be delivered by the PDN-GW to the DF2 if available.

12.4.3.17
Packet Header Information Reporting

12.4.3.17.0
Introduction

Packet Header Information Reporting can be done either on a per-packet (i.e., non-summarized) basis or in a summary report.  

12.4.3.17.1
Packet Data Header Report

This event is used to provide packet header reports on a per packet basis (non-summarized reporting) and is triggered by each packet sent or received by the target.  These elements will be delivered by the S-GW/PDN-GW either directly to the DF2 or via another network entity if available: 

	Observed MN NAI

	Observed MSISDN

	Observed IMEI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Logical Function Information

	Correlation number

	Lifetime

	UE Address Info

	Access Technology Type

	Serving Network

	Home address

	Care of Address

	APN

	Location information

	Source IP Address

	Source Port Number

	Destination IP Address

	Destination Port Number

	Transport Protocol (e.g., TCP)

	Packet Size

	Flow Label (IPv6 only)


12.4.3.17.2
Packet Data Summary Report

This event is used to report:

1)
the source and destination information derived from the packet headers, including:

a)
source and destination IP Addresses,

b)
IP next-layer protocol,

c)
Layer-4 ports, and

d)
Flow label, if the packet is IPv6

2)
summary information for the number of packets and bytes transmitted or received by the target for each unique packet flow within an EPS bearer,  and 

3)
the date and the time of the first and last packets associated with that packet flow.  A packet flow is defined as the 6-tuple of source/destination IP address/port number and the layer 4 protocol and EPS bearer. 

IP addresses and the IP next-layer protocol are always reported, the flow label is reported if the packet is IPv6, and the layer-4 ports are reported.    

The event provides packet summary reports for each unique packet data session (EPS bearer) and packet flow, and is triggered by one of the following:

-
start of a packet flow associated with an EPS bearer

-
an interim report for a packet flow associated with an EPS bearer is to be reported

-
end of a packet flow associated with an EPS bearer (including end of the EPS bearer itself).

An interim report can be triggered by

-
The expiration of a configurable timer per intercept (called a Summary Timer). The Summary Timer is configurable in units of seconds;

-
A per-intercept configurable count threshold is reached.

These elements will be delivered either directly to the DF2 or via a MF for each packet flow if available:

	Observed MN NAI

	Observed MSISDN

	Observed IMEI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Logical Function Information

	Correlation number

	Lifetime

	UE Address Info

	Access Technology Type

	Serving Network

	Home address

	Care of Address

	APN

	Location information

	Source IP Address

	Source Port Number

	Destination IP Address

	Destination Port Number

	Transport Protocol (e.g., TCP)

	Flow Label (IPv6 only)

	Summary Period

	Packet Count (for this summary period)

	Sum of Packet Sizes (for this summary period)


If the packets are IPv4, the sum of all observed packet sizes is the sum of the values contained in the Total Length field of each packet as specified in IETF RFC 791[39].
If the packet is IPv6, the sum of all observed packet sizes is the sum of the values contained in the Payload Length field for each packet as specified in IETF RFC 2460 [40].
If no packets were detected for the duration of the Summary Timer, then the Packet Data Summary Report shall not be sent.
12.5
Functional requirements for LI in case of untrusted non-3GPP IP access
12.5.0
Introduction 
This section specifies functional requirements applicable to the PDN-GW and HSS. In addition, this section specifies requirements applicable to the ePDG in case this node is using a GTPv2 based protocol over s2b interface as specified in 3GPP TS 23.402 [23].
The e-PDG not using a GTPv2 based protocol over s2b interface and the AAA server are subjected to all the requirements specified in this document for PDG and AAA server for the case of I-WLAN interworking.

Interception in the PDN-GW is a national option.

Interception in the PDN-GW shall be based on IMSI or NAI. In case of GTPv2 based protocol, interception at the ePDG and PDN-GW shall be based on IMSI.
NOTE:
The NAI may be a temporary ID, therefore the use of  IMSI is recommended.
For the delivery of the CC and IRI, the PDN-GW and ePDG provides correlation number and target identity to the DF2 and DF3 which is used there in order to select the different LEAs where the product shall be delivered.

12.5.1
Provision of Intercept Related Information

12.5.1.0
General
Intercept Related Information (Events) shall be sent at attach/tunnel activation on interfaces s2b and s2c, detach/tunnel deactivation, session modification, start of interception with active tunnel, Serving Evolved Packet System. 
In case of GTPv2 based s2b, Intercept Related Information shall be sent at attach/bearer activation, detach/bearer deactivation, bearer modification and start of interception with active bearer.
Serving Evolved Packet System reporting is a national option.

12.5.1.1
X2-interface

The following information needs to be transferred from the PDN-GW, ePDG or the HSS to the DF2 in order to allow a DF2 to perform its functionality:

-
target identity;

-
events and associated parameters as defined in clause 12.5.1.2 and 12.5.3 may be provided;

-
the target location (if available) or the IAs in case of location dependent interception; (FFS)
-
correlation number;

-
Quality of Service (QoS) information, if available;
-
encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.
The PDN-GW detect packets containing packet header information in the communications path but the information needed for Packet Header Information Reporting may need to be transferred from the PDN-GW either directly to the DF2 or via another network entity in order to allow the DF2 to perform its functionality. 
The IRI should be sent to DF2 using a reliable transport mechanism.

12.5.1.2
Structure of the events

There are several different events in which the information is sent to the DF2 if this is required. The events for interception are configurable (if they are sent to DF2) in the PDN-GW, ePDG or the HSS and can be suppressed in the DF2.

The following events are applicable to the PDN-GW:

-
PMIP attach/tunnel activation;

-
PMIP detach/tunnel deactivation;

-
PMIP session modification;

- 
Start of interception with active PMIP tunnel;

-
DSMIP registration/tunnel activation;

-
DSMIP deregistration/tunnel deactivation;

-
DSMIP session modification;

-
Start of interception with active DSMIP tunnel;

-
DSMIP HA Switch;

-
PMIP Resource Allocation Deactivation ;

-
Packet Header Information 
-
Bearer activation;

-
Bearer deactivation;

-
Bearer modification;

-
Start of interception with active bearer.

The following events are applicable to the ePDG:

-
Bearer activation;

-
Bearer deactivation;

-
Bearer modification:

-
Start of interception with active bearer.
The following events is applicable to the HSS:

-
Serving Evolved Packet System

A set of elements as shown below can be associated with the events. The events trigger the transmission of the information from the nodes to DF2. Available IEs from this set of elements as shown below can be extended in the nodes, if this is necessary as a national option. DF2 can extend available information if this is necessary as a national option. When the GTP protocol is used over the s2b interface, elements from table 12.2.1.2 are included in the applicable events.
	Observed MN NAI

The Network Access Identifier of the Mobile Node (target identity).

	Observed IMSI

The IMSI of the target.

	Event type

Indicates which type of event is delivered: PMIP attach/tunnel activation, PMIP detach/tunnel deactivation, Start of interception with active PMIP tunnel, DSMIP registration/tunnel activation, DSMIP deregistration/tunnel deactivation, Start of interception with active DSMIP tunnel, DSMIP HA Switch, PMIP resource Allocation Deactivation, Serving Evolved Packet System.

	Event time

Time of the event generation in the ICE. Time stamp shall be generated relative to ICE internal clock.

	Event date

Date of the event generation in the ICE.

	Correlation number

The correlation number is used to correlate CC and IRI.

	Network Element Identifier

Unique identifier for the ICE reporting the event.

	Logical Function Information

Used to distinguish between multiple logical functions operating in a single physical network element.

	Lifetime

Indicates the lifetime of the tunnel; must be set to a nonzero value in the case of registration or lifetime extension; is set to zero in case of deregistration.

	Failed attach reason

Reason for the failed attach/tunnel deactivation of the target.

	Session modification failure reason

Reason for a failure of a session modification attempt for the target 

	Access technology type

Indicates the Radio Access Type.

	Handover indicator

Provides information on whether the triggered as part of a handover.

	APN

The Access Point Name used for the connection.

	UE address info

Includes one or more IP addresses allocated to the UE.

	Additional Parameters

Additional information provided by the UE, such as protocol configuration options.

	Home Agent address

Contains the IP address of the Home Agent.

	Care of Address

The Local IP address assigned to the UE by the Access Network, used as Care of Address for DSMIPv6 over S2c reference point.

	HSS/AAA address

The address of the HSS/AAA triggering the PDN-GW reallocation.

	Target PDN-GW address

The address of the PDN-GW which the UE will be reallocated to.

	Revocation trigger

Contains the cause for the revocation procedure.

	Foreign domain address

The relevant IP address in the foreign domain.

	Visited network identifier

An identifier that allows the home network to identify the visited network [24].

	Requested IPv6 Home Prefix

The IPv6 Home Prefix requested by the UE.

	IPv6 home prefix

The IPv6 home prefix assigned by the PDN GW to the UE.

	Home address

Contains the UE Home IP address.

	Destination IP Address

The IP address, including type IPv4 or IPv6, of the destination of the IP packet.

	Destination Port Number

The port number of the destination of the IP packet.

	Flow Label (IPv6 only)

The field in the IPv6 header that is used by a source to label packets of a flow (see RFC 3697 [41]).

	Packet Count 

The number of packets detected and reported (for a particular summary period).

	Packet Data Summary Reason

The reason for a Packet Data Summary message being sent to the LEMF (e.g., timed out, counter expiration, end of session)

	Packet Size

The size of the packet. (i.e., Total Length Field in IPv4 or Payload Length field in IPv6)

	Source IP Address

The IP address, including type IPv4 or IPv6, of the source of the IP packet.

	Source Port Number

The port number of the source of the IP packet.

	Sum of Packet Sizes (for a particular summary period)

The sum of values contained in the Total Length fields of the IPv4 packets or the sum of the values contained in the Payload Length fields of the IPv6 packets.

	Summary Period

Includes the dates and times of the first and last packets in a particular packet data interval.  

	Transport Protocol (e.g., TCP)

The identification of the transport protocol of the packet or packet flow being reported.


12.5.2
X3-interface

The access method for the delivering of PDN-GW and/or ePDG Intercept Product is based on duplication of packets without modification at the intercepting node. The duplicated packets with additional information in a header are sent to DF3 for further delivery to the LEA.




[image: image52.emf] 

PDN - GW, ePDG  

other   party  

Target  

LEA  

Delivery   Function 3  

Duplication of packets  


Figure 12.5.2.1: Configuration for interception of PDN-GW, ePDG product data
In addition to the intercepted content of communication, the following information needs to be transferred from the PDN-GW and/or ePDG to the DF3 to perform its functionality:

· target identity;

· correlation number;

· time stamp (optional);

· direction (indicates whether T-PDU is MO or MT) – optional;

· the target location (if available) or the IAs in case of location dependent interception.

NOTE: location dependent interception for EPC is FFS.

12.5.3
LI events for untrusted Non-3GPP IP access

12.5.3.1
Initial Attach and PDN connection activation with PMIPv6 on S2b

In the VPLMN, LI shall be done at the ePDG according to LI requirements for I-WLAN; no additional requirement applies to the S-GW for this case.

When the attach or PDN connectivity activation is detected over PMIP at the PDN-GW, a PMIP attach/tunnel activation event shall be generated. The following elements will be delivered to the DF2 if available:

	Observed MN NAI

	Observed IMSI

	Event Type

	Event Time

	Event Date

	Correlation number

	Logical Function Information

	Network Element Identifier

	Lifetime

	Failed attach reason

	Access Technology Type

	Handoff Indicator

	APN

	UE Address Info

	Additional Parameters


12.5.3.2
Initial attach and PDN connection activation for S2c in untrusted non-3GPP IP access

In the VPLMN, LI shall be done at the ePDG according to LI requirements for PDG for I-WLAN.
When the attach or PDN connectivity activation is detected over DS-MIPv6 at the PDN-GW, a DSMIP registration/tunnel activation event shall be generated. The following elements will be delivered to the DF2 if available:

	Observed MN NAI

	Observed IMSI

	Event Type

	Event Time

	Event Date

	Correlation number

	Network Element Identifier

	Logical Function Information

	Lifetime

	Failed attach reason

	Home address

	Care of Address

	APN

	Requested IPv6 Home Prefix


12.5.3.3
UE/ePDG-initiated Detach Procedure and UE Requested PDN disconnection with PMIP

In the VPLMN, LI shall be done at the ePDG according to LI requirements for PDG for I-WLAN; no additional requirement applies to the S-GW for this case.

When the detach or UE requested PDN disconnection is detected over PMIP at the PDN-GW, a PMIP detach/tunnel deactivation event shall be generated. The following elements will be delivered to the DF2 if available:

	Observed MN NAI

	Observed IMSI

	Event Type

	Event Time

	Event Date

	Correlation number

	Network Element Identifier

	Logical Function Information

	APN


12.5.3.4
Detach and PDN Disconnection for S2c in Un-trusted Non-3GPP IP access

In the VPLMN, LI shall be done at the ePDG according to LI requirements for PDG for I-WLAN.

When the detach or PDN disconnection is detected over DS-MIPv6 at the PDN-GW, a DSMIP deregistration/tunnel deactivation event shall be generated. The following elements will be delivered to the DF2 if available:

	Observed MN NAI

	Observed IMSI

	Event Type

	Event Time

	Event Date

	Correlation number

	Network Element Identifier

	Home address

	Logical Function Information

	Initiator

	Care of Address


12.5.3.5
Serving Evolved Packet System

The Serving Evolved Packet System report event is generated at the HSS, when the HSS has detected that the target has roamed. The elements will be delivered to the DF2 if available:

	Observed MSISDN

	Observed IMSI

	Observed ME Identity

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Logical Function Information

	Visited Network Identifier


12.5.3.6 
Start of interception with active tunnel/bearer
When interception is started at the PDN-GW/ePDG and the target has an already active tunnel/bearer, a start of interception with active tunnel/bearer shall be generated. The event shall be detected by the same node for which tunnel/bearer activation reporting is applicable and reported. Separate events are defined for the specific protocol (PMIP, DSMIP, GTP). When the GTP protocol is used for the s2b interface, the event Start of interception with active bearer is applicable as specified in section 12.2.3.6. The parameter applicable to the tunnel activation event,  as defined in the related sections, will be delivered to the DF2 if available. One event shall be sent for each active tunnel.

12.5.3.7
PDN-GW reallocation upon initial attach on s2c

When a PDN GW reallocation procedure is detected by the PDN-GW, a DSMIP HA Switch event shall be generated. The following elements will be delivered to the DF2 if available:

	Observed MN NAI

	Observed IMSI

	Event Type

	Event Date

	Event Time

	Network Element Identifier

	Logical Function Information

	HSS/AAA address

	Target PDN-GW address


12.5.3.8
PDN GW initiated Resource Allocation Deactivation with S2b PMIP

When a PDN GW initiated resource allocation deactivation is detected, a PMIP Resource Allocation Deactivation event shall be sent. The following elements will be delivered to DF2 if available:
	Observed MN NAI

	Observed IMSI

	Event Type

	Event Date

	Event Time

	Network Element Identifier

	Logical Function Information

	Revocation trigger

	UE address info

	Correlation number


12.5.3.9
PMIP session modification

In the VPLMN, LI shall be done at the ePDG according to LI requirements for PDG for I-WLAN; no additional requirement applies to the S-GW for this case.

When a session modification is detected at the PDN-GW, a PMIP session modification event shall be generated by the PDN-GW. The following elements will be delivered to the DF2 if available:

	Observed MN NAI

	Observed IMSI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Logical Function Information

	Correlation number

	Lifetime

	UE Address Info

	Access Technology Type

	Additional Parameters

	Session failure modification reason

	Handover indicator


12.5.3.10
DSMIP session modification

In the VPLMN, LI shall be done at the ePDG according to LI requirements for PDG for I-WLAN.
When the session modification is detected over DS-MIPv6 at the PDN-GW, a DSMIP session modification event shall be generated. The following elements will be delivered to the DF2 if available:

	Observed MN NAI

	Observed IMSI

	Event Type

	Event Time

	Event Date

	Correlation number

	Network Element Identifier

	Logical Function Information

	Lifetime

	Session failure modification reason

	Home address

	Care of Address

	APN

	Requested IPv6 Home Prefix


12.5.3.11
Packet Header Information Reporting

12.5.3.11.0
General

Packet Header Information Reporting can be done either on a per-packet (i.e., non-summarized) basis or in a summary report.  

12.5.3.11.1
Packet Data Header Report

This event is used to provide packet header reports on a per packet basis (non-summarized reporting) and is triggered by each packet sent or received by the target.  These elements will be delivered by the PDN-GW either directly to the DF2 or via another network entity if available:

	Observed MN NAI

	Observed MSISDN

	Observed IMEI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Correlation number

	Logical Function Information

	Lifetime

	UE Address Info

	Access Technology Type

	Serving Network

	Home address

	Care of Address

	APN

	Location information

	Source IP Address

	Source Port Number

	Destination IP Address

	Destination Port Number

	Transport Protocol (e.g., TCP)

	Packet Size

	Flow Label (IPv6 only)


12.5.3.11.2
Packet Data Summary Report

This event is used to report:

1)
the source and destination information derived from the packet headers, including:

a)
source and destination IP Addresses,

b)
IP next-layer protocol,

c)
Layer-4 ports, and

d)
Flow label, if the packet is IPv6

2)
summary information for the number of packets and bytes transmitted or received by the target for each unique packet flow within an EPS bearer,  and 

3)
the date and the time of the first and last packets associated with that packet flow.  A packet flow is defined as the 6-tuple of source/destination IP address/port number and the layer 4 protocol and EPS bearer.   

IP addresses and the IP next-layer protocol are always reported, the flow label is reported if the packet is IPv6, and the layer-4 ports are reported.    

The event provides packet summary reports for each unique packet data session (EPS bearer) and packet flow, and is triggered by one of the following:

-
start of a packet flow associated with an EPS bearer

-
an interim report for a packet flow associated with an EPS bearer is to be reported

-
end of a packet flow associated with an EPS bearer (including end of the EPS bearer itself).

An interim report can be triggered by

-
The expiration of a configurable timer per intercept (called a Summary Timer). The Summary Timer is configurable in units of seconds;

-
A per-intercept configurable count threshold is reached.

These elements will be delivered either directly to DF2 or via DF3 for each packet flow if available:

	Observed MN NAI

	Observed MSISDN

	Observed IMEI

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Logical Function Information

	Correlation number

	Lifetime

	UE Address Info

	Access Technology Type

	Serving Network

	Home address

	Care of Address

	APN

	Location information

	Source IP Address

	Source Port Number

	Destination IP Address

	Destination Port Number

	Transport Protocol (e.g., TCP)

	Flow Label (IPv6 only)

	Summary Period

	Packet Count (for this summary period)

	Sum of Packet Sizes (for this summary period)


If the packets are IPv4, the sum of all observed packet sizes is the sum of the values contained in the Total Length field of each packet as specified in IETF RFC 791[39].
If the packet is IPv6, the sum of all observed packet sizes is the sum of the values contained in the Payload Length field for each packet as specified in IETF RFC 2460 [40].
If no packets were detected for the duration of the Summary Timer, then the Packet Data Summary Report shall not be sent.
12.5.3.12
Bearer activation
When the Attach is handled by the ePDG over the GTP based s2b interface (TS 23.402 [23]), or the Dedicated bearer activation on the GTP based S2b interface (TS 23.402 [23]) is detected by the ePDG, or a Bearer activation is detected at the PDN-GW, a Bearer activation event shall be generated. The elements listed in section 12.2.3.3 will be delivered to the DF2 if available.
12.5.3.13
Bearer deactivation
When the Detach is handled by the ePDG over GTP S2b interface (TS 23.402 [23]), or a Bearer deactivation is detected at the PDN-GW, or the PDN GW initiated Resource Allocation Deactivation is detected by the ePDG on GTP based s2b interface, a Bearer deactivation event shall be generated. The elements listed in section 12.2.3.4 will be delivered to the DF2 if available.
12.5.3.14

Bearer modification

When a Bearer Modification is handled by the ePDG over GTP S2b interface (TS 23.402 [23]), or a Bearer modification is detected at the PDN-GW, a Bearer modification event shall be generated. The elements listed in section 12.2.3.5 will be delivered by the ePDG to the DF2 if available.

12.6
Functional requirements for LI in case of Handovers between E-UTRAN and CDMA2000 Accesses.

When an handover is performed from CDMA2000 Access to E-UTRAN, the MME shall intercept the attach event received from the HRPD AN based on IMSI. 

Interception at S-GW and PDN-GW shall be done according to the requirements given in section 12.2 or 12.3 and related subsections, depending on the protocol used over the S5/S8 interface.

12.7
Functional requirements for LI in case of interworking between SGSN and EPS nodes over S4/S12 interfaces

The SGSN and the HSS are subjected to the requirements applicable to these nodes for PS interception, as specified throughout this document.

The S-GW is subjected to the requirements specified in section 12.2 and subsections. The applicable events shall be reported also when received from the SGSN over S4 interface. CC shall be also reported when received over S4/S12 interfaces. The network procedures for which the events applicable to the S-GW, defined in section 12.2 and subsections, are generated when the S-GW is connected over S4/S12 interfaces to a SGSN are defined in [10].

The PDN-GW is subjected to the requirements specified in section 12.2 or 12.3 and related subsections, depending on the protocol used on S5/S8 interfaces, which are applicable also to the case in which the PDN-GW is involved for a target for which a S4 based SGSN is used.

12.8
Functional requirements for LI in case of interworking between SGSN and PDN-GW over Gn/Gp interfaces 

According to [10] and [22] a PDN-GW may provide a Gn/Gp interface for interworking with the SGSN. When this interface is provided, from LI perspective the PDN-GW acts as a GGSN towards the involved SGSN. In this case, in addition to the requirements specified in this clause, all the requirements specified by this document for the GGSN are applicable to the PDN-GW. 

The PDN-GW shall use the same correlation number in records when the PDP context/EPS bearer modification signalling is detected due to the handover between different accesses involving a Gn/Gp interface (i.e. from E-UTRAN to 2G/3G and vice versa).  After the handover, the PDN-GW shall report the events applicable to the new access and continue to use the same correlation number inside the same PDP context/EPS bearer.

The SGSN is subjected to the requirements applicable to this node for PS interception, as specified throughout this document.

13
Lawful Interception for 3GPP H(e)NBs

13.0
General

Home Node B (HNB) and Home enhanced Node B (HeNB) are jointly referred to as H(e)NB as defined in TS 22.220 [31].  As identified in TS 33.106 [7], lawful interception for 3GPP H(e)NBs can be based on three different targets:  a target accessing a H(e)NB, a target CSG of a H(e)NB, and a target H(e)NB.   

LI for a target CSG is FFS.

LI for a target H(e)NB is FFS.

LI for Local IP Access (LIPA) via a H(e)NB is FFS.

13.1
Provision of Intercepted Content of Communications for 3GPP H(e)NBs

The access method for the delivery of intercepted content of communications (CC) is based on duplication of packets without modification. 

See clause 13.4 for UMTS HNB specifics and 13.5 for HeNB specifics.

Note:  In the case where the UE is the target of intercept, from the perspective of the core network, a H(e)NB is treated the same as a NodeB or eNodeB for CC interception purposes (i.e., no additional LI functionality is required).

13.2
Provision of Intercept Related Information for 3GPP H(e)NBs

13.2.1
X2-interface

The following information needs to be transferred to the DF2 in order to allow a DF2 to perform its functionality:

-
target identity (IMSI, IMEI, MSISDN, ME Id); 

-
events and associated parameters as defined in sections 13.4.3 may be provided;

-
the H(e)NB location (if available);

-
H(e)NB ID;

H(e)NB location information needs to be transferred from the location verifying nodes per TS 33.320 [34]to the DF2 in order to allow the DF2 to perform its functionality.  The manner that the location verifying node provides the DF2 with the H(e)NB location is outside the scope of this document.

The IRI should be sent to DF2 using a reliable transport mechanism.

13.3

3GPP H(e)NB LI Events and Event Information

For a target UE that is attached to any H(e)NB, LI events and messages for 3GPP H(e)NBs defined in this clause shall be reported in addition to the LI events and messages defined in other clauses of this document.  H(e)NB LI events and event information are included in 13.4 for UMTS HNBs and 13.5 for HeNBs.

A set of possible elements as shown below is used to generate the events. Information associated with the events is transmitted from the IRI ICES to DF2.

Table 13: Information Events for H(e)NB Event Records

	Element
	Definition/Usage

	Cause
	Reason for an error or an action

	Context-Id
	Unique identifier for a UE used by the HNB and HNB GW.

	CSG Identity
	Uniquely identifies a CSG within one PLMN.  Note:  Open H(e)NBs do not have associated CSGs.

	CSG List
	Identifies the membership of a given CSG (i.e., CSG Identities and associated expiration data for the UEs).

	Destination cell ID
	Resultant cell ID after handover (HNB ID or PLMN cell ID)

	Event type
	Description which type of event is delivered

	Event date
	Date of the event generation

	Event time
	Time of the event generation.

	Handover Direction


	Identifies if the handover is inbound (from macro network to H(e)NB), outbound (from H(e)NB to macro network) or intra-H(e)NB (between H(e)NBs).  

	H(e)NB Identity
	Uniquely identifies a H(e)NB (i.e., H(e)NB equipment ID and H(e)NB name)

	H(e)NB IP Address
	Reports the location of the H(e)NB used during location verification..

	H(e)NB Location
	When authorized, reports the location of the H(e)NB used during location verification prior to H(e)NB activation.

	IAs
	The observed Interception Areas

	Initiator


	The initiator of an action (e.g., network or specific network entity, target, associate)

	ISP Operator Identity
	Identifies the ISP through which the H(e)NB is connected to the SeGW

	Network Identifier


	Unique identifier for the operator and the element carrying out the LI operations

	Observed MSISDN
	MSISDN of the target.

	Observed IMSI
	IMSI of the target.

	Observed IMEI
	IMEI of the target.  

	Observed ME Id
	ME Id of the target; when it coincides with the IMEI, it shall be checked for each activation over the radio interface

	Security Gateway IP Address
	The IP Address of the Security Gateway used by the H(e)NB to terminate the tunnel from the H(e)NB

	Source Cell ID
	Original cell ID prior to handover (HNB ID or PLMN cell ID)

	Tunnel Protocol
	The tunnel protocol used between the H(e)NB and the SeGW


Editor’s note:  Identification of which HNB IP Address is reported is needed.
13.4
UMTS Home Node B (HNB)
13.4.0
General

Figures 13-1 shows the reference architectures upon which Lawful Interception for 3GPP HNBs is based.    


[image: image53.emf] 

UE  

HNB  

SeGW  

insecure  link  

HNB GW  

HMS  

HMS  

AAA  Server/HSS  

3G GSN (PS)  

MSC/VLR/  MGW (CS)  

Operator ’ s core network  

Figure 13-1: 3GPP UMTS HNB Architecture Basis for Lawful Interception

13.4.1
Intercepted Content of Communications for 3GPP UMTS HNBs

Editor’s Note:  This section is a place holder for the scenarios where the target of interception is either a CSG or a HNB.

13.4.2
Intercept Related Information

13.4.2.0
General
Figures 13-2 show the transfer of intercept related information to the DF2. 



[image: image55.emf] 

HNB location   v erifying node  

LEMF  

DF  2  

Other Party  

HNB  

Target  

HNB   GW   SGSN (PS)   MSC/VLR (CS)  

SeGW  


Figure 13-2: Provision of Intercept Related Information for 3GPP UMTS HNB

13.4.2.1
X2-interface

The following information needs to be transferred from the HNB GW to the DF2 in order to allow a DF2 to perform its functionality:

-
target identity (IMSI, IMEI, MSISDN, ME Id); 

-
events and associated parameters as defined in section 13.4.3 may be provided;

-
the HNB location (if available);

-
HNB Identity;

HNB location information needs to be transferred from the location verifying node to the DF2 in order to allow the DF2 to perform its functionality.

The IRI should be sent to DF2 using a reliable transport mechanism.

13.4.3
3GPP UMTS HNB LI Events and Event Information

The following events are applicable at the HNB GW:

-
Target UE Registration to HNB;

-
Target UE De-Registration from HNB;

-
Start of Interception with HNB attached UE;

-
Target UE HNB Handover

A set of possible elements used to generate the events is found in clause 13.3 in Table 13. Information associated with the events is transmitted from the HNB GW to DF2.

13.4.4
Structure of HNB Events

13.4.4.1
Target UE Registration to HNB 

This event reports when a target UE is attempting to register to any HNB.  This event is generated when 

· a HNB GW sends a UE REGISTRATION ACCEPT message towards a target UE, or 

· a HNB GW sends a UE REGISTRATION REJECT message towards a target UE, or 

· a HNB GW receives an SCCP Connection Confirm (CC) or Connection Refused (CREF) messages from the Core Network 

The elements, shown in Table 13a, will be delivered to the DF2, if available.

Table 13a: UE Registration to HNB

	Observed MSISDN 

	Observed IMSI

	Observed IMEI

	Observed ME Id

	Event Type

	Event Time

	Event Date

	Network Identifier

	Context-ID (for successful connection)

	H(e)NB Identity

	H(e)NB Location  

	H(e)NB IP Address 

	Security Gateway IP address

	Tunnel Protocol

	ISP Operator Identity

	Cause (of failed connection, e.g., “Refusal Cause” of SCCP CREF)

	CSG Identity (if closed/hybrid HNB)

	CSG List (if closed/hybrid HNB) – See Note 1

	IAs (if applicable)


Note 1:  In a HNB GW, the CSG List is the Access Control List. 

13.4.4.2
Target UE De-Registration from HNB

This event reports a when a target UE is de-registered to any HNB.  This event is generated when 

· a HNB GW receives a UE DE-REGISTER message from the HNB, or 

· a HNB GW receives a RANAP Release Iu Connection Command message from the Core Network 

The elements, shown in Table 13b, will be delivered to the DF2, if available.

Table 13b: UE De-Registration from HNB

	Observed MSISDN 

	Observed IMSI

	Observed IMEI

	Observed ME Id

	CSG Identity (if closed or hybrid H(e)NB)

	Event Type

	Event Time

	Event Date

	Network Identifier

	H(e)NB Identity

	H(e)NB Location  

	Initiator (i.e., HNB or Network)

	Cause (of de-registration action, if known)

	IAs (if applicable)


13.4.4.3
Start of Intercept with HNB attached UE 

This event will be generated if interception for a target UE is started when the target UE has already registered and is receiving service from a HNB.  The elements, shown in Table 13c, will be delivered to the DF2, if available.

Table 13c: Start of Intercept with Target UE active on a HNB 
	Observed MSISDN 

	Observed IMSI 

	Observed IMEI

	Observed ME Id

	H(e)NB Identity

	CGS Identity (if closed or hybrid H(e)NB)

	Event Time

	Event Date

	Network Identifier

	H(e)NB IP Address 

	Security Gateway IP address

	Tunnel Protocol

	ISP Operator Identity

	CSG List (if closed or hybrid HNB) – See Note 1

	H(e)NB Location 

	IAs (if applicable)


Note 1:  In a HNB GW, the CSG List is the Access Control List. 

13.4.4.4
Target UE HNB Handover 

This event reports a when a registered target UE moves from a cell on the serving PLMN to a HNB, from a HNB to a cell on the serving PLMN, or from a HNB to another HNB.  This event is generated when 

· a HNB GW receives an inbound UE relocation trigger (e.g., RANAP Relocation Request message from the Core Network), or 
· a HNB GW receives a HNBAP:  UE RELOCATION COMPLETE message from the Destination HNB (i.e., the “Target HNB” per TS 25.467[33]) , or

· a HNB GW acts as a Iurh proxy and sends a RADIO LINK RESTORE INDICATION message from the “Drift HNB” to the “Serving HNB” per TS 25.467[33]) (i.e., a target UE is involved in a soft handover between HNBs)

The elements, shown in Table 13d, will be delivered to the DF2, if available.

Table 13d: UE Handover

	Observed MSISDN 

	Observed IMSI

	Observed IMEI

	Observed ME Id

	Event Type

	Event Time

	Event Date

	Network Identifier

	Context-ID (for successful connection)

	Cause (of failed connection, e.g., “Refusal Cause” of SCCP CREF)

	CSG Identity (if closed/hybrid HNB)

	CSG List (if closed/hybrid HNB) – See Note 1

	Handover Direction

	Source cell ID (HNB ID or PLMN cell ID)

	Destination cell ID (HNB ID or PLMN cell ID)

	IAs (if applicable)


Note 1:  In a HNB GW, the CSG List is the Access Control List. 

The reporting of a soft handover between HNBs that are directly connected and the HNB GW is not involved is FFS.

13.5
Home enhanced Node B (HeNB)
Figure 13-3 show the reference architectures upon which Lawful Interception for 3GPP HeNBs is based.  Per TS 36.300 [32], HeNB GW is optional.  


[image: image56.emf] 

UE  

H e NB  

SeGW  

insecure  link  

Operator ’ s core network  

HeNB GW  

HeMS  

HeMS  

AAA  Server/HSS  

S - GW (UP)   MME (CP)  


Figure 13-3: 3GPP HeNBs Architecture Basis for Lawful Interception

In the case where the UE is the target of intercept, LI functionality is specified in clause 12.

14
Interception of Generic Bootstrapping Architecture (GBA) Secured Communications

14.1
Introduction

The Generic Bootstrapping Architecure (GBA) is defined in the 3GPP TS 33.220 [35]. This section details the stage 2 Lawful Interception architecture and functions that are needed to provide the GBA based application specific encryption keys from the GBA architecture towards the DF2 for a subscriber that is target of interception.

Figure 14.1 shows the LI architecture for the GBA where the BSF provides the events and associated information towards the DF2 over the X2 interface.


[image: image57.emf] 

LEMF  

Mediation  

Function  

Delivery  

Function 2  

Mediation  

Function  

ADMF  

X2  

X1_2  

X1_1  

HI1  

HI2  

BSF  


Figure 14.1: GBA Intercept Configuration

14.2
Provision of Content of Communications

The GBA interception provides the application specific cryptographic keys (aka GBA application specific keys) which are used to decrypt the intercepted communication secured using those GBA application specific keys. Interception of the content of communications for GBA secured services is not part of this section and can be achieved via other methods outlined in this specification. The Ua protocol Id and the NAF Id along with the GBA application specific keys will allow the LEMF to decrypt the received intercepted packets. 

Note 1: The details of LI capabilities for GBA in a roaming scenario is for further study.  

Note 2: The delivery by the CSP of intercepted packets in a decrypted form is for further study. 

14.3
Provision of Intercept Related Information

14.3.1
Provision of Intercept Related Information Data Flow

Figure 14.2 shows the transfer of intercept related information to the DF2. If an event related to a target occurs, the BSF shall send the relevant data to the DF2.




[image: image59.emf] 

  BSF  

Target  

Delivery   Function 2  

LEMF  


Figure 14.2: Provision of Intercept Related Information

14.3.2
X2-interface

The following information needs to be transferred from the BSF to the DF2 in order to allow a DF2 to perform its functionality:

-
target identity;

-
events and associated parameters as defined in clauses 14.3.3 may be provided;

The IRI should be sent to DF2 using a reliable transport mechanism.

14.3.3
GBA LI Events and Event Information

Intercept Related Information (Events) are necessary for the following;

· Bootstrapping

· Query from NAF

· Start of interception with GBA key

A set of possible elements as shown in Table 14.3.1 are used to generate the events.

Table 14.3.1: Information Events for GBA Event Records

	Element

	Observed IMSI

IMSI of the target.

	Observed Other Identity

Other Identity of the target.

	Event type

Description which type of event is delivered: Bootstrapping, Query from NAF, 
Start of interception with GBA key

	Event date

Date of the event generation in the BSF

	Event time

Time of the event generation in the BSF. 

	Network Element Identifier

Unique identifier for the element reporting the BSF.

	B-TID

Bootstrapping transaction identifier, TS 33.220 [35].

	Key lifetime

The lifetime of the key material is set according to the local policy of the BSF, TS 33.220 [35].

	Bootstrapping time

The timestamp of the bootstrapping event.

	Ks_int_NAF

GBA application specific key (internal), if GBA_U has been used, TS 33.220 [35].

	Ks_ext_NAF

GBA application specific key (external), if GBA_U has been used, TS 33.220 [35].

	Ks_NAF

GBA application specific key, if GBA_ME has been used, TS 33.220 [35].

	Ua protocol id

Ua interface security protocol id defined in clause Annex H in TS 33.220 [35].

	NAF_Id

The FQDN of the NAF, concatenated with the Ua security protocol identifier, TS 33.220 [35].


14.4
Structure of GBA Events

14.4.1
Bootstrapping

This event will be generated when the UE triggers a bootstrapping procedure towards the BSF when the UE wants to interact with a NAF. The actual bootstrapping procedure is defined in the TS 33.220 [35], in sections 4.5.2 and in 5.3.2. The information elements shown in Table 14.4.1 table, if available, will be delivered to the DF2, by the BSF.

Table 14.4.1: Bootstrapping
	Observed IMSI

	Observed Other Identity

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	B-TID

	Key lifetime

	Bootstrapping time


14.4.2
Query from NAF

The Query from NAF event is generated when the BSF receives an application specific key query from a NAF in order to retrieve GBA based application specific keys and related information. A new event is generated for each individual query events. The information elements shown in Table 14.4.2 will be delivered to the DF2, if available, by the BSF. 

Table 14.4.2: Query from NAF
	Observed IMSI

	Observed Other Identity

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Ks_ext_NAF

	NAF_Id

	Ks_int_NAF

	Ks_NAF

	Key lifetime

	Bootstrapping time

	Ua protocol id


14.4.3
Start of Interception with GBA key

For start of interception where GBA application specific key is already in use a Start of Interception with GBA key event is generated. The elements, shown in Table 14.4.3 will be delivered to the DF2, if available, by the BSF. 

Table 14.4.3: Start of Interception with GBA key

	Observed IMSI

	Observed Other Identity

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	B-TID [Note]

	NAF_Id [Note]

	Ks_ext_NAF [Note]

	Ks_int_NAF [Note]

	Ks_NAF [Note]

	Key lifetime [Note]

	Bootstrapping time [Note]

	Ua protocol id [Note]


Note: These are repeated for each GBA application specific key associated with the target. 

15
Invocation of Lawful Interception for IMS-based VoIP

15.1
Overview of VoIP Interception 

The capabilities defined in this clause apply when the interception of content of communications for IMS-based VoIP is to be separated from the interception of content of communication at the packet data network. 

The network nodes, involved in providing the interception of an IMS-based VoIP call, shall be determined based on the deployment configuration and the call scenario. The scenarios where the media transport nodes and signalling nodes are handled by different CSPs are beyond the scope of this standard. 

Note 1: Lawful interception of VoIP as it applies SR-VCC (see 23.237 [46]) is for further study.  

The interception of IRI for a VoIP call shall be done according to 15.3. The interception of VoIP CC shall be done according to 15.2. 
15.2
Provision of Content of Communications 

As the interception of CC needs to be done at a network node that has access to the voice media, and that interception of CC is required for all targeted calls, including forwarded calls and transferred calls, the CSP needs to support the capability to dynamically trigger CC interception for a call at a network node that has access to the voice media. Depending on the CSP’s network configuration and the call scenario, different network elements will intercept the CC.

The interception and delivery of CC for VoIP may be done at the following functional element: 

1)
PDN-GW/GGSN;

2)
IMS-AGW;

3)
TrGW;

4)
IM-MGW;

5)
MRF. 

Note 2:  Other functional elements may also be applicable in specific deployment scenarios. 
Note 3:  The redirection of target communications to a specific network element purely for LI purposes is undesirable.
The functional elements that provide the signalling to generate the trigger for the CC interception may be any of the following functional elements:  

-
P-CSCF, for PDN-GW/GGSN and IMS-AGW;

-
IBCF for TrGW;

-
MGCF for IM-MGW
-
S-CSCF or AS for MRF. 

At any given time, for a specific target and for any given call, only one functional element is required to provide the CC interception. The functional element that provides the CC interception may vary, primarily, based on the call scenario. 

Annex E shows scenarios where the use of the above functional elements are applicable. 





15.2.1
General Principles of CC Interception

15.2.1.1
Intercept Trigger

As the interception of IRI and CC is required for all targeted VoIP calls, including forwarded and transferred calls, the CC shall be correlated with the IRI. The CC Interception Triggering Function triggers the CC interception for a call at the CC Intercept Function. The placement of the CC Interception Triggering Function is dependent on CSP network implementation, the call scenario, and the placement of network nodes that have access to the voice media.

The CC Interception Triggering Functions sends a CC intercept trigger to the CC Interception Function to activate CC interception for a call.

The intercept trigger, at the minimum, shall consist of the following: 

-
Correlation Identifier (the mechanics of correlation is for further study);

-
Media Identifier

Editor’s Note: The re-use of the existing reference point or whether to use a new reference point is for further study. 

The Correlation Identifier is used correlate the CC with the corresponding IRI data and is delivered from the CC Intercept Function in the intercepted media packet (i.e., CC) over the X3 interface to the Delivery Function 3. 

The Media Identifier is used to identify the media packets that have to be intercepted. The technique used in defining the Media Identifier is implementation specific. 

The information passed in this CC intercept trigger shall adhere to the security requirements outlined in clause 8.  

15.2.1.2
X3-Interface

For the delivery of intercepted media packets, the following information shall be passed from the CC Intercept Function to the Delivery Function 3 in addition to the intercepted media packets:

-
target identity;

-
correlation identifier;

-
time stamp (optional);

-
direction (indicates media is from or to the target) – optional;

Editor’s Note: The use of the target identity for the information passed to the Delivery Function 3 requires further study.

The Delivery Function 3 delivers the information to the LEMF over the HI3 interface based on the national regulations.  
15.2.2
VoIP CC Interception 

The capabilities defined in this clause apply for the following cases: 

-
When a target originates a call or receives an incoming call – the target’s media passes through the indicated CC Intercept Function.

-
When an incoming call to the target is forwarded, the media of the forwarded call passes through the indicated CC Intercept Function. 

The term "CC Intercept Function" is a generic term used to denote a network function that has access to the voice media of an intercepted call. The term "CC Interception Triggering Function" is a generic term used to denote a network function that provides a trigger to intercept the CC. The examples of CC Intercept Function and CC Interception Triggering Function are listed at the beginning of clause  15.2 

Figure 15.1 illustrates the CC interception at the CC Intercept Function for a basic call. Figure 15.2 illustrates the CC interception at the CC Intercept Function for a forwarded call.

 


[image: image61.emf] 

CC Interception   Triggering   Function  

CC Intercept   Function  

Other  Party  

Target  

LEMF  

Delivery Function 3  

Duplicato r  of Media Packets  

X3  

HI 3  




Figure 15.1: VoIP CC Interception for basic calls

In figure 15.1, the Target is the target and the Other Party is the called party when the target originates a call; and the Other Party is the calling party when the target receives an incoming call. In both cases, the media passes through the CC Intercept Function present on the side of target’s access network.  

In figure 15.2 (below), there is no Target (i.e., target) shown because this is the scenario where an incoming call to a target gets forwarded. The figure 15.2 shows the calling party who originated call and the forwarded-to-party who receives the forwarded call. The media passes through the CC Intercept Function associated with the forwarded-to-party.   


[image: image62.emf]Forwarded-to-Party

Calling Party

Delivery Function 3

LEMF

Duplicator of Media Packets

CC Intercept 

Function

CC Interception 

Triggering 

Function

X3

HI3



 
Figure 15.2: VoIP CC Interception for forwarded calls

The CC Interception Triggering Function sends the CC intercept trigger to the CC Intercept Function according 15.2.1.1.   The CC Intercept Function intercepts media packets for the call (identified based on the Media Identifier information received over the intercept trigger) and delivers the media packets as according to 15.2.1.2. 

Editor’s Note: Exact use of figures 15.1 and 15.2 in call forwarding scenarios needs to be clarified. 





























15.3
Provision of Intercept Related Information for VoIP

See clause 7.A. 
16
LI for Group Communications using GCSE
16.1
Background

There are several scenarios possible for the interception of group communications involving GCSE (see TS 22.468 [51] and TS 23.468[53]).  First is where the GCSE AS is part of an operator’s network.  Second is where the GCSE AS is outside of the intercepting operator’s network.  This clause specifies LI solutions for both cases.

16.2
GCSE AS in Operator Network
16.2.0
General

In the case where the GCSE AS is in the operator’s network, the ICE in this case will be the GCSE AS as it is fully aware of the group communications as well as the parties on the communications.  The solution is very similar to the conferencing solution specified in Clause 11, where the main difference is that a single functional entity (the GCSE AS) is utilized for GCSE, rather than two functional entities.  


[image: image63]
Figure 16.1:
GCSE AS Intercept configuration
16.2.1
Provision of Content of Communications
16.2.1.0
General
Figure 16.2 shows the interception of the content of communications for GCSE at the GCSE AS is performed based on identifying the target of interception being a member of a group communication at the GCSE AS.  


[image: image64]
Figure 16.2: Provision of Intercept Product from GCSE AS
16.2.1.1
X3-interface

In addition to the intercepted content of communications, the following information may need to be transferred from the GCSE AS to the DF3 in order to allow the DF3 to perform its functionality:

-
identity used for interception include IMSI, IMEI, ProSe UE ID (see TS 22.278 [50] and TS 23.303 [51]);

-
correlation number;

-
the identity of source (i.e., group communications party identity) of a media stream;

-
time stamp;

-
direction (from target or to target).

16.2.2
Provision of Intercept Related Information

16.2.2.0
General

Figure 16.3 shows the transfer of intercept related information to the DF2. If an event for / from a GCSE user occurs, the GCSE ASshall send the relevant data to the DF2.


[image: image65]
Figure 16.3: Provision of Intercept Related Information

16.2.2.1
X2-interface

The following information needs to be transferred from the GCSE AS to the DF2 in order to allow a DF2 to perform its functionality:

-
target identity include IMSI, IMEI, ProSe UE ID;

-
events and associated parameters as defined in clauses X.1.2.2 may be provided;

The IRI should be sent to DF2 using a reliable transport mechanism.

16.2.2.2
GCSE AS LI Events and Event Information

16.2.2.2.0
General

Intercept Related Information events to be reported by the GCS AS include:

-
When GCSE communications group involving a target of interception is activated (enabled for communications)

-
When GCSE communications group involving a target of interception is deactivated (no longer enabled for communications)

-
When a User is added to an active GCSE communications group 

-
When a User is dropped from an active GCSE communications group 

-
Start of Interception with an Active GCSE communications Group

-
End of Interception with an Active GCSE communications Group

-
Modification of Target Connection to GCS AS.

A set of possible elements as shown in Table 16.4 are used to generate the events.

Table 16.4: Information Events for GCS AS Event Records

	Element

	Observed IMSI

IMSI of the target.

	Observed IMEI

IMEI of the target.

	Observed ProSe UE ID

ProSe UE ID of the target.

	Observed Other Identity

Other Identity of the target

	Event type

Description which type of event is delivered: Group Activated, Group Deactivated. Group Add Member, Group Drop Member, Start of Intercept with Active Group, End of Intercept with Active Group, Modification of Active Group.

	Event date

Date of the event generation in the GCS AS.

	Event time

Time of the event generation in the GCS AS. Timestamp shall be generated relative to the GCS AS internal clock.

	Observed Communications Group ID

Identifies the GCSE communications group at the GCS AS.

	GCSE Group Communication Characteristics
Details of the Group Communications Service to which the Target is a member including such characteristics such as voice, video, and data communications.

	GCSE Communications Group Membership List

List of all users that are members of the GCSE communications group. Not all members may be participants in a group communications.

	GCSE Communications Group Participants

List of all users that are participating in the GCSE communications group.

	Correlation Number

The correlation number is used to correlate CC and IRI. The correlation number is also used to allow the correlation of IRI records.  

	Network Element Identifier

Unique identifier for the element reporting the ICE.

	Added User ID

Identity of the party successfully added to an active GCSE Communications Group.

	Dropped User ID

Identity of the party successfully dropped from an active GCSE Communications Group.

	Target Connection Method

Identifies the current target connection method with the GCS AS including whether the target is connected at all.

	Modified Target Connection Method

Identifies the modified target connection method with the GCS AS when the target connection method changes including whether the target is connected at all.

	Identity of Visited Network

Identifies the visited network from which the target is connecting to the GCS AS.

	Length of TMGI Reservation Time

Identifies the length of time reserved for use of a TMGI for a GCSE Communications Group.

	Reserved TMGI

Identifies the TMGI reserved for use by a GCSE Communications Group.

	Location information

Location information of the target, e.g., Cell ID as known by the GCS AS.


NOTE: Generation of Correlation Number is FFS. 

16.2.2.2.1
Activation of GCSE Communications Group 

When a GCSE communications group is activated at the GCS AS (i.e., enabled for communications), an Activation of GCSE Communications Group event is generated in the following cases:

-
When the GCS AS successfully activates a GCSE communications group in which a member is a target of interception.  

The fields, shown in Table 16.5, will be delivered to the DF2, if available, by the GCS AS.
Table 16.5 Activation of GCSE Communications Group
	Observed IMSI 

	Observed IMEI

	Observed ProSe UE ID

	Observed Other Identity

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Correlation Number

	Target Connection Method

	GCSE communications group membership list

	GCSE communications group participants

	Group Communications Characteristics

	Observed Communications Group ID

	Reserved TMGI

	Length of TMGI reservation time (if known)

	Identity of Visited Network (if known)


16.2.2.2.2
Deactivation of GCSE Communications Group

When a GCSE communications group is deactivated (not enabled for communications) at the GCS AS, a Deactivation of GCSE Communications Group event is generated in the following cases:

-
When the GCS AS successfully releases a GCSE communications group in which a member is a target of interception.  

The fields, shown in Table 16.6, will be delivered to the DF2, if available, by the GCS AS.
Table 16.6 Deactivation of GCSE Communications Group
	Observed IMSI 

	Observed IMEI

	ProSe UE ID

	Observed Other Identity

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Correlation Number

	GCSE communications group membership list

	Observed Communications Group ID

	Reserved TMGI

	Length of TMGI reservation time (if known)

	Identity of Visited Network (if known)

	Location Information 


16.2.2.2.3
User Added

A User Added-event is generated in the following cases:

-
When a user is successfully added to a GCSE communications group in which a member is a target of interception.

The fields, shown in Table 16.7, will be delivered to the DF2, if available, by the GCS AS.

Table 16.7 User Added

	Observed IMSI 

	Observed IMEI

	Observed ProSe UE ID

	Observed Other Identity

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Correlation Number

	Added User ID

	GCSE communications group membership list

	Observed Communications Group ID

	GCSE communications group participants

	Reserved TMGI

	Identity of Visited Network (if known)


16.2.2.2.4
User Dropped

A User Dropped-event is generated in the following cases:

-
When a user is successfully dropped from a GCSE communications group in which a member is a target of interception.

The fields, shown in Table 16.8, will be delivered to the DF2, if available, by the GCS AS.

Table 16.8 User Dropped

	Observed IMSI 

	Observed IMEI

	Observed ProSe UE ID

	Observed Other Identity

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Correlation Number

	Dropped User ID

	GCSE communications group membership list

	Observed Communications Group ID

	GCSE communications group participants

	Reserved TMGI

	Identity of Visited Network (if known)


16.2.2.2.5
Start of Intercept with an Active GCSE Communications Group

When an intercept is started with an active GCSE communications group, a Start of Intercept for GCSE Communications Group event is generated in the following cases:

-
When a target of interception is successfully added to an active GCSE communications group.

-
When interception is activated for a target of interception who is already a member of an active GCSE communications group.

The fields, shown in Table 16.9, will be delivered to the DF2, if available, by the GCS AS.
Table 16.9 Start of Intercept with an Active GCSE Communications Group
	Observed IMSI 

	Observed IMEI

	Observed ProSe UE ID

	Observed Other Identity

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Correlation Number

	Target Connection Method

	GCSE communications group membership list

	Group Communications Characteristics

	Observed Communications Group ID

	GCSE communications group participants

	Reserved TMGI

	Length of TMGI reservation time (if known)

	Identity of Visited Network (if known)

	Location Information


16.2.2.2.6
End of Intercept with an Active GCSE Communications Group

When an intercept is ended with an active GCSE communications group, an End of Intercept for GCSE Communications Group event is generated in the following cases:

-
When a target of interception is successfully dropped from an active GCSE communications group.

The fields, shown in Table 16.10, will be delivered to the DF2, if available, by the GCS AS.
Table 16.10 End of Intercept with an Active GCSE Communications Group
	Observed IMSI 

	Observed IMEI

	Observed ProSe UE ID

	Observed Other Identity

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Correlation Number

	GCSE communications group membership list

	Observed Communications Group ID

	GCSE communications group participants

	Reserved TMGI

	Length of TMGI reservation time (if known)

	Identity of Visited Network (if known)

	Location Information


16.2.2.2.7
Modification of Target Connection to GCS AS

When a modification to a target connection to the GCS AS occurs, a Modification of Target Connection event is generated in the following cases:

-
When a target of interception changes the current downlink communications reception method to now receive only via a unicast link, only via a multicast link, or via both unicast and multicast links.

-
When the target of interception changes the uplink and downlink connection method from not connected to one of the connected connection methods, and vice versa.

The fields, shown in Table 16.11, will be delivered to the DF2, if available, by the GCS AS.
Table 16.11 Modification of Target Connection to GCSE AS
	Observed IMSI 

	Observed IMEI

	Observed ProSe UE ID

	Observed Other Identity

	Event Type

	Event Time

	Event Date

	Network Element Identifier

	Correlation Number

	Modified Target Connection Method

	GCSE communications group membership list

	Group Communications Characteristics

	Observed Communications Group ID

	GCSE communications group participants

	Reserved TMGI

	Length of TMGI reservation time (if known)

	Identity of Visited Network (if known)

	Location Information


16.3
GCS AS outside Intercepting CSP Network
Interception of group communications by the intercepting CSP when the GCS AS is outside of the CSP’s network, is not provided in this release.  Packet data interception capabilities can be used to intercept and report a target’s communication.

17
Interception for Proximity Services

17.1
ProSe Direct Discovery

17.1.1
General

Proximity Service (ProSe) are specified in TS 23.303 [54]. This includes Direct Discovery where two UEs may discover that they are in proximity using direct signalling between the UEs, where such signalling both controlled by and reported to the ProSe Function. 


[image: image66.wmf] 

LEMF

 

Mediation

 

Function

 

Delivery

 

Function 2

 

Mediation

 

Function

 

ADMF

 

X2

 

X1_2

 

X1_1

 

HI1

 

HI2

 

ProSe

 

Function

 


Figure 17.1-1: ProSe Direct Discovery Intercept configuration

Figure 17.1-1 shows the CC interception configuration for ProSe Direct Discovery.  The HI2 interface represents the interface between the LEA and the delivery function. The delivery function is used to distribute the Intercept Related Information (IRI) to the relevant LEA(s) via HI2.  See Clause 4 for more information on the ADMF and other interfaces.

The target identity for ProSe Direct Discovert interception is the IMSI.  The activation, deactivation, and interrogation of interception regarding the ProSe Function shall follow the requirements of Clause 5.
17.1.2
Provision of Inteception of Call Content 

Interception of direct discovery does not have a call content component as all the information useful to the LEA is provided as part of the IRI interception. 

17.1.3
Provision of Intercept Related Information
17.1.3.1
General

Figure 17.1.3.1-1 shows the transfer of intercept related information (IRI) to the DF2. If an event involving a target occurs, the ProSe Function shall send the relevant data to the DF2. A UE always contacts the ProSe Function in its HPLM, which then contacts the other relevant ProSe Functions to complete the UEs request.In the case of Match Report event, it is possible that a non-target monitoring UE will tigger interception of a target UE when it reports a code announced by  that target UE.

This is illustrated in the following figure where it should be noted that only one subcriber to HPLMN ProSe Function interaction is needed to trigger an interception event.




[image: image68.emf] 

HPLMN - O   ProSe   Function  

HPLMN - I   ProSe   Function  

Target  

Other   subscriber  

LEMF  

Delivery   Function 2  


Figure 17.1.3.1-1: Provision of Intercept Related Information for discovery

17.1.3.2
X2-interface

The following information needs to be transferred from the ProSe Function to the DF2 in order to allow a DF2 to perform its functionality:

-
target identity (IMSI);
-
events and associated parameters, as defined in section 17.1.3.3; 

The IRI should be sent to DF2 using a reliable transport mechanism.
17.1.3.3
ProSe LI Events and Event Information
17.1.3.3.1
ProSe LI Events

The following events are applicable to the interception of direct discovery:
-
Discovery Request

-
Match Report

Interception of these events is mandatory in the ProSe Function of the PLMN that is used for direct discovery.

17.1.3.3.2
ProSe LI Event Information

A set of possible elements as shown below is used to generate the events.
Table: 17.1.3.3.2-1: Information Events for ProSe Event Records
	Element

	Observed IMSI
IMSI of the target 

	Event type

Description which type of event is delivered:- Discovery Request, Match Report

	Event date

Date of the event generation in the ProSe Function

	Event time

Time of the event generation in the ProSe Function. Timestamp shall be generated relative to the ProSe Function internal clock.

	Role of the target
Whether the target is an announcing or a monitoring UE

	Discovery PLMN identity 

PLMN used or to be used for the discovery

	ProSe Application ID Name

Identity of a user within the context of a specific application

	Metadata

Metadata relating  to a ProSe Application ID Name of the announcing UE  

	Network Element Identifier

Unique identifier for the element reporting the ICE.

	Timer

The”Validity Timer” or “Time to Live” value assigned by the network to a specific ProSe Application Code or Filter, that controls how long the UE can announce/monitor it

	Identity of the other UE
In Match reports, there is a second UE involved.

	ProSe Application Code

Bitstring that is actually announced over the air or included in a discovery filter applied by UE

	ProSe App Mask

Bitmask that allows the monitoring UE to perform full or partial matching. Multiple Masks may be included in a Discovery Filter. The length of the mask is the same as the length of ProSe Application Code


17.1.3.3.3
Structure of ProSe Events
17.1.3.3.3.1
Discovery Request

For ProSe Discovery Requests, a Discovery Request event is generated. The elements shown in Table 17.1.3.3.1-1 will be delivered by the ProSe Function to the DF2, if available. A new Discovery Request Event shall be generated for each individual ProSe Discovery Request received by the ProSe Function.

Table 17.1.3.3.3.1-1: Discovery Request
	Observed IMSI

	Event Type

	Event Time

	Event Date

	Role of the target

	Network Element Identifier

	Discovery PLMN identity

	ProSe Application ID Name





	Timer

	Metadata 



                            (If Applicable)

	ProSe Application Code

	ProSe App Mask                                 (If Applicable)


17.1.3.3.3.2
Match Report
For ProSe Match Report, a Match Report event is generated. The elements shown in Table 17.1.3.3.2-1 will be delivered by the ProSe Function to the DF2, if available. A new Match Report Event shall be generated for each individual ProSe Match Report received by the ProSe Function.
Table 17.1.3.3.3.2-1: Match Report
	Observed IMSI

	Event Type

	Event Time

	Event Date

	Role of the target

	Network Element Identifier

	Discovery PLMN identity

	ProSe Application ID Name





	Timer

	Metadata 



                                (If Applicable)

	Identity of the other UE                               (If Available)

	ProSe Application Code


17.2
ProSe One To Many Communications 
17.2.1
General

Proximity Service (ProSe) are specified in TS 23.303 [54]. This includes one to many communications among Public Safety ProSe UEs  with such communication occuring while the UEs are in proximity. There are several procedures that takes place with network elements as part of a typical one-to-many commuication. The UE needs to be authorised by the ProSe Function, and receive keys from the ProSe Key Management Function (PKMF - see TS 33.303[57]). After such a communication, the UE reports the relevant usage information to the ProSe Function. This subclause details the interception at the ProSe Function and ProSe Key Management Function. 

NOTE: The present document does not provide a solution for the interception of content of communication.

Editor’s note: Interception of ProSe one-to-many communication with the group as the target is FFS.


[image: image69.emf]ProSe 

Function or 

ProSe Key 

Management 

Function 




Figure 17.2-1: ProSe One To Many Communications Intercept configuration

Figure 17.2-1 shows the IRI interception configuration for ProSe one to many communications. The HI2 interface represents the interface between the LEMF and the delivery function. The delivery function is used to distribute the Intercept Related Information (IRI) to the relevant LEMF(s) via HI2. See clause 4 for more information on the ADMF and other interfaces.

The target identity for interception is  the IMSI.The activation, deactivation, and interrogation of interception regarding the ProSe Function shall follow the requirements of clause 5.
17.2.2
Provision of Intercept Product – One-To-Many Communications 

17.2.2.1
General

Figure 17.2.2.1-1 shows the transfer of IRI relating to the one to many communications from the ProSe Function and ProSe Key Management Function to the DF2 and to the LEMF.  If a target UE interacts with the ProSe Function or ProSe Key Management Function relating to one-to-many communications, a One-To-Many IRI event, is generated and sent via the Delivery Function 2 to the LEMF. 
If an event involving an target occurs, the ProSe Function or ProSe Key Management Function shall send the relevant data to the DF2 for formatting and delivery to the LEMF.  

[image: image71.emf]ProSe UE A

ProSe UE B

ProSe UE C

ProSe Function 

or PKMF

Signalling

Communication

DF2

LEMF

X2 

interface

Target

HI2




Figure 17.2.2.1-1: Provision of Intercept Product for Public Safety One-To-Many Communications
NOTE: There is signalling between the non-target ProSe UEs and the ProSe Function and PKMF, which is not shown in figure 17.2.2.1-1. This signalling is not relevant to the interception of the target UE.
17.2.2.2
X2-interface

The following information needs to be transferred from the ProSe Function or ProSe Key Management Function to the DF2 in order to allow a DF2 to perform its functionality:

-
target identity (IMSI);
-
events and associated parameters, as defined in section 17.2.2.3; 

-
the target location (if available) or the IAs in case of location dependent interception;
-
Quality of Service (QoS) identifier;
-
Encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.
The IRI should be sent to DF2 using a reliable transport mechanism.
17.2.2.3
ProSe LI One-To-Many Events and Event Information
17.2.2.3.1
Overview of ProSe LI One-To-Many Events

The following event are applicable to the interception of one to many communications:
-
ProSe Function sends authorisation to perform one-To-many communications to the target UE. 
-
ProSe Key Management Function sends keys to use in one-to-may commuications to the target UE.

-
ProSe Function receives a one-to-many communications usage information report from the target UE.
Interception of these events is mandatory. .

17.2.3.3.2
Structure of ProSe LI One-To-Many Event Information

A set of possible elements as shown below is used to generate the events.
Table 17.2.3.3.2-1: Information Elements for Event Records relating to one-to-many communications
	Element

	Observed IMSI

IMSI of the target

	


	


	Event date

Date of the event generation.

	Event time

Time of the event generation. Timestamp shall be generated relative to the intercepting node’s internal clock.

	PLMN Identity

The PLMN that the UE is being authorised to perform one-to-many communication in.

	Sender ID

Identifies the sender of the One-To-Many Communications – also known as the ProSe UE ID or the Group member identity.

	ProSe Layer-2 Group ID
Identifies the group of the One-To-Many Communications – also known as the destination ID.

	


	


	Network Identifier

Operator ID plus unique identifier for the element reporting the ICE.

	IP address type

The type of IP address, i.e IPv4 or IPv6, used for this one-to-many communication.

	IP multicast address

The IP address to be used when doing one-to-many communication 

	PKMF address

The IP address of the ProSe Key Management Function that will provides keys for this one-to-may communication

	Source IP address

The IP source address to be used by the UE as a source address

	Communication Authorisation Validity timer

Indicates in minutes how long the authoristaion is valid for

	Confidentiality algorithm

The confidentiality algorithm used with this group 

	PGK

ProSe Group Key that can be used to protect data for this group 

	PGK ID

The identity of the PGK within this group

	PGK lifetime

The lifetime of the PGK

	


	


	Location list

List of locations of the UE when in coverage and the corresponding timestamps

	E-UTRAN coverage timestamps

List of timestamsp when the UE goes in/out of E-UTRAN coverage

	First communication timestamp
Timestamp of the first one-to-may communication transmission/reception

	Transmitter identities
Identities of the transmitters in the one-to-many communication session

	Data transmitted in coverage

List of amount of data transmitted by UE when in E-UTRAN coverage at each location, with ECGI and the corresponding timestamps 

	Data transmitted out of coverage

List of amount of data transmitted by UE for each E-UTRAN out of coverage and the corresponding timestamps

	Data received in coverage

List of amount of data received by UE when in E-UTRAN coverage at each location, with ECGI and the corresponding timestamps

	Data received out of coverage
List of amount of data received by UE for each out of E-UTRAN coverage period and the corresponding timestamps


17.2.2.3.3
ProSe LI One-To-Many Events

If ProSe Function sends authorisation to use ProSe one-to-many communication to the target UE, a One-To-Many-Auth Event shall be generated. These elements if available will be delivered to the DF2 (see TS 24.333 [58] for more information on the elements):
Table 17.2.2.3.3-1 One-To-Many-Auth Event

	Observed IMSI

	Event type

	Event Time

	Event Date

	Network Identifier (including network element identifier)

	PLMN Identity

	Communication Authorisation Validity timer

	ProSe Layer-2 Group ID

	IP address type

	IP multicast address

	PKMF address (if applicable)

	Source IP address (if applicable)


If a ProSe Key Management Function sends keys for one-to-many communication to a target UE, a One-To-Many-Keys Event shall be generated. These elements if available will be delivered to the DF2 (see TS 33.303 [57] for more information on the elements):
Table 17.2.2.3.3-2 One-To-Many-Keys Event

	Observed IMSI

	Event type

	Event Time

	Event Date

	Network Identifier (including network element identifier)

	ProSe Layer-2 Group ID

	Sender ID

	Confidentiality algorithm

	PGK

	PGK ID

	PGK lifetime




	

	

	

	

	

	

	

	

	

	

	



If a ProSe Function receives a usage information report for ProSe one-to-many communication to a ProSe Function from a target UE, a One-To-Many-Usage Event shall be generated. These elements if available will be delivered to the DF2 (see TS 32.277 [59] for more information on the elements):

Table 17.2.2.3.3-3 One-To-Many-Usage Event

	Observed IMSI

	Event type

	Event Time

	Event Date

	Network Identifier (including network element identifier)

	Location list

	E-UTRAN coverage timestamps

	Sender ID

	ProSe Layer-2 Group ID

	IP address type

	IP multicast address

	Source IP address (if applicable)

	First communication timestamp

	Transmitter identities

	Data transmitted in coverage

	Data transmitted out of coverage

	Data received in coverage

	Data received out of coverage


NOTE: The UE periodically generates usage event reports for charging purposes from which this event is generated. Therefore there may be a significantly delay between the generation of this event and the communications for which the usage report is generated by the UE.
Editor’s note: The above table is based on the stage 2 of the PC3ch interface as decribed in TS 32.277[59]. Some of the above information may need to be changed once the stage 3 for PC3ch is complete. 

Editor’s Note: The application of LI requirements in 33.106 to the CTF (TS 32.277[59]) within the ProSe function when it is physically separated from the ProSe Function is FFS.
Annex A (informative):
Information flows for Lawful Interception invocation of circuit switched services

The following figures show the information flows for the invocation of Lawful Interception for various types of calls. The figures show some of the basic signalling messages of the target calls and the events on the X2 and X3-interfaces. The call control messages to and from the network are shown for informational purposes only; some of them may not be sent or may be combined in certain networks. The handling of the bearers for the basic calls is not shown. The bearer points are established in a manner to minimise content loss without delaying the call to the target. The bearer establishment to agency will be in parallel or immediately following the bearer establishment to the target. The flows portray both forward and backward bearer establishment and release to the agency.

A.1
Mobile originated circuit switched calls

Figure A.1 shows the interception of a basic mobile originated circuit switched speech or data call where the originating mobile (A) is the target for interception. B is not necessarily also a mobile subscriber and resides on a different exchange.


[image: image73.wmf] 

SETUP

 

IAM

 

ACM

 

Call Establishment Attempt

 

CPG(alert)

 

ANM

 

Answer

 

Stublines contain CC of AB

 

DISCONNECT

 

REL

 

Release

 

ALERTING

 

DF

2

 

MSC Server

 

DF3 

Bearer

 

MS A

 

B

 

DF3 

Signalling

 

MGW

 

setup of stublines

 

Establish Bearer/

 

Prepare Bearer Establishment

 

 

Bearer Establishment

 

Prepare Bearer Establishment/

 

Establish Bearer

 

release of stublines

 

Release Resource/Release Bearer 

 

Release Bearer/Release Resource

 

Bearer Release

 

CONNECT

 


Figure A.1: Interception of mobile originated circuit switched calls

In figure A.1 the result (answer) of the set-up of the stublines is not shown. This assumes no special action is taken in case of failure.

A.2
Mobile terminated circuit switched calls

Figure A.2 shows the interception of a basic mobile terminated circuit switched speech or data call where the terminating mobile (B) is the target for interception. A is not necessarily also a mobile subscriber and resides on a different exchange.


[image: image74.wmf] 

IAM

 

ACM

 

SETUP

 

Call Establishment Attempt

 

CPG(alert)

 

CONNECT

 

ANM

 

Answer

 

stublines contain CC of AB

 

DISCONNECT

 

REL

 

Release

 

ALERTING

 

DF

2

 

MSC Server

 

DF3 

Bearer

 

MS B

 

A

 

DF3 

Signalling

 

MGW

 

setup of stublines

 

Establish Bearer/

 

Prepare Bearer Establishment

 

 

Bearer Establishment

 

Prepare Bearer Establishment/

 

Establish Bearer

 

release of stublines

 

Release Resource/Release Bearer 

 

Release Bearer/Release Resource

 

Bearer Release

 


Figure A.2: Interception of mobile terminated circuit switched calls

A.3
Call hold / call waiting

Figures A.3 and A.4 show the interception of calls involving call hold / call waiting. Figure A.3 covers the case where one pair of stublines is used per target, figure A.4 covers the case where a separate pair of stublines is used for each target call. The mobile that receives the waiting call (A) is the target for interception.

[image: image75.wmf]... release of AB call and interception as in

figure A1 ...

... setup and interception of AB call as in figure

A1 ...

Release(AB)

SETUP(AB)

Answer(AB)

Stublines

  contain CC of AB

IAM

ACM

SETUP(CA)

Call Establishment 

Attempt(CA, CWAIT)

CPG(alert)

HOLD(AB)

Suppl. 

Service(AB, CHOLD)

CONNECT(CA)

ANM

Answer(CA)

stublines contain CC of CA

DISCONNECT(CA)

REL

Release(CA)

RETRIEVE(AB)

Suppl. 

Service(AB, CRETR)

Stublines

  contain CC of AB

ALERTING(CA)

RETRIEVE 

ACK(AB)

HOLD 

ACK(AB)

DISCONNECT(AB)

REL

DF2

MSC Server

DF3

Bearer

MS A

B

DF3

Signaling

MGW

C


Figure A.3: Interception of call hold / call waiting - stublines per target


[image: image76.wmf]... release of AB call and interception as in

figure A1 ...

... setup and interception of AB call as in figure

A1 ...

Release(AB)

SETUP(AB)

Answer(AB)

Stublines’ contain CC of AB

IAM

ACM

SETUP(CA)

Call Establishment 

Attempt(CA, CWAIT)

CPG(alert)

HOLD(AB)

Suppl. 

Service(AB, CHOLD)

CONNECT(CA)

ANM

Answer(CA)

stublines” contain CC of CA

DISCONNECT(CA)

REL

Release(CA)

RETRIEVE(AB)

Suppl. 

Service(AB, CRETR)

stublines

’ contain CC of AB

ALERTING(CA)

HOLD 

ACK(AB)

DISCONNECT(AB)

REL

DF2

MSC Server

DF3

Bearer

MS A

B

DF3

Signaling

MGW

C

RETRIEVE 

ACK(AB)

setup of 

stublines”

Establish Bearer/

Prepare Bearer Establishment

Bearer Establishment

Prepare Bearer Establishment/

Establish Bearer

release of 

stublines”

Release Resource/ Release Bearer

Release Bearer/Release Resource

Bearer Release


Figure A.4: Interception of call hold / call waiting - stublines per target call

A.4
Multiparty calls

Figures A.5 and A.6 show the interception of multiparty calls. Figure A.5 covers the case where one pair of stublines is used per target, figure A.6 covers the case where a separate pair of stublines is used for each target call. The mobile setting up the multiparty call (A) is the target for interception.

[image: image77.wmf]... setup and interception of AC call as in figure A1

without setup of stublines ...

... release of  AC call and interception as in figure A1

without release of the stublines ...

... release of AB call and interception as in

figure A1 ...

... setup and interception of AB call as in figure

A1 ...

DISCONNECT(AC)

REL

Stublines

  contain CC of AB

SETUP(AB)

Answer(AB)

BuildMPTY

(AB)

Suppl. 

Service(AB, BMPTY)

Suppl. 

Service(AC, BMPTY)

Stublines

  contain CC of ABC

Stublines

  contain CC of ABC

Release(AC)

HOLD(AB)

Suppl. 

Service(AB, CHOLD)

HOLD 

ACK(AB)

Stublines

  contain CC of AC

SETUP(AC)

Answer(AC)

RetrieveMPTY

(AB)

Suppl. 

Service(AB, RMPTY)

RetrieveMPTY 

ACK(AB)

Stublines contain CC of AB

DISCONNECT(AB)

REL

Release(AB)

BuildMPTY 

ACK(AB)

DF2

MSC Server

DF3

Bearer

MS A

B

DF3

Signaling

MGW

C


Figure A.5: Interception of multiparty calls - stublines per target


[image: image78.wmf]... setup and interception of AC call as in figure A1 ...

... release of  AC call and interception as in figure A1 ...

without release of stublines

... release of AB call and interception as in

figure A1 ...

... setup and interception of AB call as in figure

A1 ...

DISCONNECT(AC)

REL

Stublines’ contain CC of AB

SETUP(AB)

Answer(AB)

BuildMPTY

(AB)

Suppl. 

Service(AB, BMPTY)

Suppl. 

Service(AC, BMPTY)

Stublines’ contain CC of ABC

Release(AC)

HOLD(AB)

Suppl. 

Service(AB, CHOLD)

HOLD 

ACK(AB)

Stublines” contain CC of AC

SETUP(AC)

Answer(AC)

RetrieveMPTY

(AB)

Suppl. 

Service(AB, RMPTY)

RetrieveMPTY 

ACK(AB)

Stublines’ contain CC of AB

DISCONNECT(AB)

REL

Release(AB)

BuildMPTY 

ACK(AB)

DF2

MSC Server

DF3

Bearer

MS A

B

DF3

Signaling

MGW

C

Stublines” contain CC of ABC

Release Bearer/Release Resource

Release Bearer/Release Resource

Release of 

stublines


Figure A.6: Interception of multiparty calls - stublines per target call

A.5
Call forwarding / call deflection

A.5.0
General

The following pictures show the information flows for the interception of forwarded calls. Information flows will be given for three typical cases of call forwarding. All other types of call forwarding / call deflection are intercepted similar to one of these.

A.5.1
Unconditional call forwarding

Figure A.7 shows the interception of unconditionally forwarded calls. The mobile that activated unconditional call forwarding (B) is the target for interception. In this case interception will be performed at the 3G GMSC, where the Service Request Indicator (SRI) request for B is issued and subsequently the SRI response indicating that the call shall be forwarded is received.


[image: image79.wmf]SRI request for B

SRI response

IAM

ACM

IAM

Release

CPG(alert)

Call Establishment Attempt

CPG(alert)

ANM

Answer

ANM

REL

REL

Stublines

  contains CC of AC

Suppl. 

Service(CFU)

ACM

DF2

3G GMSC

DF3

Bearer

HLR

A

DF3

Signaling

MGW

C

setup of 

stublines

Establish Bearer/

 Prepare Bearer Establishment

Establishment/

Bearer Establishment

Prepare Bearer Establishment/

Establish Bearer

release of 

stublines

Release Resource/ Release Bearer

Release Bearer/Release Resource

Bearer Release


Figure A.7: Interception of unconditional call forwarding

A.5.2
Call forwarding on not reachable (IMSI detached)

Call forwarding on not reachable because the IMSI is detached is also handled on the 3G GMSC. Interception of this type of call forwarding is similar to interception of unconditional call forwarding.

A.5.3
Call forwarding on busy (network determined)

Figure A.8 shows the interception of call forwarding on busy (network determined). The mobile that activated call forwarding on busy (B) is the target for interception. In this case interception will be performed at the 3G MSC where B resides, where the busy condition is detected and the call is forwarded.


[image: image80.wmf]... setup and interception of BD call as in figure A1 ...

... release of BD call and interception as in figure A1 ...

IAM

ACM

IAM

Release(AB)

CPG(alert)

Call Establishment 

Attempt(AB)

CPG(alert)

ANM

Answer(AB)

ANM

REL

REL

Stublines” contain CC of AC

Suppl. 

Service(AB, CFB)

ACM

SETUP(BD)

Answer(BD)

stublines

’ contain CC of BD

DISCONNECT(BD)

REL

Release(BD)

DF2

MSC Server

DF3

Bearer

MS B

C

DF3

Signaling

MGW

D

A

setup of 

stublines”

Establish Bearer/

 Prepare Bearer Establishment

Establishment/

Bearer Establishment

Prepare Bearer Establishment/

Establish Bearer

release of 

stublines”

Release Resource/ Release Bearer

Release Bearer/Release Resource

Bearer Release


Figure A.8: Interception of call forwarding on busy (network determined)

A.5.4
Call forwarding on not reachable (no response to paging/radio channel failure)

Call forwarding on not reachable because of no response to paging or radio channel failure is also handled on the 3G MSC similar to call forwarding on busy (network determined). Interception of this type of call forwarding is therefore done in the same way (see clause A.5.3).

A.5.5
Call forwarding on no reply

Figure A.9 shows the interception of call forwarding on no reply. The mobile that activated call forwarding on no reply (B) is the target for interception. In this case interception will be performed at the 3G MSC where B resides, where the no reply condition is detected and the call is forwarded. Initially, the interception is similar to the interception of a basic mobile terminated circuit switched speech of data call. On no reply time-out, the interception will continue on the forwarded call to C.


[image: image81.wmf]IAM

ACM

IAM

Release

CPG(alert)

Call Establishment Attempt

CPG(alert)

ANM

Answer

ANM

REL

REL

Stublines

  contain CC of AC

Suppl. 

Service(CFNR)

ACM

CPG(alert)

No reply timeout

CPG(forward)

SETUP

ALERTING

DF2

MSC Server

DF3

Bearer

MS B

A

DF3

Signaling

MGW

C

setup of 

stublines

Establish Bearer/

 Prepare Bearer Establishment

Bearer Establishment

Prepare Bearer Establishment/

Establish Bearer

release of 

stublines

Release Resource/ Release Bearer

Release Bearer/Release Resource

Bearer Release


Figure A.9: Interception of call forwarding on no reply

In figure A.9 the release of the stublines is done after the forwarded call is released by A or C. It is a national option not to support interception of forwarded calls. In that case, the release of the stublines is done after the call is forwarded and B is no longer involved.

A.5.6
Call forwarding on busy (user determined)/call deflection

Call forwarding on busy (user determined) and call deflection are also handled on the 3G MSC similar to call forwarding on no reply. Interception of this type of call forwarding is therefore done in the same way (see A5.5).

A.5.7
Call waiting / call forwarding on no reply

Figures A.10 and A.11 show the interception of a call involving both call waiting and call forwarding on no reply. Figure A.10 covers the case where one pair of stublines is used per target, figure A.11 covers the case where a separate pair of stublines is used for each target call. The mobile that activated call forwarding on no reply and receives the waiting call (B) is the target for interception. In figure A.10 a new pair of stublines needs to be set up when the call is forwarded since the first pair of stublines is still used for the initial call.

[image: image82.wmf]... setup and interception of BD call as in figure A1 ...

... release of BD call and interception as in figure A1 ...

IAM

ACM

IAM

Release(AB)

CPG(alert)

Call Establishment 

Attempt(AB, CWAIT)

CPG(alert)

ANM

Answer(AB)

ANM

REL

REL

Stublines” contain CC of AC

Suppl. 

Service(AB, CFNR)

ACM

CPG(alert)

No reply timeout

CPG(forward)

SETUP(AB)

ALERTING(AB)

SETUP(BD)

Answer(BD)

stublines

’ contain CC of BD

DISCONNECT(BD)

REL

Release(BD)

DF2

MSC Server

DF3

Bearer

MS B

C

DF3

Signaling

MGW

D

A

setup of 

stublines”

Establish Bearer/

Prepare Bearer Establishment

Bearer Establishment

Prepare Bearer Establishment/

Establish Bearer

release of 

stublines”

Release Resource/ Release Bearer

Release Bearer/Release Resource

Bearer Release


Figure A.10: Interception of call waiting / call forwarding on no reply - stublines per target


[image: image83.wmf]... setup and interception of BD call as in figure A1 ...

... release of BD call and interception as in figure A1 ...

IAM

ACM

IAM

Release(AB)

CPG(alert)

Call Establishment 

Attempt(AB, CWAIT)

CPG(alert)

ANM

Answer(AB)

ANM

REL

REL

Stublines” contain CC of AC

Suppl. 

Service(AB, CFNR)

ACM

CPG(alert)

No reply timeout

CPG(forward)

SETUP(AB)

ALERTING(AB)

SETUP(BD)

Answer(BD)

stublines

’ contain CC of BD

DISCONNECT(BD)

REL

Release(BD)

DF2

MSC Server

DF3

Bearer

MS B

C

DF3

Signaling

MGW

D

A

setup of 

stublines”

Establish Bearer/

Prepare Bearer Establishment

Bearer Establishment

Prepare Bearer Establishment/

Establish Bearer

release of 

stublines”

Release Resource/ Release Bearer

Resource

Release Bearer/Release Resource

Bearer Release


Figure A.11: Interception of call waiting / call forwarding on no reply - stublines per target call

A.6
Explicit call transfer

Figures A.12 and A.13 show the interception of explicit call transfer. Figure A.12 covers the case where one pair of stublines is used per target, figure A.13 covers the case where a separate pair of stublines is used for each target call. The mobile transferring the call (B) is the target for interception.


[image: image84.wmf]... setup and interception of BC call as in figure A1

without setup of stublines ...

... setup and interception of AB call as in figure

A2 ...

IAM

Release(AB)

ECT(AB)

REL

REL

Stublines contain CC of AC

Stublines

  contain CC of AB

Answer(AB)

HOLD(AB)

Suppl. 

Service(AB, CHOLD)

HOLD 

ACK(AB)

stublines contain CC of BC

SETUP(BC)

Answer(BC)

Suppl. 

Service(AB, ECT)

Suppl. 

Service(BC, ECT)

Release(BC)

DF2

MSC Server

DF3

Bearer

MS B

A

DF3

Signaling

MGW

C

release of 

stublines

Release Resource/ Release Bearer

Release Bearer/Release Resource

Bearer Release


Figure A.12: Interception of explicit call transfer - stublines per target


[image: image85.wmf]... setup and interception of BC call as in figure A1 ...

... setup and interception of AB call as in figure

A2 ...

IAM

Release(AB)

ECT(AB)

REL

REL

Stublines’ contain CC of AC

Stublines

’  contain CC of AB

Answer(AB)

HOLD(AB)

Suppl. 

Service(AB, CHOLD)

HOLD 

ACK(AB)

Stublines” contain CC of BC

SETUP(BC)

Answer(BC)

Suppl. 

Service(AB, ECT)

Suppl. 

Service(BC, ECT)

Release(BC)

DF2

MSC Server

DF3

Bearer

MS B

A

DF3

Signaling

MGW

C

Stublines” contain CC of AC

release of 

stublines’

Release Resource/ Release Bearer

Bearer/Release

Release Bearer/Release Resource

Bearer Release

release of 

stublines”

Release Resource/ Release Bearer

Release Bearer/Release Resource

Bearer Release


Figure A.13: Interception of explicit call transfer - stublines per target call

In figures A.12 and A.13 the release of the stublines is done after the transferred call is released by A or C. It is a national option not to support interception of transferred calls. In that case, the release of the stublines is done after the call is transferred and B is no longer involved.

Annex B (informative):
Information flows for Lawful Interception invocation of GSN Packet Data services
B.0
General
The following figures show the information flows for the invocation of Lawful Interception for Packet Data and typical scenarios. The figures show some of the basic signalling messages of the target Packet Data communication and the events on the X2 and X3 interfaces. The dotted lines indicate signalling depending on whether CC and/or IRI information has been requested. The Gateway 3G GGSN may setup/release packet tunnels and send IRI information depending on national requirements.

The use of the Gateway 3G GGSN for interception is a national option.

B.1
Mobile Station Attach

Figure B.1 shows the interception of a basic Mobile Station Attach where the mobile (A) is the target for interception.

[image: image86.wmf] 

Mobile Station Attach

 

Attach Accept

 

Signalling

 

Attach Request

 

 

 

DF2

 

DF3

 

MS A

 

3G SGSN

 


Figure B.1: Interception of mobile originated Mobile Station Attachment

B.2
Mobile Initiated Mobile Station Detach

Figure B.2 shows the interception of a Mobile Initiated Mobile Station Detach where the originating mobile (A) is the target for interception.


[image: image87.wmf] 

Mobile Station Detach

 

Detach Accept

 

Detach Request

 

 

 

DF2

 

DF3

 

MS A

 

3G SGSN

 

MS initiated

 

PDP Context Deactivation

 

See B.8 when context is deactivated

 


Figure B.2: Interception of mobile originated Mobile Station Detachment

B.3
Network initiated Mobile Station Detach

Figure B.3 shows the interception of a network initiated (by 3G SGSN or HLR) Mobile Station Detach where the mobile (A) is the target for interception.


[image: image88.wmf] 

Cancel Location *

 

Mobile Station Detach Indication

 

Detach Accept

 

Detach Request

 

DF2

 

DF3

 

MS A

 

3G SGSN

 

Network initiated PDP

 

Context Deactivation

 

See B.9 when context is deactivated

 

HLR

 

MSC/VLR

 

Cancel Location Ack *

 

Mobi

le Station Detach

 


NOTE:
* Additional signals in case of HLR initiated.

Figure B.3: Interception of network initiated Mobile Station Detach

B.4
Intra 3G GSN Routing Area Update

Figure B.4 shows the interception of an Intra Routing Area Update where the mobile (A) is the target for interception. The sequence is the same for the combined RA / LA Update procedure but additional signalling is performed between the current 3G SGSN and the prior 3G SGSN before the Routing Area Update Accept message is sent to the MS.


[image: image89.wmf] 

RA update

 

Routing Area Update Accept

 

Security functions

 

Routing Area Update Request

 

 

 

DF2

 

DF3

 

MS A

 

3G SGSN

 

Routing Area Update Complete

 


Figure B.4: Interception of an Intra Routing Area Update

B.5
Inter 3G GSN Routing Area Update

Figure B.5 shows the interception of an Inter Routing Area Update where the mobile (A) is the target for interception. The sequence is the same for the combined RA / LA Update procedure but additional signalling is performed between the 3G GSN, HLR and the old 3G GSN before the Routing Area Update Accept message is sent to the MS. In case of PDP context not being active less signalling is required.


[image: image90.wmf] 

RA Update

 

Set

-

up of Packet Data tunnel

 

Routing Area Update Complete

 

Routing Area Update Accept

 

Additional Signalling

 

Rele

ase of Packet Data tunnel

 

3G GSN Context Acknowledge

 

Security Functions

 

3G GSN Context Response

 

3G GSN Context Request

 

Routing Area Update Request

 

DF2

 

DF3

 

MS A

 

new 3G SGSN

 

old 3G SGSN

 

3G SGSN

 

HLR

 


Figure B.5: Interception of an Inter Routing Area Update

B.6
PDP Context Activation

Figure B.6 shows the interception of a PDP Context activation where the mobile (A) is the target for interception. The sequence for a network initiated PDP Context activation is analogous but is preceded by the 3G GSN sending a Request PDP Context Activation to the MS.


[image: image91.wmf] 

Setup of Packet Data tunnel

 

PDP Context Activation

 

Activate PDP Context Accept

 

Setup of Packet Data tunnel

 

Create PDP Context Response

 

Create PDP Context Request

 

Security functions

 

Activate PDP Context Request

 

 DF2

 

 DF3

 

MS A

 

 3G SGSN

 

 3G GGSN

 

PDP Context Activation

 


Figure B.6: Interception of a PDP Context Activation

B.7
Start of interception with PDP context active

A tunnel is established to DF3 and an event is sent to DF2.

B.8
MS initiated PDP Context Deactivation

Figure B.7 shows the interception of a MS initiated PDP Context deactivation where the mobile (A) is the target for interception.


[image: image92.wmf] 

Release of Packet Data tunnel

 

PDP Context Deactivation

 

Deactivate PDP Context 

Accept

 

Release of Packet Data tunnel

 

Delete PDP Context Response

 

Delete PDP Context Request

 

Security functions

 

Deactivate PDP Context Request

 

 DF2

 

 DF3

 

MS A

 

 3G SGSN

 

 3G GGSN

 

PDP Context Deactivation

 


Figure B.7: Interception of a PDP Context Deactivation

B.9
Network initiated PDP Context Deactivation

Figure B.8 shows the interception of a Network initiated PDP Context deactivation where the mobile (A) is the target for interception. The 3G GGSN may send, (depending on national requirements) the PDP Context deactivation and release the Packet Data tunnel after the Delete PDP Context Response has been sent or received, (signalling between the 3G SGSN and the 3G GGSN is not shown here).


[image: image93.wmf] 

 PDP Context Deactivation

 

Release of Packet Data tunnel

 

 Deactivate PDP Context Accept

 

 Deactivate PDP context Request

 

 DF2

 

 DF3

 

MS A

 

 3G SGSN

 


Figure B.8: Interception of a Network initiated PDP Context Deactivation

B.10
SMS

Figures B.9a and B.9b show the interception of a Mobile-terminated SMS. Figures B.10a and B.10b show the interception of a Mobile-originated SMS. In all the scenarios, the mobile subscriber (A) is the target for interception.


[image: image94.wmf] 

Forward Short Message Result

 

SMS

 

Message Transfer

 

Forward Short Message

 

 DF3

 

 MS A

 

3G SGSN

 

SMSC

 

 DF2

 


Figure B.9a: MT-SMS interception after 3G SGSN receives notification of SMS delivery to MS(A)


[image: image95.wmf]SMS

Forward Short Message

 

 

DF2P

 

 DF3P

 MS A

3G SGSN

SMSC


Figure B.9b: MT-SMS interception after 3G SGSN receives SMS from SMSC


[image: image96.wmf] 

SMS

 

Delivery Report

 

Forward Short Message Result

 

Message Transfer

 

Forward Short Message

 

 DF2

 

 DF3

 

MS A

 

3G SGSN

 

SMSC

 


Figure B.10a: MO-SMS interception after 3G SGSN receives notification of SMS delivery from SMSC


[image: image97.wmf]SMS

Message Transfer

 DF2P

 DF3P

MS A

3G SGSN

SMSC


Figure B.10b: MO-SMS interception after 3G SGSN receives SMS from MS(A)

Annex C (informative):
Information flows for the invocation of Lawful Interception for Packet Data with multimedia
C.0
General
The following figures show the information flows for the invocation of Lawful Interception for Packet Data with multimedia. The figures show some of the basic signalling messages of the target Packet Data communication and the events on the X2 interfaces. The dotted lines indicate signalling depending on whether IRI information has been requested. The figures illustrate interception in the visited network.

The figures in this annex only apply to scenarios where the P-CSCF is located in the visited network. In some operator deployment scenarios, the P-CSCF will be in the Home Network. Where the P-CSCF is located in the Home Network and UE to P-CSCF signalling encryption is applied, all SIP messages between the P-CSCF and the UE will be encrypted within the visited network and therefore plain text interception in the visited network may not be possible.

C.1
Multimedia registration

Figures C.1.1 and C.1.2 show the intercept of the Multimedia registration for the case of visited network interception (refer to TS 23.228 clauses 5.3.2.4 and 5.3.2.5).


[image: image98.wmf] 

P

-

CSCF

 

HSS

 

I

-

CSCF

 

Register

 

 Register

 

 

 Cx

 

-

Query

 

UE

 

Visited Network

 

Home 

Network

 

 

Cx

 

-

Query

 

 Resp

 

Continuation of registration

 

 Serving 

 

Network 

Selection

 

 

 Cx

 

-

Select

-

pull

 

 

 Cx

 

-

Select

-

pull Resp

 

DF2

 

 Register

 



Figures C.1.1 and C.1.2 show the intercept of the Multimedia registration for the case of visited network interception, where the P-CSCF is located in the Visited Network (refer to TS 23.228 clauses 5.3.2.4 and 5.3.2.5).

Figure C.1.1: Intercept of Start of Multimedia Registration


[image: image99.wmf] 

P

-

CSCF

 

HSS

 

I

-

CSCF

 

 

 Cx

 

-

Pull

 

 

 Cx

 

-

Pull

 

 Resp

 

Register

 

S

-

CSCF

 

UE

 

Visited Network

 

Home 

Network

 

 200 OK

 

200 OK

 

200 OK

 

 S

-

CSCF selection

 

 

 Cx

 

-

put

 

 

 Cx

 

-

put

 

 Resp

 

DF2

 

200 OK

 


Figure C.1.2: Intercept of Continuation of Multimedia Registration

NOTE:
The same SIP Registration command is used for the initial registration and any registration updates. Registration deletion request is accomplished with a Registration command that indicates a '*' contact or zero expiration time.

C.2
Multimedia Session Establishment and Answer

Figure C2 shows the intercept of the Multimedia Establishment and Answer in the visited network, where the P-CSCF is located in the Visited Network (refer to 3G TS 23.228, clause 5.7.1).


[image: image100.wmf] 

 INVITE

 

Ringing

 

UE

 

P

-

CSCF

 

S

-

CSCF

 

a. SDP

 

  SDP

 

 Final SDP

 

a.  Ringing

 

a. 200 OK

 

 200 OK

 

 

 Reserv

 

 Success

 

ACK

 

SDP

 

INVITE

 

Ringing

 

200 OK

 

 Serv

ice Control

 

Visited Network

 

Home Network

 

Resource

 

Reservation

 

 Authorize

 

 QoS

 

 Resources

 

I

-

CSCF

 

(Firewall)

 

a. INVITE

 

b1. INVITE

 

b2. INVITE

 

b1.  SDP

 

b2.  SDP

 

b1.  Ringing

 

b2.  Ringing

 

b1. 200 OK

 

b2. 200 OK

 

Service Control

 

Final SDP

 

Final SDP

 

 

 

R

eserv

 

 Success

 

Success

 

 ACK

 

ACK

 

 

 Ringback

 

2Approval of

 

 QoS

 

 Commit

 

Start Media Flow

 

DF2

 

 INVITE

 

  SDP

 

Final  SDP

 

 

 

Reserv

 

 Success

 

Ringing

 

200 OK

 

ACK

 


Figure C.2 Intercept of Multimedia Establishment and Answer at Visiting Network

C.3
Multimedia Release

Figure C.3 shows the intercept of the Multimedia Release in the visited network, where the P-CSCF is located in the Visited Network (3G TS 23.228, clause C.2.1 reference available).


[image: image101.wmf] 

UE B

 

S

-

CSCF

 

UE A

 

Home Network

 

P

-

CSCF

 

GPRS

 

Visited Network

 

 

 Rls

 

 Response

 

SIP OK

 

GPRS

 

Visited Network

 

 

 

Hangup

 

P

-

CSCF

 

SIP OK

 

Release PDP

 

SIP OK

 

S

-

CSCF

 

Home Network

 

 

 Hangup

 

 

 Hangup

 

SIP OK

 

 Remove Resource 

 

Reservation

 

 Service Control

 

Service Control

 

 

 Hangup

 

 Remove Resource 

 

Reservation

 

 

 Hangup

 

 

 Rls

 

 R

esponse

 

Release PDP

 

SIP OK

 

DF2

 

 

 

Hangup

 

SIP OK

 


Figure C.3 Intercept of Multimedia Release at Visiting Network

C.4
Multimedia with Supplementary Service – Call Forwarding

Not defined in this release.

C.5
Multimedia with Supplementary Service – Explicit Call Transfer

Not defined in this release.

C.6
Multimedia with Supplementary Service – Subscriber Controlled input

Not defined in this release.

Annex D (informative):
Information flows for Lawful Interception invocation at the MGW using H.248
D.0
General
The following figures show the use of H.248 in setting up a bearer intercept point at the MGW.

D.1
Mobile to Mobile call, originating side is target

Figure D.1 shows the network model for interception of a mobile-to-mobile call, where the originating mobile subscriber is the target for interception.

Figure D.2 message sequence only shows the H.248 elements related to the necessary topology, which could be used in this example.

Normal call establishment using other H.248 elements shall be in accordance with TS 23.205. 
It should be noted that other means exist with H.248 to achieve similar interception.

[image: image102.wmf] 

DF3

 

 

MSC1

 

 

MSC2

 

 

 

CTX2

 

T3

 

T4

 

 

 

RNC A

 

 

 

 

MGW2

 

DF3

 

 Signalling

 

 

 

CTX3

 

T7

 

T8

 

 

 

CTX4

 

T9

 

T10

 

 

 

 

 

 

      MGW1

 

CTX1

 

T1

 

T5

 

T6

 

T2

 

 

 

 

DF3 Bearer

 

 

 

LEMF

 

 

 

 

RNC B

 

Internal Interface

 

Siganalling

 

Bearer

 

Mc Interface

 


Figure D.1: Mobile to Mobile call originating side is target (network model)

[image: image103.wmf] 

Setup

Call Proceeding

Ctx

=$, 

Add(T$)

Ctx

=1, 

Accept(T1)

RAB Assignment 

Req

RAB Assignment 

Resp

.

Ctx

=1, 

Topology(T1,T$,

oneway

),

Add(T$)

Ctx

=1, 

Accept(T5)

UE

RNC A

MGW1

MSC1

MSC2

DF

-

3

CTX1

T1

T

T5

T

CTX1

T1

T

T

T

First Outgoing Call and Bearer

Establishment between MSC1 and DF3

Second Outgoing Call and Bearer

Establishment between MSC1 and DF3

Ctx

=1, 

Topology({

T*,T$,isolate},{T1,T$,

bothway

}),

Add(T$)

Ctx

=1, 

Accept(T2)

Ctx

=1, 

Topology({

T*,T$,isolate},{T2,T$,

oneway

}),

Add(T$)

Ctx

=1, 

Accept(T6)

CTX1

T1

T2

T5

T

CTX1

T1

T2

T5

T6

Bearer 

Establishment 

between MSC1 & MSC2

for AB Call


Figure D.2: Mobile to Mobile call originating side is target
Annex E (Informative) IMS-based VoIP Lawful Interception Call Scenarios

E.1
Overview 

This informative annex provides the examples of call scenarios that illustrate the interception and delivery of CC interception for an IMS-based VoIP call.
E.2
Background

One of the use-cases of IMS-based VoIP service is VoLTE.  The term "VoLTE" is used to refer to an IMS-based VoIP service when EPS (see 3GPP TS 23.401 [22]) happens to be the access network. When EPS is not the access network, the lawful interception capabilities defined in this informative annex applies for any IMS-based VoIP service with the presumption that in those cases the media on the target’s access goes through an IMS-AGW (see 3GPP TS 29.334 [42]) or a PDN-GW (see 3GPP TS 23.401 [22] and 3GPP TS 402 [23]) or a GGSN (UMTS network).  

Note 1: Even with the EPS-based access network, the media may still go through the IMS-AGW. And, in this case, a VoLTE shall be treated very similar to any other VoIP service. 

Furthermore, it is presumed that an inter-CSP call enters or leaves an IMS network via an IBCF/TrGW or an MGCF/IM-MGW depending on whether the inter-working CSP network is an IMS-based network or a CS-based network (see 3GPP TS 23.228 [43]). Also, for an IMS roaming scenario, it is presumed that the signalling and media enters or leaves the home CSP through the IBCF/TrGW. 

The figure E.1 illustrates the VoIP configuration considered for the lawful interception capabilities defined in this clause: 

 
[image: image104.emf]Rx

Gx

Ix/Mn

EPC + LTE

Non-LTE Access, 

interconnection to EPC

Non-3GPP Access, no   

interconnection to EPC  

Iq

Access Network

Media Nodes

AS

3GPP Access, no

interconnection to EPC

GGSN

IMS-AGW

IMS-UE

PDN-GW

Gx

TrGW/

IM-MGW

Voice Services offered in 

another CSP’s network

PCRF

IMS-UE

IMS-UE

IMS-UE

I-BCF/

MGCF

P-CSCF

TrGW

Ix

S-CSCF

I-BCF

Signaling 

Nodes

IMS-UE

IMS Roaming


Figure E.1: IMS-based VoIP Configuration
Note 2: The configuration scenario where the media does not go through the GGSN, or PDN-GW or the IMS-AGW is outside the scope of this document. 

Note 3: In the remaining part of this informative annex, the PDN-GW/GGSN are shown in one box and should read as either the GGSN (UMTS) or the PDN-GW (EPS). 

In figure E.1, the term "media node" is used to denote the network node present on media path and the term "signalling node" is used to denote the network node present on the signalling path.  

The general concepts of VoIP LI is shown in figure E.2

 
[image: image105.emf]Delivery Function 2

SIP Signaling Nodes

IRI ICE

Delivery Function 3

Media Nodes

(CC Intercept Function)

CC ICE

LEMF

X2

X3

HI2

HI3

SIP Signaling Nodes

(CC Interception 

Trigger)


Figure E.2: General Principles of VoIP Interception

In clause 15, the SIP signalling nodes provide signalling information to the CC Interception Triggering Function.  The CC Interception Triggering Function triggers the CC interception at a media node that implements the CC Intercept Function.
The following functional elements provide the CC Intercept Function in the example call scenarios:
-
PDN-GW/GGSN;
-
IMS-AGW;
-
TrGW;
-
IM-MGW
-
MRF. 

The following functional elements provide the signalling to the CC Intercept Triggering Function:

-
P-CSCF for PDN-GW/GGSN and IMS-AGW;
-
IBCF for TrGW;
-
MGCF for IM-MGW
-
S-CSCF for MRF.

At any given time, for a specific target and for any given call, only one functional element is required to provide the CC interception. The functional element that provides the CC interception may vary, primarily, based on the CSP network implementation and the call scenario. 

E.3
Originating Call from the Target with CC Interception at the PDN-GW/GGSN
E.3.0
General

Figure E.3 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC, when a target originates a call with PDN-GW (or GGSN) providing the CC interception. 


[image: image106.emf]PDN-GW/GGSN

P-CSCF

S-CSCF/AS

Target’s 

UE

ADMF

Mediation

Function

Delivery 

Function 3

Mediation

Function

LEMF

Delivery 

Function 2

Mediation

Function

HI1

HI2 HI3

X1_1

X1_2

X1_3

X2

X3

PCRF

Voice Services

Called Party

SIP

SIP

Voice Media Voice Media

SIP


Figure E.3: VoIP lawful interception for an originating call with CC interception at the PDN-GW/GGSN 

The cloud shown with the label "voice services" is to indicate that the target is making a voice call and the called party can be within the same CSP’s network or in another CSP’s network. 

Figure E.3 shows that the IRI interception is done at S-CSCF or AS. However, as specified in 7A, the IRI interception can also be done at the P-CSCF (not shown in figure E.3). The CC interception is done at the PDN-GW/GGSN. The P-CSCF sends the CC intercept trigger to the PDN-GW/GGSN.    

Note 4: PCRF is defined in 3GPP TS 23.203 [44]. 

The activation of intercept is done by the ADMF for IRI interception and CC interception using the same target identity (SIP URI,  TEL URI or IMEI). 

E.3.1
Originating Call from the Target with CC Interception at the MRF

Figure E.3.1 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC when a target originates a call with an MRF providing the CC interception. The S-CSCF provides the CC Interception Triggering Function for the MRF.


[image: image107.emf]X1_3

IMS-AGW

P-CSCF

Target’s 

UE

Delivery

Function 3

Mediation

Function

LEMF

HI1 HI2 HI3

X1_1

X1_2

X2

X3

Voice Services

Called Party

SIP

SIP

Voice Media Voice Media

SIP

S-CSCF

MRF

AS

ADMF

Mediation

Function

Delivery 

Function 2

Mediation

Function

SIP

Voice Media


Figure E.3.1: VoIP lawful interception for an originating call with CC interception at the MRF
The cloud shown with the label "voice services" indicates that the target is making a voice call and the called party can be within the same CSP’s network or in another CSP’s network.

IRI interception is done at the S-CSCF. The CC interception is done at an MRF that functions as the CC ICE.

The MRF is deployed in a network configuration with an IP border access controller that serves to hide the presence of the MRF. The IP access border controller supports IP-level topology hiding for any voice media streams.  Specifically, the border gateway supports NAT functionality; always presents its own IP address to the user; prevents ICMP Ping & ICMP Traceroute requests from being forwarded across the gateway; and, resets TTL field in the IP header.
Note 3.1: MRF is defined in 3GPP TS 23.228 [43].

The ADF activates interception at the S-CSCF for IRI interception and CC interception using the same target identity (SIP URI,  TEL URI or IMEI).  The S-CSCF dynamically triggers CC interception at the MRF for the call.

E.4
Originating Call from the Target with CC Interception at the IMS-AGW

Figure E.4 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC, when a target originates a call with IMS-AGW providing the CC interception. 


[image: image108.emf]IMS-AGW

P-CSCF

S-CSCF/AS

Target’s 

UE

ADMF

Mediation

Function

Delivery 

Function 3

Mediation

Function

LEMF

Delivery 

Function 2

Mediation

Function

HI1

HI2 HI3

X1_1

X1_2

X1_3

X2

X3

Voice Services

SIP

SIP

Voice Media

Voice Media

SIP

Called Party


Figure E.4: VoIP lawful interception for an originating call with CC interception at the IMS-AGW  

The cloud shown with the label "voice services" is to indicate that the target is making a voice call and the called party can be within the same CSP’s network or in another CSP’s network. 

Figure E.4 shows that the IRI interception is done at S-CSCF or AS. However, as specified in 7A, the IRI interception can also be done at the P-CSCF (not shown in figure Z.4). The CC interception is done at the IMS-AGW. The P-CSCF sends the CC intercept trigger to the IMS-AGW.      

The activation of intercept is done by the ADMF for IRI interception and CC interception using the same target identity (SIP URI,  TEL URI or IMEI). 

E.5
Terminating Call to the Target with CC Interception at the PDN-GW/GGSN
E.5.0
General

Figure E.5 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC, when a target receives an incoming call with PDN-GW/GGSN providing the CC interception. 

[image: image109.emf]PDN-GW/GGSN

P-CSCF

S-CSCF/AS

Target’s 

UE

ADMF

Mediation

Function

Delivery 

Function 2

Mediation

Function

Delivery 

Function 3

Mediation

Function

HI1

HI3 HI2

X1_1

X1_3

X1_2

X2 X3

Voice Services

LEMF

PCRF

Calling Party

SIP

Voice Media

SIP

SIP

Voice Media


Figure E.5: VoIP lawful interception for a terminating call with CC interception at the PDN-GW/GGSN  

The cloud shown with the label "voice services" is to indicate that the target is receiving an incoming voice call and the calling party can be within the same CSP’s network or in another CSP’s network. The network nodes such as I-CSCF, HSS involved in the handling of an incoming call to the target are not shown for simplification of the drawing. 

Figure E.5 shows that the IRI interception is done at S-CSCF or AS. However, as specified in 7A, the IRI interception can also be done at the P-CSCF (not shown in figure Z.5). The CC interception is done at the PDN-GW/GGSN. The P-CSCF sends the CC intercept trigger to the PDN-GW/GGSN.      

The activation of intercept is done by the ADMF for IRI interception and CC interception using the same target identity (SIP URI,  TEL URI or IMEI). 

E.5.1
Terminating Call to the Target with CC Interception at the MRF

Figure E.5.1 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC when the target receives an incoming call with an MRF providing the CC interception. The S-CSCF provides the CC Interception Triggering Function for the MRF.

The MRF is deployed in a network configuration with an IP border access controller that serves to hide the presence of the MRF. The IP access border controller supports IP-level topology hiding for any voice media streams.  Specifically, the border gateway supports NAT functionality; always presents its own IP address to the user; prevents ICMP Ping & ICMP Traceroute requests from being forwarded across the gateway; and, resets TTL field in the IP header.

[image: image110.emf]X1_3

IMS-AGW

P-CSCF

Target’s 

UE

Delivery

Function 3

Mediation

Function

LEMF

HI1 HI2 HI3

X1_1

X1_2

X2

X3

Voice Services

Calling Party

SIP

SIP

Voice Media Voice Media

SIP

S-CSCF

MRF

AS

ADMF

Mediation

Function

Delivery 

Function 2

Mediation

Function

SIP

Voice Media


Figure E.5.1: VoIP lawful interception for a terminating call with CC interception at the MRF
The cloud shown with the label "voice services" is to indicate that the target is receiving an incoming voice call and the calling party can be within the same CSP’s network or in another CSP’s network. The network nodes such as I-CSCF, HSS involved in the handling of an incoming call to the target are not shown for simplification of the figure.

IRI interception is done at the S-CSCF. The CC interception is done at an MRF that functions as the CC ICE.

The ADMF activates interception at the S-CSCF for IRI interception and CC interception using the same target identity (SIP URI,  TEL URI or IMEI).  The S-CSCF dynamically triggers CC interception at the MRF for the call.

E.6
Terminating Call to the Target with CC Interception at the IMS-AGW 

Figure E.6 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC, when a target receives an incoming call with IMS-AGW providing the CC interception. 


[image: image111.emf]IMS-AGW

P-CSCF

S-CSCF/AS

Target’s 

UE

ADMF

Mediation

Function

Delivery 

Function 2

Mediation

Function

Delivery 

Function 3

Mediation

Function

HI1

HI3 HI2

X1_1

X1_3

X1_2

X2 X3

Voice Services

LEMF

Calling Party

SIP

Voice Media

SIP SIP

Voice Media


Figure E.6: VoIP lawful interception for a terminating call with CC interception at the IMS-AGW  

The cloud shown with the label "voice services" is to indicate that the target is receiving an incoming voice call and the calling party can be within the same CSP’s network or in another CSP’s network. The network nodes such as I-CSCF, HSS involved in the handling of an incoming call to the target are not shown for simplification of the drawing. 

Figure E.6 shows that the IRI interception is done at S-CSCF or AS. However, as specified in 7A, the IRI interception can also be done at the P-CSCF (not shown in figure E.6). The CC interception is done at the IMS-AGW. The P-CSCF sends the CC intercept trigger to the IMS-AGW.    

The activation of intercept is done by the ADMF for IRI interception and CC interception using the same target identity (SIP URI,  TEL URI or IMEI). 

E.7
Intra-CSP Forwarded Call with CC Interception at the PDN-GW/GGSN
E.7.0
General

Figure E.7 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC, when a target receives an incoming call with PDN-GW/GGSN providing the CC interception and the call is forwarded to another IMS subscriber within the CSP’s network.   

 
[image: image112.emf]S-CSCF/AS

ADMF

Delivery 

Function 2

Mediation

Function

HI1

HI2

X1_1

X1_3

X1_2

X2

LEMF

P-CSCF

Forward

-to-

Party’s 

UE

Delivery 

Function 3

Mediation

Function

HI3

X3

Voice Services

PDN-GW/GGSN

S-CSCF/AS

PCRF

Mediation

Function

Calling Party

PDN-

GW/

GGSN

P-CSCF

Target’s 

UE

P-CSCF, PCRF, 

PDN-GW/GGSN, 

associated with the 

Target’s UE, not 

used in this 

interception

PCRF

SIP

Voice Media

SIP

Voice 

Media

SIP SIP


Figure E.7: VoIP lawful interception for an intra-CSP forwarded call with CC interception at the PDN-GW/GGSN 

The cloud shown with the label "voice services" is to indicate that the target is receiving an incoming voice call and the calling party can be within the same CSP’s network or in another CSP’s network. The network nodes such as I-CSCF, HSS involved in the handling of an incoming call to the target and to the forwarded-to-party are not shown for simplification of the drawing. 

Figure E.7 shows that the IRI interception is done at S-CSCF or AS. In this scenario, the IRI interception cannot occur in the P-CSCF since the P-CSCF that provides the proxy functions to the target is not on the signalling path.  The CC interception is done at the PDN-GW/GGSN. The P-CSCF (that provides the proxy functions to the forwarded-to-party) sends the CC intercept trigger to the PDN-GW/GGSN.    

The activation of intercept is done by the ADMF for IRI interception and CC interception using the same target identity (SIP URI or TEL URI). 

E.7.1
Intra-CSP Forwarded Call with CC Interception at the MRF
Figure E.7.1 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC, when a target receives an incoming call with the MRF providing the CC interception and the call is forwarded to another IMS subscriber within the CSP’s network. The S-CSCF provides the CC Interception Triggering Function for the MRF.

The MRF is deployed in a network configuration with an IP border access controller that serves to hide the presence of the MRF. The IP access border controller supports IP-level topology hiding for any voice media streams.  Specifically, the border gateway supports NAT functionality; always presents its own IP address to the user; prevents ICMP Ping & ICMP Traceroute requests from being forwarded across the gateway; and, resets TTL field in the IP header.

[image: image113.emf]X1_3

IMS-AGW

P-CSCF

Forward

-to-

Party’s 

UE

Delivery

Function 3

Mediation

Function

LEMF

HI1 HI2 HI3

X1_1

X1_2

X2

X3

Calling Party

SIP

SIP

Voice Media

SIP

S-CSCF

MRF

AS

ADMF

Mediation

Function

Delivery 

Function 2

Mediation

Function

SIP

Voice Media

Voice Services

S-CSCF

SIP

Voice 

Media

AS

SIP


Figure E.7.1: VoIP lawful interception for an intra-CSP forwarded call with CC interception at the MRF

The cloud shown with the label "voice services" is to indicate that the target is receiving an incoming voice call and the calling party can be within the same CSP’s network or in another CSP’s network. The network nodes such as P-CSCF, IMS-AGW, HSS involved in the handling of an incoming call to the target and to the forwarded-to-party are not shown for simplification of the drawing. 

Figure E.7.1 shows that the IRI interception is done at S-CSCF. The CC interception is done at the MRF that functions as the CC ICE.

The ADMF activates interception at the S-CSCF for IRI interception and CC interception using the same target identity (SIP URI or TEL URI).  The S-CSCF dynamically activates CC interception at the MRF.

E.8
Intra-CSP Forwarded Call with CC Interception at the IMS-AGW
Figure E.8 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC, when a target receives an incoming call with IMS-AGW providing the CC interception and the call is forwarded to another IMS subscriber within the CSP’s network.   


[image: image114.emf]S-CSCF/AS

ADMF

Mediation

Function

Delivery 

Function 2

Mediation

Function

HI1

HI2

X1_1

X1_3

X1_2

X2

LEMF

P-CSCF

Forward

-to-

Paty’s 

UE

Delivery 

Function 3

Mediation

Function

HI3

X3

Voice Services

IMS-AGW

S-CSCF/AS

Calling Party

IMS-

AGW

P-CSCF

Target’s 

UE

P-CSCF/IMS-AGW 

associated with the 

Target’s UE, not 

used in this 

interception

SIP

Voice Media

Voice 

Media

SIP

SIP

SIP


Figure E.8: VoIP lawful interception for an intra-CSP forwarded call with CC interception at the IMS-AGW  

The cloud shown with the label "voice services" is to indicate that the target is receiving an incoming voice call and the calling party can be within the same CSP’s network or in another CSP’s network. The network nodes such as I-CSCF, HSS involved in the handling of an incoming call to the target and to the forwarded-to-party are not shown for simplification of the drawing. 

Figure E.8 shows that the IRI interception is done at S-CSCF or AS. In this scenario, the IRI interception cannot occur in the P-CSCF since the P-CSCF that provides the proxy functions to the target is not on the signalling path.  The CC interception is done at the IMS-AGW. The P-CSCF (that provides the proxy functions to the forwarded-to-party) sends the CC intercept trigger to the IMS-AGW.      

The activation of intercept is done by the ADMF for IRI interception and CC interception using the same target identity (SIP URI or TEL URI). 

E.9
Inter-CSP Forwarded Call to a CS Domain
Figure E.9 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC, when a target receives an incoming call and the call is forwarded to a subscriber on the CS domain of another CSP’s network.  

 
[image: image115.emf]S-CSCF/AS

ADMF

Mediation

Function

Delivery 

Function 2

Mediation

Function

HI1

HI2

X1_1

X1_3

X1_2

X2

LEMF

MGCF

Delivery 

Function 3

Mediation

Function

HI3

X3

Voice Services

IM-MGW

Signaling (CS)

Forwarded-to-

Party

Calling Party

Voice Media

SIP

SIP

Voice Media 

(CS)

Voice Services

(CS domain)

PDN-

GW/

GGSN

P-CSCF

Target’s 

UE

PCRF

IMS-

AGW

P-CSCF, PCRF, 

PDN-GW/GGSN or 

IMS-AGW 

associated with the 

Target’s UE, not 

used in this 

interception


Figure E.9: VoIP lawful interception for an inter-CSP forwarded call to a CS Domain

The cloud shown with the label "voice services" is to indicate that the target is receiving an incoming voice call and the calling party can be within the same CSP’s network or in another CSP’s network. The network nodes such as I-CSCF, HSS involved in the handling of an incoming call to the target are not shown for simplification of the drawing. 

Figure E.9 shows that the IRI interception is done at S-CSCF or AS. In this scenario, the IRI interception cannot occur in the P-CSCF since the P-CSCF that provides the proxy functions to the target is not on the signalling path.  The CC interception is done at the IM-MGW. The MGCF sends the CC intercept trigger to the IM-MGW.  

The activation of intercept is done by the ADMF for IRI interception and CC interception using the same target identity (SIP URI or TEL URI). 

E.10
Inter-CSP Forwarded Call to an IMS Domain
Figure E.10 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC, when a target receives an incoming call and the call is forwarded to a subscriber on the IMS domain of another CSP’s network.  

 
[image: image116.emf]S-CSCF/AS

ADMF

Mediation

Function

Delivery 

Function 2

Mediation

Function

HI1

HI2

X1_1

X1_3

X1_2

X2

LEMF

IBCF

Delivery 

Function 3

Mediation

Function

HI3

X3

Voice Services

TrGW

SIP (Ici)

Voice Services 

(another IMS CSP)

Forwarded-to-

Party

Calling Party

PDN-

GW/

GGSN

P-CSCF

Target’s 

UE

PCRF

IMS-

AGW

P-CSCF, PCRF, 

PDN-GW/GGSN or 

IMS-AGW 

associated with the 

Target’s UE, not 

used in this 

interception

Voice Media

SIP

SIP

Voice Media 

(Izi)


Figure E.10: VoIP lawful interception for an inter-CSP forwarded call to an IMS Domain

The cloud shown with the label "voice services" is to indicate that the target is receiving an incoming voice call and the calling party can be within the same CSP’s network or in another CSP’s network. The network nodes such as I-CSCF, HSS involved in the handling of an incoming call to the target are not shown for simplification of the drawing.

Figure E.10 shows that the IRI interception is done at S-CSCF or AS. In this scenario, the IRI interception cannot occur in the P-CSCF since the P-CSCF that provides the proxy functions to the target is not on the signalling path. The CC interception is done at the TrGW. IBCF sends the CC intercept trigger to the TrGW.    

The activation of intercept is done by the ADMF for IRI interception and CC interception using the same target identity (SIP URI or TEL URI). 

E.11
Originating Call from the Target  with IMS Roaming
Figure E.11 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC, when a target originates a call with IMS roaming. 

 
[image: image117.emf]P-CSCF

IMS 

Roaming 

Target’s 

UE

ADMF

Mediation

Function

Delivery 

Function 3

Mediation

Function

LEMF

Delivery 

Function 2

Mediation

Function

HI1

HI2 HI3

X1_1

X1_2

X1_3

X2

X3

PCRF

Voice Services

Called Party

SIP SIP

Voice 

Media

Voice Media

SIP

S-CSCF/AS

IBCF

IMS-AGW

TrGW

PDN-GW/

GGSN

IMS Roaming

SIP

Voice 

Media

Voice 

Media

Voice 

Media


Figure E.11: VoIP lawful interception for an originating call with IMS Roaming  

The cloud shown with the label "voice services" is to indicate that the target is making a voice call and the called party can be within the same CSP’s network or in another CSP’s network. 

Figure E.11 shows that the IRI interception is done at S-CSCF or AS. The IRI interception at the P-CSCF does not apply to this configuration due to the fact that the P-CSCF resides at the visited CSP as a result of IMS roaming. The CC interception is done at the TrGW. The I-BCF sends the CC intercept trigger to the TrGW.    

The activation of intercept is done by the ADMF for IRI interception and CC interception using the same target identity (SIP URI,  TEL URI or IMEI). 

E.12
Terminating Call to the Target  with IMS Roaming
Figure E.12 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC, when a target with IMS roaming receives an incoming call. 

 
[image: image118.emf]S-CSCF/AS

ADMF

Mediation

Function

Delivery 

Function 2

Mediation

Function

HI1

HI2

X1_1

X1_3

X1_2

X2

LEMF

IBCF

Delivery 

Function 3

Mediation

Function

HI3

X3

Voice Services

TrGW

Calling Party

Voice Media

SIP

SIP

PCRF

IMS-AGW

PDN-GW/

GGSN

IMS Roaming

P-CSCF

SIP

Voice 

Media

IMS 

Roaming 

Target’s 

UE

SIP

Voice 

Media

Voice 

Media

Voice 

Media


Figure E.12: VoIP lawful interception for a terminating call with IMS Roaming  

The cloud shown with the label "voice services" is to indicate that the target is receiving an incoming voice call and the calling party can be within the same CSP’s network or in another CSP’s network. The network nodes such as I-CSCF, HSS involved in the handling of an incoming call to the target are not shown for simplification of the drawing.

Figure E.12 shows that the IRI interception is done at S-CSCF or AS. The IRI interception at the P-CSCF does not apply to this configuration due to the fact that the P-CSCF resides at the visited CSP as a result of IMS roaming. The CC interception is done at the TrGW. The I-BCF sends the CC intercept trigger to the TrGW.    

The activation of intercept is done by the ADMF for IRI interception and CC interception using the same target identity (SIP URI, or TEL URI or IMEI). 

E.13
Intra-CSP Forwarded Call  with IMS Roaming
Figure E.13 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC, when an incoming call to a target gets forwarded to another subscriber who is IMS roaming. The target may or may not be IMS roaming. 


[image: image119.emf]S-CSCF/AS

ADMF

Mediation

Function

Delivery 

Function 2

Mediation

Function

HI

1

HI

2

X1_1

X1_3

X1_2

X2

LEMF

IBCF

Delivery 

Function 3

Mediation

Function

HI

3

X3

Voice Services

TrGW

Calling 

Party

PDN-GW/

GGSN

P-CSCF

Target’

s UE

PCRF

IMS-

AGW

P-CSCF, PCRF, 

PDN-GW/GGSN 

or IMS-AGW 

associated with 

the Target’s UE, 

not used in this 

interception

Voice Media

SIP

SIP

PCRF

IMS-

AGW

PDN-GW/

GGSN

IMS Roaming

P-CSCF

SIP

IMS 

Roaming 

Forwarded-

to Party’s 

UE

SIP

Voice 

Media

Voice 

Media

Voice 

Media

S-CSCF/

AS

SIP

Voice 

Media

 
Figure E.13: VoIP lawful interception for a intra-CSP forwarded call  with IMS Roaming  

The cloud shown with the label "voice services" is to indicate that the target is receiving an incoming voice call and the calling party can be within the same CSP’s network or in another CSP’s network. The network nodes such as I-CSCF, HSS involved in the handling of an incoming call to the target are not shown for simplification of the drawing. 

Figure E.13 shows that the IRI interception is done at S-CSCF or AS of the target.  In this scenario, the IRI interception cannot occur in the P-CSCF since the P-CSCF that provides the proxy functions to the target is not on the signalling path. Since the forwarded-to-party is IMS roaming, the CC interception is done at the TrGW. IBCF sends the CC intercept trigger to the TrGW.        

The activation of intercept is done by the ADMF for IRI interception and CC interception using the same target identity (SIP URI or TEL URI). 

Note 5: If the target is IMS roaming, but not the forwarded-to-party, then the CC interception for an intra-CSP forwarded call is done at the PDN-GW/GGSN (as illustrated in Z.9) or IMS-AGW (as illustrated in Z.8). 
Annex F (Informative) Examples of IMS-based VoIP Lawful Interception Call Flows
F.1
General Remarks

All the call flows illustrate that the CC delivery begins once the SDP offer and answer is completed (i.e., when the media bearer is setup). In all the call flows, the first reliable response is SIP 200 OK.  

In all the call flows, the originating end of the call sends the SDP offer and terminating end gives the SDP answer. Since, the first reliable response is SIP 200 OK, the SDP answer is always given in the SIP 200 OK message. 

The call flows assume that per clause 7.A, the IRI for VoIP is nothing but the delivery encapsulated SIP messages. The call flows do not show the method used for correlating the IRI with IRI and IRI with CC. It is presumed that those are stage 3 details. 

All the call flows assume the presence of a Voice Application Server (shown as AS) that provides the voice services like digit translation, invoking the call forwarding etc,  

IRI in the visited CSP is intercepted by the P-CSCF and IRI in the home CSP is intercepted by the S-CSCF. 

The call flows show that CC interception is done at the IP-CAN (and it should be interpreted to mean that the interception is done in the PDN-GW or GGSN depending on the packet core network), or at the TrGW or at the IM-MGW. The other possible CC interception options (e.g., IMS-AGW) are not shown. 

Not all the functional elements are shown in the call flows. For example, the call flows do not show I-CSCF, HSS, PCRF.  

All the call flows show a summary of SIP messages that are delivered to the LEA (not all SIP messages are shown). The term LEMF, used in some call flows,  means it is an equivalent of LEA.
For each call flow, references are required to identify MMTEL service that it illustrates (for further study).  

F.2
Call Originations from Target in Home CSP

F.2.0
Introduction

This clause gives 2 call flows to illustrate the call origination scenarios. 

Figure F.1 illustrates the case where the Party_A (target) calls Party_B.  

Figure F.2 illustrates the case where the Party_A (target) dials a special number (e.g., a speed call number or an 800-number), which is translated to Party_B by the AS.  

F.2.1
Target Originated Call - Target (Party_A) Calls Party_B


[image: image120.emf]Party_A

[Target]

IP-CAN P-CSCF S-CSCF AS Party_B DF3 LEA

Home CSP [Target Domain] MF/DF

AAR/RAR

AAA/RAA

Media 

Media 

CC

D1

INVITE Party_B

[SDP]

200 OK

[SDP]

AAR/RAR

AAA/RAA

D10

ACK

DF2

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

200 OK

[SDP]

200 OK

[SDP]

200 OK

[SDP]

200 OK

[SDP]

{200 OK}

{ACK}

ACK

ACK

ACK

ACK

180 Ringing

180 Ringing

180 Ringing 180 Ringing

180 Ringing

{180 Ringing}

{INVITE Party-B}

Figure F.1: Target originated call – target calls Party_B  

F.2.2
Target Originated Call – Target (Party_A) dials a Special Number   

[image: image121.emf]Party_A

[Target]

IP-CAN P-CSCF S-CSCF AS Party_B DF3 LEA

Target Domain MF/DF

AAR/RAR

AAA/RAA

Media 

Media 

CC

D1

INVITE SP.NO.

[SDP]

200 OK

[SDP]

AAR/RAR

AAA/RAA

D10

ACK

DF2

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

200 OK

[SDP]

200 OK

[SDP]

200 OK

[SDP]

200 OK

[SDP]

{INVITE SP.NO.}

{200 OK}

{ACK}

ACK

ACK

ACK

ACK

180 Ringing

180 Ringing

180 Ringing 180 Ringing

180 Ringing

{180 Ringing}

INVITE SP.NO.

[SDP]

INVITE SP.NO.

[SDP]

{INVITE Party-B}


Figure F.2: Target originated call – target dials a special number  

F.3
Call Terminations to Target – Home CSP
F.3.0
Introduction

This clause gives 1 call flow to illustrate the  call termination scenario.   

Figure F.3 illustrates the case where the Party_A calls target (Party_B).     


[image: image122.emf]Party_B

Target

IP-CAN P-CSCF

S-CSCF

AS Party_A

DF3 LEA

Target Domain MF/DF

AAA/RAA

Media 

Media 

D1

AAR/RAR

D10

AAA/RAA

AAR/RAR

DF2

CC

INVITE Party_B

[SDP]

200 OK

[SDP]

INVITE Party_B

[SDP]

200 OK

[SDP]

{200 OK}

{ACK}

ACK

ACK

180 Ringing

180 Ringing

{180 Ringing}

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

180 Ringing

180 Ringing

180 Ringing

200 OK

[SDP]

200 OK

[SDP]

200 OK

[SDP]

ACK

ACK ACK

{INVITE Party-B}


Figure F.3: Target receives an incoming call.

F.4
Call Forwarding – Non Roaming
F.4.0
Introduction

This clause gives 4 call flows to illustrate call forwarding scenarios.   

Figure F.4 illustrates the case of an intra-CSP call forwarding unconditional. Here, the Party_A calls target (Party_B).  The AS determines that all incoming calls to the target have to be forwarded to Party_C served by the same CSP.  

Figure F.5 illustrates the case first part of an intra-CSP call forwarding no answer. Figure F.6 illustrate the second part of an intra-CSP call forwarding no answer. Here,  the Party_A calls target (Party_B). .  The target does not answer and the AS determines that target has a call forwarding no answer enabled to Party_C served by the same CSP.  

Figure F.7 illustrates the case of inter-CSP call forwarding unconditional. Here, the Party_A calls target (Party_B).  The AS determines that all incoming calls to the target have to be forwarded to Party_C served by a different CSP.  

F.4.1
Intra-CSP Call Forwarding Unconditional 


[image: image123.emf]Party_C IP-CAN

S-CSCF/

P-CSCF

S-CSCF

AS Party_A

DF3 LEA

Target Domain MF/DF

AAA/RAA

Media 

Media 

D1

AAR/RAR

D10

AAA/RAA

AAR/RAR

DF2

CC

INVITE Party_B

[SDP]

200 OK

[SDP]

INVITE Party_C

[History Info: Party_B]

[SDP]

200 OK

[SDP]

{200 OK}

{ACK}

ACK

ACK

180 Ringing

180 Ringing

{180 Ringing}

INVITE Party_B

[SDP]

180 Ringing

180 Ringing

180 Ringing

200 OK

[SDP]

200 OK

[SDP]

200 OK

[SDP]

ACK

ACK ACK

{INVITE Party-B}

P-

CSCF

Party_B

Target

IP-

CAN

INVITE Party_C

[History Info: Party_B]

[SDP]

INVITE Party_C

[History Info: Party_B]

[SDP]

{INVITE Party-C}

Forwarded-to-Party’s Domain

181 Call Is Being 

Forwarded

181 Call Is Being 

Forwarded

{Call Is Being 

Forwarded}

Call Forwarding 

Unconditional


Figure F.4: Incoming call to target is forwarded within the CSP

F.4.2
Intra-CSP Call Forwarding No Answer


[image: image124.emf]Party_B

Target

IP-CAN P-CSCF

S-CSCF

AS Party_A

DF3 LEA

Target Domain MF/DF

AAA/RAA

D1

AAR/RAR

D10

DF2

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

180 Ringing

{180 Ringing}

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

{INVITE Party-B}

180 Ringing 180 Ringing

180 Ringing

180 Ringing

{CANCEL}

487 Terminated

SAA/RAA

SAR/RAR

CANCEL

CANCEL

CANCEL

487 Terminated

487 Terminated

ACK

ACK

ACK

{487 TERMINATED}

{ACK}

Call Forwarding No Answer 

Timer Expires

The flow continues…...


Figure F.5: Incoming call to target is forwarded due to call forwarding no answer within the CSP (flow 1 of 2)


[image: image125.emf]Party_C IP-CAN

S-CSCF/

P-CSCF

S-CSCF

AS Party_A

DF3 LEA

Target Domain MF/DF

D20

DF2

INVITE Party_C

[History Info: Party_B]

[SDP]

180 Ringing 

180 Ringing

{180 Ringing}

{Call Is Being 

Forwarded}

P-CSCF

Party_B

Target

IP-

CAN

INVITE Party_C

[History Info: Party_B]

[SDP]

INVITE Party_C

[History Info: Party_B]

[SDP]

{INVITE Party-C}

181 Call Is Being 

Forwarded

181 Call Is Being 

Forwarded

180 Ringing 180 Ringing 

180 Ringing

Media 

Media 

AAA/RAA

AAR/RAR

CC

{200 OK}

{ACK}

ACK 

ACK   ACK  

200 OK

[SDP]

200 OK

[SDP]

200 OK

[SDP]

200 OK

[SDP]

200 OK

[SDP]

ACK

ACK

Call Forwarding No Answer 

Timer Expires

AAA/RAA

AAR/RAR

Forward-to-Party Domain

Figure F.6: Incoming call to target is forwarded due to call forwarding no answer within the CSP (flow 2 of 2)
F.4.3
Inter-CSP Call Forwarding Unconditional


[image: image126.emf]Party_C

TrGW/

IM-MGW

IBCF/

MGCF

S-CSCF

AS Party_A

DF3 LEA

Target Domain MF/DF

Media 

Media 

D1

D10

DF2

CC

INVITE Party_B

[SDP]

200 OK

[SDP]

INVITE Party_C

[History Info: Party_B]

[SDP]

200 OK

[SDP]

{200 OK}

{ACK}

ACK

ACK

180 Ringing

180 Ringing

{180 Ringing}

INVITE Party_B

[SDP]

180 Ringing

180 Ringing

180 Ringing

200 OK

[SDP]

200 OK

[SDP]

200 OK

[SDP]

ACK

ACK ACK

{INVITE Party-B}

P-CSCF

Party_B

Target

INVITE Party_C

[History Info: Party_B]

[SDP]

INVITE Party_C

[History Info: Party_B]

[SDP]

{INVITE Party-C}

Add Context

Add Context Ack

Modify Context

Modify Context Ack

IP-

CAN/

IMS-

AGW

181 Call Is Being 

Forwarded

181 Call Is Being 

Forwarded

{Call Is Being 

Forwarded}

Call Forwarding 

Unconditional


Figure F.7: Incoming call to target is forwarded outside the CSP

F.5
IMS Roaming

F.5.0
General
This clause gives 2 call flows to illustrate the case of IMS roaming. 

Figure F.8 illustrate the case where the roaming target originates a call. Here, roaming target (Party_A) calls Party_B who is served by the same CSP as that of target. Party_B is not roaming.  

Figure F.9 illustrates the case where a roaming target receives an incoming call. Here, non-roaming Party_A, who is served by the same CSP as that of target, calls the target (Party_B).     

F.5.1
Roaming Target Originates a Call


[image: image127.emf]Party_A

[Target]

TrGW IBCF S-CSCF AS Party_B DF2 LEA

MF/DF

Media 

Media 

CC

D1

INVITE Party_B

[SDP]

200 OK

[SDP]

D10

ACK

DF2

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

200 OK

[SDP]

200 OK

[SDP]

200 OK

[SDP]

200 OK

[SDP]

{200 OK}

{ACK}

ACK

ACK

ACK

ACK

180 Ringing

180 Ringing

180 Ringing 180 Ringing

180 Ringing

{180 Ringing}

{INVITE Party-B}

Add Context

Add Context Ack

Modify Context

Modify Context Ack

Home CSP [Target Domain]

Visited CSP 


Figure F.8: Roaming target originates a call

F.5.2
Call Termination to a Roaming Target


[image: image128.emf]TrGW IBCF

S-CSCF

AS Party_A

DF3 LEA

Home CSP [Target Domain] MF/DF

Media 

Media 

D1

D10

DF2

CC

INVITE Party_B

[SDP]

200 OK

[SDP]

200 OK

[SDP]

{200 OK}

{ACK}

ACK

ACK

180 Ringing

180 Ringing

{180 Ringing}

INVITE Party_B

[SDP]

180 Ringing

180 Ringing

180 Ringing

200 OK

[SDP]

200 OK

[SDP]

200 OK

[SDP]

ACK

ACK ACK

{INVITE Party-B}

Add Context

Add Context Ack

Modify Context

Modify Context Ack

Party_B

Target

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

Visited CSP 


Figure F.9: Roaming target receives an incoming call.

F.6
Interception in Visited CSP

F.6.0
General
This clause gives 3 call flows to illustrate the case of interception in the visited CSP. In all these flows, the IRI interception happens at the P-CSCF. Both IRI and CC interception happen in the visited CSP. 

Figure F.10 illustrates the case where the target (Party_A) in the visited CSP originates a call dialing a special number. The special number is translated into Party_B in the home CSP. The flow also assumes that the interception is done only in the visited CSP.    

Figure F.11 illustrates the case where the target (Party_B) in the visited CSP receives an incoming call from Party_A served by the same Home CSP.  The flow assumes that the interception is done only in the visited CSP.  

Figure F.12 illustrates the case where an incoming call to the target (Party_B) gets forwarded in the Home CSP due to call forwarding no answer. The flow also assumes that the interception is done only in the visited CSP.  

F.6.1
Interception in Visited CSP – Target Originated Call


[image: image129.emf]Party_A

[Target]

IP-CAN S-CSCF AS Party_B DF3 LEMF

Visited CSP

MF/DF

DF2 IBCF IBCF TRGW TRGW

Home CSP

P-CSCF

AAR/RAR

AAA/RAA

Media 

CC

D1

D10

ACK

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

{INVITE SP.NO.}

{ACK}

ACK

ACK

ACK

ACK

180 Ringing

180 Ringing

180 Ringing 180 Ringing 180 Ringing

{180 Ringing}

INVITE SP.NO.

[SDP]

INVITE SP.NO.

[SDP]

INVITE SP.NO.

[SDP]

INVITE SP.NO.

[SDP]

INVITE SP.NO.

[SDP]

180 Ringing 180 Ringing

200 OK [SDP]

{200 OK}

200 OK [SDP]

200 OK [SDP]

200 OK [SDP]

200 OK [SDP]

200 OK [SDP]

200 OK [SDP]

Media 

Media 

ACK

ACK

AAR/RAR

AAA/RAA

Add Context Ack

Add Context

Add Context

Add Context Ack

Modify Context

Modify Context Ack

Modify Context Ack

Modify Context

Media 


Figure F.10: Roaming target originates a call – interception in the visited CSP

F.6.2
Interception in Visited CSP – Target Terminating Calls. 


[image: image130.emf]Party_A

IP-CAN

S-CSCF AS

Party_B

[Target]

DF3 LEMF

Visited CSP MF/DF

DF2 IBCF IBCF TRGW TRGW

Home CSP

P-CSCF

D1

D10

INVITE Party_B

[SDP]

180 Ringing

180 Ringing

{180 Ringing}

INVITE Party_B

[SDP]

180 Ringing

180 Ringing

{INVITE Party-B}

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

180 Ringing 180 Ringing

180 Ringing

Add Context Ack

Add Context

Add Context

Add Context Ack

200 OK [SDP]

200 OK [SDP]

{200 OK}

200 OK [SDP]

200 OK [SDP]

200 OK [SDP] 200 OK [SDP]

200 OK [SDP]

AAR/RAR

AAA/RAA

AAR/RAR

AAA/RAA

Modify Context

Modify Context Ack

Modify Context Ack

Modify Context

Media 

CC

Media 

Media 

Media 

ACK

ACK

ACK

ACK ACK ACK

ACK

{ACK}


Figure F.11: Roaming target receives a call – interception in the visited CSP. 

F.6.3
Incoming Call to Roaming Target is forwarded due to Call Forwarding No Answer


[image: image131.emf]Party_A

IP-CAN

S-CSCF AS

Party_B

[Target]

DF3 LEMF

Visited CSP MF/DF

DF2 IBCF IBCF TRGW TRGW

Home CSP

P-CSCF

D1

D10

INVITE Party_B

[SDP]

180 Ringing

180 Ringing

{180 Ringing}

INVITE Party_B

[SDP]

180 Ringing

180 Ringing

{INVITE Party-B}

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

INVITE Party_B

[SDP]

180 Ringing 180 Ringing

180 Ringing

Add Context Ack

Add Context

Add Context

Add Context Ack

487 

Terminated

{CANCEL}

AAA/RAA

CFNR Timer Expires

CANCEL CANCEL

CANCEL CANCEL CANCEL

487 

Terminated

487 

Terminated

487 

Terminated

487 

Terminated

AAR/RAR

SAA/RAA

SAR/RAR

Delete Context

Delete Context Ack

Delete Context Ack

Delete Context

ACK

ACK

ACK

ACK

ACK

181 Call Is Being Forwarded

181 Call Is 

Being 

Forwarded

Call Forwarding occurs (steps not shown) 

as those messages are not intercepted. 

CC is not intercepted. 

{487 

Terminated}

{ACK}


Figure F.12: Incoming call to target is forwarded in the home CSP due to Call Forwarding No Answer – interception in visited CSP.  
Annex G (informative):
Change history

	Change history

	Date
	TSG #
	TSG Doc.
	CR
	Rev
	Cat
	Subject/Comment
	Old
	New
	WI

	
	SA_03
	
	
	-
	
	Approved at SA#6 and placed under TSG SA Change Control
	1.0.0
	3.0.0
	

	
	SA_10
	SP-000625
	0001
	-
	
	Addition of parameters to the X3-Interface
	3.0.0
	3.1.0
	Security

	2000-03
	SP-11
	SP-010137
	0002
	-
	
	Correction of Location information parameters in interception event records
	3.1.0
	3.2.0
	Security

	2000-03
	SP-11
	SP-010146
	0003
	-
	
	Update of TS 33.107 for Release 4 - Inclusion of PS LI requirements
	3.1.0
	4.0.0
	Security

	2000-06
	SP-12
	SP-010374
	0004
	1
	B
	Update of TS 33.107 for Release 5
	4.0.0
	5.0.0
	SEC1-LI

	2001-12
	SP-14
	SP-010612
	0010
	-
	A
	Start of secondary interception of an active PDP context
	4.1.0
	5.1.0
	SEC1-LI

	2001-12
	SP-14
	SP-010613
	0011
	-
	C
	Alignment of TS 33.107 for Release 5 Network Architecture
	4.1.0
	5.1.0
	SEC1-LI

	2001-12
	SP-14
	SP-010614
	0014
	-
	A
	Correct the MO-SMS and MT-SMS events
	4.1.0
	5.1.0
	SEC1-LI

	2001-12
	SP-14
	SP-010615
	0016
	-
	A
	Source of PDP context initiation
	4.1.0
	5.1.0
	SEC1-LI

	2002-03
	SP-15
	SP-020109
	0017
	-
	B
	PDP context Deactivation cause
	5.1.0
	5.2.0
	SEC1-LI

	2002-03
	SP-15
	SP-020110
	0018
	-
	B
	The use of H.248 in setting up a bearer intercept point at the MGW
	5.1.0
	5.2.0
	SEC1-LI

	2002-03
	SP-15
	SP-020111
	0021
	-
	B
	Inter-SGSN RA update with active PDP context
	5.1.0
	5.2.0
	SEC1-LI

	2002-03
	SP-15
	SP-020112
	0022
	-
	B
	Addition of PDP context modification Event and Transferring the QoS information element across the X2 interface
	5.1.0
	5.2.0
	SEC1-LI

	
	-
	-
	-
	-
	-
	Change History new version corrected for SP-15 CRs
	5.2.0
	5.2.1
	SEC1-LI

	2002-06
	SP-16
	SP-020345
	0023
	-
	B
	Changes to 33.107 to support interception at a GGSN
	5.2.1
	5.3.0
	SEC1-LI

	2002-06
	SP-16
	SP-020345
	0024
	-
	B
	Addition of SMS type information
	5.2.1
	5.3.0
	SEC1-LI

	2002-06
	SP-16
	SP-020345
	0025
	-
	C
	Inclusion of Serving System IRI in TS 33.107
	5.2.1
	5.3.0
	SEC1-LI

	2002-09
	SP-17
	SP-020511
	0026
	-
	F
	Essential clarification to the Timestamp IE
	5.3.0
	5.4.0
	SEC1-LI

	2002-09
	SP-17
	SP-020511
	0027
	-
	F
	Additional X3-interface parameters
	5.3.0
	5.4.0
	SEC1-LI

	2002-12
	SP-18
	SP-020702
	0028
	-
	F
	Event Time
	5.4.0
	5.5.0
	SEC1-LI

	2002-12
	SP-18
	SP-020704
	0029
	-
	F
	Essential correction to the LI events generated during inter-SGSN RAU, when PDP context is active
	5.4.0
	5.5.0
	SEC1-LI

	2002-12
	SP-18
	SP-020703
	0030
	-
	F
	Essential correction to the LI events generated during inter-SGSN RAU, when PDP context is active
	5.4.0
	5.5.0
	SEC1-LI

	2002-12
	SP-18
	SP-030478
	0031
	-
	F
	Missing QoS Parameter in IRI
	5.5.0
	5.6.0
	SEC1-LI

	2003-09
	SP-21
	SP-030479
	0032
	-
	B
	TEL URL for IMS interception identity (Release 6)
	5.5.0
	6.0.0
	SEC1-LI

	2003-09
	SP-21
	SP-030479
	0032
	-
	D
	Stereo delivery to LEMF
	5.5.0
	6.0.0
	SEC1-LI

	2003-12
	SP-22
	SP-030590
	0034
	-
	F
	MSISDN/IMEI clarification for GPRS interception
	6.0.0
	6.1.0
	SEC1-LI

	2003-12
	SP-22
	SP-030591
	0035
	-
	F
	Reporting TEL URL
	6.0.0
	6.1.0
	SEC1-LI

	2004-06
	SP-24
	SP-040397
	0036
	-
	F
	Correction on Network initiated Mobile Station Detach signalling flow
	6.1.0
	6.2.0
	SEC1-LI

	2004-06
	SP-24
	SP-040398
	0037
	-
	F
	TEL-URL missing in activation of LI in the CSCFs
	6.1.0
	6.2.0
	SEC1-LI

	2004-06
	SP-24
	SP-040399
	0038
	-
	F
	Correction on the use of session initiator parameter
	6.1.0
	6.2.0
	SEC1-LI

	2004-06
	SP-24
	SP-040400
	0039
	-
	F
	Correction to HLR interception event name
	6.1.0
	6.2.0
	SEC1-LI

	2004-06
	SP-24
	SP-040401
	0040
	-
	B
	Clarification for Push to talk over Cellular
	6.1.0
	6.2.0
	SEC1-LI

	2004-06
	SP-24
	SP-040402
	0041
	-
	F
	Adding an encryption parameter to IRI across X2 interface
	6.1.0
	6.2.0
	SEC1-LI

	2004-06
	SP-24
	SP-040403
	0042
	-
	F
	References
	6.1.0
	6.2.0
	SEC1-LI

	2004-06
	SP-24
	SP-040404
	0043
	-
	F
	Enhancements for the Functional Architecture chapter
	6.1.0
	6.2.0
	SEC1-LI

	2004-09
	SP-25
	SP-040693
	0044
	-
	F
	Correction on the use of session initiator parameter
	6.2.0
	6.3.0
	SEC1-LI

	2004-09
	SP-25
	SP-040693
	0045
	-
	F
	ICE (Intercepting Control Elements), INE (Intercepting Network Elements) definition
	6.2.0
	6.3.0
	SEC1-LI

	2004-09
	SP-25
	SP-040693
	0046
	-
	F
	Clarification to SMS interception
	6.2.0
	6.3.0
	SEC1-LI

	2004-09
	SP-25
	SP-040693
	0047
	-
	F
	Replace SIP URL with SIP URI
	6.2.0
	6.3.0
	SEC1-LI

	2004-12
	SP-26
	SP-040850
	0048
	-
	B
	Lawful Interception for WLAN Interworking
	6.3.0
	6.4.0
	SEC1-LI

	2004-12
	SP-26
	SP-040850
	0049
	-
	F
	33.107 Cleanup
	6.3.0
	6.4.0
	SEC1-LI

	2004-12
	SP-26
	SP-040850
	0050
	-
	B
	Clarification on MMS interception
	6.3.0
	6.4.0
	SEC1-LI

	2005-06
	SP-28
	SP-050256
	0052
	-
	F
	Correction on the use of identities for I-WLAN lawful interception
	6.4.0
	6.5.0
	SEC1-LI

	2005-06
	SP-28
	SP-050257
	0051
	1
	F
	Clarifications for the usage of the notion of a service in distributed IP networks
	6.4.0
	7.0.0
	SEC-LI

	2005-06
	SP-28
	SP-050257
	0053
	-
	C
	Correlation for IMS intercption
	6.4.0
	7.0.0
	SEC1-LI

	2005-09
	SP-29
	SP-050570
	0054
	-
	F
	Clarifications to the RAU event
	7.0.0
	7.1.0
	SEC1-LI

	2005-09
	SP-29
	SP-050570
	0055
	-
	C
	Simplifications to LDI handling
	7.0.0
	7.1.0
	SEC1-LI

	2005-12
	SP-30
	SP-050779
	0054
	-
	B
	Start of interception for already attached UE
	7.1.0
	7.2.0
	LI-7A

	2005-12
	SP-30
	SP-050763
	0056
	-
	A
	Availability of IMSI at PDG
	7.1.0
	7.2.0
	IMS2 (SEC1-LI)

	2006-03
	SP-31
	SP-060064
	0057
	-
	F
	WLAN Interworking - Additional Details for TS 33.107
	7.2.0
	7.3.0
	LI-7A

	2006-09
	SP-33
	SP-060659
	0058
	1
	F
	Editorial Update by Rapporteur
	7.3.0
	7.4.0
	LI-7A

	2007-03
	SP-35
	
	0060
	-
	B
	Stage 2 MBMS Interception
	7.4.0
	7.5.0
	LI-7A

	2007-03
	SP-35
	SP-070156
	0061
	1
	F
	SMS IRI Reporting for WLAN Interworking
	7.4.0
	7.5.0
	LI-7A

	2007-06
	SP-36
	SP-070331
	0063
	-
	B
	Direct Tunnel LI
	7.5.0
	7.6.0
	LI-7A

	2007-06
	SP-36
	SP-070332
	0062
	-
	B
	NSAPI (Network layer Service Access Point Identifier) optional in IRI
	7.6.0
	8.0.0
	LI8

	2007-09
	SP-37
	SP-070601
	0065
	-
	B
	WLAN IRI at AAA for re-authentication
	8.0.0
	8.1.0
	LI8

	2007-09
	SP-37
	SP-070599
	0064
	-
	A
	Stage 2 MBMS Interception
	8.0.0
	8.1.0
	LI-7A

	2007-12
	SP-38
	SP-070788
	0066
	-
	C
	P-CSCF IMS LI Optional
	8.1.0
	8.2.0
	LI8

	2007-12
	SP-38
	SP-070788
	0067
	-
	C
	MBMS IRI Registration
	8.1.0
	8.2.0
	LI8

	2008-03
	SP-39
	SP-080172
	0068
	1
	D
	CR on P-CSCF IMS LI Optional
	8.2.0
	8.3.0
	LI8

	2008-03
	SP-39
	SP-080172
	0069
	1
	D
	Removing "P" suffix from references
	8.2.0
	8.3.0
	LI8

	2008-03
	SP-39
	SP-080172
	0070
	1
	B
	Changes for Interception of IRI and CC at a WAG
	8.2.0
	8.3.0
	LI8

	2008-06
	SP-40
	SP-080262
	0071
	-
	F
	CSCF SIP Event reporting
	8.3.0
	8.4.0
	LI8

	2008-06
	SP-40
	SP-080262
	0072
	-
	B
	Conference Event Reporting
	8.3.0
	8.4.0
	LI8

	2008-06
	SP-40
	SP-080262
	0073
	-
	D
	Editorial corrections
	8.3.0
	8.4.0
	LI8

	2008-09
	SP-41
	SP-080514
	0074
	-
	B
	Updates to TS 33.107 to support LI for EPSs
	8.4.0
	8.5.0
	LI8

	2008-12
	SP-42
	SP-080762
	0077
	-
	F
	Editorial corrections to 33.107
	8.5.0
	8.6.0
	LI8

	2008-12
	SP-42
	SP-080762
	0075
	-
	F
	Corrections and clarifications of LI for EPS and alignment with latest version of SAE stage 2 specs.
	8.5.0
	8.6.0
	LI8

	2008-12
	SP-42
	SP-080762
	0076
	-
	F
	Clarification on 3G DT with the GGSN
	8.5.0
	8.6.0
	LI8

	2009-03
	SP-43
	SP-090133
	0078
	-
	F
	Alignment with SAE stage 2 specifications approved by TSG SA#42
	8.6.0
	8.7.0
	LI8

	2009-04
	
	
	
	
	
	Editorial correction to cover page
	8.7.0
	8.7.1
	

	2009-06
	SP-44
	SP-090272
	0079
	-
	F
	Correction on UE requested bearer resource modification - Alignment with SAE stage 2 specification
	8.7.1
	8.8.0
	LI8

	2009-06
	SP-44
	SP-090272
	0080
	-
	F
	Clarification on parameter APN
	8.7.1
	8.8.0
	LI8

	2009-06
	SP-44
	SP-090272
	0081
	-
	F
	Clarification on the handover between 2G/3G access and E-UTRAN with Gn/Gp
	8.7.1
	8.8.0
	LI8

	2009-06
	SP-44
	SP-090272
	0082
	-
	F
	Clarification on parameter PDN type
	8.7.1
	8.8.0
	LI8

	2009-09
	SP-45
	SP-090522
	0083
	-
	F
	Correction on identities and parameters for LI in case of E-UTRAN access and PMIP based S5/S8
	8.8.0
	8.9.0
	LI8

	2009-09
	SP-45
	SP-090522
	0084
	-
	F
	Correction on Serving Evolved Packet System event
	8.8.0
	8.9.0
	LI8

	2009-10
	--
	--
	--
	--
	--
	Correction of misimplementation of CR0084
	8.9.0
	8.9.1
	

	2009-12
	SP-46
	SP-090817
	86
	-
	F
	Correction on events names
	8.9.1
	8.10.0
	LI8

	2009-12
	SP-46
	SP-090817
	87
	-
	F
	Restoring section header 9.4.5
	8.9.1
	8.10.0
	LI8

	2009-12
	SP-46
	SP-090817
	88
	-
	F
	Correction on PDP context modification event
	8.9.1
	8.10.0
	LI8

	2009-12
	SP-46
	SP-090817
	85
	-
	F
	Correction on LI correlation for S4-SGSN
	8.9.1
	8.10.0
	LI8

	2009-12
	-
	-
	-
	-
	-
	Update to Rel-9 version (MCC)
	8.10.0
	9.0.0
	-

	2010-06
	SP-48
	SP-100364
	89
	-
	F
	Correction in IMS Conference text
	9.0.0
	9.1.0
	LI9

	2010-06
	SP-48
	SP-100253
	90
	-
	F
	Reporting of Dual Stack PDP address from the SGSN
	9.1.0
	10.0.0
	LI10

	2010-10
	SP-49
	SP-100570
	92
	-
	A
	Correction in IMS Conference Service X2 interface
	10.0.0
	10.1.0
	TEI9

	2010-10
	SP-49
	SP-100570
	93
	-
	A
	IMS Conference Service configuration for CC interception
	10.0.0
	10.1.0
	TEI9

	2010-10
	SP-49
	SP-100481
	91
	-
	F
	Unsuccessful bearer modificatio
	10.0.0
	10.1.0
	LI10

	2010-10
	SP-49
	SP-100481
	94
	-
	B
	LI architecture and functions for KMS based IMS Media Security
	10.0.0
	10.1.0
	LI10

	2010-10
	--
	--
	--
	--
	--
	ToC update
	10.1.0
	10.1.1
	--

	2010-12
	SP-50
	SP-100845
	97
	1
	A
	IMSI based activation
	10.1.1
	10.2.0
	LI10

	2010-12
	SP-50
	SP-100865
	98
	1
	B
	MME start of interception with bearer active
	10.1.1
	10.2.0
	LI8

	2010-12
	SP-50
	SP-100865
	103
	1
	C
	Corrections and Alignment for IMS Conferencing
	10.1.1
	10.2.0
	LI8

	2011-03
	SP-51
	SP-110023
	104
	-
	B
	Location information from trusted non-3GPP access
	10.2.0
	10.3.0
	LI10

	2011-03
	SP-51
	SP-110023
	106
	-
	C
	Security requirements for LI in KMS based IMS media security
	10.2.0
	10.3.0
	LI10

	2011-03
	SP-51
	SP-110023
	111
	-
	F
	Initiator parameter definition
	10.2.0
	10.3.0
	LI10

	2011-03
	SP-51
	SP-110021
	109
	-
	A
	IMS Conf LI 33.107
	10.2.0
	10.3.0
	LI8

	2011-06
	SP-52
	SP-110260
	112
	-
	C
	TLS and IPsec profiling for Xk interface
	10.3.0
	10.4.0
	LI10

	2011-09
	SP-53
	SP-110511
	113
	-
	B
	Reporting of PMIP and DSMIP session modification
	10.4.0
	11.0.0
	LI11

	2012-03
	SP-55
	SP-12-0034
	114
	
	F
	Correction on MIP specific parameters provided over the X2 interface.
	11.0.0
	11.1.0
	LI11

	2012-06
	SP-56
	SP-120336
	114a
	1
	C
	IMSI in untrusted non-3GPP access
	11.1.0
	11.2.0
	LI11

	2012-06
	SP-56
	SP-120336
	115
	1
	C
	SGs received location transfer over the X2 interface
	11.1.0
	11.2.0
	LI11

	2012-06
	SP-56
	SP-120336
	116
	2
	F
	UE IP Address at X2 interface
	11.1.0
	11.2.0
	LI11

	2012-06
	SP-56
	SP-120336
	117
	1
	F
	Handover indication at X2 interface
	11.1.0
	11.2.0
	LI11

	2012-06
	SP-56
	SP-120336
	118
	1
	F
	IMS Conference Services
	11.1.0
	11.2.0
	LI11

	2012-06
	SP-56
	SP-120336
	119
	1
	F
	IMS Provison of CC
	11.1.0
	11.2.0
	LI11

	2012-09
	SP-57
	SP-120627
	120
	1
	F
	Reference list correction to align with the corrected TS 29.212 title
	11.2.0
	11.3.0
	TEI11

	2012-09
	SP-57
	SP-120600
	121
	-
	B
	H(e)NB Support in TS 33.107
	11.3.0
	12.0.0
	LI12

	2012-12
	SP-58
	SP-120854
	122
	-
	B
	LI events for trusted non-3GPP access on GTP S2a, Rel12
	12.0.0
	12.1.0
	LI12

	2012-12
	SP-58
	SP-120854
	123
	-
	F
	Removal of LTE
	12.0.0
	12.1.0
	LI12

	2013-03
	SP-59
	SP-130034
	124
	-
	B
	Start of interception for an already established IMS media secured session
	12.1.0
	12.2.0
	LI12

	
	
	
	125
	-
	F
	Correcting 33.107 and 33.108 differences - TFT is applicable only to dedicated bearer
	
	
	

	2013-06
	SP-60
	SP-130248
	126
	-
	C
	Provision on Unencrypted CC
	12.2.0
	12.3.0
	LI12

	
	
	
	127
	-
	B
	Mid Session Interception for IMS
	
	
	

	2013-09
	SP-61
	SP-130401
	128
	-
	B
	Addition of LI to GBA
	12.3.0
	12.4.0
	LI12

	
	
	
	129
	-
	F
	Adding version to non 3GPP references
	
	
	

	
	
	
	130
	-
	F
	Updating Tel URL to Tel URI
	
	
	

	2013-12
	SP-62
	SP-130661
	131
	-
	B
	ULI timestamp reporting
	12.4.0
	12.5.0
	LI12

	
	
	
	132
	-
	D
	Editorial Correction on header
	
	
	

	
	
	
	133
	-
	B
	107 UMTS IRI Packet Header Information Reporting
	
	
	

	
	
	
	134
	-
	B
	107 WLAN IRI Packet Header Information Reporting
	
	
	

	
	
	
	135
	-
	B
	107 LTE IRI Packet Header Information Reporting
	
	
	

	
	
	
	136
	-
	F
	Correction to I-WLAN LI location information reporting
	
	
	

	
	
	
	137
	-
	D
	Editorial Fix of implementation of SA3LI13_073r3
	
	
	

	2014-03
	SP-63
	SP-140020
	138
	-
	C
	Handling of unsuccessful LI procedures in getting encryption keys from the KMS
	12.5.0
	12.6.0
	LI12

	
	
	
	139
	-
	B
	Basic architecture  to deliver location information based Civic Addresses
	
	
	

	
	
	
	140
	-
	F
	Clarification to EPS Interception for Trusted Non-3GPP IP Access
	
	
	

	
	
	
	141
	-
	B
	Addition of VoIP LI Functions
	
	
	

	2014-06
	SP-64
	SP-140310
	142
	-
	F
	Editorial clean-up of target & monitored subscriber
	12.6.0
	12.7.0
	LI12

	
	
	
	143
	-
	F
	Editorial clean-up of incorrect paragraph numbering
	
	
	

	
	
	
	144
	-
	A
	IMS IMEI Interception
	
	
	

	
	
	
	145
	-
	B
	LI for GCSE Group Communications
	
	
	

	
	
	
	147
	-
	C
	IMS-Based VoIP LI Enhancements
	
	
	

	
	
	
	148
	-
	B
	Adding the interception of ProSe Direct Discovery
	
	
	

	
	
	
	149
	-
	B
	IMS-based VoIP Stage 2 Call Flows
	
	
	

	2014-09
	SP-65
	SP-140586
	150
	-
	B
	LI functionalities for GTP based s2b interfaces
	12.7.0
	12.8.0
	LI12

	
	
	
	151
	-
	D
	Editorial Corrections
	
	
	

	
	
	
	152
	-
	F
	Clarification on interception of VoIP CC for targeted calls
	
	
	

	
	
	
	153
	-
	C
	Change of 'Decryption for IMS Media Plane Security' clause: making it more specific regarding E2E/E2AE modes
	
	
	

	
	
	
	154
	-
	F
	Intercept trigger and X3 alignment for IMS based VOIP stage 2
	
	
	

	
	
	
	155
	-
	A
	IMEI-based LI stage 2 description
	
	
	

	
	
	
	157
	-
	C
	Clarifications to IMS Interception relative to CC
	
	
	

	
	
	
	158
	-
	D
	Hanging paragraph repair
	
	
	

	2014-12
	SP-66
	SP-140821
	159
	-
	B
	Adding the interception feature of any XCAP usages
	12.8.0
	12.9.0
	

	
	
	
	160
	-
	F
	Editorial Correction to Stage 2 call flows (VoIP)
	
	
	

	
	
	
	161
	-
	C
	Group Communications LI with GSC AS outside the intercepting CSP’s network
	
	
	

	
	
	
	162
	-
	B
	Adding the Mask parameter to the interception of ProSe direct discovery
	
	
	

	
	
	
	163
	-
	C
	Alignment of LI for GCSE with HI2 & HI3
	
	
	

	
	
	
	164
	-
	B
	LI for ProSe One to Many Communications – In Network Coverage
	
	
	

	2015-03
	SP-67
	SP-150075
	165
	-
	F
	Correction to referenced in-house clauses
	12.9.0
	12.10.0
	LI12

	
	
	
	166
	-
	F
	Adding a minor note on how to understand the architecture that support the annex M of TS 33.108 on Hi1 over Hi2
	
	
	

	
	
	
	167
	-
	F
	Clarification on X interfaces
	
	
	

	
	
	
	168
	-
	F
	Adding logical function information
	
	
	

	
	
	
	169
	-
	F
	Uniform use of “target”
	
	
	

	
	
	
	170
	-
	B
	Adding the details of ProSe one-to-many communication interception
	
	
	

	
	
	
	171
	-
	B
	WebRTC Interworking LI Stage 2
	
	
	


 





intercepted
























































































































































MME





HI2





function 2





 





delivery





 





LEA





 





center





 





service





 





SMS





Target





HI1





X1_1





X1_2





X2





ADMFuuu   





LEAs  


 





Other�party





Function 





Mediation uuu 





 





SubscriberTarget





 








 





 





 





LEMF





3G  





Function 2





Delivery





DF2





HI3  





Function 3





HI2    





S-CSCF





GSN  





CSCF





-





P





Delivery





X3  





X2  





ADMF





 





 





 





 





3G GSN








packets





of





Duplicator





LEA





Function 3





Delivery





party





other





subscriberTarget





intercepted





3G GSN
































































































































 





LEAs





 





HI3





 





HI2





 





Function 3   





 





Delivery





 





X3 





 





X2





 





 





3G ICE





 





Function 2





 





Delivery





 





  





 





LEAs





 





HI3





 





HI2





 





Function 3





 





Delivery





 





X3





 





X2





 





 





3G ICE





 





Function 2





 





Delivery





 





  





 





packets





 





of





 





Duplicator





 





LEA





 





Function 3





 





Delivery





 





party





 





other





 





subscribeTargetr





 





intercepted





 





PDG





 





 





 





 





 





 





SC





-





BM





 





 





 





LEMF





 





 





 





Function 2   





 





 





 





Delivery





 





 





 





 





 





 





 





subscriberTarget





 





intercepted





 





 





 
































…
















































































 





 





 





Party





 





 





 





Other





 





 





 





LEMF











S-GW, PDN-GW








SubjectTarget














intercepted








 





 





 





DF2





 





 





 





Party





 





 





subscriberTarget





 





Other








 








AS/MRFC














Function 2 





Delivery 





Function 





Mediation 





LEMF





other





party





Delivery





Function 3





LEA





Duplicator





of





intercepted





S-GW, PDN-GW





packets





of





Duplicator





LEA





Function 3





Delivery





party





other





subscriberTarget





intercepted





PDN-GW





packets




















packets





of





Duplicator





LEA





Function 3





Delivery





party





other





subscriberTarget











 



































 





 





 





 





 





 





 





 





 





 





 





LEMF





 





Mediation





 





Function





 





Delivery





 





Function 2





 





Mediation





 





Function





 





ADMF





 





GCSE AS 





 





X2





 





X3





 





X1_2





 





X1_1





 





X1_3





 





HI1





 





HI2





 





Mediation





 





Function





 





Delivery





 





Function 3





 





HI3





 





 





 





 








 





subscriberTarget





 





 





 





 





 





 





 





Delivery





 





 





 





Function 3   





 





 





 





LEMF





 





 





 





GCSE AS





 





 





 





 





 





 





 





 





intercepted





 





subscriber





 





 





 





 





 





 





 





Delivery





 





 





 





Function 2   





 





 





 





LEMF





 





 





 





GCSE   AS








 





 





 





 





 
























































_1453185376.vsd
PDN-GW/GGSN


P-CSCF


S-CSCF/AS


Target’s UE


Voice Services


ADMF


Mediation
Function


Delivery Function 3


Mediation
Function


LEMF


Delivery Function 2


Mediation
Function


PCRF


HI1


HI2


HI3


X1_1


X1_2


X1_3


X2


X3


Called Party


SIP


SIP


Voice Media


Voice Media


SIP



_1459807884.vsd
Party_C


IP-CAN


S-CSCF/P-CSCF


S-CSCF


AS


Party_A


DF3


LEA


Target Domain


MF/DF


D20


DF2


INVITE Party_C
[History Info: Party_B]
[SDP]


180 Ringing 


180 Ringing


{180 Ringing}


{Call Is Being Forwarded}


P-CSCF


Party_B
Target


IP-CAN


INVITE Party_C
[History Info: Party_B]
[SDP]


INVITE Party_C
[History Info: Party_B]
[SDP]


{INVITE Party-C}


181 Call Is Being Forwarded


181 Call Is Being Forwarded


180 Ringing


180 Ringing 


180 Ringing


Media 


Media 


AAA/RAA


AAR/RAR


AAA/RAA


AAR/RAR


Forward-to-Party Domain


CC


{200 OK}


{ACK}


ACK 


ACK  


ACK  


200 OK
[SDP]


200 OK
[SDP]


200 OK
[SDP]


200 OK
[SDP]


200 OK
[SDP]


ACK


ACK


Call Forwarding No Answer Timer Expires



_1483436658.vsd
ProSe Function or ProSe Key Management Function 



_1487501983.doc


LEMF







Delivery



Function 2







Target







other



party











3G GSN







HLR












_1487503027.doc


Other



party







Delivery



Function 3







LEMF







Target







Other



party







MRFP







Duplicator of



packets












_1487504109.doc


SeGW







SGSN (PS)



MSC/VLR (CS)







HNB



GW







Target







HNB







Other Party







DF 2







LEMF







HNB location



verifying node












_1487504735.doc


Duplicator of Media Packets







Delivery Function 3







LEMF







Target







Other Party







CC Intercept



Function







CC Interception



Triggering



Function







X3







HI3












_1487505106.doc


Delivery



Function 2







LEMF







Other



subscriber







Target







HPLMN-I



ProSe



Function







HPLMN-O



ProSe



Function












_1487504209.doc


LEMF







Delivery



Function 2







Target











BSF












_1487503577.doc


Duplication of packets







Delivery



Function 3







LEA







Target







other



party







PDN-GW, ePDG












_1487502319.doc


DF2







LEMF







Target







Other



Party











PDG/WAG







AAA Server












_1487494062.doc


LEMF







Delivery



Function 2







Target







SMS



service



center







3G MSC












_1487501644.doc


LEMF







Delivery



Function 2







Target







Other



party







3G MSC












_1487493790.doc


Two stublines







Target







MGW







other party







Delivery Function 3







LEMF












_1483441339.vsd
ProSe UE A


ProSe UE B


ProSe UE C


ProSe Function or PKMF


HI2


Signalling


Communication


DF2


LEMF


X2 interface


Target



_1460438479.doc


ProSe



Function







X1_1







X2







Function 2







Delivery







Function







Mediation







HI2







HI1







X1_2







ADMF







Function







Mediation







LEMF












_1466495659.vsd
X1_2


LEMF


Mediation Function


Delivery Function 2


Mediation Function


Delivery Function 3


Mediation Function


ADMF


S-GW,
PDN-GW,
ePDG


X3


X2


X1_3


X1_1


HI1


HI2


HI3



_1479889717.doc

[image: image2.emf]



 




LEMF 




Mediation 
Function 




Delivery 
Function 2 




Mediation 
Function 




ADMF 




GSN 




X2 




X3 




X1_2 




X1_1 




X1_3 




HI1 




HI2 




Mediation 
Function 




Delivery 
Function 3 




HI3 
















eNB











[image: image1]
_1479889797.doc

[image: image1]

ProSe UE A







ProSe UE B







ProSe UE C







eNB







DF 2







LEMF







Target 







Target



Group ID:



  abcdef







    X 



Interface












_1466495444.vsd
other party


intercepted subscriber


PDN-GW, ePDG


Delivery Function 3


LEA


Duplicator of packets



_1460464110.vsd
Delivery Function 2


SIP Signaling Nodes
IRI ICE


Delivery Function 3


Media Nodes (CC Intercept Function)
CC ICE


LEMF


X2


X3


HI2


HI3


SIP Signaling Nodes
(CC Interception Trigger)



_1459809498.vsd
Party_A
[Target]


TrGW


IBCF


S-CSCF


AS


Party_B


DF2


LEA


Home CSP [Target Domain]


MF/DF


Add Context


Add Context Ack


Media 


Media 


CC


D1


INVITE Party_B
[SDP]


200 OK
[SDP]


Modify Context


Modify Context Ack


D10


ACK


DF2


INVITE Party_B
[SDP]


INVITE Party_B
[SDP]


INVITE Party_B
[SDP]


INVITE Party_B
[SDP]


200 OK
[SDP]


200 OK
[SDP]


200 OK
[SDP]


200 OK
[SDP]


{200 OK}


{ACK}


ACK


ACK


ACK


ACK


180 Ringing


180 Ringing


180 Ringing


180 Ringing


180 Ringing


{180 Ringing}


{INVITE Party-B}


Visited CSP 



_1459810137.vsd
Party_A
[Target]


IP-CAN


P-CSCF


S-CSCF


AS


Party_B


DF3


LEMF


Visited CSP


MF/DF


DF2


IBCF


IBCF


TRGW


TRGW


Home CSP


AAR/RAR


AAA/RAA


Media 


Media 


CC


D1


200 OK [SDP]


200 OK [SDP]


200 OK [SDP]


200 OK [SDP]


{200 OK}


D10


ACK


INVITE Party_B
[SDP]


INVITE Party_B
[SDP]


200 OK [SDP]


200 OK [SDP]


200 OK [SDP]


{INVITE SP.NO.}


{ACK}


ACK


ACK


ACK


ACK


180 Ringing


180 Ringing


180 Ringing


180 Ringing


180 Ringing


{180 Ringing}


INVITE SP.NO.
[SDP]


INVITE SP.NO.
[SDP]


180 Ringing


180 Ringing


INVITE SP.NO.
[SDP]


INVITE SP.NO.
[SDP]


INVITE SP.NO.
[SDP]


Media 


Media 


ACK


ACK


AAR/RAR


AAA/RAA


Add Context


Add Context Ack


Add Context


Add Context Ack


Modify Context


Modify Context Ack


Modify Context Ack


Modify Context



_1459810388.vsd
Party_A


IP-CAN


S-CSCF


AS


Party_B
[Target]


DF3


LEMF


Visited CSP


MF/DF


DF2


IBCF


IBCF


TRGW


TRGW


Home CSP


P-CSCF


D1


D10


INVITE Party_B
[SDP]


180 Ringing


180 Ringing


{180 Ringing}


INVITE Party_B
[SDP]


180 Ringing


180 Ringing


{INVITE Party-B}


INVITE Party_B
[SDP]


INVITE Party_B
[SDP]


INVITE Party_B
[SDP]


INVITE Party_B
[SDP]


INVITE Party_B
[SDP]


180 Ringing


180 Ringing


180 Ringing


Add Context Ack


Add Context


Add Context


Add Context Ack


SAA/RAA


SAR/RAR


487 Terminated


487 Terminated


{CANCEL}


487 Terminated


487 Terminated


Delete Context


487 Terminated


Delete Context Ack


AAR/RAR


AAA/RAA


Delete Context Ack


Delete Context


ACK


ACK


ACK


ACK


ACK


181 Call Is Being Forwarded


181 Call Is Being Forwarded


Call Forwarding occurs (steps not shown) as those messages are not intercepted. CC is not intercepted. 


{487 Terminated}


{ACK}


CFNR Timer Expires


CANCEL


CANCEL


CANCEL


CANCEL


CANCEL



_1460368905.vsd
Intercepted subscriber


Other subscriber


HPLMN-I ProSe Function


HPLMN-O ProSe Function


Delivery Function 2


LEMF



_1459810285.vsd
Party_A


IP-CAN


S-CSCF


AS


Party_B
[Target]


DF3


LEMF


Visited CSP


MF/DF


DF2


IBCF


IBCF


TRGW


TRGW


Home CSP


P-CSCF


Add Context Ack


Add Context


D1


Add Context


D10


200 OK [SDP]


200 OK [SDP]


{200 OK}


200 OK [SDP]


Add Context Ack


200 OK [SDP]


INVITE Party_B
[SDP]


200 OK [SDP]


200 OK [SDP]


180 Ringing


180 Ringing


{180 Ringing}


INVITE Party_B
[SDP]


180 Ringing


180 Ringing


200 OK [SDP]


AAR/RAR


{INVITE Party-B}


180 Ringing


180 Ringing


AAA/RAA


AAR/RAR


180 Ringing


AAA/RAA


Modify Context


Modify Context Ack


Modify Context Ack


Modify Context


Media 


INVITE Party_B
[SDP]


INVITE Party_B
[SDP]


INVITE Party_B
[SDP]


INVITE Party_B
[SDP]


INVITE Party_B
[SDP]


Media 


CC


ACK


ACK


ACK


{ACK}


ACK


ACK


Media 


Media 


ACK


ACK



_1459809524.vsd
INVITE Party_B
[SDP]


TrGW


IBCF


S-CSCF


AS


Party_A


DF3


LEA


Home CSP [Target Domain]


MF/DF


Media 


Media 


D1


D10


DF2


CC


INVITE Party_B
[SDP]


200 OK
[SDP]


INVITE Party_B
[SDP]


200 OK
[SDP]


{200 OK}


{ACK}


ACK


ACK


180 Ringing


180 Ringing


{180 Ringing}


INVITE Party_B
[SDP]


180 Ringing


180 Ringing


180 Ringing


200 OK
[SDP]


200 OK
[SDP]


200 OK
[SDP]


ACK


ACK


ACK


{INVITE Party-B}


INVITE Party_B
[SDP]


Visited CSP 


Add Context


Add Context Ack


Modify Context


Modify Context Ack


Party_B
Target



_1459808034.vsd
Party_C


TrGW/IM-MGW


IBCF/MGCF


S-CSCF


AS


Party_A


DF3


LEA


Target Domain


MF/DF


Add Context


Media 


Media 


D1


Add Context Ack


D10


Modify Context


Modify Context Ack


DF2


CC


INVITE Party_B
[SDP]


200 OK
[SDP]


INVITE Party_C
[History Info: Party_B]
[SDP]


200 OK
[SDP]


{200 OK}


{ACK}


ACK


ACK


180 Ringing


180 Ringing


{180 Ringing}


INVITE Party_B
[SDP]


180 Ringing


180 Ringing


180 Ringing


200 OK
[SDP]


200 OK
[SDP]


200 OK
[SDP]


ACK


ACK


ACK


{INVITE Party-B}


P-CSCF


Party_B
Target


INVITE Party_C
[History Info: Party_B]
[SDP]


INVITE Party_C
[History Info: Party_B]
[SDP]


{INVITE Party-C}


IP-CAN/IMS-AGW


181 Call Is Being Forwarded


181 Call Is Being Forwarded


{Call Is Being Forwarded}


Call Forwarding Unconditional



_1459334557.vsd
LEMF


LEMF


LEMF


ADMF


Mediation
Function


Delivery Function 3


Mediation
Function


HI3


X1_3


X3


CC Intercept Function


CC Interception Triggering Function


X1_1


HI1



_1459805400.vsd
Party_A
[Target]


IP-CAN


P-CSCF


S-CSCF


AS


Party_B


DF3


LEA


Target Domain


MF/DF


AAR/RAR


AAA/RAA


Media 


Media 


CC


D1


INVITE SP.NO.
[SDP]


200 OK
[SDP]


AAR/RAR


AAA/RAA


D10


ACK


DF2


INVITE SP.NO.
[SDP]


INVITE SP.NO.
[SDP]


INVITE Party_B
[SDP]


INVITE Party_B
[SDP]


200 OK
[SDP]


200 OK
[SDP]


200 OK
[SDP]


200 OK
[SDP]


{INVITE SP.NO.}


{200 OK}


{ACK}


ACK


ACK


ACK


ACK


180 Ringing


180 Ringing


180 Ringing


180 Ringing


180 Ringing


{180 Ringing}


{INVITE Party-B}



_1459806875.vsd
Party_C


IP-CAN


S-CSCF/P-CSCF


S-CSCF


AS


Party_A


DF3


LEA


Target Domain


MF/DF


AAA/RAA


Media 


Media 


D1


AAR/RAR


D10


AAA/RAA


AAR/RAR


DF2


CC


INVITE Party_B
[SDP]


200 OK
[SDP]


INVITE Party_C
[History Info: Party_B]
[SDP]


200 OK
[SDP]


{200 OK}


{ACK}


ACK


ACK


180 Ringing


180 Ringing


{180 Ringing}


INVITE Party_B
[SDP]


INVITE Party_C
[History Info: Party_B]
[SDP]


INVITE Party_C
[History Info: Party_B]
[SDP]


180 Ringing


180 Ringing


180 Ringing


200 OK
[SDP]


200 OK
[SDP]


200 OK
[SDP]


ACK


ACK


ACK


{INVITE Party-B}


P-CSCF


Party_B
Target


IP-CAN


{INVITE Party-C}


181 Call Is Being Forwarded


181 Call Is Being Forwarded


{Call Is Being Forwarded}


Call Forwarding Unconditional


Forwarded-to-Party’s Domain



_1459807710.vsd
Party_B
Target


IP-CAN


P-CSCF


S-CSCF


AS


Party_A


DF3


LEA


Target Domain


MF/DF


AAA/RAA


D1


AAR/RAR


D10


DF2


INVITE Party_B
[SDP]


INVITE Party_B
[SDP]


180 Ringing


{180 Ringing}


INVITE Party_B
[SDP]


INVITE Party_B
[SDP]


INVITE Party_B
[SDP]


{INVITE Party-B}


{CANCEL}


487 Terminated


180 Ringing


180 Ringing


180 Ringing


180 Ringing


SAA/RAA


SAR/RAR


CANCEL


CANCEL


CANCEL


487 Terminated


487 Terminated


ACK


ACK


ACK


{487 TERMINATED}


{ACK}


Call Forwarding No Answer Timer Expires


The flow continues…...



_1459806656.vsd
Party_B
Target


IP-CAN


P-CSCF


S-CSCF


AS


Party_A


DF3


LEA


Target Domain


MF/DF


AAA/RAA


Media 


Media 


D1


DF2


CC


INVITE Party_B
[SDP]


200 OK
[SDP]


INVITE Party_B
[SDP]


200 OK
[SDP]


{200 OK}


{ACK}


ACK


ACK


180 Ringing


180 Ringing


{180 Ringing}


AAR/RAR


INVITE Party_B
[SDP]


INVITE Party_B
[SDP]


D10


INVITE Party_B
[SDP]


180 Ringing


180 Ringing


180 Ringing


200 OK
[SDP]


200 OK
[SDP]


200 OK
[SDP]


ACK


ACK


ACK


{INVITE Party-B}


AAA/RAA


AAR/RAR



_1459667991.vsd
S-CSCF


IMS-AGW


P-CSCF


Target’s UE


ADMF


Mediation
Function


Delivery Function 3


Mediation
Function


LEMF


SIP


Voice Media


HI1


HI2


HI3


X1_1


X1_2


X1_3


X2


X3


Voice Services


Called Party


SIP


SIP


Voice Media


Voice Media


SIP


MRF


AS


Delivery Function 2


Mediation
Function



_1459805305.vsd
Party_A
[Target]


Home CSP [Target Domain]


180 Ringing


MF/DF


IP-CAN


P-CSCF


S-CSCF


AS


Party_B


DF3


LEA


AAR/RAR


AAA/RAA


Media 


Media 


CC


D1


INVITE Party_B
[SDP]


INVITE Party_B
[SDP]


INVITE Party_B
[SDP]


INVITE Party_B
[SDP]


200 OK
[SDP]


INVITE Party_B
[SDP]


200 OK
[SDP]


200 OK
[SDP]


200 OK
[SDP]


200 OK
[SDP]


{INVITE Party-B}


AAR/RAR


AAA/RAA


{200 OK}


{ACK}


ACK


ACK


ACK


ACK


180 Ringing


ACK


180 Ringing


180 Ringing


180 Ringing


{180 Ringing}


D10


DF2



_1459667918.vsd
S-CSCF


IMS-AGW


P-CSCF


Target’s UE


ADMF


Mediation
Function


Delivery Function 3


Mediation
Function


LEMF


SIP


Voice Media


HI1


HI2


HI3


X1_1


X1_2


X1_3


X2


X3


Voice Services


Calling Party


SIP


SIP


Voice Media


Voice Media


SIP


MRF


AS


Delivery Function 2


Mediation
Function



_1459667753.vsd
S-CSCF


IMS-AGW


P-CSCF


Forward -to-Party’s UE


ADMF


Mediation
Function


Delivery Function 3


Mediation
Function


LEMF


AS


SIP


Voice Media


HI1


HI2


HI3


X1_1


X1_2


X1_3


X2


X3


Voice Media


Voice Services


Calling Party


SIP


SIP


Voice Media


SIP


MRF


AS


Delivery Function 2


Mediation
Function


SIP


SIP


S-CSCF



_1453185471.vsd
PDN-GW/GGSN


P-CSCF


S-CSCF/AS


PCRF


ADMF


Mediation
Function


Delivery Function 2


Mediation
Function


S-CSCF/AS


Target’s UE


HI1


P-CSCF, PCRF, PDN-GW/GGSN, associated with the Target’s UE, not used in this interception


HI2


PCRF


X1_1


X1_3


X1_2


X2


SIP


Voice Media


SIP


Voice Services


LEMF


PDN-GW/GGSN


P-CSCF


Forward-to-Party’s UE


Delivery Function 3


Mediation
Function


HI3


X3


Calling Party


Voice Media


SIP


SIP



_1453186548.vsd
PDN-GW/GGSN


P-CSCF


S-CSCF/AS


Target’s UE


ADMF


Mediation
Function


Delivery Function 2


Mediation
Function


HI1


HI2


X1_1


X1_3


X1_2


X2


LEMF


IBCF


Voice Services (another IMS CSP)


Delivery Function 3


Mediation
Function


HI3


X3


Voice Services


TrGW


PCRF


SIP (Ici)


IMS-AGW


Calling Party


Forwarded-to-Party


P-CSCF, PCRF, PDN-GW/GGSN or IMS-AGW associated with the Target’s UE, not used in this interception


Voice Media


SIP


SIP


Voice Media (Izi)



_1453186567.vsd
S-CSCF/AS


ADMF


Mediation
Function


Delivery Function 2


Mediation
Function


HI1


HI2


X1_1


X1_3


X1_2


X2


LEMF


MGCF


Delivery Function 3


Mediation
Function


HI3


X3


Voice Services


IM-MGW


Signaling (CS)


Voice Services
 (CS domain)


Forwarded-to-Party


Calling Party


PDN-GW/GGSN


P-CSCF


Target’s UE


PCRF


IMS-AGW


P-CSCF, PCRF, PDN-GW/GGSN or IMS-AGW associated with the Target’s UE, not used in this interception


Voice Media


SIP


SIP


Voice Media (CS)



_1453186507.vsd
S-CSCF/AS


ADMF


Mediation
Function


Delivery Function 2


Mediation
Function


HI1


HI2


X1_1


X1_3


X1_2


X2


LEMF


IBCF


Delivery Function 3


Mediation
Function


HI3


X3


Voice Services


TrGW


Calling Party


Voice Media


SIP


SIP


PCRF


IMS-AGW


PDN-GW/GGSN


IMS Roaming


P-CSCF


SIP


Voice Media


IMS Roaming Target’s UE


SIP


Voice Media


Voice Media


Voice Media



_1453186520.vsd
P-CSCF


IMS Roaming Target’s UE


ADMF


Mediation
Function


Delivery Function 3


Mediation
Function


LEMF


Delivery Function 2


Mediation
Function


HI1


HI2


HI3


X1_1


X1_2


X1_3


X2


X3


PCRF


Voice Services


Called Party


SIP


SIP


Voice Media


Voice Media


SIP


S-CSCF/AS


IBCF


IMS-AGW


TrGW


PDN-GW/GGSN


IMS Roaming


SIP


Voice Media


Voice Media


Voice Media



_1453185488.vsd
S-CSCF/AS


ADMF


Mediation
Function


Delivery Function 2


Mediation
Function


HI1


HI2


X1_1


X1_3


X1_2


X2


LEMF


P-CSCF


Forward-to-Paty’s UE


Delivery Function 3


Mediation
Function


HI3


X3


Voice Services


IMS-AGW


S-CSCF/AS


IMS-AGW


P-CSCF


Target’s UE


P-CSCF/IMS-AGW associated with the Target’s UE, not used in this interception


Calling Party


SIP


Voice Media


Voice Media


SIP


SIP


SIP



_1453186478.vsd
S-CSCF/AS


ADMF


Mediation
Function


Delivery Function 2


Mediation
Function


HI1


HI2


X1_1


X1_3


X1_2


X2


LEMF


IBCF


Delivery Function 3


Mediation
Function


HI3


X3


Voice Services


TrGW


Calling Party


PDN-GW/GGSN


P-CSCF


Target’s UE


PCRF


IMS-AGW


P-CSCF, PCRF, PDN-GW/GGSN or IMS-AGW associated with the Target’s UE, not used in this interception


Voice Media


SIP


SIP


PCRF


IMS-AGW


PDN-GW/GGSN


IMS Roaming


P-CSCF


SIP


IMS Roaming Forwarded-to Party’s UE


SIP


Voice Media


Voice Media


Voice Media


S-CSCF/AS


SIP


Voice Media



_1453185436.vsd
PDN-GW/GGSN


P-CSCF


S-CSCF/AS


Target’s UE


Voice Services


ADMF


Mediation
Function


Delivery Function 2


Mediation
Function


LEMF


Delivery Function 3


Mediation
Function


HI1


HI3


HI2


X1_1


X1_3


X1_2


X2


X3


PCRF


Calling Party


SIP


Voice Media


SIP


SIP


Voice Media



_1453185456.vsd
IMS-AGW


P-CSCF


S-CSCF/AS


Target’s UE


ADMF


Mediation
Function


Delivery Function 2


Mediation
Function


Delivery Function 3


Mediation
Function


HI1


HI3


HI2


X1_1


X1_3


X1_2


X2


X3


Voice Services


LEMF


SIP


SIP


Calling Party


SIP


Voice Media


Voice Media



_1453185417.vsd
IMS-AGW


P-CSCF


S-CSCF/AS


Target’s UE


ADMF


Mediation
Function


Delivery Function 3


Mediation
Function


LEMF


Delivery Function 2


Mediation
Function


HI1


HI2


HI3


X1_1


X1_2


X1_3


X2


X3


Voice Services


Called Party


SIP


SIP


Voice Media


Voice Media


SIP



_1143528322.doc


LEMF







Mediation







Function







Delivery







Function 2







Mediation







Function







ADMF







P-CSCF/



S-CSCF







X2











X1_2







X1_1











HI1







HI2








































_1409062104.vsd

_1448280440.doc


LEMF







Mediation







Function







Delivery







Function 2







Mediation







Function







ADMF







BSF







X2











X1_2







X1_1











HI1







HI2








































_1453101930.vsd
Forwarded-to-Party
  


Calling Party


Delivery Function 3


LEMF


Duplicator of Media Packets


CC Intercept Function


CC Interception Triggering Function


 


X3


HI3



_1453103591.vsd
Intercepted Subscriber


Other Party


Delivery Function 3


LEMF


Duplicator of Media Packets


CC Intercept Function


CC Interception Triggering Function


X3


HI3



_1448280441.doc

[image: image1]

 







 







 







intercepted







 







subscriber







 







 







 







 







 







 







 







Delivery







 







 







 







Function 2   







 







 







 







LEMF







 







 







 







BSF







 







 







 







 







 












_1453093053.vsd
Rx


Gx


Ix/Mn


EPC + LTE


Non-LTE Access, 
interconnection to EPC


Non-3GPP Access, no   interconnection to EPC  


Iq


Access Network


Media Nodes


AS


3GPP Access, no
interconnection to EPC


GGSN


IMS-AGW


IMS-UE


PDN-GW


Gx


TrGW/
IM-MGW


Voice Services offered in another CSP’s network


PCRF


IMS-UE


IMS-UE


IMS-UE


I-BCF/
MGCF


P-CSCF


TrGW


Ix


S-CSCF


I-BCF


Signaling Nodes


IMS-UE


IMS Roaming



_1448275278.doc

[image: image1]

 







 







 







intercepted







 







subscriber







 







HLR







 







 







 







 







 







 







 







other







 







 







 







party







 







 







 







Delivery







 







 







 







Function 2 







 







 







   







LEMF







 







 







 







3G GSN







 







 







 







 







 












_1448276670.doc

[image: image1]

 







AAA Server







 







 







 







 







 







PDG/WAG







 







 







Other







 







 







 







Party







 







 







 







DF2







 







 







 







Intercept







 







 







 







Subject







 







 







 







LEMF







 







 







 












_1409121806.doc

[image: image1]

 











 







HNB location verifying node







 







 







HNB 







 







 







Targeted Subscriber 







 







 







Other Party 







 







SGSN (PS)



 







MSC/VLR (CS)







 







 







SeGW







 







 







HNB GW







 







 







DF 2







 







 







 







 







LEMF







 







 







 












_1409122688.doc

[image: image2.wmf]

S-GW (UP)



MME (CP)







AAA Server/HSS







HeMS







HeMS







HeNB GW







Operator’s core network







insecure link







SeGW







HeNB







UE











[image: image1]
_1409063160.doc

[image: image2.wmf]

Operator’s core network







MSC/VLR/ MGW (CS)







3G GSN (PS)







AAA Server/HSS







HMS







HMS







HNB GW







insecure link







SeGW







HNB







UE











[image: image1]
_1270539101.doc

[image: image1]

HI1







HI3







HI2







X3







X1_3







X1_2







X2































LEMF











		Delivery



		Function 3











		



		ADMF











		Delivery



		Function 2







Mediation



Function







Mediation



Function







Mediation



Function















MRFP















AS/MRFC







X1_1












_1341908728.doc

[image: image1]

LEMF







Mediation







Function







Delivery







Function 2







Mediation







Function







ADMF







P-CSCF/



S-CSCF







X2











X1_2







X1_1











HI1







HI2



















Xk







KMS




















_1343216190.doc

[image: image1]

party







MRFP







Intercepted







subscriber







Other







party







Delivery







Function 3







LEMF







Duplicator







of







packets







Other







…












_1288078025.doc
[image: image1.jpg]K oy







_1164520087.doc










MGW











Bearer Establishment







Establish Bearer/



Prepare Bearer Establishment











Release Bearer/Release Resource







Release Resource/Release Bearer 







release of stublines



































DF3 Signalling











A











MS B











Bearer Release







IAM







ACM







SETUP







Call Establishment Attempt







CPG(alert)







setup of stublines







ALERTING







CONNECT







ANM







Answer







DF3 Bearer







Release







REL







DISCONNECT







MSC Server







Prepare Bearer Establishment/



Establish Bearer







stublines contain CC of AB







DF2












_1164520209.doc


 







DF2







DF3







MS A







3G SGSN







Detach Request







MS initiated



PDP Context Deactivation



See B.8 when context is deactivated







Detach Accept







Mobile Station Detach












_1231579723.doc

[image: image1]

HI1







HI3







HI2







X3







X1_3







X1_2







X2































LEMF











		Delivery



		Function 3











		



		ADMF











		Delivery



		Function 2







Mediation



Function







Mediation



Function







Mediation



Function















PDG/WAG















AAA







X1_1












_1164520259.doc


Cancel Location Ack *







DF2







DF3







MS A







3G SGSN







Detach Request







Network initiated PDP



Context Deactivation



See B.9 when context is deactivated







Detach Accept







Mobile Station Detach Indication







HLR







MSC/VLR







Cancel Location *







Mobile Station Detach












_1164520183.doc


 







DF2







DF3







MS A







3G SGSN







Attach Request







Signalling







Attach Accept







Mobile Station Attach












_1159172872.doc


Delivery�Function 2















3G ICE







ADMF







Delivery�Function 3







X1_2







X1_3







X1_1







HI1












_1164520062.doc










MGW











Bearer Establishment







Establish Bearer/



Prepare Bearer Establishment











Release Bearer/Release Resource







Release Resource/Release Bearer 







release of stublines



































DF3 Signalling











B











MS A











Bearer Release







SETUP







IAM







ACM







Call Establishment Attempt







CPG(alert)







setup of stublines







ALERTING







CONNECT







ANM







Answer







DF3 Bearer







Release







REL







DISCONNECT







MSC Server







Prepare Bearer Establishment/



Establish Bearer







Stublines contain CC of AB







DF2












_1151828596.doc
... deactivation of delivery functions ...




response for lawful interception deactivation







request for lawful interception deactivation







lawful interception deactivation







lawful interception deactivation ack











           3G ICE







ADMF












_1151828615.doc
... interrogation for activation status on X1_1




response for lawful interception interrogation







request for lawful interception interrogation











lawful interception interrogation







        3G ICE







ADMF







lawful interception interrogation ack












_1151828510.doc
... activation of delivery functions ...




lawful interception activation







lawful interception activation ack







request for lawful interception activation







response for lawful interception activation







           



           3G ICE







ADMF












_1045473191.doc
... setup and interception of BD call as in figure A1 ...


... release of BD call and interception as in figure A1 ...




Call Establishment Attempt(AB)







ACM







IAM







Bearer Release







D







MGW







DF3 Signaling







C







MS B







DF3 Bearer







Answer(BD)







SETUP(BD)







CPG(alert)







CPG(alert)







ANM







IAM







Answer(AB)







ANM







REL







REL







Release(AB)







Prepare Bearer Establishment/



Establish Bearer







Stublines” contain CC of AC







stublines’ contain CC of BD







ACM







Suppl. Service(AB, CFB)







MSC Server







Release(BD)







REL







DISCONNECT(BD)







DF2







A







Release Bearer/Release Resource







Release Resource/ Release Bearer







release of stublines”







Bearer Establishment







Establish Bearer/



 Prepare Bearer Establishment Establishment/











setup of stublines”












_1055678712.doc


Set-up of Packet Data tunnel







DF2







DF3







MS A







new 3G SGSN







old 3G SGSN







3G SGSN







HLR







Routing Area Update Request







3G GSN Context Request







3G GSN Context Response







Security Functions







3G GSN Context Acknowledge







Release of Packet Data tunnel







Additional Signalling







Routing Area Update Accept







Routing Area Update Complete







RA Update












_1066654217.doc


 DF2







 DF3







 MS A







3G SGSN







SMSC







Forward Short Message







Message Transfer







SMS







Forward Short Message Result












_1143527712.doc


LEMF







Mediation







Function







Delivery







Function 2







Mediation







Function







ADMF







MGW







MSC Server/







GMSC Server







Mc







X2







X3







X1_2







X1_1







X1_3







HI1







HI2











Mediation







Function







Delivery







Function 3







Signaling







HI3











Mediation







Function







Delivery







Function 3







Bearer
















_1143528273.doc


LEMF







Mediation







Function







Delivery







Function 2







Mediation







Function







ADMF







HLR







X2











X1_2







X1_1











HI1







HI2








































_1143527579.doc


LEMF







Mediation







Function







Delivery







Function 2







Mediation







Function







ADMF







GSN







X2







X3







X1_2







X1_1







X1_3







HI1







HI2







Mediation







Function







Delivery







Function 3







HI3




















_1058276584.doc


 DF2P







 DF3P







MS A







3G SGSN







SMSC







Message Transfer



















SMS












_1058276724.doc
[image: image1.wmf] 


 


DF2P


 




�







 DF3P







 MS A







3G SGSN







SMSC







Forward Short Message











SMS
















_1055678976.doc


P-CSCF







HSS







I-CSCF















 Cx







-Pull















 Cx







-Pull







 Resp







Register







S-CSCF







UE







Visited Network







Home Network







 200 OK







200 OK







200 OK







 S-CSCF selection















 Cx







-put















 Cx







-put







 Resp







DF2







200 OK












_1055679516.doc


 INVITE







a. INVITE







Ringing







UE







P-CSCF







S-CSCF







a. SDP







  SDP







 Final SDP







a.  Ringing







a. 200 OK







 200 OK















 Reserv







 Success







ACK







SDP







INVITE







Ringing







200 OK







 Service Control







Visited Network







Home Network







Resource







Reservation







 Authorize







 QoS







 Resources







I-CSCF







(Firewall)







b1. INVITE







b2. INVITE







b1.  SDP







b2.  SDP







b1.  Ringing







b2.  Ringing







b1. 200 OK







b2. 200 OK







Service Control







Final SDP







Final SDP







 







Reserv







 Success







Success







 ACK







ACK















 Ringback







2Approval of







 QoS







 Commit







Start Media Flow







DF2







 INVITE







  SDP







Final  SDP







 







Reserv







 Success







Ringing







200 OK







ACK












_1055679665.doc


UE B







S-CSCF







UE A







Home Network







P-CSCF







GPRS







Visited Network















 Rls







 Response







SIP OK







GPRS







Visited Network







 







Hangup







P-CSCF







SIP OK







Release PDP







SIP OK







S-CSCF







Home Network















 Hangup















 Hangup







SIP OK







 Remove Resource 







Reservation







 Service Control







Service Control















 Hangup







 Remove Resource 







Reservation















 Hangup















 Rls







 Response







Release PDP







SIP OK







DF2







 







Hangup







SIP OK












_1055678870.doc


P-CSCF







HSS







I-CSCF







Register







 Register















 Cx







-Query







UE







Visited Network







Home Network















Cx







-Query







 Resp







Continuation of registration







 Serving 







Network Selection















 Cx







-Select-pull















 Cx







-Select-pull Resp











DF2







 Register












_1047237812.doc


 DF2







 DF3







MS A







 3G SGSN







 3G GGSN







Activate PDP Context Request







Security functions







Create PDP Context Request







Create PDP Context Response







Setup of Packet Data tunnel







PDP Context Activation







Activate PDP Context Accept







PDP Context Activation







Setup of Packet Data tunnel












_1047237934.doc


 DF2







 DF3







MS A







 3G SGSN







 Deactivate PDP context Request







 Deactivate PDP Context Accept







Release of Packet Data tunnel







 PDP Context Deactivation












_1047238096.doc


 DF2







 DF3







MS A







3G SGSN







SMSC







Message Transfer







Forward Short Message







Forward Short Message Result







Delivery Report







SMS












_1047237887.doc


 DF2







 DF3







MS A







 3G SGSN







 3G GGSN







Deactivate PDP Context Request







Security functions







Delete PDP Context Request







Delete PDP Context Response







Release of Packet Data tunnel







PDP Context Deactivation







Deactivate PDP Context Accept







PDP Context Deactivation







Release of Packet Data tunnel












_1047237667.doc


 







DF2







DF3







MS A







3G SGSN







Routing Area Update Request







Security functions







Routing Area Update Accept







RA update







Routing Area Update Complete












_1005705767.doc
... activation on X1_2 ...


... activation on X1_1...




DF 3







ADMF







response for lawful interception activation







request for lawful interception activation















lawful interception activation







lawful interception activation ack












_1045422120.doc
... release of AB call and interception as in figure A1 ...


... setup and interception of AB call as in figure A1 ...




RETRIEVE ACK(AB)







Bearer Establishment







SETUP(AB)







Establish Bearer/



Prepare Bearer Establishment











DF3 Signaling







B







MS A







DF3 Bearer







MSC Server







HOLD ACK(AB)







Release Bearer/Release Resource







Release Resource/ Release Bearer







setup of stublines”







release of stublines”







REL







C







Answer(AB)







DISCONNECT(AB)







Prepare Bearer Establishment/



Establish Bearer







Bearer Release







Release(AB)







ALERTING(CA)







DF2







IAM







ACM







SETUP(CA)







Call Establishment Attempt(CA, CWAIT)







CPG(alert)







HOLD(AB)







Suppl. Service(AB, CHOLD)







CONNECT(CA)







ANM







Answer(CA)







Suppl. Service(AB, CRETR)







Release(CA)







REL







DISCONNECT(CA)







RETRIEVE(AB)







Stublines’ contain CC of AB







stublines” contain CC of CA







stublines’ contain CC of AB







MGW












_1045422795.doc
... setup and interception of BD call as in figure A1 ...


... release of BD call and interception as in figure A1 ...




Call Establishment Attempt(AB, CWAIT)







ACM







IAM







Bearer Release







A







D







MGW







DF3 Signaling







C







MS B







No reply timeout







CPG(alert)







CPG(alert)







CPG(alert)







ANM







IAM







Answer(AB)







ANM







REL







REL







Release(AB)







Prepare Bearer Establishment/



Establish Bearer







Stublines” contain CC of AC







CPG(forward)







ACM







Suppl. Service(AB, CFNR)







ALERTING(AB)







SETUP(AB)







DF3 Bearer







stublines’ contain CC of BD







Answer(BD)







SETUP(BD)







Release(BD)







REL







DISCONNECT(BD)







MSC Server







DF2







Release Bearer/Release Resource







Release Resource/ Release Bearer Resource







release of stublines”







Bearer Establishment







Establish Bearer/



Prepare Bearer Establishment











setup of stublines”












_1045422899.doc
... setup and interception of BC call as in figure A1 ...


... setup and interception of AB call as in figure A2 ...




SETUP(BC)







C







IAM







Answer(BC)







DF3 Signaling







A







MS B







DF3 Bearer







MSC Server







DF2







HOLD ACK(AB)







Suppl. Service(AB, CHOLD)







ECT(AB)







MGW







Stublines” contain CC of AC







Bearer Release







Suppl. Service(BC, ECT)







Release(BC)







REL







REL







Release(AB)







Release Bearer/Release Resource







Stublines’ contain CC of AC







Bearer Release







Stublines” contain CC of BC







Suppl. Service(AB, ECT)







HOLD(AB)







Answer(AB)







Stublines’  contain CC of AB







Release Bearer/Release Resource







Release Resource/ Release Bearer



Bearer/Release 







release of stublines’







Release Resource/ Release Bearer







release of stublines”












_1045472957.doc
[image: image1.wmf]Release Bearer/Release Resource


[image: image2.wmf]Release Bearer/Release Resource


... setup and interception of AC call as in figure A1 ...


... release of  AC call and interception as in figure A1 ...


without release of stublines


... release of AB call and interception as in figure A1 ...


... setup and interception of AB call as in figure A1 ...




Release(AB)







C











MGW







DF3 Signaling







B







MS A







DF3 Bearer







MSC Server







HOLD(AB)















REL







DISCONNECT(AB)











Stublines” contain CC of ABC







Answer(AB)







SETUP(AB)







DISCONNECT(AC)







HOLD ACK(AB)







Suppl. Service(AB, CHOLD)







Stublines’ contain CC of AB







DF2







Stublines” contain CC of AC







�







BuildMPTY ACK(AB)







Release of stublines











Suppl. Service(AC, BMPTY)







Stublines’ contain CC of ABC







BuildMPTY(AB)







RetrieveMPTY ACK(AB)







RetrieveMPTY(AB)







Suppl. Service(AB, BMPTY)







Suppl. Service(AB, RMPTY)







REL







Answer(AC)







Release(AC)







Stublines’ contain CC of AB







SETUP(AC)







�












_1045422855.doc
... setup and interception of BC call as in figure A1 without setup of stublines ...


... setup and interception of AB call as in figure A2 ...




SETUP(BC)







C







IAM







Answer(BC)







DF3 Signaling







A







MS B







DF3 Bearer







MSC Server







DF2







HOLD ACK(AB)







Suppl. Service(AB, CHOLD)







ECT(AB)







MGW







Release Bearer/Release Resource







Bearer Release







Suppl. Service(BC, ECT)







Release(BC)







REL







REL







Release(AB)







Release Resource/ Release Bearer







Stublines contain CC of AC







release of stublines







stublines contain CC of BC







Suppl. Service(AB, ECT)







HOLD(AB)







Answer(AB)







Stublines  contain CC of AB












_1045422711.doc


Call Establishment Attempt







ACM







IAM







Bearer Release







C







MGW







DF3 Signaling







A







MS B







DF3 Bearer







No reply timeout







CPG(alert)







CPG(alert)







CPG(alert)







ANM







IAM







Answer







ANM







REL







REL







Release







Prepare Bearer Establishment/



Establish Bearer







Stublines  contain CC of AC







CPG(forward)







ACM







Suppl. Service(CFNR)







ALERTING







SETUP







MSC Server







DF2







Release Bearer/Release Resource







Release Resource/ Release Bearer







release of stublines







Bearer Establishment







Establish Bearer/



 Prepare Bearer Establishment











setup of stublines












_1041834607.doc


LEMF







Delivery�Function 3















other



party







MGW







intercepted�subscriber







Two stublines












_1044426815.doc


Call Establishment Attempt







ACM







IAM







Bearer Release







MGW







DF3 Signaling







A







HLR







DF3 Bearer







3G GMSC







SRI request for B







SRI response







CPG(alert)







CPG(alert)







ANM







IAM







Answer







ANM







REL







REL







Release







Prepare Bearer Establishment/



Establish Bearer







Stublines  contains CC of AC







C







ACM







Suppl. Service(CFU)







DF2







Release Bearer/Release Resource







Release Resource/ Release Bearer







release of stublines







Bearer Establishment







Establish Bearer/



 Prepare Bearer Establishment Establishment/











setup of stublines












_1005662284.doc
... deactivation on X1_1 and X1_2 ...




response for lawful interception deactivation







request for lawful interception deactivation







lawful interception deactivation







DF 3







ADMF







lawful interception deactivation ack












_1005663652.doc


LEMF







Delivery�Function 2







3G MSC















SMS�service�center







intercepted�subscriber












_1005705702.doc
... activation on X1_3 ...


... activation on X1_1 




DF 2







ADMF







response for lawful interception activation







request for lawful interception activation















lawful interception activation







lawful interception activation ack












_1005662614.doc
... interrogation for activation status on X1_2 and X1_3 ...




response for lawful interception interrogation







request for lawful interception interrogation











lawful interception interrogation







DF2/DF3











ADMF







lawful interception interrogation ack












_1005663611.doc


LEMF







Delivery�Function 2







3G MSC







Other



party







intercepted�subscriber












_1005662175.doc
... deactivation on X1_3 


... deactivation on X1_1 




lawful interception deactivation







response for lawful interception deactivation







request for lawful interception deactivation







 DF 2







ADMF







lawful interception deactivation ack












