3GPP TSG SA WG3 (Security) Meeting #80	S3-151761

24-28 August 2015, Tallinn (Estonia)

Source:	NEC

Title:	Adding a test case related to the requirement 5.3.3.1.3 “Automatic launch of removable media”

Document for:	Discussion/approval

Agenda Item:	7.10

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution: This contribution proposes to add a test case related to the requirement 5.3.3.1.3 of TS 33.117.

# Introduction

This contribution proposes to add the test case related to the requirement 5.3.3.1.3 of TS 33.117

Note that the Requirements Reference in the test case template used in TR 33.806 is no longer needed here since in TS 33.117 the test case immediately follows the requirement.

# Proposed pCR

***	BEGIN OF FIRST CHANGE	***

##### 5.3.3.1.3	Automatic launch of removable media

Requirement Name: Automatic launch of removable media

Requirement Reference: TBA

Requirement Description:

The network product shall not automatically launch any application when a removable media device such as CD-, DVD-, USB-Sticks or USB-Storage drives is connected. If the operating system supports an automatic launch, it shall be deactivated.

Threat References: TBA

Security Objective References: TBA

Test Case:

Test Name: TC_AUTO_LAUNCH_OF_REMOVABLE_MEDIA

Purpose:

To verify that the network product doesn’t launch any applications automatically when a removable media devices is connected. Any such feature should be deactivated.

Procedure and execution steps:

Pre-Condition

The network product should be provisioned with physical ports/drives (CD/DVD drive, USB port, etc.).

Execution Steps

- The tester log in the network product.

- The tester inserts a removable media device (CD-, DVD-, USB-Sticks and/or USB-Storage drives) in the network product.

Expected Results:

The network product doesn’t launch any applications to open the contents in the removable media device,

In Linux machines, the removable media device is not automatically mounted in the filesystem.

Expected format of evidence:

Evidence can be presented in the form of screenshot/screen-capture on how the network product responds when any removable media device is attached to it.

***	END OF FIRST CHANGE	***