3GPP TSG SA WG3 (Security) Meeting #80	S3-152011

24-28 August 2015 Tallinn (Estonia)		revision of S3-151876

Source:	Alcatel-Lucent

Title:	SCAS: Mapping TR.33.806 Annex D.3.3.2.3.1 test case to TS’s.

Document for:	 Discussion/approval

Agenda Item:	7.10

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution: Abstract of the contribution: This contribution proposes to map test cases in Annex D.3.3.2.3.1 – Traffic protection mechanisms on OAM interface testing to the relevant sub-clauses in TS.33.117 (Generic requirements). A separate pCR (S3-152012) will propose text for the test case for requirement 5.2.3.2.4 of the MME specific TS33.116.

The first section of the contribution copies the text from TR 33.806, D.3.3.2.3.1. The current contribution maps Annex D.3.3.2.3.1 to TS.33.117. 5.2.3.2.4.

The second section of the contribution provides a pCR to TS 33.117.

# Annotated text from TR 33.806

### D.3.3.2.3.1	Traffic protection mechanisms on OAM interface

Test Name: TC_PROTECT_DATA_INFO_TRANSFER_1

Requirements:

Requirements Reference - Protecting data and information in transfer (cf. B.3.3.2.3 for the general case), (cf. B.3.6.1 for  HTTPS)

Purpose:

Verify the mechanisms implemented to protect data and information in transfer to and from the MME OAM interface. In particular for testing a connection protected with the security protocol implemented by the vendor (e.g. SSHv2 or HTTPS) is used and the test shall verify that the traffic is correctly encrypted and cannot be tampered with.

Procedure and execution steps:

Pre-Conditions:

The MME shall be configured for secure OAM communications according to vendor documentation.

Client implementing the security protocol (e.g. SSH client supporting SSHv2 or HTTPS client) as OAM peer shall be available.

Execution Steps

- Turn on a network analyzer  (e.g. tcpdump, wireshark) on MME OAM interface

- Configure the client to use cryptographic algorithms considered strong by the industry for message authentication, and verbose logging. For HTTPS, the cryptographic algorithms that are considered strong follow from the requirement on HTTPS (cf. B.3.6.1).

- Start a protected communication between MME and OAM network

- Analyze the client logs and the traffic in transit and verify that the generated traffic is properly protected, and that insecure options are not accepted by the MME (e.g. SSHv1 or SSL). For HTTPS, proper protection of the traffic is defined by the requirements on HTTPS (cf. B.3.6.1). The tester shall take these requirements one by one and check that they are satisfied.

Expected Results:

The traffic is properly protected , and insecure options are not accepted by the MME.

Expected format of evidence:

Save the logs and the  communication flow in a .pcap file.

# 2 pCR to TS 33.117 (Generic requirements)

# ***	BEGIN OF SECOND CHANGE	***

##### 5.2.3.2.4	Protecting data and information in transfer

Requirement Name: tba

Requirement Reference: to be done later

Requirement Description:

-	Usage of cryptographically protected network protocols is required.

-	The transmission of data with a need of protection shall use industry standard network protocols with sufficient security measures and industry accepted algorithms. In particular, a protocol version without known vulnerabilities or a secure alternative shall be used.

Security Objective references: tba

Test case:

Test Name: TC_PROTECT_DATA_INFO_TRANSFER_1

Requirements:

Requirements Reference - Protecting data and information in transfer (cf. B.3.3.2.3 for the general case), (cf. B.3.6.1 for  HTTPS)

Note: Assurance of this requirement is in scope of Basic Vulnerability Testing (BVT), cf. 5.4. The test lab must document explicitly which BVT test case is applicable.

Purpose:

Verify the mechanisms implemented to protect data and information in transfer to and from the Network Product’s MME OAM interface. In particular for testing a connection protected with the security protocol implemented by the vendor (e.g. SSHv2 or HTTPS) is used and the test shall verify that the traffic is correctly encrypted and cannot be tampered with.

Procedure and execution steps:

Pre-Conditions:

The Network Product MME shall be configured for secure OAM communications according to vendor documentation.

Client implementing the security protocol (e.g. SSH client supporting SSHv2 or HTTPS client) as OAM peer shall be available.

Execution Steps

- Turn on a network analyzer  (e.g. tcpdump, wireshark) on the Network Product’s MME OAM interface

- Configure the client to use cryptographic algorithms considered strong by the industry for message authentication, and verbose logging. For HTTPS, the cryptographic algorithms that are considered strong follow from the requirement on HTTPS (cf. 5.2.5.1).

- Start a protected communication between the Network Product MME and OAM network

- Analyze the client logs and the traffic in transit and verify that the generated traffic is properly protected, and that insecure options are not accepted by the Network Product (e.g. SSHv1 or SSL). For HTTPS, proper protection of the traffic is defined by the requirements on HTTPS (cf. 5.2.5.1). The tester shall take these requirements one by one and check that they are satisfied.

Expected Results:

The traffic is properly protected , and insecure options are not accepted by the Network Product MME

Expected format of evidence:

Save the logs and the  communication flow in a .pcap file.

***	END OF SECOND CHANGE	***