3GPP TSG SA WG3 (Security) Adhoc Meeting on FS_NSA	S3-161489

27 – 29 September, 2016, San Diego, USA 	revision of S3-161359

Source:	NEC

Title:	pCR to TR 33.899: Solution #1.z Key hierarchy for NextGen

Document for:	Approval

Agenda Item:	4.1

Work Item / Release:	FS_NSA / Rel-14

Abstract: This pCR proposes a solution to Key Issues of “#1 Architectural aspects of NextGen security“

#### Introduction

As described in key issues # 1.2 and #1.7 of TR 33.899, there is a need for security anchor in the NextGen network and the requirement for a corresponding key hierarchy to provision confidentiality and integrity requirement. TR 23.799 provisions two new entities CP-AU and SCM to handle the issues of security anchoring and security context management respectively and another entity CP-CN to handle MM (Mobility Management) and SM (Session Management) functions. This pCR proposes a solution to the key hierarchy based on new architecture given in TR 23.799 ver 0.8.0 09/2016, considering CP-AU and SCM.

#### Proposal

#### **************BEGIN OF CHANGES************

#### 5.1.4.z	Solution #1.x: Key hierarchy for NextGen

##### 5.1.4.z.1	Introduction

This solution proposes a key hierarchy for NextGen network in accordance to TR 23.799 ver 0.8.0 proposed reference architecture for unified authentication framework for non-roaming scenario which introduces a new module Security Context Management (SCM) as a part of Core Network. SCM deals with management of the security context for core networks and access network of NextGen.

CP-AU, the security anchor, is a function in the core network that performs UE authentication process and interacts with Subscriber Repository Function for retrieving authentication materials. Security Context Management function (SCM) is a function in the core network that upon successful UE authentication maintains an authentication security context from which the security contexts for the specific functionality as applicable for the CN and the AN will be derived.

The solution describes a key hierarchy that includes CP-AU and SCM keys to handle the issues of security anchoring and security context management respectively. The details of the solution are provided in the following clause.

NOTE: A Single C-plane at network level is assumed.

Editor’s Note:  “It is FFS to consider how to realize security isolation among network slices for single C-plane in core network”

##### 5.1.4.z.2	Solution details

###### 5.1.4.z.2.1	Architecture

Figure 5.1.4.z.2.1-1 describes the reference architecture (copy of Figure 6.12.1.1.1-1 of version 0.8.0 of TR 23.799 [2]) defined in  TR 23.799 v 0.8.0 for a unified authentication framework in non-roaming scenario.

User Plan Protection

CP-AU

Subscriber repository

AN

CN

Supplicant

UE

DN

Data Network

Control
Plane of

Slice #1

User
Plane of

Slice #1

User Plan Protection

Trusted Storage or Computing

CN signaling Protection

CP-CN

CP-AN signaling Protection

CP-AN Signaling Protection

User Plan Protection

User Plan Protection

AU-UE

SubRep-AU

AU-AN

AU-SCM

Common Control
Plane

SCM

SCM-CN

Figure 5.1.4.z.2.1-1: Reference architecture for Unified authentication framework for in non-roaming scenario

The architecture includes a Control Plane Authentication function (CP-AU) that authenticates a UE connecting to the NextGen core network (CN) via different access networks. CP-AU, the security anchor of the NextGen network, is decoupled from the security context management (SCM) function and the mobility management (MM) and session management (SM) functions, which are abstracted as a part of the Control Plane-Core Network (CP-CN) functions. The Security Context Management (SCM) module, maintains an authentication security context which is the basis to derive the security contexts for the specific functionality as applicable for the CN and the AN, upon successful UE authentication. It also maintains the security context of the UE for an authentication session and derives/provides keys for other network functions to enable those functions to establish a security context for the UE.

Decoupling of the security anchor functionality from the MM and SM functionalities and security context management enables the security anchor to reside in a physically secure location in the NextGen CN independent of the MM, SM and SCM location (closer to the AN). It also helps to reduce security configuration complexity between network entities/functions. This functional split requires a new key hierarchy and a corresponding security and key derivation procedure. In addition, the generation of keys during slicing in NextGen Systems is also required to be addressed by this key hierarchy.

###### 5.1.4.z.2.2	Key Hierarchy

A key hierarchy for the NextGen system is shown in Figure 5.1.4.z.2.2-1. In general, the key hierarchy is similar to that of SAE/LTE with the differences like  (1) Introduction of two additional layers in key hierarchy that allows the security anchor  key resulting from the authentication to be held in a secure location (in effect this is splitting the KASME into KCP-AU, KSCM and KCP-CN); (2) Ability to generate keys (KUP-GW) for termination of the UP security at a user plane gateway (Key Issue # 1.4); (3) Ability to generate keys for each slice at the UP-GW for encryption and integrity checks.

The description of each key in the key hierarchy is as follows.

| K | - The subscriber credential that is held in the UE and AAA. |
| --- | --- |
| KCP-AU | - Security Anchor Key for the authentication from which subsequent SCM key is derived. |
| KSCM | - Key for management of security context from which subsequent CN and AN keys are derived. |
| KCP-CN | - Control Plane Key that is used to derive NAS integrity and encryption protection keys. |
| KAN/NH | - Key provided to the AN (similar to KeNB/NH in LTE) to derive RRC and UP integrity and encryption protection keys. |
| KUP-GWx | - User plane key for UP-GW of slice ‘x’ when the user plane security terminates at the UP-GW. |

The details on each key derivation (e.g., key derivation algorithm, input parameters) are FFS.

K

KCP-AU

KSCM

KAN/NH

KRRCenc

KRRCint

KUPenc

KUPint

KUP-GW

Slice n

KUP-GW1enc

KUP-GW1int

UE/AAA

UE/CP-AU

UE/AN

UE/SCM

KCP-CN

KNASenc

KNASint

UE/CP-CN

KUP-GW

Slice1

KUP-GW1enc

KUP-GW1int

UE/UP-GW

Figure 5.1.4.z.2.2-1: Key hierarchy of the NextGen system

Editor’s Note: Terminology used in this solution need to be updated in accordance with SA3 agreement

Editor’s Note:  It is FFS whether the proposed levels of key hierarchy are all needed.

****************End of Changes*************