3GPP TSG-SA3 Meeting #86	S3-170185

# Sophia-Antipolis, France, 6th Feb 2017 - 10th Feb 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.401 | CR | 0597 | rev | 2 | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding BEST Service to TS 33.401 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | BEST_MTC_Sec |  |  |  |  |  | Date: |  |  | 2017-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The reason for this CR that it was agreed to add the results of the BEST Study item in normative specifications. This CR proposes how to capture the normative changes into TS 33.401 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR adds a chapter for the BEST service and an annex that shows a possible transport protocol |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The BEST service not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.3, #B (new), Annex A.19 (new section), 3 new Annexes |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 33.220 CR 0185 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[3]	3GPP TS 23.003: "Numbering, addressing and identification".

[4]	3GPP TS 33.102: "3G security; Security architecture".

[5]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[6]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[7]	IETF RFC 4303: "IP Encapsulating Security Payload (ESP)".

[8]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic bootstrapping architecture".

[9]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[10] – [11]	Void.

[12]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification"

[13]	3GPP TS 31.102: "Characteristics of the Universal Subscriber Identity Module (USIM) application".

[14]	3GPP TS 35.215: "Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 1: UEA2 and UIA2 specifications"

[15]	NIST: "Advanced Encryption Standard (AES) (FIPS PUB 197) "

[16]	NIST Special Publication 800-38A (2001): "Recommendation for Block Cipher Modes of Operation".

[17]	NIST Special Publication 800-38B (2001): "Recommendation for Block Cipher Modes of Operation: The CMAC Mode for Authentication".

[18] – [20]	Void.

[21]	3GPP TS 36.331:"Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC); Protocol specification".

[22]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[23]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Service aspects; Service principles".

[24]	3GPP TS 25.331: "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Radio Resource Control (RRC); Protocol Specification ".

[25]	3GPP TS 44.060: "3rd Generation Partnership Project; Technical Specification Group GSM/EDGE Radio Access Network; General Packet Radio Service (GPRS); Mobile Station (MS) - Base Station System (BSS) interface; Radio Link Control/Medium Access Control (RLC/MAC) protocol.

[26]	3GPP TS 23.122: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[27]	3GPP TS 33.320: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Security of Home Node B (HNB) / Home evolved Node B (HeNB)".

[28]	(void)

[29]	ETSI TS  102 484 V10.0.0: "Smart Cards; Secure channel between a UICC and an end-point terminal".

[30]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[31]	3GPP TS 31.116 "Remote APDU Structure for (Universal) Subscriber Identity Module (U)SIM Toolkit applications".

[32]	ETSI TS 102 221 V9.2.0: "Smart Cards; UICC-Terminal interface; Physical and logical characteristics".

[33]	3GPP TS 35.221: "Confidentiality and Integrity Algorithms EEA3 & EIA3; Document 1: EEA3 and EIA3 specifications".

[34]	RFC 4301: "Security Architecture for the Internet Protocol".

[35]	3GPP TS 22.346: "Isolated Evolved Universal Terrestrial Radio Access Network (E-UTRAN) operation for public safety; Stage 1".

[36]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[37]	3GPP TS.33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[38]	IETF RFC 7296: " Internet Key Exchange Protocol Version 2 (IKEv2)".

[39]	IEEE 802.11, Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications, IEEE Std.".

[40]	3GPP TS 36.463: " Evolved Universal Terrestrial Radio Access Network (E-UTRAN) and Wireless LAN (WLAN); Xw application protocol (XwAP)".

[41]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[42]	3GPP TS 55.241: "Specification of the GIA4 integrity algorithm for GPRS; GIA4 specification"

[43]	3GPP TS 55.251: " Specification of the GEA5 encryption and GIA5 integrity algorithms for GPRS; GEA5 and GIA5 algorithm specification"

[44]	3GPP TS 35.205: " 3G Security; Specification of the MILENAGE algorithm set: An example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 1: General"

**** End of First Change ****

**** Second Change ****

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 33.102 [4] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Access Security Management Entity: entity which receives the top-level keys in an access network from the HSS. For E-UTRAN access networks, the role of the ASME is assumed by the MME

Activation of security context: the process of taking into use a security context.

Authentication data: Data that is part of a security context or of authentication vectors.

BEST: Battery Efficient Security service for Cellular IoT

BEST Capable UE: A UE that is enabled for the BEST service

Chaining of KeNB: derivation of a new KeNB from another KeNB (i.e., at cell handover)

Current EPS security context: The security context which has been activated most recently. Note that a current EPS security context originating from either a mapped or native EPS security context may exist simultaneously with a native non-current EPS security context.

ECM-CONNECTED state: This is as defined in TS 23.401 [2]. The term ECM-CONNECTED state corresponds to the term EMM-CONNECTED mode used in TS 24.301 [9].

ECM-IDLE state: As defined in TS 23.401 [2]. The term ECM-IDLE state corresponds to the term EMM-IDLE mode used in TS 24.301 [9].

Enterprise Key: A secret key shared by the Enterprise Application Server and the UE for application in the BEST service.

EPS-Authentication Vector: KASME, RAND, AUTN, XRES

EPS security context: A state that is established locally at the UE and a serving network domain. At both ends "EPS security context data" is stored, that consists of the EPS NAS security context, and the EPS AS security context.

NOTE 1:	An EPS security context has type "mapped", "full native" or "partial native". Its state can either be "current" or "non-current". A context can be of one type only and be in one state at a time. The state of a particular context type can change over time. A partial native context can be transformed into a full native. No other type transformations are possible.

EPS AS security context: the cryptographic keys at AS level with their identifiers, the Next Hop parameter NH, the Next Hop Chaining Counter parameter NCC used for next hop access key derivation, the identifiers of the selected AS level cryptographic algorithms, counters used for replay protection and SCG Counter used as freshness input into S-KeNB derivations. Note that the EPS AS security context only exists when cryptographically protected radio bearers are established and is otherwise void.

NOTE 2: NH and NCC need to be stored also at the MME during connected mode.

EPS AS Secondary Cell security context: This context consists of the cryptographic keys for SeNB (KUPenc), the identifier of the selected AS SC level cryptographic algorithm and counters used for replay protection.

EPS NAS security context: This context consists of KASME with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values. In particular, separate pairs of NAS COUNT values are used for each EPS NAS security contexts, respectively. The distinction between native and mapped EPS security contexts also applies to EPS NAS security contexts. The EPS NAS security context is called "full" if it additionally contains the keys KNASint and KNASenc and the identifiers of the selected NAS integrity and encryption algorithms.

Extended User Plane protection: Within the context of TS 33.401 the extended user plane protection means the activation of a BEST secure session.

Full native EPS security context: A native EPS security context for which the EPS NAS security context is full according to the above definition. A full native EPS security context is either in state "current" or state "non-current".

Forward security: In the context of KeNB key derivation, forward security refers to the property that, for an eNB with knowledge of a KeNB, shared with a UE, it shall be computationally infeasible to predict any future KeNB, that will be used between the same UE and another eNB. More specifically, n hop forward security refers to the property that an eNB is unable to compute keys that will be used between a UE and another eNB to which the UE is connected after n or more handovers (n=1 or 2).

Legacy security context: A security context which has been established according to TS 33.102 [4].

Mapped security context: Security context created by converting the current security context in the source system to a security context for the target system in inter-system mobility, e.g., UMTS keys created from EPS keys. The EPS NAS security context of a mapped security context is full and current.

Native EPS security context: An EPS security context whose KASME was created by a run of EPS AKA.

Non-current EPS security context: A native EPS security context that is not the current one. A non-current EPS security context may be stored along with a current EPS security context in the UE and the MME. A non-current EPS security context does not contain an EPS AS security context. A non-current EPS security context is either of type "full native" or of type "partial native".

Partial native EPS security context: A partial native EPS security context consists of KASME with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values, which are initially set to zero before the first NAS SMC procedure for this security context. A partial native EPS security context is created by an EPS AKA, for which no corresponding successful NAS SMC has been run. A partial native context is always in state "non-current".

Re-derivation of NAS keys: derivation of new NAS keys from the same KASME but including different algorithms (and no freshness parameter)

Refresh of KeNB: derivation of a new KeNB from the same KASME and including a freshness parameter

Re-keying of KeNB: derivation of a new KeNB from a new KASME in ECM-CONNECTED (i.e. to activate a partial native EPS security context, or to re-activate a non-current full EPS security context)

Re-keying of NAS keys: derivation of new NAS keys from a new KASME

UE security capabilities: The set of identifiers corresponding to the ciphering and integrity algorithms implemented in the UE. This includes capabilities for EPS AS and NAS, and includes capabilities for UTRAN and GERAN if these access types are supported by the UE.

UE EPS security capabilities: The UE security capabilities for EPS AS and NAS.

UE-to-HSE: UE to Home  Security Endpoint

UE-to-EAS: UE to Enterprise Application Server

User plane: Within the context of TS 33.401, this means the data path between UE and Serving Gateway that does NOT go via the MME.

(User) Data via MME: User Data sent to or from the UE that uses an RRC connection established using the Control Plane CIoT EPS optimisation specified in TS 23.401[2].

IOPS-capable eNB: an eNB that has the capability of IOPS mode operation, which provides local IP connectivity and Public Safety services to IOPS-enabled UEs via a Local EPC when the eNB has lost backhaul to the Macro EPC or it has no backhaul to the Macro EPC.

IOPS network: an IOPS network consists of one or more eNBs operating in IOPS mode and connected to a Local EPC.

Local EPC: a Local EPC is an entity which provides functionality that eNBs in IOPS mode of operation use, instead of the Macro EPC, in order to support Public Safety services.

Macro EPC: the EPC which serves an eNB when it is not in IOPS mode of operation.

Nomadic EPS: a deployable system which has the capability to provide radio access (via deployable IOPS-capable eNB(s)), local IP connectivity and Public Safety services to IOPS-enabled UEs in the absence of normal EPS.IOPS-enabled UE: is an UE that is configured to use networks operating in IOPS mode.

**** End of Second Change ****

**** Third Change ****

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AES	Advanced Encryption Standard

AK	Anonymity Key

AKA	Authentication and Key Agreement

AMF	Authentication Management Field

AN	Access Network

AS	Access Stratum

AUTN	Authentication token

AV	Authentication Vector

ASME	Access Security Management Entity

Cell-ID	Cell Identity as used in TS 36.331 [21]

CK	Cipher Key

CKSN	Cipher Key Sequence Number

C-RNTI	Cell RNTI as used in TS 36.331 [21]

CRL	Certificate Revocation List

DeNB	Donor eNB

DoS	Denial of Service

DSCP	Differentiated Services Code Point

EARFCN-DL	E-UTRA Absolute Radio Frequency Channel Number-Down Link

EAS	Enterprise Application Server

ECM	EPS Connection Management

EEA	EPS Encryption Algorithm

EIA	EPS Integrity Algorithm

eKSI	Key Set Identifier in E-UTRAN

EMM	EPS Mobility Management

eNB	Evolved Node-B

EPC	Evolved Packet Core

EPS	Evolved Packet System

EPS-AV	EPS authentication vector

E-UTRAN	Evolved UTRAN

GERAN	GSM EDGE Radio Access Network

GUTI	Globally Unique Temporary Identity

HE	Home Environment

HFN	Hyper Frame Number

HO	Hand Over

HSS	Home Subscriber Server

HSE	HPLMN Security Endpoint

IK	Integrity Key

IKE	Internet Key Exchange

IMEI	International Mobile Station Equipment Identity

IMEISV	International Mobile Station Equipment Identity and Software Version number

IMSI	International Mobile Subscriber Identity

IOPS	Isolated E-UTRAN Operation for Public Safety

IRAT	Inter-Radio Access Technology

ISR	Idle Mode Signaling Reduction

KDF	Key Derivation Function

KSI	Key Set Identifier

LWIP	LTE WLAN RAN Level Integration using IPSec

LSB	Least Significant Bit

LSM	Limited Service Mode

LWA	LTE-WLAN Aggregation

MAC-I	Message Authentication Code for Integrity (terminology of TS36.323 [12])

MACT	Message Authentication Code T used in AES CMAC calculation

MeNB	Master eNB

ME	Mobile Equipment

MME	Mobility Management Entity

MME-RN	MME serving the RN

MS	Mobile Station

MSC	Mobile Switching Center

MSIN	Mobile Station Identification Number

NAS	Non Access Stratum

NAS-MAC	Message Authentication Code for NAS for Integrity (called MAC in TS24.301 [9])

NASDVM	Non Access Stratum - Data via MME

NCC	Next hop Chaining Counter

NH	Next Hop

OCSP	Online Certificate Status Protocol

OTA	Over-The-Air (update of UICCs)

PCI	Physical Cell Identity as used in TS 36.331 [21]

PDCP	Packet Data Convergence Protocol

PLMN	Public Land Mobile Network

PRNG	Pseudo Random Number Generator

PSK	Pre-shared Key

P-TMSI	Packet- Temporary Mobile Subscriber Identity

RAND	RANDom number

RAU	Routing Area Update

RN	Relay Node

RRC	Radio Resource Control

SCG	Secondary Cell Group

SEG	Security Gateway

SGSN	Serving GPRS Support Node

SIM	Subscriber Identity Module

SMC	Security Mode Command

SeNB	Secondary eNB

SN	Serving Network

SN id	Serving Network identity

SQN	Sequence Number

SRB	Source Route Bridge

SRVCC	Single Radio Voice Call Continuity

S-TMSI	S-Temporary Mobile Subscriber Identity

TAI	Tracking Area Identity

TAU	Tracking Area Update

UE	User Equipment

UEA	UMTS Encryption Algorithm

UIA	UMTS Integrity Algorithm

UICC	Universal Integrated Circuit Card

UMTS	Universal Mobile Telecommunication System

UP	User Plane

USIM	Universal Subscriber Identity Module

UTRAN	Universal Terrestrial Radio Access Network

WT	WLAN Termination as used in TS 36.300 [30]

XRES	Expected Response

**** End of Third Change ****

**** Fourth Change ****

# #B	Security Procedures for Battery Efficient Security for very low Throughput Machine Type Communication Devices (BEST)

## #B.1	Introduction

This clause deals with extended user plane protection, i.e. protection of user data from the UE to the HSE in the HE or beyond the HSE to an Enterprise Application Server (EAS). This service is called "BEST".

## #B.2	Service Description

### #B.2.1	Architecture

Figure #B.2.1-1: The architecture of the extended user plane protection service

The extended user plane data service requires the following components:

- Home Security Environment (HSE) – This is the termination point in the home network that performs the following functions:

  - Terminating the signalling plane for BEST between the UE and the HSE

  - Terminating the secure communication for BEST between the UE and the HSE if UE-to-HSE security is selected.

  - Routing the user plane traffic for BEST between the UE and the Enterprise Application Server (EAS) if UE-to-EAS security is selected.

- End to Middle Key Server (EMKS) – This is an optional key server element that manages the key communication with the HSS for retrieving AVs and stores keys to reduce load on the HSE and HSS. The EMKS has interfaces to the HSS (S6g) and the HSE (S6f’).

The extended user plane service requires the following interfaces:

- S6f between the HSS and the HSE

- S6g between the HSS and EMKS

- S6f' between the EMKS and the HSE

- SGi' between the HSE and the Enterprise Application Server

## #B.3	Procedures between the UE and the HSE

### #B.3.1	UE initiated BEST-session

A BEST-session can only be started by the HSE in the HE. A BEST-capable UE may request the HSE to start a BEST-session. The procedure is shown in figure #B.3.1-1.

Figure #B.3.1-1: Request and start of BEST-session

In the request, the UE shall use the "Start EMSDP session request" command to request the setup of a new BEST session. In this command the UE shall set the Enterprise TLV field to the preconfigured value. To request a UE-to-EAS secure connection the UE shall set the UE-to-EAS flag to 1. To request a UE-to-HSE secure connection the UE shall set the UE-to-EAS flag to 0 to request a UE-to-HSE secure connection.

The HSE shall respond with the "Start new EMSDP session" command. The HSE shall set the HSE Identity to its own identity. If new keys need to be derived, the HSE shall include a RAND and AUTN as specified in 3GPP TS 24.008 [41]. To start a UE-to-EAS BEST session, the HSE shall set the UE-to-EAS flag to 1 and include an EAS container if one is received from the EAS.

Note 1: The HSE decides whether or not to start a UE-to-EAS BEST session, although it will only do so if the UE has indicated UE-to-EAS capabilities. The HSE decides based on configuration, local regulations, resources available, operator settings, etc.

### #B.3.2	HSE initiated BEST-session

An HSE that initiates a BEST-session shall send the Start EMSDP session command as detailed in #B.3.1.

### #B.3.3	UE initiated termination of BEST-session

To terminate a BEST session, the UE shall send the "Terminate EMSDP session request" command to the HSE. The HSE shall repond with the "Terminate EMDSP session" command to the UE.

Figure #B.3.3-1: UE Initiated terminated of BEST-session

### #B.3.4	HSE initiated termination of BEST-session

To terminate a BEST-session, the HSE shall send the "Terminate EMSDP-session" command to the UE.

### #B.3.5	UE initiated BEST-key refresh procedure

The UE may request a new AKA run for the BEST keys by sending the "Manage EMSDP keys request" command to the HSE. The HSE may respond with the "Manage EMSDP Keys" command, in which the HSE shall include a fresh AUTN and RAND. If the BEST session is a UE-to-EAS session, the HSE shall set the UE-to-EAS flag to 1 and include a new EAS Container. The HSE shall set the UE-to-EAS flag to 0 for UE-to-HSE sessions.

Figure #B.3.5-1: UE Initiated key refresh request

### #B.3.6	HSE initiated BEST-key refresh procedure

The HSE may trigger a new key derivation by sending the "Manage EMSDP Keys" command. The HSE shall include a fresh AUTN and RAND. If the BEST session is a UE-to-EAS session, the HSE shall set the UE-to-EAS flag to 1 and include a new EAS Container. The HSE shall set the UE-to-EAS flag to 0 for UE-to-HSE sessions.

### #B.3.7	UE initiated BEST-session modification request

The UE may request the modification of an existing BEST-session between a UE-to-HSE and a UE-to-EAS session. In order to do so, the UE shall send a "Modify EMSDP session request" command to the HSE. The UE shall set the UE-to-EAS flag to 0 to modify an existing session to a BEST UE-to-HSE session. The UE shall set the UE-to-EAS to 1 to modify an existing session to a BEST UE-to-EAS session.

Figure #B.3.7-1: UE initiated BEST-session modification request

The HSE shall respond with a "Modify EMSDP-session" command or a "Request rejected" command.

If the HSE sends a "Modify EMSDP-session" command it shall contain the elemements as described in #B.3.8.

In the case the UE requests a modification from one type to the same type (i.e. from UE-to-HSE to UE-to-HSE), the HSE may either accept or may reject the request modification.

### #B.3.8	HSE initiated BEST-session modification request

The HSE may modify an existing BEST-session by sending the "Modify EMSDP-session" command to the UE. This command shall contain the following, based on the current session:

-	To modify a BEST UE-to-HSE session to a UE-to-EAS session, the HSE shall set UE-to-EAS flag equal to 1 and includes a new EAS Container.

-	To modify a BEST UE-to-EAS session to a UE-to-HSE session, the HSE shall set the "UE-to-EAS" flag to 0 and provides no EAS container.

## #B.4	Procedures between the HSE and the Enterprise Application Server

### #B.4.1	Signalling between the HSE and the EAS

The signaling format between the HSE and the EAS is left up to the operator to agree with the enterprise. Reasonable choices are to use an XML-message or for example JSON-RPC.

### #B.4.2	HSE initiated start of UE-to-HSE session

The HSE shall inform the EAS of a UE-to-HSE session by means of a "UE-to-HSE Session Started".

This command shall contain:

-	The identity of the UE. This shall be either the IMSI or the MSISDN

The EAS shall respond with a "UE-to-EAS Session Accepted" message.

This command shall contain:

-	No command options.

Upon succesfull completion of the procedure between the HSE and the UE, the HSE shall send a "Start UE-to-EAS Session" command which shall contain:

-	The identity of the UE

The EAS shall reply with a "UE-to-HSE Session Started" message.

### #B.4.3	HSE initiated start of UE-to-EAS session

The HSE shall inform the EAS of a UE-to-EAS session by means of a "UE-to-EAS Session Started".

This command shall contain:

-	The identity of the UE. This shall be either the IMSI or the MSISDN

-	The E2E intermediate (KInt_E2E)  key

The EAS shall respond with a "UE-to-EAS Session Accepted" message.

This command shall contain:

-	The enterprise container

Upon succesfull completion of the procedure between the HSE and the UE, the HSE shall send a "Start UE-to-EAS Session" command which shall contain:

-	The identity of the UE

The EAS shall reply with a "UE-to-EAS Session Started" message.

### #B.4.4	EAS initiated start of UE-to-HSE or UE-to-EAS session

The EAS may request a UE-to-EAS session by sending a "UE-to-EAS Session Requested by EAS" to the HSE.

This command shall contain:

-	The identity of the UE. This shall be the IMSI or the MSISDN or another identifier agreed between EAS and HSE.

-	A UE-to-EAS flag which shall be set to 0 if UE-to-HSE is requested and shall be set to 1 if UE-to-EAS is requested

The HSE shall reply with:

-	"UE-to-HSE Session Started", according to #B.4.2 if UE-to-EAS is not allowed

-	"UE-to-HSE Session Started", according to #B.4.3 if UE-to-EAS is allowed

-	"UE not found" if the identifiers cannot be found.

### #B.4.5	HSE initiated key refresh of UE-to-EAS session

The HSE shall inform the EAS of key refresh for an UE-to-EAS session by means of a "UE-to-EAS Session Key Refresh" message.

This command shall contain:

-	The identity of the session

The EAS shall respond with the "UE-to-EAS Key Refresh Accepted" command, which shall contain:

-	The Enterprise Container.

Upon succesfull key agreement with the UE, the HSE shall respond to the EAS by means of a "UE-to-EAS Keys Refreshed".

This command shall contain:

-	The identity of the UE. This shall be either the IMSI or the MSISDN

-	The E2E intermediate (KInt_E2E)  key

Upon reception of this message, the HSE shall derive the new keys and shall use the newly derived keys for any communication to and from the UE.

### #B.4.6	EAS initiated key refresh of UE-to-EAS session

The EAS may trigger a key refresh by means of the "UE-to-EAS Key Refresh" command.

This command shall contain:

-	The identity of the session

The HSE shall reply with:

-	The "UE-to-EAS Session Key Refresh" command if key refresh is permitted by the network

-	The "UE-to-EAS Session Key Refresh Denied" if the key refresh is not permitted by the network

### #B.4.7	EAS initiated key refresh of UE-to-HSE session

The EAS may trigger a key refresh by means of the "UE-to-HSE Key Refresh" command.

This command contains:

-	The identity of the session

The HSE shall reply with:

-	The "UE-to-HSE Session Key Refreshed" successful if key refresh was successfully performed by the network.

-	The "UE-to-HSE Session Key Not Refresh" if the key refresh was unsuccessful.

## #B.5	Starting a BEST service session

### #B.5.1	UE initiated BEST session

2. Start EMSDP session request command over the SGi interface (UE initialised session).

UE

HSE

PDN gateway

1. Start EMSDP session request command over the Air interface / core network interfaces (UE initialised session).

2. Start EMSDP session request command over the SGi interface (UE initialised session).

3. Start new EMSDP session command over the SGi interface (UE or HSE initialised session).

4. Start new EMSDP session command over the Air interface / core network interfaces (UE  or HSE initialised session).

3. Key Agreement run if required

Figure #B.5.1-1 UE initiated BEST session startup flow diagram

Once the data session is live (following the BEST service discovery procedure), if there is no current session running and typically when the UE has some data to transfer, the UE checks the key status and the BEST container received from the BEST service discovery procedure and does the following:

- If the BEST service is not activated in the BEST container then the session shall not be started.

- If the UE determines that the key needs to be re-agreed then the UE sends a "Start EMSDP session request" command through the SGI PtP tunnel (This is described in detail in clause 6.8.2.8) with the key id set to "00", Session id set to "00..00", no encryption and the MAC set to "00..00".

- If the UE determines that the key does not need to be re-agreed then the UE sends a "Start EMSDP session request" command with the indicated key and algorithms.  The HSE may trigger a key re-agreement if it chooses to do so.

On reception of the "Start EMSDP session request" command from the UE via the PDN gateway, the HSE checks the security of the message, the UE's subscription, the BEST service status for the location of the UE and the validity of the BEST service keys and carries out one of the following:

- If the HSE determines that the key does not need to be re-agreed then the HSE sends a "Start new EMSDP session" command with the indicated key, encrypted and integrity protected as indicated in the service discovery.  This message has one HSE Identity TLV containing the identity of the HSE and a Key agreement TLV with its length set to 00 and no AUTN IE or RAND IE.

- If the new keys need to be agreed for the BEST service, then the HSE includes one HSE Identity TLV containing the identity of the HSE and the Key Agreement TLV in the "Start new EMSDP session" command.  The Key Agreement TLV has the RAND and AUTN of the authentication vector to be used, in the RAND IE and AUTN IE.

On reception of the "Start new EMSDP session" command in the UE, the BEST session is considered setup and user plane messages are allowed to be sent.

### #B.5.2	HSE initiated BEST session

UE

HSE

PDN gateway

1. Start new EMSDP session command over the SGi interface (UE or HSE initialised session).

2. Start new EMSDP session command over the Air interface / core network interfaces (UE  or HSE initialised session).

3. Key Agreement run if required

Figure #B.5.2-1 HSE initiated BEST session startup flow diagram

Once the data session is live (following the BEST service discovery procedure), if there is no current session running and typically when the HSE has some data to transfer, the HSE checks the UE's subscription, the BEST service status for the location of the UE and the validity of the BEST service keys and carries out one of the following:

-	If the HSE determines that the key does not need to be re-agreed then the HSE sends a "Start new EMSDP session" command with the indicated key, encrypted and integrity protected as indicated in the service discovery.  This message has one HSE Identity TLV containing the identity of the HSE and a Key agreement TLV with its length set to 00 and no AUTN IE or RAND IE.

-	If the new keys need to be agreed for the BEST service, then the HSE includes one HSE Identity TLV containing the identity of the HSE and the Key Agreement TLV in the "Start new EMSDP session" command.  The Key Agreement TLV has the RAND and AUTN of the authentication vector to be used, in the RAND IE and AUTN IE.

On reception of the "Start new EMSDP session" command in the UE, the BEST session is considered setup and user plane messages are allowed to be sent.

### #B.5.3	Resuming a BEST session following a power cycle at the UE or a re-attach

If the new session required is not set and the other BEST parameters in the HSE BEST IE are identical to the previous agreed HSE BEST IE, the UE may continue a previously started session by using the security details of the last agreed session.

Storage of keys is described in clause #B.7 "Key Handling"

### #B.5.4	BEST service session operation

Once a BEST session is setup, boith signalling messages and user plan messages can be sent and received between the UE and the HSE secured by the agreed security mechanisms via the PDN gateway.

### #B.5.5	Ending a BEST service session

The HSE or the UE can end a BEST session by sending a Terminate EMSDP request command.  This may be due to a request to do so by the enterprise or the user or due to an error condition.

## #B.6	Key Management

### #B.6.1	Key Agreement and Refreshing

#### #B.6.1.1	Key setup messaging between HSE and UE

New keys are agreed either at the start of a BEST session or as required due to key aging or counter thresholds being met.  Key agreement is based on the 3GPP AKA mechanism detailed in TS 33.102 [4] and the AKA challenge is transported between the HSE and the UE in any of the following EMSDP commands:

-  Start new EMSDP session EMSDP command from the HSE as part of a new session being setup by the HSE.

-	Manage new EMSDP Keys EMSDP command from the HSE as a result of key aging or counter thresholds being met.

Once in a session, a UE can request new keys by sending a Manage EMSDP Keys request or command to the HSE. The way a session modification is handled is described in #B.5.1.2.

Figure #B.6.1.1-1 shows the key agreement process

With EMKS

Without EMKS

UE

HSE

PDN gateway

1. Start EMSDP session request command over the Air interface / core network interfaces (UE initialised session) or Manage EMSDP Keys request command if in a session.

EMKS

HSS

2. Start EMSDP session request command over the SGi interface (UE initialised session) or Manage EMSDP Keys request command if in a session.

3. Keys required?

4. Authentication-Information-Request over S6x interface (to request authentication vectors)

5. Authentication-Information-Answer over S6x interface (to deliver authentication vectors)

d. Authentication-Information-Answer over the new2 interface (to deliver authentication vectors)

c. Authentication-Information-Answer over S6y interface (to deliver authentication vectors –if required)

a. Authentication-Information-Request over the new2 interface (to request authentication vectors)

b. Authentication-Information-Request over S6y interface (to request authentication vectors - if required)

Authentication vectors needed?

9. Start new EMSDP session command over the SGi interface (UE or HSE initialised session).

10. Start new EMSDP session command over the Air interface / core network interfaces (UE  or HSE initialised session).

8. Calculate UE Keys

6. Calculate HSE Keys

EAS

7. UE E2E Session Request over the new-1 interface.

8. E2E Session Accept with a EAS service container

9. Calculate E2E Keys

Figure #B.6.1.1-1 BEST key agreement process

The Key agreement steps are:

1.	Start EMSDP session request or Manage EMSDP Keys request command over the Air interface / core network interfaces. – The UE sends the a "Start EMSDP session request" command through a SGI PtP tunnel between the UE and the P-GW to set up a new BEST session. If the UE is already in a session and wants to renegotiate the keys it sends a or Manage EMSDP Keys request command secured with the current keys to the PDN gateway through the SGI PtP tunnel.

2.	Start EMSDP session request command or Manage EMSDP Keys request command over the SGi interface.  –The "Start EMSDP session request" or Manage EMSDP Keys request command is forwarded to the HSE using the SGi interface.

3.	Keys required? - the HSE checks to see if there are valid keys with valid counter values available in the HSE for that UE then the following is checked:

-	If the "Start EMSDP session request " has the correct counter value, is correctly encrypted and is correctly integrity protected then the HSE sends a "Start new EMSDP session" – use current keyset, to the UE correctly encrypted, integrity protected and with a correct counter to the UE (step 6).

-	If the "Start EMSDP session request" is not integrity protected or has any of the security parameters wrong (against the information the HSE has) and there are new authentication vectors available in the HSE for that UE, then the HSE goes to step 6 and uses one of the available unused authentication vectors.

-	If the "Start EMSDP session request " is not integrity protected or has any of the security parameters wrong (against the information the HSE has) and there are no authentication vectors available in the HSE for that UE and the HSE has a direct connection to the HSS, then the HSE goes to step 4.

-	If the "Start EMSDP session request " is not integrity protected or has any of the security parameters wrong (against the information the HSE has) and there are no authentication vectors available in the HSE for that UE and the HSE has an indirect connection to the HSS via a EMKS, then the HSE goes to step a.

4. Authentication-Information-Request over S6f interface – The HSE uses the S6f interface to the HSS to request one or more authentication vectors using the UE IMSI.  These vectors will be specific to the BEST service.  The IMSI of the UE is the IMSI passed in the accounting request (Start) for that UE when the SGi interface was setup.

5. Authentication-Information-Answer over S6f interface – The HSS uses the S6f interface to the HSE to return the requested authentication vectors.  These vectors are specific to the BEST service.

a. Authentication-Information-Request over the S6f’ interface – Where an EMKS is used, the HSE uses the S6f’ interface to the EMKS to request an authentication vector using the UE IMSI.  These vectors will be specific to the BEST service.  The IMSI of the UE is the IMSI passed in the accounting request (Start) for that UE when the SGi interface was setup.

b. Authentication-Information-Request over the S6g interface – The EMKS uses the S6g interface to the HSS to request one or more authentication vectors using the UE IMSI.  These vectors will be specific to the BEST service.

c. Authentication-Information-Answer over the S6g interface – The HSS uses the S6g interface to the EMKS to return the requested authentication vectors.  These vectors may be stored on the EMKS for later use.

d. Authentication-Information-Answer over the S6f’ interface – The EMKS uses the S6f’ interface to the HSE to return the requested authentication vector.

6. Calculate HSE Keys -  The HSE calculates the KE2Menc key using the BEST key derivation mechanism, CK, IK, a HSE id and the key id with the Algorithm id set to "BEST encryption Key". The HSE calculates the KE2Mint key using the BEST key derivation mechanism, CK, IK, a HSE id and the key id with the Algorithm id set to "BEST integrity Key". The HSE calculates the (KInt_E2E)  key using the BEST key derivation mechanism, CK, IK, a HSE id and the key id with the Algorithm id set to "BEST end2end Intermediate Key".

Note: the calculation of the KInt_E2E  may be omitted if no UE-to-EAS session is to be set up.

7. The HSE sends a "UE-to-EAS Session Request" to the EAS over the new-1 interface – The HSE uses the new-1 interface to the EAS to inform the Enterprise Server of the new UE-to-EAS session request and forwards the intermediate E2E key (KInt_E2E)  to the Enterprise Server.

8. The Enterprise Server sends a "UE-to-EAS Session Accept" to the HSE with an EAS container – The Enterprise Server replies with the Enterprise service container to be send to the UE.

9. Start new EMSDP session command over the SGi interface -  The HSE sends a "Start new EMSDP session" command to the PDN Gateway using the SGi interface as follows:

-	If the new keys need to be agreed for the BEST service, then the HSE includes the Key Agreement TLV in the "Start new EMSDP session" command.  The Key Agreement TLV has the RAND and AUTN of the authentication vector used in the RAND IE and AUTN IE.

- If a UE-to-EAS session is to be set up, the HSE sets the UE-to-EAS flag to 1 and includes the EAS container that was obtained from the EAS.

10. Start new EMSDP session command over the Air interface / core network interface -  The PDN Gateway sends a "Start new EMSDP session " command to the UE using the core network and air interfaces.

11. Calculate UE Keys - On successful reception of the "Start new EMSDP session " command, the UE checks the AKA challenge as detailed in 3GPP TS 33.102 [4] and if it is valid, calculates the AKA response CK and IK.  Unlike the normal AKA challenge/response, the AKA response in not sent to the HSE. The UE calculates the UE-to-HSE keys as follows:

-	KE2Menc key using the BEST key derivation mechanism, CK, IK, a HSE id and the key id with the Algorithm id set to "BEST encryption Key". The UE calculates the KE2Mint key using the BEST key derivation mechanism, CK, IK, a HSE id and the key id with the Algorithm id set to "BEST integrity Key".  The UE may choose to calculate one or a set of KE2Menc and KE2Mint key values depending on its optimal power strategy.  Each key id refers to a pair of KE2Menc and KE2Mint key values. The UE calculates the (KInt_E2E)  key using the BEST key derivation mechanism, CK, IK, a HSE id and the key id with the Algorithm id set to " BEST end2end Intermediate Key".

The UE calculates the UE-to-EAS keys as follows:

The UE calculates the KE2Eenc key using the BEST key E2E derivation mechanism using the Enterprise key, optional additional information from the EAS container, the BEST end-to-end Intermediate Key (KInt_E2E) and the Algorithm ID set to "BEST e2e encryption key" and the UE calculates the KE2Eenc key using the BEST key E2E derivation mechanism using the Enterprise key, optional additional information from the Enterprise container,  the BEST end2end Intermediate Key (KInt_E2E) and the Algorithm ID set to "BEST e2e integrity key (KE2Eenc)".

NOTE: The UE may omit the calculation of the UE-to-EAS keys if UE-to-HSE is selected

12. Calculate E2E Keys - The Enterprise Server calculates the KE2Eenc key using the BEST key E2E derivation mechanism using the Enterpise key and optional additional information, the BEST end-to-end Intermediate Key (KInt_E2E) and the Algorithm ID set to "BEST e2e encryption key". The Enterprise Server calculates the KE2Eenc key using the BEST key E2E derivation mechanism using the Enterprise key and optional additional information from the Enterprise container,  the BEST end2end Intermediate Key (KInt_E2E) and the Algorithm ID set to "BEST e2e integrity key (KE2Eenc)".

#### #B.6.1.2	Usage of Keys

For UE-to-HSE BEST sessions, the following keys shall be used:

-	The KE2Menc shall be used by the UE and the HSE for the encryption of the user plane and the signalling plane, according to the agreed encryption algorithm.

-	The KE2Mint shall be used by the UE and the HSE for the integrity protection of the user plane and the signaling plane according to the agreed integrity protection algorithm.

For UE-to-EAS BEST sessions, the following keys shall be used:

-	The KE2Menc shall be used by the UE and the HSE for the encryption of the signalling plane, according to the agreed encryption algorithm.

-	The KE2Mint shall be used by the UE and the HSE for the integrity protection of the signaling plane according to the agreed integrity protection algorithm.

-	The KE2Eenc shall be used by the UE and the EAS for the encryption of the user plane, according to the agreed encryption algorithm.

-	The KE2Eint shall be used by the UE and the EAS for the integrity protection of the user plane according to the agreed integrity protection algorithm.

#### #B.6.1.3	Key Setup for BEST session modification

The modification of a BEST UE-to-EAS session to a BEST UE-to-HSE session does not require any new calculations of keys.

The modification of a BEST UE-to-HSE session to a BEST UE-to-EAS session requires new keys to be calculated when no EAS keys are available in the EAS. Before sending the EMSDP Modification command the HSE does the following:

-	Checks with the EAS whether it has keys in storage

-	If not, obtains a EAS container from the EAS and forwards this to the UE in the EMSDP Modification command.

### #B.6.2	BEST Key Hierarchy

#### #B.6.2.1	Introduction

This clause describes the key hierarchies for BEST for the following options:

-	Best Key Hierarchy in case a separate BEST domain is used.

#### #B.6.2.2	BEST Key Hierarchy for Separate BEST Domain

Figure #B.6.2.2-1: Key Hierarchy for Separate BEST Domain

### #B.6.3	BEST Key Handling

Editor’s Note: This section should describe the key handling of BEST keys for different UE modes, such as idle and off.  For example, whether keys (and if so, which keys) are to be stored when the UE is turned off.

**** End of Fourth Change ****

**** Fifth Change ****

A.19 Derivation of BEST Keys

## A.19.1	BEST key derivation for separate BEST Domain

## A.19.1.1	Derivation of UE-to-HSE keys

The following input string is used when the UE and the HSE derive the BEST keys KE2Menc or KE2Mint from CK, IK:

-	FC = 0x60,

-	P0 = HSE id,

-	L0 = length of HSE id (i.e. 0x00 0x03),

-	P1 = SQN  AK

-	L1 = length of SQN  AK (i.e. 0x00 0x06)

-	P2 = algorithm id

-	L2 = length of algorithm id (i.e. 0x00 0x01)

Table A.19.1.1-1: Algorithm type distinguishers

| Algorithm distinguisher | Value |
| --- | --- |
| BEST encryption key (KE2Menc) | 0x01 |
| BEST integrity Key (KE2Mint) | 0x02 |
| BEST end2end Intermediate Key (KInt_E2E) | 0x03 |

The input key shall be equal to the concatenation CK || IK of CK and IK.

## A.19.1.2	Derivation of UE-to-EAS keys

The following input string is used when the UE and the EAS derive the BEST keys a KE2Eenc or KE2Eint from KInt_E2E.:

-	FC = 0x61,

-	P0 = Enterprise Application Server id,

-	L0 = length of Enterprise Application Server id (i.e. 0x00 0x03),

-	P1 = algorithm id

-	L1 = length of algorithm id (i.e. 0x00 0x01)

-	P2 = RAND

-	L2 = length of rand (0 if no rand found in the Enterprise container)

Note: The RAND in this derivation is the one provided by inside the EAS container. If no RAND is provided, it should be set to zero (0..0).

Table A.19.1.2-1: Algorithm type distinguishers

| Algorithm distinguisher | Value |
| --- | --- |
| BEST e2e encryption key (KE2Eenc) | 0x01 |
| BEST e2e integrity Key (KE2Eint) | 0x02 |

The input key shall be equal to the concatenation KInt_E2E || Enterprise_Key of KInt_E2E  and Enterprise_Key. When the Enterprise container specifies no Enterprise key, the Enterprse_Key shall be set to zero (0..0).

Note: The Enterprise_Key provisioning is outside the scope of 3GPP.

**** End of Fifth Change ****

**** Sixth Change ****

Annex X (normative): End to Middle Secured Data Protocol (EMSDP)

## X.1	Introduction

The EMSDP protocol is used for the BEST service. This protocol runs between the UE and the HSE and has been optimised for efficient, secure transfer of both user plane and signalling plane data.

## X.2	EMSDP Protocol Description

### X.2.1	Data Stack

Figure X.2.1-1 shows the data stack for the EMSDP transfers (based on 3GPP TS 23.401 [2]) for data transfer between the HSE and the UE.

EMSDP

PDCP

RLC

MAC

L1

UE

RLC

MAC

E-UTRAN

UDP/IP

L2

L1

L1

PDCP

GTP-U

Relay

LTE - Uu

UDP/IP

L2

Serving GW

UDP/IP

L2

L1

L1

GTP-U

GTP-U

Relay

S1-U

UDP/IP

L2

IP

L2

L1

L1

S5

PDN GW

Transformation

GTP-U

UDP/IP

EMSDP

UDP/IP

IP

L2

L1

HSE

SGi

New-1

Figure X.2.1-1: data stack for the EMSDP transfers

The UE selects the non-IP PDN data type as defined in section 4.3.17.8 of 3GPP TS 23.401[2]. The PDN GW adds the IP and UDP layers for the communication with the HSE.

The dashed box indicates that for UE-to-EAS BEST sessions, the HSE may forward the EMSDP user plane unmodified to the EAS over the New-1 interface. The signalling plane always terminates in the HSE.

Note: It is up to the operator to decide the protocol between the HSE and the EAS. For example, an operator may agree to use XML over the New-1 interface. In that case, the HSE will have to encapsulate the user data in an agreed XML message.

### X.2.2	EMSDP general structure

This clause details a type 01 signalling plane EMSDP message and a type 01 user plane EMSDP message.

Figure X.2.2-1 shows the general structure of the EMSDP type 01 message:

| Signalling Plane type 01 message: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UP / SP Flag | Key ID | SP COUNTER | Session ID | EMSDP Command | Cmd Options | MAC |
| 1 bit | 4 bits | Note 4 | Note 1 Note 2 | Note 1 Note 2 Note 3 | Note 2 Note 3 | Note 1 Note 3 |
| User Plane type 01 message: |  |  |  |  |  |  |
| UP / SP Flag | Key ID | UP COUNTER | Session ID | Data Length | Data | MAC |
| 1 bit | 4 bits | Note 4 | Note 1 Note 2 | Note 1 Note 2 Note 3 | Note 2 Note 3 | Note 1 Note 3 |

Note 1: The length of these fields is indicated in the BEST service discovery from the HSE.

Note 2: These fields are included in the integrity protection calculation.

Note 3: These fields are encrypted when encryption is used.  When encrypted these fields are replaced by the encrypted output.

Note 4: The length of this field is as defined in the BEST Counter Scheme being used.

Figure X.2.2-1: data stack for the EMSDP transfers

UP / SP Flag: 	This is a 1 bit field that when set means that the data packet is a User Plane message and when clear means that the message is a signalling plane message.

Key ID:	This is a 4 bit field that indicates the key being used for encryption and Integrity protection.

SP COUNTER:	This is a counter used for signalling plain data packets that is incremented every data packet.  It is used to protect data packets against replay and its length is set based on the counter scheme indicated in the BEST service discovery (minimum length is 3 bits).  There are separate counter values for UE to HSE and HSE to UE.

UP COUNTER:	This is a counter used for user plain data packets that is incremented every data packet.  It is used to protect data packets against replay and its length is set based on the counter scheme indicated in the BEST service discovery (minimum length is 3 bits). There are separate counter values for UE to HSE and HSE to UE.

Session ID:	This indicates the identifier for the current session.  The value is assigned by the HSE.  For signalling messages sent outside of a session, the Session ID shall be all 0's.  Its length is the value set in the BEST service discovery.

EMSDP Command:	This is a 1 byte field that contains the signalling command.  The defined commands are detailed below.

Cmd Options:	This is a TLV container that contains TLV elements that detail the options for the EMSDP command. The defined options TLVs are detailed below.

MAC:	This contains the truncated integrity result for this data packet calculated using the agreed integrity algorithm. Its length is set in the BEST service discovery.

Data length:	This holds the length of the following data in this data packet. Its length is set in the BEST service discovery.

Data:	This is the data being transferred.

### X.2.3	EMSDP Counter Schemes

#### X.2.3.1	Optimised EMSDP counter scheme

The optimised EMSDP counter scheme has a 3 bit counter length followed by the counter value.  This allows the counter length to be changed on a per message basis.  The 3 bits are used to indicate the number of octets the counter is on, the value "000" is reserved for future use.  So for instance:

Counter value "1" is represented as "001 00000001"

Counter value "257" is represented as "010 00000001 00000001"

The optimised EMSDP counter scheme is defined as scheme 01.

### X.2.4	EMSDP Integrity protection

The integrity protection algorithm to be used and the length of the MAC is indicated in the BEST service discovery.

Integrity protection is mandatory for all signalling plane and user plan data packets except for the following signalling plane commands when no valid keyset is agreed between the UE and HSE:

-	a Start EMSDP session request originating from the UE or the HSE

-	a Start EMSDP session response originating from the HSE

For these commands the MAC shall be set to "0…0".

For all other signalling plane and user plane data packets the MAC shall be computed as follows:

First the following fields are calculated (where needed for the chosen algorithm):

INPUT-I	set to the Data Packet Counter Value expanded and right padded with 0's to a fixed size of 4 bytes.

COUNT-C	set to the Data Packet Counter Value expanded and right padded with 0's to a fixed size of 4 bytes

M (GSM)	set to the length of Message in bytes. It is coded on 2 bytes.

LENGTH 	set to the length of Message in bytes. It is coded on 2 bytes.

MESSAGE	the fields marked for integrity protection in figure 6.8.2.4.3.1: "data stack for the EMSDP transfers" concatenated in the order they appear in the data packet.

M (LTE)	the fields marked for integrity protection in figure 6.8.2.4.3.1: "data stack for the EMSDP transfers" concatenated in the order they appear in the data packet.

DIRECTION	The DIRECTION bit shall be "0" for UE to HSE data packets and set to "1" for HSE to UE data packets.

BEARER	For signalling data packets this shall be set to "00000" and for user plane data packets this shall be set to "10101"

FRAMETYPE	For signalling data packets this shall be set to "00" and for user plane data packets this shall be set to "AA".

KI128	This is the agreed integrity key value truncated to the lowest 128 bits.

IK	This is the agreed integrity key value truncated to the lowest 128 bits.

If GIA4 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 55.241 [42] shall be used to generate the MAC value.  If the MAC length indicated in the BEST service discovery is less than the length of the MAC produced by the GIA4 function, then the MAC shall be truncated to the correct size from the right.

If GIA5 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 55.251 [43] shall be used to generate the MAC value. If the MAC length indicated in the BEST service discovery is less than the length of the MAC produced by the GIA5 function, then the MAC shall be truncated to the correct size from the right.

If UIA1 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.205 [44] shall be used to generate the MAC-I value.  The MAC shall be set to the MAC-I truncated to the correct size from the right.

If UIA2 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.215 [14] shall be used to generate the MAC-I value.  The MAC shall be set to the MAC-I truncated to the correct size from the right.

If 128-EIA1 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.401 [45] shall be used to generate the MAC-I value.  The MAC shall be set to the MAC-I truncated to the correct size from the right.

If 128-EIA2 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.401 [45] shall be used to generate the MAC-I value.  The MAC shall be set to the MAC-I truncated to the correct size from the right.

If 128-EIA3 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.221 [33] shall be used to generate the MAC-I value.  The MAC shall be set to the calculated MAC-I value truncated to the correct size from the right.

### X.2.5	EMSDP Encryption

The encryption protection algorithm to be used is indicated in the BEST service discovery.  If EEA0 is indicated then the data packet shall not be encrypted.

Encryption is mandatory for all signalling plane and user plan data packets when an encryption algorithm is indicated in the BEST service discovery, except for the following signalling plane commands when no valid keyset is agreed between the UE and HSE:

-	a Start EMSDP session request originating from the UE or the HSE

-	a Start EMSDP session response originating from the HSE

For all other data packets the following encryption shall be applied the fields indicated in figure X.2.1-1: "data stack for the EMSDP transfers" to be encrypted.

First the following are computed (where relevant for the algorithm being used):

INPUT-I	set to the Data Packet Counter Value expanded and right padded with 0's to a fixed size of 4 bytes.

COUNT-C	set to the Data Packet Counter Value expanded and right padded with 0's to a fixed size of 4 bytes

M (GSM)	set to the length of Message in bytes. It is coded on 2 bytes.

LENGTH 	set to the length of Message in bytes. It is coded on 2 bytes.

MESSAGE	the fields marked for encryption protection in figure 6.8.2.4.3.1: "data stack for the EMSDP transfers" concatenated in the order they appear in the data packet.

M (LTE)	the fields marked for encryption protection in figure 6.8.2.4.3.1: "data stack for the EMSDP transfers" concatenated in the order they appear in the data packet.

DIRECTION	The DIRECTION bit shall be "0" for UE to HSE data packets and set to "1" for HSE to UE data packets.

BEARER	For signalling data packets this shall be set to "00000" and for user plane data packets this shall be set to "10101"

FRAMETYPE	For signalling data packets this shall be set to "00" and for user plane data packets this shall be set to "AA".

KI128	This is the agreed encryption key value truncated to the lowest 128 bits.

CK	This is the agreed encryption key value truncated to the lowest 128 bits.

If GEA4 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 55.241 [42] shall be used to generate the OUTPUT value. The OUTPUT value replaces the fields that are encrypted in the data packet.

If GEA5 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 55.251 [43] shall be used to generate the OUTPUT value. The OUTPUT value replaces the fields that are encrypted in the data packet.

If UEA1 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.205 [44] shall be used to generate the OBS value.  The OBS value replaces the fields that are encrypted in the data packet.

If UEA2 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.215 [14] shall be used to generate the OBS value.  The OBS value replaces the fields that are encrypted in the data packet.

If 128-EEA1 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.401 [45] shall be used to generate the OBS value.  The OBS value replaces the fields that are encrypted in the data packet.

If 128-EEA2 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.401 [45] shall be used to generate the OBS value.  The OBS value replaces the fields that are encrypted in the data packet.

If 128-EEA3 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.221 [33] shall be used to generate the OBS value.  The OBS value replaces the fields that are encrypted in the data packet.

### X.2.6	EMSDP Commands

#### X.2.6.1	Overview

The following EMSDP commands are proposed Table X.2.6.1-1:

Table X.2.6.1-1: EMSDP commands

| Code (Hex) | Command |
| --- | --- |
| 10 | Start EMSDP session request |
| 11 | Start new EMSDP session |
| 30 | Terminate EMSDP session request |
| 31 | Terminate EMSDP session response |
| 40 | Manage EMSDP Keys request |
| 41 | Manage EMSDP Keys response |
| 50 | Modify EMSDP session request |
| 51 | Modify EMSDP session response |
| 60 | Request Rejected |

##### X.2.6.1.1		Start EMSDP session request

The Start EMSDP session request command is used by the UE to trigger a new BEST session from the HSE.

The cmd options for the Start EMSDP session request command are as follows:

Enterprise Setup Information Element TLV: This is a TLV element that contains information from the UE that is used by the HSE to setup the HSE to enterprise connection.

UE-to-EAS flag: If set to 1 it indicates that the UE is capable of UE-to-EAS BEST secure session. It set to 0 it means that the UE requests a UE-to-HSE BEST secure session.

##### X.2.6.1.2		Start new EMSDP session

The Start new EMSDP session response command is used by the HSE to setup a new BEST session.  This message can be sent in response to a start EMSDP session request command from the UE or as an initial message from the HSE.

The Start new EMSDP session response command has the following cmd options:

HSE Identity TLV: this contains a 4 octet numeric identifier for the HSE.  This should be unique to the HSE being used within the home network.

Key agreement TLV: this contains the RAND IE and AUTN IE specified in 3GPP TS 24.008 [41] as detailed in table 6.8.2.4.6.2.1:

Table 6.8.2.4.6.2.1: Key Agreement TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| Key Agreement TLV | 1 byte | M | TBD |
| Length | 1 byte | M | 0 or 35 |
| RAND IE | 17 bytes | C | See 10.5.3.1 in 3GPP TS 24.008 [41] |
| AUTN IE | 18 bytes | C | See 10.5.3.1.1 in 3GPP TS 24.008 [41] |

If the Length of the Key agreement is set to 0 then this means use current keyset.

UE-to-EAS flag: Indicates that the session will be a BEST UE-to-EAS session if set to 1. If set to zero or omitted, the BEST session will be a UE-to-HSE session.

Enterprise Container: [optional] a container that contains the provided information for the key E2E derivation for the UE as provided by EAS. The container may contain an identifier for the a private key to be used and optionally other values for key derivation, such as a RAND.

Note: The enterprise container may be omitted whenever the UE-to-EAS is not set or set to zero.

##### X.2.6.1.3		Terminate EMSDP session request

The Terminate EMSDP session request EMSDP command is used by either the UE or the HSE to terminate an existing EMSDP session.

This command has no cmd options.

##### X.2.6.1.4	Manage EMSDP Keys request

The Manage EMSDP Keys request EMSDP command is used by the HSE to negotiate new key material.

This command has no cmd options.

##### X.2.6.1.5		Manage new EMSDP Keys

The Manage EMSDP Keys response command is used by the HSE to agree new keys.

The Manage EMSDP Keys response command has the following cmd options:

Key agreement TLV: this contains the RAND IE and AUTN IE specified in 3GPP TS 24.008 [41] as detailed in table X.2.6.1.5-1:

Table X.2.6.1.5-1: Key Agreement TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| Key Agreement TLV | 1 byte | M | TBD |
| Length | 1 byte | M | 35 |
| RAND IE | 17 bytes | C | See 10.5.3.1 in 3GPP TS 24.008 [41] |
| AUTN IE | 18 bytes | C | See 10.5.3.1.1 in 3GPP TS 24.008 [41] |

##### X.2.6.1.6	Modify EMSDP session request

The Modify EMSDP session request EMSDP command is used by the UE to request a modification of the EMSDP session.

The Modify EMSDP session request has the following command options:

UE-to-EAS: This contains a flag indicating whether the UE requests a session modification to a UE-to-EAS session (flag equal 1) or to a UE-to-HSE session (flag 0).

##### X.2.6.1.7		Modify EMSDP session response

The Modify EMSDP session response EMSDP command is used by the HSE to modify a EMSDP session from UE-to-HSE and UE-to-EAS and vice versa.

The Modify EMSDP session response command has the following cmd options:

UE-to-EAS: This contains a flag indicating whether the session is modified to a UE-to-EAS session (flag equal 1) or to a UE-to-HSE session (flag 0).

EAS Container: [optional] This contains the EAS container in case the UE-to-EAS flag is set to 1.

##### X.2.6.1.8		Request Rejected command

TheRequest Rejected command is used by the HSE to reject requests by the UE.

The Request Rejected command has one cmd option:

Command Rejected: The hex code of the command rejected.

**** End of Sixth Change ****

**** Seventh Change ****

Annex Y (informative): Structure of APN names for BEST

## Y.1 Introduction

Due to regulatory requirements, operators may have to disable the best service for UEs roaming in their network. A simple, but effective method is rely on the MME capability to filter specific APNs. For that reason, a BEST APN structure is proposed in this section. Operators may agree on a different method in bilateral requirements.

## Y.2 Structure of APN names for BEST

APNs that are used for the BEST service may reside in a specific subdomain of the operator. It is proposed to use the subdomain ‘best’ for this specific service.

Note 1: For example, if the operator uses operator.cc as a domain for APNs, the best servers should be in the domain ‘best.operator.cc’. So ‘apn.best.operator.cc’ would be an APN that is best capable..

**** End of Seventh Change ****

**** Eighth Change ****

# Annex Z (normative): BEST Service Management

## Annex Z.1	Service Discovery

This clause defines the BEST service discovery and initial BEST service definition.  This is run when the UE wants to use the BEST service and there is no BEST session active or as part of the power-on sequence.

Figure Z.1-1 shows the message flow for service discovery for LTE.

UE

1. PDN CONNECTIVITY REQUEST

with UE support for BEST service indicated in protocol configuration options element or flagged for later retrieval in ESM information response (see TS 24.301[x]).

2. CREATE SESSION REQUEST

with UE support for BEST service indicated in protocol configuration options and optionally an additional BEST information element containing the visited network BEST request.

MME

HSE

PGW

3. Accounting Request (START)

with UE support for BEST service indicated in protocol configuration options and optionally an additional BEST information element containing the visited network BEST request

4. Accounting Response (START)

with HSE support for BEST service indicated in protocol configuration options.

5. CREATE SESSION RESPONSE

with HSE support for BEST service indicated in protocol configuration options  (see TS TS 29.274 [x]).

6. ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST

with UE support for BEST service indicated in protocol configuration options element(see TS 24.301[x]).

Figure Z.1-1: Flow diagram showing the BEST service discovery message flow for LTE

The ability of the UE to support the BEST service shall be indicated in a BEST container within the protocol configuration options of a PDN_CONNECTIVITY_REQUEST.

The following BEST service aspects shall be indicated in the protocol configuration options as detailed in figure Z.1-2 (based on the protocol block format for the protocol configuration options IE in 3GPP TS 24.008 [41]):

| Bit 8 | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Upper byte of PCO protocol ID = UE BEST |  |  |  |  |  |  |  |
| Lower byte of PCO protocol ID = UE BEST |  |  |  |  |  |  |  |
| Length of Best protocol ID contents |  |  |  |  |  |  |  |
| BEST release supported by the UE |  |  |  | BEST counter schemes supported by the UE - BEST optimised counter scheme | Reserved for future use (set to 000) |  |  |
| BEST protocols supported - type 01 signalling plane EMSDP message | Reserved for future use (set to 000) |  |  | BEST protocols supported - type 01 user plane EMSDP message | Reserved for future use (set to 000) |  |  |
| Support for user data push to UE | BEST encryption algorithm GEA4 supported | BEST encryption algorithm GEA5 supported | BEST encryption algorithm UEA1 supported | BEST encryption algorithm UEA2 supported | BEST encryption algorithm EEA0 supported | BEST encryption algorithm 128-EEA1 supported | BEST encryption algorithm 128-EEA2 supported |
| BEST encryption algorithm 128-EEA3 supported | BEST integrity algorithm GIA4 supported | BEST integrity algorithm GIA5 supported | BEST integrity algorithm UIA1 supported | BEST integrity algorithm UIA2 supported | BEST integrity algorithm 128-EIA1 supported | BEST integrity algorithm 128-EIA2 supported | BEST integrity algorithm 128-EIA3 supported |

Figure Z.1-2: Definition of the UE BEST protocol ID element

- BEST release supported by the UE – an indicator the release of the BEST solution that the UE has been designed to.  If no release is indicated this means that the BEST service is not supported.  This shall be a 4 bit field where "0000" = Rel.14 and "0001" to "1111" are RFU,

- BEST protocols supported – 1 octet where each bit is used to indicate a BEST signalling protocol that is supported from the following list: type 01 signalling plane EMSDP message and type 01 user plane EMSDP message.  All other values are reserved for future use.

- BEST encryption algorithms supported by the UE – a flag for each supported algorithm.  A further optimisation for this would be to assume that all algorithms indicated in the UE network capability information element are supported for the BEST service (currently GEA4, GEA5, UEA1, UEA2, EEA0, 128-EEA1, 128-EEA2 and 128-EEA3).

- BEST integrity algorithms supported by the UE – a flag for each supported algorithm.  A further optimisation for this would be to assume that all algorithms indicated in the UE network capability information element are supported for the BEST service (currently GIA4, GIA5, UIA1, UIA2, 128-EIA1, 128-EIA2 and 128-EIA3).

- BEST counter schemes supported by the UE – a flag for each supported scheme.  Currently only one scheme is being proposed, the optimised counter scheme, but other optimisations may be proposed in the future.

- Support for user data push to UE – a flag to indicate whether the UE supports user data pushed to the UE without a request for user data from the UE.

- Power capabilities of the UE.  This allows the home network and visited network to optimise their key agreement and signalling layers to take into account the power available.

In order to support regulatory requirements in certain countries (e.g. LI or where encryption is prohibited) then an MME that supports the BEST service may inspect the PCO provided by the UE in the PDN_CONNECTIVITY_REQUEST.  The MME may add a VISITED NETWORK BEST INDICATION container to the PCO with the following information:

- BEST encryption Restricted – A flag that indicates that encryption is not allowed through this visited network.

The HSE defines how the BEST service shall be setup in a BEST container within the protocol configuration options of a ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST.

The following BEST service aspects shall be indicated in the protocol configuration options as detailed in figure Z.1-3 (based on the protocol block format for the protocol configuration options IE in 3GPP TS 24.008 [41]):

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Upper byte of PCO protocol ID = HSE BEST |  |  |  |  |  |  |  |
| Lower byte of PCO protocol ID = HSE BEST |  |  |  |  |  |  |  |
| Length of Best protocol ID contents |  |  |  |  |  |  |  |
| BEST Service Activated | BEST encryption algorithm GEA4 to be used | BEST encryption algorithm GEA5 to be used | BEST encryption algorithm UEA1 to be used | BEST encryption algorithm UEA2 to be used | BEST encryption algorithm EEA0 to be used | BEST encryption algorithm 128-EEA1 to be used | BEST encryption algorithm 128-EEA2 to be used |
| BEST signalling plane protocol identifier |  |  |  |  |  |  |  |
| BEST user plane protocol identifier |  |  |  |  |  |  |  |
| BEST encryption algorithm 128-EEA3 to be used | BEST integrity algorithm GIA4 to be used | BEST integrity algorithm GIA5 to be used | BEST integrity algorithm UIA1 to be used | BEST integrity algorithm UIA2 to be used | BEST integrity algorithm 128-EIA1 to be used | BEST integrity algorithm 128-EIA2 to be used | BEST integrity algorithm 128-EIA3 to be used |
| Reserved for future use (set to 000) |  |  | Local BEST configuration management allowed | EMSDP session ID length |  |  |  |
| Reserved for future use (set to 00) | New session required | EMSDP MAC length |  | Size of EMSDP Data Length |  |  |  |

Figure Z.1-3: Definition of the HSE BEST protocol ID element

- BEST Service Activated – a bit flag that when set instructs the UE to use the BEST service and when clear instructs the UE not to use the BEST service,

- BEST signalling plane protocol identifier – 1 octet that is used to determine the BEST signalling protocol to be used from the following list (only one shall be indicated): 01 = type 01 signalling plane EMSDP message. All other values are reserved for future use.

- BEST user plane protocol identifier – 1 octet that is used to determine the BEST signalling protocol to be used from the following list (only one shall be indicated): 01 = type 01 user plane EMSDP message. All other values are reserved for future use.

- BEST encryption algorithm to be used – 1 octet that is used to define which of the following algorithms to use for encryption: GEA0, GEA4, GEA5, UEA0, UEA1, UEA2, EEA0, 128-EEA1, 128-EEA2 and 128-EEA3. Only one algorithm from this list shall be indicated.  If the Visited network indicated that BEST encryption is restricted, then the HSE shall indicate EEA0.

- BEST integrity algorithm to be used – 1 octet that is used to define which one of the following algorithms to use for integrity:GIA4, GIA5, UIA1, UIA2, 128-EIA1, 128-EIA2 and 128-EIA3). Only one algorithm from this list shall be indicated.

- BEST counter scheme to be used – a flag for each supported scheme.  4 bits that are used to define which one of the following counter schemes to use: "0001" = the optimised EMSDP counter scheme. All other values are reserved for future use. Only one counter scheme from this list is allowed to be indicated.

- Local BEST configuration management allowed – a flag to indicate that the software connected to the UE is allowed to manage the BEST service.

- EMSDP session ID length – 4 bits that indicates how many octets in the EMSDP data packet the EMSDP session ID will be on.

- New Session Required – 1 bit that indicates if a new session is required.  If this bit is set to 0 then the details agreed for the last session can be used and a new session is not required to be setup.

- EMSDP MAC length – 2 bits that indicates how many octets in the EMSDP data packet the integrity checksum (MAC) will be on, as follows: "00"=4 octets, "01"=8 octets, "10"=12 octets and "11"= 16 octets.  This value shall not be set to a size that is greater than MAC size produced by the chosen algorithm.

- Size of EMSDP Data Length – 4 bits that indicate how many octets are used for the EMSDP Data Length. "0000" is reserved for future use.

Any remaining bits are reserved for future use and are set to "0..0".

In order to ensure that the visited networks encryption restriction is honoured by the HSE, an MME that has indicated BEST encryption is restricted, should inspect the BEST information element in the PCO within the start session response from the PGW and if it indicates an encryption algorithm that is not EEA0, the MME should reject the start session response.  It is the HSE's responsibility to send a corrected start session response.

## Z.2	Ability to Enable and Disable the BEST service

The HSE may allow the enterprise to manage the BEST service.  The service shall allow enable / disable of the BEST service down to an individual UE level. It is anticipated that the interface for the management of the BEST service between the enterprise and the HSE will be out of scope for 3GPP.  However it is expected that such an interface would have the following abilities:

- a mechanism that allows the service to be removed for a specific subscriber at the network end (probably re-using  a service provisioning service).

- the ability for the enterprise to enable / disable the E2M security service for one or many of their endpoints through communication with the operators CIoT interface.  (This may be because other security mechanisms are being used).

- The ability for the home network to disable the BEST service due to non-payment or security concerns.

If the BEST service parameters in the HSE change as a result of the management of the BEST service for a specific UE and the BEST service is not currently active with that UE then the changes shall take effect the next BEST session.

If the BEST service parameters in the HSE change as a result of the management of the BEST service for a specific UE and the BEST service is currently active with that UE then the HSE shall end the current BEST session and where appropriate start a new BEST session using the new BEST service parameters.

The UE may allow the application connected to it to manage the BEST service if the Local BEST configuration management flag within the protocol configuration options of a ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST indicates this is allowed.

If the BEST service parameters in the UE change as a result of the management of the BEST service and the BEST service is not currently active then the changes shall take effect the next BEST session.

If the BEST service parameters in the UE change as a result of the management of the BEST service and the BEST service is currently active in that UE then the UE shall end the current BEST session and where appropriate start a new BEST session using the new BEST service parameters.

The UE may choose not to accept data that does not meet the defined security requirements.

When the BEST service is disabled the HSE and UE shall act as though the BEST service is not supported.

**** End of Eighth Change ****